| U.S. Radiocommunications Sector Fact Sheet |  |
| --- | --- |
| Working Party: ITU-R WP 7B | Document No: US7B_27-029_R1 |
| Ref: Document 7B/153, Document 7B/131, WRC-27 Agenda Item 1.15. WRC-27 Agenda Item 1.11 | Date: 14 August 2025 |
| Document Title: PRELIMINARY CALCULATION OF INTERFERENCE FROM LUNAR SURFACE SRS TO MSS (SPACE-TO-SPACE) IN FREQUENCY BANDS OVERLAPPED BY WRC-27 AGENDA ITEMS 1.15 AND 1.11 |  |
| Author(s)/Contributors(s): Name: Scott Kotler Org: Lockheed Martin Corporation Name: Steve Baruch Org: NWSP for Lockheed Martin Corporation Name: Catherine Sham Org: NASA Name: Dennis Lee Org: NASA (JPL) | Phone: (703) 789-3923 Email: scott.kotler@LMCO.com Phone: (240) 476-2600 Email: sbaruch@newwavespectrum.com Phone: (281) 483-0124 Email: catherine.c.sham@nasa.gov Phone: (818) 205-7623 Email: dennis.k.lee@jpl.nasa.gov |
| Purpose/Objective: The purpose of this contribution is to provide preliminary analysis of potential interference from SRS stations on the lunar surface, and SRS systems in lunar orbit communicating with systems on the lunar surface, to MSS (space-to-space) in the 2 483.5 – 2 500 MHz band overlapped by WRC-27 Agenda Items 1.15 and 1.11. |  |
| Abstract: There is frequency overlap in the 2 483.5 – 2 500 MHz frequency range considered by WRC-27 Agenda Item 1.15 for SRS stations on the lunar surface, and SRS systems in lunar orbit communicating with systems on the lunar surface, in accordance with Resolution 680 (WRC-23), and also considered by WRC-27 Agenda item 1.11 for MSS (space-to-space) in accordance with Resolution 249 (WRC-23). This contribution will provide an assessment of the potential impact to MSS (space-to-space) operations under WRC-27 AI 1.11 from lunar SRS operations under WRC-27 AI 1.15, in accordance with the reply liaison statement from WP 4C in Doc 7B/153. |  |

| Radiocommunication Study Groups |  |
| --- | --- |
| Received: xx September 2025 | Document 7B/xxx-E |
|  | 16 September 2025 |
|  | English only |
| United States of America |  |
| PRELIMINARY CALCULATION OF INTERFERENCE FROM LUNAR SURFACE SRS TO MSS (SPACE-TO-SPACE) IN FREQUENCY BANDS OVERLAPPED BY WRC-27 AGENDA ITEMS 1.15 AND 1.11 |  |

Introduction

The 2 483.5 – 2 500 MHz frequency range is being considered for a possible new allocation for SRS stations on the lunar surface, and SRS systems in lunar orbit communicating with systems on the lunar surface, under WRC-27 agenda item in accordance with Resolution 680 (WRC-23).  This same frequency range is also being considered by WRC-27 Agenda item 1.11 for MSS (space-to-space) in accordance with Resolution 249 (WRC-23).

This study provides calculations to determine the impact, if any, to MSS (space-to-space) links under WRC-27 AI 1.11 from lunar SRS operations under WRC-27 AI 1.15, using the guidance from the WP 4C reply liaison statement in Doc 7B/153.

ATTACHMENT

# 1	Studies between direct connectivity mobile satellite service (space-to-space) and lunar SRS operating in the frequency band 2 483.5 - 2 500 MHz

The 2 483.5 – 2 500 MHz frequency range is being considered for a possible new allocation for SRS stations on the lunar surface, and SRS systems in lunar orbit communicating with systems on the lunar surface, under WRC-27 agenda item in accordance with Resolution 680 (WRC-23).  Under WRC-27 agenda item 1.11, the frequency range 2 483.5-2 500 MHz is also being considered for MSS (s-s) in accordance with Resolution 249 (WRC-23).

This study will examine the effect from SRS systems on the lunar surface being studied under AI 1.15 to MSS (s-s) being studied under AI 1.11.  Per Resolution 249 (WRC-23), the MSS (s-s) links under AI 1.11 are limited to operate in the same direction as the existing MSS allocation.  For the 2 483.5 – 2 500 MHz band, this means that the MSS (s-s) emissions will operate in a direction towards earth, typically from a GSO or non-GSO MSS service provider to a user space station at a lower altitude (see Figure 1 from Annex 2 of Document 4C/356).

## 1.1 	SRS Technical and operational characteristics

The technical and operational characteristics of lunar orbit space station transmitter characteristics to the lunar surface are provided in the Draft New Report ITU-R SA.[LUNAR.SRS STATIONS CHAR], and the relevant parameters used in this study are summarized in Table 1 below.

Table 1

Lunar orbit space station transmitter characteristics to lunar surface

| Station | Lunar Orbit-to-Surface Transmitter |
| --- | --- |
| Orbital Characteristics Apolune (km) Perilune (km) | 7 000-11 000 600-2 700 |
| Frequency range | 2 483.5 – 2 500 MHz |
| Polarization | CP |
| Tx Antenna Peak Gain | 16 dBi |
| EIRP Density | -48 dBW/Hz |
| Max. EIRP | 24 dBW |
| Transmit Antenna Pattern | Rec. ITU-R S.672-4 |
| Channel BW | 16 MHz |
| Deployment characteristics Number of orbiting stations | 4-8 |

## 1.2 	MSS (space-to-space) technical and operational characteristics

The technical and operational characteristics of MSS (s-s) systems in the 2 483.5 –   2 500 MHz band under WRC-27 agenda item 1.11 are provided in Annex 2 to the WP 4C Chair’s Report (Document 4C/356).  Additional characteristics for the MSS (space-to-space) systems in this band can be found in Document 4C/251.  The relevant parameters for the MSS (s-s) user space station used in this calculation are summarized in Table 2 below.

Table 2

Technical characteristics of MSS (space-to-space) user space station

| Parameter | MSS (LEO user space station) |
| --- | --- |
| Altitude | 500 km |
| Frequency range | 2 483.5 – 2 500 |
| User G/T | -24 dB/K |
| Polarization | CP |
| Channel BW | 16 MHz |

## 1.3	Propagation model

This study assumes the free space propagation model in Recommendation ITU-R P.525, per the Working Party 3J/3M guidance in Document 4C/92.

## 1.4	Methodology

The methodology employed in this study is based on a static analysis of worst-case interference. It evaluates the interference levels under conditions of perfect antenna coupling between the MSS(s-s) and lunar orbiting SRS systems using the worst case geometry.

#### 1.5 	Study results

For this analysis, the following assumptions were considered:

- The MSS (s-s) user space station receiver is assumed to be in LEO orbit with 500 km altitude, with its antenna pointed at a MSS (s-s) service provider.  Figure 1 shows the geometry of the worst case interference scenario considered, where the MSS (s-s) user space station antenna is pointed towards the MSS (s-s) service provider which happens to be aligned with the lunar orbiting SRS satellite.

FIGURE 1.  SRS–MSS (s-s) Interference Scenario

- In the present study, it is assumed that both systems operate at the same central frequency. A bandwidth correction factor (CF) is applied when the bandwidth of the potential interferer exceeds the bandwidth of the victim receiver under analysis. If the bandwidth of the interferer is equal to or smaller than that of the victim receiver, the bandwidth correction factor is 0

CF = 10 * log10bandwidthvictimbandwidthinterferer

- The interference-to-noise ratio (I/N) is calculated considering Lunar SRS transmitter pointing towards the MSS (s-s) user space station receiver using the following equation:

INTotal= 1n (EIRPnθ,φ+CF-FSPL+Grxθ,φ)-Nrx)

where n is the number of lunar surface transmitters;  EIRPnθ,φ is the equivalent isotropic radiated power of each lunar transmitter; FSPL is the free space path loss calculated using Rec. ITU-R P.525; (Grx)θ,φ is the MSS (s-s) user space station maximum antenna gain; and Nrx is the thermal noise power in the MSS (s-s) user space station receiver bandwidth.

## 1.5 	Summary and analysis of the results

The calculation of interference from a single lunar orbiting transmitter to the MSS (space-to-space) user space station receiver is provided in Table 3, using the worst case assumption of perfect alignment between the MSS and lunar orbiting SRS antennas.  The worst case MSS receiver I/N calculated for a single lunar interferer is -54.5 dB.

As shown in Table 1, the deployment of 4 - 8 lunar orbiting satellites is expected in this band, of which only half will be visible to the MSS user space station in LEO orbit at any particular time.  Thus the aggregate interference from 4 of these lunar orbiting satellites would result in a worst case MSS receiver I/N of -48.5 dB, which would have negligible impact on the MSS receiver performance.

These results suggest that WRC-27 AI 1.11 can be treated independently from AI 1.15 with regards to any potential impact of lunar orbiting SRS operations to MSS (s-s) user space station receivers in the 2 483.5 – 2 500 MHz band overlapped by the two agenda items.

Table 3

SRS – MSS (s-s) user space station interference calculation

| Category | Item | Unit | Scenario 1 |
| --- | --- | --- | --- |
| Direction | From |  | Lunar Orbiting space station |
|  | To |  | MSS user space station (LEO 500 km) |
| Band | Frequency | MHz | 2492 |
|  | Bandwidth | MHz | 16 |
| Lunar Orbiting Transmitter | EIRP | dBW | 24 |
| MSS user space station receiver | G/T | dB/K | -24 |
| Propagation | Distance | km | 343 500 |
|  | Free space loss | dB | 211 |
| Results | I/N | dB | -54.5 |

Table 4

Summary of SRS – MSS (s-s) user space station interference results

| SRS Station | Frequency Band | MSS space station | Analysis | I/N result |
| --- | --- | --- | --- | --- |
| Lunar Orbit Transmitter | 2483.5-2500 MHz | LEO orbit (500 km) | Single Entry (worst case) | -54.5 dB |
|  |  |  | Aggregate (4 lunar orbiting transmitters) | -48.5 dB |

____________________