| U.S. Radiocommunications Sector Fact Sheet |  |
| --- | --- |
| Working Party: ITU-R WP 7B | Document No: US7B_27-030_R1 |
| Ref: Document 7B/153, Document 7B/131, WRC-27 Agenda Item 1.15. WRC-27 Agenda Item 1.13 | Date: 14 August 2025 |
| Document Title: PRELIMINARY CALCULATION OF INTERFERENCE FROM LUNAR SURFACE SRS TO DC-MSS-IMT IN FREQUENCY BANDS OVERLAPPED BY WRC-27 AGENDA ITEMS 1.13 AND 1.15 |  |
| Author(s)/Contributors(s): Name: Scott Kotler Org: Lockheed Martin Corporation Name: Steve Baruch Org: NWSP for Lockheed Martin Corporation Name: Catherine Sham Org: NASA Name: Dennis Lee Org: NASA (JPL) | Phone: (703) 789-3923 Email: scott.kotler@LMCO.com Phone: (240) 476-2600 Email: sbaruch@newwavespectrum.com Phone: (281) 483-0124 Email: catherine.c.sham@nasa.gov Phone: (818) 205-7623 Email: dennis.k.lee@jpl.nasa.gov |
| Purpose/Objective: The purpose of this contribution is to provide preliminary analysis of potential interference from SRS stations on the lunar surface, and SRS systems in lunar orbit communicating with systems on the lunar surface, to DC-MSS-IMT in the 2 500 – 2 60 MHz band overlapped by WRC-27 Agenda Items 1.15 and 1.13. |  |
| Abstract: There is frequency overlap in the 2 500 – 2 690 MHz frequency range considered by WRC-27 Agenda Item 1.15 for SRS stations on the lunar surface, and SRS systems in lunar orbit communicating with systems on the lunar surface, in accordance with Resolution 680 (WRC-23), and also considered by WRC-27 Agenda item 1.13 for MSS direct connectivity between space stations and IMT user equipment in accordance with Resolution 253 (WRC-23). This contribution will provide an assessment of the potential impact to DC-MSS-IMT operations under WRC-27 AI 1.13 from lunar SRS operations under WRC-27 AI 1.15, in accordance with the reply liaison statement from WP 4C in Doc 7B/153. |  |

| Radiocommunication Study Groups |  |
| --- | --- |
| Received: xx September 2025 | Document 7B/134-E |
|  | 16 September 2025 |
|  | English only |
| United States of America |  |
| PRELIMINARY CALCULATION OF INTERFERENCE FROM LUNAR SURFACE SRS TO DC-MSS-IMT IN FREQUENCY BANDS OVERLAPPED BY WRC-27 AGENDA ITEMS 1.13 AND 1.15 |  |

Introduction

The 2 500 – 2 690 MHz frequency range is being considered for a possible new allocation for SRS stations on the lunar surface, and SRS systems in lunar orbit communicating with systems on the lunar surface, under WRC-27 agenda item in accordance with Resolution 680 (WRC-23).  This same frequency range is also being considered by WRC-27 Agenda item 1.13 for MSS direct connectivity between space stations and IMT user equipment in accordance with Resolution 253 (WRC-23).

This study provides calculations to determine the impact, if any, to DC-MSS-IMT uplinks under WRC-27 AI 1.13 from lunar SRS operations under WRC-27 AI 1.15, using guidance from the WP 4C reply liaison statement in Doc 7B/153.  Analysis of the effect to lunar SRS operations from non-GSO DC-MSS-IMT downlinks has been considered separately in Doc 4C/150.

Attachment:	1

ATTACHMENT

# 1	Studies between direct connectivity mobile satellite service (DC-MSS) and lunar SRS operating in the frequency band 2 500-2 690 MHz

The 2 500 – 2 690 MHz frequency range is being considered for a possible new allocation for SRS stations on the lunar surface, and SRS systems in lunar orbit communicating with systems on the lunar surface, under WRC-27 agenda item in accordance with Resolution 680 (WRC-23).  Under WRC-27 agenda item 1.13, the band 2 500-2 570 MHz is being considered for DC-MSS uplinks (from IMT UEs) and the band 2 620-2 690 MHz is being considered for DC-MSS downlinks (to IMT UEs).

This study will examine the impact of SRS systems on the lunar surface being studied under AI 1.15 to non-GSO DC-MSS uplinks being studied under AI 1.13.

## 1.1 	SRS Technical and operational characteristics

The technical and operational characteristics of SRS systems operating on the lunar surface are provided in the Draft New Report ITU-R SA.[LUNAR.SRS STATIONS CHAR], and the relevant parameters used in this study are summarized in Table 1 below.

Table 1

Technical characteristics of the lunar SRS system (surface-to-surface)

| Station | Lunar Surface-to-Surface Transmitter |
| --- | --- |
| Frequency range | 2 500 – 2 690 MHz |
| Polarization | Linear ±45º |
| Tx Antenna Peak Gain | 16 dBi |
| EIRP Density | 19 dB(W/MHz) |
| Max. EIRP | 29 dBW |
| Channel BW | 10 MHz |
| Deployment characteristics Coverage area Deployment density Activity factor (average) | Wide area (10 km radius) 1-3 LCTs and 1-6 EVAs per cell 20%, 50% |

## 1.2 	DC-MSS technical and operational characteristics

The technical and operational characteristics of DC-MSS systems are provided in Annex 7 to the WP 4C Chair’s Report (Document 4C/356).  For this analysis, the relevant parameters for the DC-MSS uplink are derived from System 7 in Annex 7 – Part 1 of 4C/356 and are summarized in Table 2 below.

Table 2

Technical characteristics of the DC-MSS NGSO Uplink (Earth-to-space)

|  | System 7 (Annex 7 – Part 1 of 4C/356) |
| --- | --- |
| Parameter | MSS space station receiver (NGSO) |
| Frequency range | 2 500 – 2 570 |
| Orbit | 40 000 km altitude (HEO) |
| Antenna gain | 47.5 dBi (12-m antenna) |
| Antenna pattern | Rec. ITU-R S.1528 Ln = -20 dB |
| System Noise Temperature | 600 K |
| Bandwidth | 5 MHz |
| Minimum Elevation (User Terminal to MSS space station) | 35 deg |

## 1.3	Propagation model

This study assumes the free space propagation model in Recommendation ITU-R P.525, per the Working Party 3J/3M guidance in Document 4C/92.

## 1.4	Methodology

The methodology employed in this study is based on a static analysis of worst-case interference. It evaluates the interference level for the DC-MSS space station receiver from the lunar surface transmitter using the worst case geometry.

#### 1.5 	Study results

For the worst-case analysis, the following assumptions were considered:

- The MSS space station is in HEO orbit with 40,000 km maximum altitude.  The minimum elevation angle from the User Terminal on the Earth’s surface to the MSS space station is 35 degrees.

- Due to the relatively high gain of the DC-MSS space station, the worst case interference geometry does not necessarily occur when the DC-MSS space station is closest to the Moon.  Rather it is when the DC-MSS antenna is pointed closest to the Moon as shown Figure 1.  The smallest off-boresight angle for the DC-MSS antenna to the Moon is 1.435 degrees

FIGURE 1

SRS–MSS Scenario (worst case interference geometry)

- In the present study, it is assumed that both systems operate at the same central frequency. A bandwidth correction factor (CF) is applied when the bandwidth of the potential interferer exceeds the bandwidth of the victim receiver under analysis. If the bandwidth of the interferer is equal to or smaller than that of the victim receiver, the bandwidth correction factor is 0

CF = 10 * log10bandwidthvictimbandwidthinterferer

- The interference-to-noise ratio (I/N) is calculated considering Lunar SRS transmitter pointing towards the DC-MSS victim receiver using the following equation:

INTotal= 1n (EIRPnθ,φ+CF-FSPL+Grxθ,φ)-Nrx)

where n is the number of lunar surface transmitters;  EIRPnθ,φ is the equivalent isotropic radiated power of each lunar transmitter; FSPL is the free space path loss calculated using Rec. ITU-R P.525; (Grx)θ,φ is the DC-MSS victim receiver maximum antenna gain; and Nrx is the thermal noise power in the DC-MSS victim receiver bandwidth

- The receiver thermal noise power is calculated from the system noise temperature using the formula provided in Recommendation ITU-R P.372-17

## 1.5 	Summary and analysis of the results

The calculation of interference from the lunar surface SRS transmitter to the HEO DC-MSS uplink receiver is provided in Table 3, using the geometry shown in Figure 1.  The worst case MSS receiver I/N calculated for a single lunar surface interferer is -25.4 dB.  Note that normally the lunar surface antenna will be pointed towards another lunar surface asset so the antenna gain towards Earth will be considerably less than what was assumed in the calculation.

Based on the lunar surface deployment characteristics described in Doc 7/26 and assuming that half of the lunar surface transmitters are visible to Earth, potentially up to 8 lunar surface transmitters (5 clusters visible to Earth x 3 base stations per cluster x 50% activity factor) could be active at any particular time.  Again making a worst case assumption that all the lunar surface transmit antennas are pointed in the direction the DC-MSS space station, the aggregate DC-MSS receiver I/N is -16.3 dB.

These static interference calculation results suggest that WRC-27 AI 1.13 can be treated independently from AI 1.15 with regards to any potential impact of lunar surface SRS operations to DC-MSS space station receivers in the 2 500 – 2 690 MHz band overlapped by the two agenda items.

Table 3

SRS – DC-MSS space station (uplink) interference calculation

| Category | Item | Unit | Scenario 1 |
| --- | --- | --- | --- |
| Direction | From |  | Lunar surface transmitter |
|  | To |  | DC-MSS space station (System 7 HEO 40 000 km) |
| Band | Frequency | MHz | 2535 |
|  | Bandwidth | MHz | 5 |
| Lunar Surface Transmitter | EIRP | dBW | 29 |
|  | Channel BW | MHz | 10 |
|  | EIRP density | dB(W/MHz) | 19 |
| DC-MSS space station | Max antenna gain | dBi | 47.5 |
|  | Antenna gain towards lunar transmitter | dBi | 27.5 |
|  | Noise temperature | K | 600 |
|  | Channel BW | MHz | 5 |
|  | Noise Power | dBW | -133.8 |
| Propagation | Distance | km | 407 360 |
|  | Free space loss | dB | 212.7 |
| Results | Interference Power | dBW | -159.2 |
|  | I/N | dB | -25.4 |

Table 4

Summary of SRS – DC-MSS space station (uplink) interference results

| SRS Station | SRS center frequency | DC-MSS space station | Analysis | I/N result |
| --- | --- | --- | --- | --- |
| Lunar Surface Transmitter | 2535 MHz | HEO orbit (40,000 km) | Single Entry (worst case geoemetry) | -25.4 dB |
|  |  |  | Aggregate (8 active transmitters) | -16.3 dB |

____________________