| US Radiocommunication Sector FACT SHEET |  |  |  |
| --- | --- | --- | --- |
| Study Group: USWP 7B |  |  | Document No: US7B_27_037 |
| Reference: 7B/192-E Annex 10 |  |  | Date: 4 December 2025 |
| Document Title: Preliminary Draft New Recommendation ITU-R SA.[2.0 GHZ SRS & EESS CHAR] |  |  |  |
| Authors Ted Berman Peraton for NASA Richard Tseng, NASA James Brase Peraton for NASA Dennis Lee NASA JPL | Telephone 240-449-0884 301-286-9658 703-483-1575 818-354-6908 | E-Mail theodore.e.berman@nasa.gov richard.s.tseng@nasa.gov james.m.brase@nasa.gov dennis.k.lee@jpl.nasa.gov |  |
| Purpose: To document characteristics of SRS and EESS systems operating (s-s) and (E-s) links in the 2 025 – 2 110 MHz and 2 110 – 2 120 MHz bands. |  |  |  |
| Abstract: At the September 2025 WP 7B meeting, work continued on development of the preliminary draft new recommendation ITU-R SA.[2.0 GHZ SRS & EESS CHAR] to document the characteristics of SRS and EESS. This contribution will address a number of issues raised during the meeting through editor’s notes, including removal of some systems which are SoS. |  |  |  |
| Fact Sheet Preparer: Ted Berman, Peraton for NASA |  |  |  |

| Radiocommunication Study Groups |  |
| --- | --- |
| Source: Document 7B/192 (Annex 10) Subject: PDN Rec. ITU-R SA.[2 GHz SRS & EESS CHAR] | Document WP7B_27_037 |
|  | 9 December 2025 |
|  | English only |
| United States of America |  |
| PRELIMINARY DRAFT NEW RECOMMENDATION ITU-R SA.[2.0 GHz SRS & EESS CHAR] |  |

At the September 2025 WP 7B meeting, work continued on development of the preliminary draft new recommendation ITU-R SA.[2.0 GHZ SRS & EESS CHAR] to document the characteristics of SRS and EESS. This contribution will address a number of issues raised during the meeting through editor’s notes, including removal of some systems which are SoS.

Some of the editor’s notes are reproduced, along with responses.

[Editor’s note: to determine if a system is SOS several definitions in Article 1 of the ITU-R Radio Regulations should be consulted, including 1.23, 1.136, 1.133, and 1.135. SOS corresponds to TT&C and is defined broadly to include not just spacecraft command but also spacecraft payload command (e.g.: turning on equipment), and in general has narrower bandwidth signals, and typically has modulation of types such as PCM/PM and PCM/PSK/PM. SRS and EESS correspond to transmission of data, and in general has wider bandwidth signals, and typically has modulation types such as BPSK, QPSK, etc.]

[Editor’s note: this Recommendation should contain characteristics for SRS and EESS links only. It should not contain characteristics for SOS systems, as these latter are contained in Rec. ITU-R SA.2169. Administrations are invited to review the links in Table 1 below, and if any are determined to be for SOS systems, submit a contribution to the first meeting of WP 7B in 2026 to remove them, and possibly to move them to Rec. ITU-R SA.2169. Similarly, administrations are invited to review the links in Rec. ITU-R SA.2169, and if any are determined to not be for SOS systems, submit a contribution to the first meeting of WP 7B in 2026 to remove them, and possibly to move them to this Recommendation. Additionally, administrations are invited to add launch vehicle links to Rec. ITU-R SA.2169.]

[USA response: it is proposed to remove Systems A and C, as these are SOS. The USA has an input to WP7B to propose to add system C to Rec ITU-R SA.2169, as it is a Lunar system, which is not otherwise covered there. System A is similar to other systems in the Rec, so it is not proposed to add it. Additionally, a review of SA.2169 has determined that one system there is not SOS, so it is proposed to remove it from there and add it to this Recommendation (see System A1).]

[Editor’s note: as an alternative to retaining EESS characteristics for systems in the 2 025 to 2 110 MHz band in this Recommendation, administration are invited to consider moving them to a new section of Report ITU-R SA.2488, which is currently under Revision. For this latter alternative, it may be necessary to keep a reference in this Recommendation to the EESS characteristics, which says that the specific characteristics are contained in Report ITU-R SA.2488.]

[USA response: it is not proposed to do this, as we believe that there is value in keeping EESS characteristics in this document, as it is for a specific frequency band, whereas SA.2488 has several bands. The USA also believes there is value in including these values in a Recommendation instead of in a Report.]

[Editor’s note: it is noted that Report ITU-R SA.2488 contains a MetSat system in the 2 025-2 110 MHz band, but that this band does not contain a MetSat allocation, so it should be reviewed from Report ITU-R SA.2488.]

[USA response: further inquiry shows that based on RR 1.52 MetSat is a subset of EESS, so we believe it is correct to have the above Metsat system in Report SA.2488, and we recommend no action on this topic.]

[Editor’s note: a review and update to several pairs of related Recommendations is needed, in which pairing is by Recommendations in which one contains characteristics and the other contains the corresponding sharing or protection criteria. These pairs include: SA.2169 & SA.363 for SOS, SA.[2 GHz SRS & EESS Char] & SA.514 for EESS, and SA.[2 GHz SRS & EESS Char] & SA.609 for SRS.]

[USA response: we agree, but do not have an input to the current WP7B meeting on this topic.]

Attachment 1 proposes updates to the subject Recommendation under development in WP 7B.

ATTACHMENT 1

PRELIMINARY DRAFT NEW RECOMMENDATION ITU-R SA.[2.0 GHZ SRS & EESS CHAR]

Technical and operational characteristics of the space research service systems in the 2 025-2 120 MHz and Earth exploration-satellite service systems in the 2 025-2 110 MHz frequency band to be used for assessing interference and for conducting sharing and compatibility studies

(202X)

Scope

This Recommendation provides technical and operational characteristics to be used in sharing and compatibility studies for the space research service (SRS) and Earth exploration-satellite service (EESS) that use the 2 025-2 120 MHz (Earth-to-space) frequency band related to science missions.

Keywords

SRS, EESS, DRS

Related ITU-R Recommendations and Reports

Recommendation ITU-R SA.514 - Interference criteria for command and data transmission systems operating in the Earth exploration-satellite and meteorological-satellite services

Recommendation ITU-R SA.609 – Protection criteria for radiocommunication links for manned and unmanned near-Earth research satellites (Note: these criteria are applicable for SRS links in the (Earth-to-space) direction) to spacecraft at altitudes ≤ 2E6 km)

Recommendation ITU-R SA.1018 – Hypothetical reference system for systems comprising data relay satellites in the geostationary orbit and user spacecraft in low Earth-orbits

Recommendation ITU-R SA.1020 – Hypothetical reference system for the Earth exploration-satellite and meteorological satellite services

Recommendation ITU-R SA.1014 – Radiocommunication requirements for manned and unmanned deep space research

Recommendation ITU-R SA.1155 – Protection criteria related to the operation of data relay satellite systems (Note: these criteria are applicable for SRS links in the (space-to-space) direction) to spacecraft at altitudes ≤ 2E6 km)

Recommendation ITU-R SA.1157 – Protection criteria for deep space research (Note: these criteria are applicable for SRS links in the (Earth-to-Space) direction) to spacecraft at altitudes ≥ 2E6 km)

Recommendation ITU-R SA.1160 – Aggregate interference criteria for data transmission systems in the Earth exploration-satellite and meteorological-satellite services using satellites in the geostationary orbit (Note: these criteria are applicable for EESS and MetSat links in the (Earth-to-space) direction) to spacecraft at altitudes ≤ 2E6 km)

Recommendation ITU-R SA.1164 – Sharing and coordination criteria for service links in data collection systems using GSO satellites in the Earth exploration-satellite and meteorological-satellite services

Recommendation ITU-R SA.1414 – Characteristics of data relay satellite systems

Recommendation ITU-R SA.2169 – Technical and operational characteristics of the space operation service (SOS) systems that use the 2 025-2 110 MHz (Earth-to-space) (space-to-space) and 2 200-2 290 MHz (space-to-Earth) (space-to-space) frequency bands for use in assessing of interference and for conducting sharing studies

Report ITU-R SA.2488 – Characteristics to be used for assessing interference to systems operating in the Earth exploration-satellite and meteorological-satellite services, and for conducting sharing studies

The ITU Radiocommunication Assembly,

considering

a)	that the frequency band 2 025-2 110 MHz is allocated to the SRS and EESS on a primary basis among other services in the Earth-to-space and space-to-space directions;

b)	that the frequency band 2 110-2 120 MHz is allocated to the SRS (deep space) on a primary basis among other services in the Earth-to-space direction;

c)	that in order to carry out sharing studies, technical and operational characteristics of SRS and EESS systems for use in the frequency band 2 025-2 120 MHz are needed, where applicable;

d)	that technical and operational characteristics of data relay satellite (DRS) systems are contained in Recommendation ITU-R SA.1414,

[e)	that Report ITU-R SA.2488 does not contain characteristics for any EESS or SRS systems in the 2 025-2 120 MHz band;

f)	that Recommendation ITU-R SA.2169 contains characteristics for SOS systems in the 2 025-2 120 MHz band;]

g)	that this Recommendation contains characteristics of systems transmitting data (SRS or EESS),

recommends

that the technical and operational system characteristics for the SRS and EESS operating in the 2 025-2 110 MHz (Earth-to-space) (space-to-space) and for the SRS (deep space) (Earth-to-space) operating in the 2 110-2 120 MHz frequency bands detailed in the annex should be used in sharing studies.

Annex

Technical and operational characteristics of the space research service systems in the 2 025-2 120 MHz frequency band and Earth exploration-satellite service systems in the 2 025-2 110 MHz frequency band to be used for assessing interference and for conducting 
sharing and compatibility studies

This Recommendation provides the technical and operational characteristics of the SRS and EESS systems in the 2 025-2 120 MHz band.

The frequency band 2 025-2 110 MHz is allocated to the SRS and EESS (Earth-to-space) and the frequency band 2 110-2 120 MHz is allocated to the SRS (deep space) (Earth-to-space). Typical characteristics for systems in these frequency bands are listed below in Tables 1 through 4.

# 1	Earth-to-space links, 2 025-2 110 MHz band

Table 1 lists the transmit and receive parameters for Earth-to-space (E-s) links in the 2 025-2 110 MHz band for SRS and EESS systems.

TABLE 1, PART 1

E-s transmit links

| Parameter |  | Unit |  | System A1 | System B |  | System D | System E | System F |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-s transmit |  |  |  |  |  |  |  |  |  |
| Earth station locations |  |  |  |  | Fairbanks, AK, USA; Dongara, AUS; Overberg, S. Africa |  | Goldstone, CA, USA; Canberra, AUS; Madrid, Spain | Goldstone, CA, USA; Canberra, AUS; Madrid, Spain | Kaliningrad, RUS; Moscow, RUS; Zheleznogorsk, RUS; Sakhalin, RUS; Tsiolkovsky, RUS; Baikonur, RUS |
| Transmit antenna gain |  | dBi |  | 34.2/38.0 | 45.7 |  | 55.79 | 61.5 | 38 |
| Transmit antenna pattern |  |  |  | RR Appendix 8, Annex 3 |  |  |  |  | Rec. ITU-R S.465-5 |
| Transmit power |  | dBW |  | 22 | 23 |  | 33 | 43 | 25 |
| Max pwr spectral density |  | dBW/Hz |  | -42.8 | −25.1 |  | –18 | 3.5 | –20.3 / –42.8 |
| Transmit bandwidth |  | MHz |  | 6.0 | 0.128 |  | 0.128 | 2 | 0.034 / 6 |
| Modulation type |  |  |  | QPSK | BPSK |  | QPSK | QPSK | PCM/PSK/PM, BPSK, QPSK |
| Subcarrier |  | kHz |  | - | – |  | – | – | 16 |
| Range tone |  | kHz |  | - | – |  | – | – |  |
| Minimum elevation angle |  | deg |  | 5 | 5 |  | 5 | 5 | 3 |
| E-s receive |  |  |  |  |  |  |  |  |  |
| Service |  |  |  | EESS | EESS. SRS |  | SRS | SRS |  |
| Altitude |  | km |  | 510 | 407 |  | 242 350 | 1 500 000 | 450 |
| Eccentricity |  |  |  | 0 | 0 |  | 0.539 | 0 | 0 |
| Inclination |  | deg |  | 97 | 65 |  | 47 | 0 | 51.6 |
| Sun Synchronous? (Y/N) |  |  |  | Y | N |  | N | N | N |
| Receive antenna gain |  | dBi |  | 2 | 23 |  | 1.5 | 30 | –3 |
| Receiving system noise temperature (1) |  | K |  | 450 | 226 |  | 290 | 518 | 450 |
| Receive antenna pattern |  |  |  | Recommendation ITU-R S.672 |  |  |  |  | ND |
|  | (1) referred to the input terminals of the receiver |  |  |  |  |  |  |  |  |

TABLE 1, PART 2

E-s transmit links

| Parameter | Unit | System G | System H | System I | System J | System K | System L |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-s transmit |  |  |  |  |  |  |  |
| Earth station locations |  |  | Canada: Prince Albert, CAN; Gatineau, CAN; Inuvik, CAN; Saint-Hubert, CAN |  |  |  |  |
| Transmit antenna gain | dBi | 42 | 41.3 / 44.1 / 46.3 | 47 | 55.6 | 47 | 55.6 |
| Transmit antenna pattern |  | Rec. ITU-R S.465-5 | Rec. ITU-R S.465 | Rec. ITU-R S.465 | Rec. ITU-R S.465 | Rec. ITU-R S.465 | Rec. ITU-R S.465 |
| Transmit power | dBW | 14 | 18.7 / 11.9 / 11.7 | 20 | 20 | 20 | 30 |
| Max pwr spectral density | dBW/Hz | –39 | -25.2 /-32 / -32.2 | -33.8 / -16.3 | -30.9 / -16.3 | -16.3 | -20.9 / -6.3 |
| Transmit bandwidth | MHz | 0.2 | 0.004 | 1.1 | 2 | 0.044 | 2 |
| Modulation type |  |  | PCM/BPSK/ PM | BPSK, PCM/PSK/PM | BPSK, PCM/PSK/PM | PCM/PSK/PM | BPSK, PCM/PSK/PM |
| Subcarrier | kHz |  | 16 |  | 16 |  | 16 |
| Range tone | kHz |  | N/A |  | 500 |  | 500 |
| Minimum elevation angle | deg | 3 | 5 | 5 | 5 | 5 | 5 |
| E-s receive |  |  |  |  |  |  |  |
| Service |  |  | SRS |  |  |  |  |
| Altitude | km | 500 | 773 | 628 | 550 | 200-500 | 32 700 (apogee), 300 (perigee) |
| Eccentricity |  | 0 | 0 | 0 | 0 | 0 | 0.708 |
| Inclination | deg | 88.2 | 98.3 | 97.9 | 31 | 51.6 | 31 |
| Sun Synchronous? (Y/N) |  | N | N | Y | N | N | N |
| Receive antenna gain | dBi | 1.5 | 3.5 | 7.5 | 7 | 5 | 6 |
| Receiving system noise temperature (1) | K | 600 | 1892 | 515 | 502 | 537 | 789 |
| Receive antenna pattern |  | ND | ND | ND | ND | ND | ND |
| (1) referred to the input terminals of the receiver |  |  |  |  |  |  |  |

# 2	Space-to-space links, 2 025-2 110 MHz band

Tables 2 and 3 list the transmit and receive parameters, respectively, for space-to-space (s-s) links in the 2 025-2 110 MHz band for SRS and EESS systems. Systems are connected by letter: for example, System A in Table 2 is connected to Systems A1, A2, A3, A4, A5, and A6 in Table 3.

TABLE 2

s-s transmit links

| Parameter | Unit | System A | System B | System C |
| --- | --- | --- | --- | --- |
| Transmit S/C altitude | km | 35 786 | 400 | 35 786 |
| Transmit S/C location | deg | Rec. ITU-R SA.1275 | Non-GSO | 42.5E & 240E |
| Transmit antenna gain | dBi | 36.0 | 7.5 | 34.6 |
| Transmit antenna pattern |  | Rec. ITU-R S.672 | ND | Rec. ITU-R S.672 |
| Transmit power | dBW | 12.5 | -3.5 | 21 |
| Max pwr spectral density | dBW/Hz | −55.4 | -65.8 | −47.1 |
| Transmit bandwidth | MHz | 6.16 | 10(1) | 6.5 |
| (1) spread spectrum |  |  |  |  |

Table 3

s-s receive links

| Parameter | Unit | System A1 | System A2 | System A3 | System A4 | System A5 |
| --- | --- | --- | --- | --- | --- | --- |
| Service |  | EESS, SRS | EESS, SRS | EESS, SRS | EESS | EESS |
| Altitude | km | 573.3 | 350 | 400 | 400 | 835 |
| Eccentricity |  | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Inclination | deg | 28.47 | 51.6 | 51.6 | 96.8 | 98.85 |
| Sun Synchronous? (Y/N) |  | N | N | N | Y | Y |
| Receive antenna gain | dBi | 0.0 | 12.9 | 22.5 | ‒3/18 | ‒3 |
| Receiving system noise temperature | K | 549.5 | 587 | 447 | 450 | 450 |
| Receive antenna pattern |  | ND-SPACE | Rec. ITU-R S.672 | Rec. ITU-R S.672 | ND-SPACE/ ITU-R S.672 | ND-SPACE |

| Parameter | Unit | System A6 | System B | System C |
| --- | --- | --- | --- | --- |
| Service |  |  |  |  |
| Altitude | km | 200-500 | 400 | 400 |
| Eccentricity |  | 0 | 0 | 0 |
| Inclination | deg | 51.6 | 51.6 | 51.5 |
| Sun Synchronous? (Y/N) |  | N | N | N |
| Receive antenna gain | dBi | 5 | 5 | 8 |
| Receiving system Noise temperature (1) | K | 509 | 480 | 631 |
| Receive antenna pattern |  | ND | ND | Rec. ITU-R S.672 |
| (1) referred to the input terminals of the receiver |  |  |  |  |

# 3	Earth-to-space links, 2 110-2 120 MHz band

Tables 4 lists the transmit and receive parameters for Earth-space (E-s) links in the 2 110-2 120 MHz band for SRS (deep space) systems.

TABLE 4

E-s transmit links

| Parameter | Unit | System DS-A | System DS-B | System DS-C |
| --- | --- | --- | --- | --- |
| E-s transmit |  |  |  |  |
| Earth station location |  | USA | Australia | India |
| Transmit antenna gain | dBi | 55.2 | 62.7 | 53.5 |
| Transmit antenna pattern |  | Rec. ITU-R SA.509-3 |  | Rec. ITU-R S.580-6 |
| Transmit power | dBW | 43 | 54.7 | 43 |
| Max pwr spectral density | dBW/Hz | 7 | 18.7 | 4 |
| Transmit bandwidth | MHz | 2.5 | 2.5 | 2.5 |
| Modulation type |  | PCM/PSK/PM | PCM/PSK/PM | PCM/PSK/PM |
| Subcarrier frequency | kHz | 16 | 16 | 8 |
| Range tone | kHz | 1 033 | 1 033 | 1 033 |
| Minimum elevation angle | deg | 10 | 10 | 10 |
| E-s receive |  |  |  |  |
| Space station minimum distance from Earth (Launch & Early Cruise, Earth flybys) | km | 200 | 3 000 | 264 |
| Space station maximum distance from Earth | km | 4.01 × 108 | 1.52 × 109 | 4.01 × 108 |
| Receive antenna gain | dBi | −2 (LGA) | 7 (LGA) | 0 (LGA) |
|  |  | 28.7 (HGA) | 34.6 (HGA) | 32.5 (HGA) |
| Receiving system noise temperature (1) | K | 200 | 1 500 | 600 |
| Receive antenna pattern |  | Rec. ITU-R S.672 |  |  |
| (1) referred to the input terminals of the receiver |  |  |  |  |

____________________