| US Radiocommunication Sector FACT SHEET |  |  |  |
| --- | --- | --- | --- |
| Study Group: USWP 7B |  |  | Document No: US7B_27_038 |
| Reference: Rec. ITU-R SA.2169-0 |  |  | Date: 4 December 2025 |
| Document Title: Preliminary Draft Revision to Recommendation ITU-R SA.2169 |  |  |  |
| Authors Ted Berman Peraton for NASA Richard Tseng, NASA James Brase Peraton for NASA Dennis Lee NASA JPL | Telephone 240-449-0884 301-286-9658 703-483-1575 818-354-6908 | E-Mail theodore.e.berman@nasa.gov richard.s.tseng@nasa.gov james.m.brase@nasa.gov dennis.k.lee@jpl.nasa.gov |  |
| Purpose: To propose a PDR to Recommendation ITU-R SA.2169. |  |  |  |
| Abstract: At the September 2025 WP 7B meeting, work continued on development of the preliminary draft new recommendation ITU-R SA.[2.0 GHZ SRS & EESS CHAR] to document the characteristics of SRS and EESS. During this discussion, it was determining that some updates may ned to be made to Recommendation ITU-R SA.2169. This input proposes such updates. |  |  |  |
| Fact Sheet Preparer: Ted Berman, Peraton for NASA |  |  |  |

| Radiocommunication Study Groups |  |
| --- | --- |
| Source: Rec. ITU-R SA.2169 Subject: Proposed Revision to SA.2169 | Document WP7B_27_038 |
|  | 9 December 2025 |
|  | English only |
| United States of America |  |
| PRELIMINARY DRAFT REVISION TO RECOMMENDATION ITU-R SA.2169 |  |

At the September 2025 WP 7B meeting, work continued on development of the preliminary draft new recommendation ITU-R SA.[2.0 GHZ SRS & EESS CHAR] to document the characteristics of SRS and EESS. During this discussion, it was determining that some systems in this document are SOS and so these systems should be removed and possibly moved to Recommendation ITU-R SA.2169. Furthermore, SA.2169 should be reviewed to determine of any non-SOS systems are included, and these should be removed. This contribution proposes to add one Lunar system from SA.[2.0 GHZ SRS & EESS CHAR] (another USA input to WP7B proposes to remove it from SA.[2.0 GHZ SRS & EESS CHAR]) to Table 4 below (system M1) and proposes to remove system C form Table 4 below, as the large bandwidth indicates that this is not an SOS system. The USA proposes to move this system to SA.[2.0 GHZ SRS & EESS CHAR].

Attachment 1 proposes updates to the subject Recommendation under the purview of WP 7B.

ATTACHMENT 1

RECOMMENDATION  ITU-R  SA.2169-0

Technical and operational characteristics of the space operation service (SOS) systems that use the 2 025-2 110 MHz (Earth-to-space) (space-to-space) and 2 200-2 290 MHz (space-to-Earth) (space-to-space) frequency bands for 
use in assessing of interference and for conducting sharing studies

(2025)

Scope

This Recommendation provides technical and operational characteristics for use in sharing studies for the space operation service (SOS) systems that use the 2 025-2 110 MHz (Earth-to-space) (space-to-space) and 2 200-2 290 MHz (space-to-Earth) (space-to-space) frequency bands.

Keywords

Tracking, Telemetry, Command, Space Operation, TT&C, SOS, DRS, POCS

Abbreviations/Glossary

CP		Circular polarization

DRS		Data relay satellite

GSO		Geostationary-satellite orbit

HEO		Highly-elliptical orbit

LEO		Low-Earth orbit

MEO		Medium-Earth orbit

ND		Non-directional

non-GSO	Non-geostationary-satellite orbit

POCS		Proximity operations communication system

SOS		Space operation service

TT&C		Telemetry, tracking and command

Related ITU-R Recommendations and Reports

Recommendation ITU-R SA.363 – Space operation systems

Recommendation ITU-R SA.1018 – Hypothetical reference system for systems comprising data relay satellites in the geostationary orbit and user spacecraft in low Earth-orbits

Recommendation ITU-R SA.1020 – Hypothetical reference system for the Earth exploration-satellite and meteorological satellite services

Recommendation ITU-R SA.1414 – Characteristics of data relay satellite systems

The ITU Radiocommunication Assembly,

considering

a)	that the frequency band 2 025-2 110 MHz is allocated to the SOS on a primary basis among other services in the Earth-to-space and space-to-space directions;

b)	that the frequency band 2 200-2 290 MHz is allocated to the SOS on a primary basis among other services in the space-to-Earth and space-to-space directions,

recommends

that the technical and operational characteristics for the SOS systems operating in the 2 025-2 110 MHz (Earth-to-space) (space-to-space) and 2 200-2 290 MHz (space-to-Earth) (space-to-space) frequency bands detailed in the Annex should be used in sharing studies.

Annex

TABLE OF CONTENTS

Page

1	Introduction		2

2	Technical and operational characteristics of the geostationary satellites		3

2.1	Telemetry in the 2 200-2 290 MHz frequency band		3

2.2	Command in the 2 025-2 110 MHz frequency band		3

3	Technical and operational characteristics of the non-geostationary satellites		4

3.1	Telemetry/ranging in the 2 200-2 290 MHz frequency band		4

3.2	Command/ranging in the 2 025-2 110 MHz frequency band		7

4	Technical and operational characteristics of the SOS space-to-space links		10

4.1	Data Relay Satellite (DRS) systems		10

4.2	Proximity Operations Communication System (POCS)		10

# 1	Introduction

This Annex provides the technical and operational characteristics of telemetry, tracking and command (TT&C) systems operating in the space operation service in the 2 025-2 110 MHz and 2 200-2 290 MHz frequency bands.

The TT&C system performs the following functions for ensuring the successful operation of a satellite:

1	Telemetry to enable ground controllers to monitor the operational health and status of the satellite, and the measured values are transmitted from satellite to the ground control centre.

2	Tracking/ranging to enable ground controllers to determine the satellite’s location and orientation.

3	Telecommand to enable ground controllers to command the various electronic units aboard the satellite, sending commands from the ground to the satellite.

The frequency band 2 025-2 110 MHz is allocated to space operation service (SOS) (Earth-to-space) (space-to-space) and the frequency band 2 200-2 290 MHz is allocated to SOS (space-to-Earth) (space-to-space). These frequency bands are used by geostationary and non-geostationary satellites and the inter-satellite links.

# 2	Technical and operational characteristics of the geostationary satellites

Representative characteristics of the TT&C systems for GSO SOS systems are listed in Tables 1 and 2.

## 2.1	Telemetry in the 2 200-2 290 MHz frequency band

Table 1 lists the system parameters for telemetry downlinks in the frequency band 2 200-2 290 MHz for the GSO SOS systems.

TABLE 1

GSO SOS system parameters for telemetry downlinks in the frequency band 2 200-2 290 MHz

| Function | Telemetry |
| --- | --- |
| System | System A |
| Maximum necessary bandwidth (MHz) | 4.930 |
| Transmitting Satellite parameters |  |
| Satellite antenna input power (dBW) (1) | 1.8 |
| Satellite antenna type | Bicone/Crossed-dipole |
| Satellite maximum antenna gain (dBi) | 1.0 |
| Satellite antenna polarization | CP |
| Satellite antenna radiation pattern | Cardioid −13 dB at 170 degrees |
| Receiving Earth station parameters |  |
| Earth station antenna type | Parabolic |
| Earth station antenna radiation pattern | Rec. ITU-R S.465-6 |
| Earth station maximum antenna gain (dBi) | 50 |
| Earth station antenna polarization | CP |
| Earth station receiver noise temperature (K) | 130 |
| Minimum elevation angle (degree) | 5 |
| (1) “satellite antenna input power” includes the antenna feeder loss. |  |

## 2.2	Command in the 2 025-2 110 MHz frequency band

Table 2 lists the system parameters for command uplinks in the 2 025-2 110 MHz frequency band for GSO SOS systems.

TABLE 2

GSO SOS system parameters for command uplinks in the frequency band 2 025-2 110 MHz

| Function | Command |
| --- | --- |
| System | System A |
| Maximum necessary bandwidth (MHz) | 0.084 |
| Transmitting Earth station parameters |  |
| Earth station antenna input power (dBW) | 21.9 |
| Earth station antenna type | Parabolic |
| Earth station antenna radiation pattern | 34.6 dB at 0.95 degrees Rec. ITU-R S.465-6 |
| Earth station maximum antenna gain (dBi) | 49.5 |
| Earth station antenna polarization | CP |
| Minimum elevation angle (degree) | 5 |
| Receiving satellite parameters |  |
| Satellite antenna type | Crossed-dipole |
| Satellite maximum antenna gain (dBi) | 1 |
| Satellite antenna polarization | CP |
| Satellite antenna radiation pattern | −11 dB at 165 degrees |
| Satellite receiver noise temperature (K) | 650 |

# 3	Technical and operational characteristics of the non-geostationary satellites

Representative characteristics of the TT&C systems for non-geostationary-satellite orbit (non-GSO) SOS systems are listed below in Tables 3 and 4.

Non-GSO satellites operate in a variety of orbits depending on their mission objectives, and characteristics of the TT&C systems are designed according to orbital features such as shape of orbit, orbital altitude, so the characteristics of the TT&C systems are provided for representative orbits of the LEO sun-synchronous orbit, LEO low latitude orbit, MEO, HEO, and Lagrange L1/L2 orbit.

## 3.1	Telemetry/ranging in the 2 200-2 290 MHz frequency band

Table 3 lists the system parameters for telemetry/ranging downlinks in the frequency band 2 200-2 290 MHz for the non-GSO SOS systems. Ranging is used on non-GSO satellites to locate the satellite’s position. Ranging is accomplished alone or together with the telemetry transmission.

TABLE 3

Non-GSO SOS system parameters for telemetry/ranging downlinks in the frequency band 2 200-2 290 MHz

| Function | Command/ranging |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Orbit type | LEO, Sun synchronous |  |  |  |  |  |  |
| System | System B | System C | System D | System E | System F | System G | System H |
| Maximum necessary bandwidth (MHz) | 3.2 | 6 | 2.2 (2) / 2.5 (3) | 0.8 | 3 | 3.32 | 0.5 |
| Orbit information |  |  |  |  |  |  |  |  |
| Shape of orbit | Circular | Circular | Circular | Circular | Circular | Circular | Circular |
| Orbital altitude (km) | 824 | 510 | 628 | 600 | 773 | 550 | 500-800 |
| Inclination angle (degree) | 98.7 | 97 | 97.9 | 97.8 | 98.3 | 97.6 | 97-98.5 |
| Satellite parameters |  |  |  |  |  |  |  |  |
| Satellite antenna input power (1) (dBW) | 7 | −0.2 | −22.2 (2) / −5.2 (3) | −3 | 1 | 0.3 | -4 |
| Satellite antenna type | Helix | Crossed dipole with a reflector | Crossed dipoles with a reflector | Quadrifilar helix | Quadrifilar helix | Broadband patch panel | Quadrifilar Helix / Patch |
| Satellite maximum antenna gain (dBi) | 3 | 2 | 7.5 | 3 | 3.5 | 5.6 | 0 |
| Satellite antenna polarization | CP | CP | CP | CP | CP | CP | CP |
| Satellite antenna radiation pattern | ND by multiple antenna | ND by multiple antenna | ND by multiple antenna | ND by multiple antenna | ND by multiple antenna | Constant gain over ¼ of the sphere | ND |
| Receiving Earth station parameters |  |  |  |  |  |  |  |  |
| Earth station antenna type | Parabolic | Parabolic | Parabolic | Parabolic | Parabolic | Parabolic | Parabolic |
| Earth station antenna radiation pattern | Rec. ITU-R S.465 | Rec. ITU-R S.465 | Rec. ITU-R S.465 | Rec. ITU-R S.465 | Rec. ITU-R S.465 | Rec. ITU-R S.465 | Rec. ITU-R S.580-6 |
| Earth station maximum antenna gain (dBi) | 42 / 44.8 / 46.8 | 34.9 / 39 | 44.2 | 42 / 45 / 47 | 42 / 45 / 47 | 42 / 45 / 47 | 44 |
| Earth station antenna polarization | CP | CP | CP | CP | CP | CP | CP |
| Earth station receiver noise temperature (K) | 130 / 190 / 245 | 75 / 100 | 148 | 139 / 145 / 152 | 139 / 145 / 152 | 139 / 145 / 152 | 200 |
| Minimum elevation angle (degree) | 5 | 5 | 5 | 3 | 3 | 3 | 3 |

TABLE 3 (end)

| Function |  |
| --- | --- |
| Orbit type | LEO, low latitude |  |  |  | MEO | HEO | L1/L2 |
| System | System I |  | System J | System K | System L | System M | System N |
| Maximum necessary bandwidth (MHz) | 0.064 |  | 2.4 (2) / 3 (3) | 2.3 | 2.5 (2) / 2.0 (3) | 0.075 | 1.15 |
| Orbit Information |  |  |  |  |  |  |  |
| Shape of orbit | Circular |  | Circular | Circular | elliptical | Highly elliptical | Heliocentric (L1) |
| Orbital altitude (km) | 550 |  | 550 | 200-500 | 32 700 (apogee) 300 (perigee) | 41885 (apogee) 9710 (perigee) | 1 500 000 |
| Inclination angle (degree) | 24 |  | 31 | 51.6 | 31 | 63.435 | N/A |
| Satellite parameters |  |  |  |  |  |  |  |
| Satellite antenna input power (1) (dBW) | −12.0 |  | −23.5 (2) / −3 (3) | −1.5 / 1.5 | 5.1 (apogee) −14.9 (2) / 5.1 (3) (perigee) | 5.5 | 5 |
| Satellite antenna type | Quadrifilar helix |  | Crossed dipoles with a reflector | Helix | Crossed dipoles with a reflector / dipole | Quadrifilar helix | 2 omni antennas |
| Satellite maximum antenna gain (dBi) | 2.5 |  | 7 | 5 | 6 | 8 | −4.5 |
| Satellite antenna polarization | CP |  | CP | CP | CP / Vertical polarization | CP | CP |
| Satellite antenna radiation pattern | ND by multiple antenna |  | ND by multiple antenna | ND by multiple antenna | ND by multiple antenna | ND by multiple antenna | ND by multiple antenna |
| Receiving Earth station parameters |  |  |  |  |  |  |  |
| Earth station antenna type | Parabolic |  | Parabolic | Parabolic | Parabolic | Parabolic | Parabolic |
| Earth station antenna radiation pattern | Rec. ITU-R S.465 |  | Rec. ITU-R S.465 | Rec. ITU-R S.465 | Rec. ITU-R S.465 | Rec. ITU-R S.465 | RR App. 8 |
| Earth station maximum antenna gain (dBi) | 34.2 / 46.6 |  | 56.3 | 47.1 | 47.1 | 46.7 | 50.5 / 51.8 |
| Earth station antenna polarization | CP |  | CP | CP | CP | CP | CP |
| Earth station receiver noise temperature (K) | 70 / 157 |  | 70 | 147 | 147 | 247 | 251 |
| Minimum elevation angle (degree) | 5 |  | 5 | 5 | 5 | 5 | 5 |

## 3.2	Command/ranging in the 2 025-2 110 MHz frequency band

Table 4 lists the system parameters for command/ranging uplinks in the 2 025-2 110 MHz frequency band for non-GSO SOS systems. Ranging is used on non-GSO satellites to locate the satellite’s position. Ranging is accomplished alone or together with the command transmission.

TABLE 4

Non-GSO SOS system parameters for command/ranging uplinks in the frequency band 2 025-2 110 MHz

| Function | Command/ranging |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Orbit type | LEO, Sun synchronous |  |  |  |  |  |  |
| System | System B |  | System D | System E | System F | System G | System H |
| Maximum necessary bandwidth (MHz) | Command 0.032 Configuration Data 0.256 |  | 1.1 | 0.38 | 0.3 | 0.2 | 0.5 |
| Orbit information |  |  |  |  |  |  |  |  |
| Shape of orbit | Circular |  | Circular | Circular | Circular | Circular | Circular |
| Orbital altitude (km) | 824 |  | 628 | 600 | 773 | 550 | 500-800 |
| Inclination angle (degree) | 98.7 |  | 97.9 | 97.8 | 98.3 | 97.6 | 97-98.5 |
| Transmitting Earth station parameters |  |  |  |  |  |  |  |  |
| Earth station antenna input power (dBW) | 9.8 |  | 20 | 11.7 | 11.7 | 11.7 | 30 |
| Earth station antenna type | Parabolic |  | Parabolic | Parabolic | Parabolic | Parabolic | Parabolic |
| Earth station antenna radiation pattern | Rec. ITU-R S.465-6 |  | Rec. ITU-R S.465 | Rec. ITU-R S.465 | Rec. ITU-R S.465 | Rec. ITU-R S.465 | Rec. ITU-R S.580-6 |
| Earth station maximum antenna gain (dBi) | 41.4 / 42 / 46.2 |  | 43.2 | 41 / 44 / 46 | 41 / 44 / 46 | 41 / 44 / 46 | 43 |
| Earth station antenna polarization | CP |  | CP | CP | CP | CP | CP |
| Minimum elevation angle (degree) | 5 |  | 5 | 5 | 5 | 5 | 5 |
| Satellite parameters |  |  |  |  |  |  |  |  |
| Satellite antenna type | Helix |  | Crossed dipoles with a reflector | Quadrifilar helix | Quadrifilar helix | Broadband patch panel | Quadrifilar Helix / Patch |
| Satellite maximum antenna gain (dBi) | 3 |  | 7.5 | 3 | 3.5 | 5.6 | 0 |
| Satellite antenna polarization | CP |  | CP | CP | CP | CP | CP |
| Satellite antenna radiation pattern | ND by multiple antenna |  | ND by multiple antenna | ND by multiple antenna | ND by multiple antenna | Constant gain over ¼ of the sphere | ND |
| Satellite receiver noise temperature (K) | 263 |  | 515 | 999 | 1892 | 8300 | 1200 |

TABLE 4 (end)

| Function |  | Command/ranging |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Orbit type | LEO, low latitude |  |  |  |  | MEO | HEO | Lunar | L1/L2 |
| System | System I |  | System J | System K | System O | System L | System M | System M1 | System N |
| Maximum necessary bandwidth (MHz) | 0.064 |  | 2 | 0.044 | 0.095 | 2 | 0.1 | 1.0 | 1.0 |
| Orbit information |  |  |  |  |  |  |  |  |  |
| Shape of orbit | Circular |  | Circular | Circular | Circular | elliptical | Highly elliptical | Lunar | Heliocentric (L1) |
| Orbital altitude (km) | 550 |  | 550 | 200-500 | 1336 | 32 700 (apogee) 300 (perigee) | 41885 (apogee) 9710 (perigee) | 405 853.7 (apogee) 362 946.3 (perigee) | 1 500 000 |
| Inclination angle (degree) | 24 |  | 31 | 51.6 | 66 | 31 | 63.435 | 18.28 to 26,58 | N/A |
| Transmitting Earth station parameters |  |  |  |  |  |  |  |  |  |
| Earth station antenna input power (dBW) | 11 |  | 20 | 20 | 8 | 30 | 13.9 | 33.0 | 22.8 / 31 |
| Earth station antenna type | Parabolic |  | Parabolic | Parabolic | Omni | Parabolic | Parabolic | Parabolic | Parabolic |
| Earth station antenna radiation pattern | Rec. ITU-R S.465-6 |  | Rec. ITU-R S.465 | Rec. ITU-R S.465 | ND | Rec. ITU-R S.465 | Rec. ITU-R S.465 | Rec. ITU-R S.465 | RR App. 8 |
| Earth station maximum antenna gain (dBi) | 36.5 / 46.8 |  | 55.6 | 47 | 6 | 47 | 46.3 | 49.7 | 49.8 / 51.1 |
| Earth station antenna polarization | CP |  | CP | CP | Linear | CP | CP | CP | CP |
| Minimum elevation angle | 5 |  | 5 | 5 | 3 | 5 | 5 | 5 | 5 |
| Satellite parameters |  |  |  |  |  |  |  |  |  |
| Satellite antenna type | Quadrifilar helix |  | Crossed dipoles with a reflector | Helix | Omni | Crossed dipoles with a reflector / dipole | Quadrifilar helix | Parabolic | 2 omni antennas |
| Satellite maximum antenna gain (dBi) | 2.5 |  | 7 | 5 | 5.2 | 6 | 8 | 21 | −4.5 |
| Satellite antenna polarization | CP |  | CP | CP | CP | CP | CP | CP | CP |
| Satellite antenna radiation pattern | ND by multiple antenna |  | ND by multiple antenna | ND by multiple antenna | ND | ND by multiple antenna | ND by multiple antenna | ND by multiple antenna | ND by multiple antenna |
| Satellite receiver noise temperature (K) | 1697 |  | 2674 | 537 | 170 | 789 | 840 | 543 | 603 |

# 4	Technical and operational characteristics of the SOS space-to-space links

The SOS space-to-space links typically include the use of a Data Relay Satellite (DRS) system and a Proximity Operations Communication System (POCS).

## 4.1	Data Relay Satellite (DRS) systems

The hypothetical reference system of the DRS systems is described in Recommendations ITU-R SA.1018 and ITU-R SA.1020. The DRS spacecraft is typically located on the geostationary orbit, and the space-to-space links of the DRS system are established between the DRS spacecraft and low-Earth orbiting user spacecraft.

The 2 025-2 110 MHz frequency band is used for SOS Earth-to-space links. This frequency band is also used for SOS forward space-to-space links, typically for radiocommunications from DRS spacecraft to low-Earth orbiting spacecraft. The characteristics of DRS-to-spacecraft links can be found in Table 2 of Recommendation ITU-R SA.1414.

The 2 200-2 290 MHz frequency band is used for SOS space-to-Earth links. This frequency band is also used for SOS return space-to-space links, typically for radiocommunications from low-Earth orbiting spacecraft to DRS spacecraft. The characteristics of spacecraft-to-DRS links can be found in Table 3 of Recommendation ITU-R SA.1414.

## 4.2	Proximity Operations Communication System (POCS)

The proximity space links are short-range, bi-directional, fixed or mobile radio links, generally used to communicate among probes, landers, rovers, orbiting constellations, and orbiting relays. The POCS supports several communications needs between such a variety of network elements for manned and unmanned missions.

The 2 025-2 110 MHz frequency band is used for the POCS forward space-to-space radiocommunications links and the 2 200-2 290 MHz frequency band is used for the POCS return space-to-space radiocommunications links.

Table 5 lists typical POCS operation scenarios.

TABLE 5

Examples of POCS operation scenario

| System | Example 1 |
| --- | --- |
| Location to operate | Near-Earth circular orbit of about 400 km altitude |
| Communication system 1 | Visiting spacecraft |
| Communication system 2 | Manned spacecraft |
| Objectives of operations | Inter-orbit communication when visiting spacecraft approaches manned spacecraft |
| Maximum distance between POCS systems | 23 km |

### 4.2.1	Telemetry/ranging in the 2 200-2 290 MHz frequency band

Table 6 lists the system parameters of telemetry/ranging return links in the frequency band 2 200-2 290 MHz for both transmitting and receiving sides of the POCS systems. Ranging is used on the POCS links to measure distance between two POCS systems.

TABLE 6

POCS system parameters for telemetry/ranging return links 
in the frequency band 2 200-2 290 MHz

| Function | Telemetry/ranging |
| --- | --- |
| System | System P |
| Maximum necessary bandwidth (MHz) | 6 (2) |
| Communication system 1 parameters (transmitting side) |  |
| Antenna input power (1) (dBW) | −0.02 |
| Antenna type | Helix |
| Maximum antenna gain (dBi) | 5 |
| Antenna polarization | CP |
| Antenna radiation pattern | ND by multiple antenna |
| Communication system 2 parameters (receiving side) |  |
| Antenna type | Micro-strip |
| Antenna radiation pattern | ND by multiple antenna |
| Maximum antenna gain (dBi) | 7.5 |
| Antenna polarization | CP |
| Receiver noise temperature (K) | 525 |
| (1) “antenna input power” includes the antenna feeder loss. (2) Spread spectrum. |  |

### 4.2.2	Command/ranging in the 2 025-2 110 MHz frequency band

Table 7 lists the system parameters for command links in the 2 025-2 110 MHz frequency band for both transmitting and receiving sides of the POCS systems. Ranging is used on the POCS links to measure distance between two POCS systems.

TABLE 7

POCS system parameters for command/ranging forward links 
in the frequency band 2 025-2 110 MHz

| Function | Command/ranging |
| --- | --- |
| System | System P |
| Maximum necessary bandwidth (MHz) | 10 (2) |
| Communication system 2 parameters (transmitting side) |  |
| Antenna input power (1) (dBW) | −7.6 |
| Antenna type | Micro-strip |
| Antenna radiation pattern | ND by multiple antenna |
| Maximum antenna gain (dBi) | 7.5 |
| Antenna polarization | CP |
| Communication system 1 parameters (receiving side) |  |
| Antenna type | Helix |
| Maximum antenna gain (dBi) | 5.0 |
| Antenna polarization | CP |
| Antenna radiation pattern | ND by multiple antenna |
| Receiver noise temperature (K) | 455 |
| (1) “antenna input power” includes the antenna feeder loss. (2) Spread spectrum. |  |

____________________