| U.S. Radiocommunication Sector FACT SHEET |  |  |  |
| --- | --- | --- | --- |
| Study Group: WP7B |  | Document No: US7B_27_039_R2 |  |
| Reference: Annex 2 to the September 2025 WP 7B Chair’s Report Doc 7B/192, Resolution 680 (WRC-23), WRC-27 Agenda Item 1.15 |  | Date: 6 February 2026 |  |
| Document Title: WORKING DOCUMENT TOWARDS PRELIMINARY DRAFT CPM TEXT FOR WRC-27 AGENDA ITEM 1.15 |  |  |  |
| Authors | Telephone |  | E-Mail |
| Dennis Lee, NASA/JPL Cathy Sham, NASA/JSC Scott Kotler, Lockheed Martin Luciano Alexandre | 818-354-6908 281-222-1117 703-789-3923 818-393-0663 |  | dennis.k.lee@jpl.nasa.gov catherine.c.sham@nasa.gov scott.kotler@LMCO.com luciano.c.alexandre@jpl.nasa.gov |
| Purpose/Objective: The purpose of this contribution is to further progress the work on the working document for the WRC-27 Agenda item 1.15 CPM text. |  |  |  |
| Abstract: Under Agenda item 1.15 (WRC-27), possible new or modified space research service (space-to-space) allocations, in the frequency ranges identified in Resolution 680 (WRC-23) resolves to invite 1, are under study to ensure sharing and compatibility between SRS links on the lunar surface/between lunar orbit and lunar surface and incumbent services in Res 680 (WRC-23) recognizings g) to n) and RAS in adjacent and nearby bands. At the September 2025 WP 7B meeting, updates to the draft WRC-27 Agenda item 1.15 CPM text were introduced. This contribution is intended to further progress the work with additional text for sections 3-5 of the working document towards a Preliminary Draft CPM Text for WRC-27 Agenda item 1.15 (WRC-27). |  |  |  |
| Fact Sheet Preparer: Dennis Lee, NASA/JPL |  |  |  |

| Radiocommunication Study Groups |  |
| --- | --- |
| Received: XX February 2026 Source: Annex 2 to Document 7B/192 Subject: WRC-27 agenda item 1.15 | Document 7B/YYY-E |
|  | X February 2026 |
|  | English only |
| United States of America |  |
| PRELIMINARY DRAFT CPM TEXT FOR WRC-27 AGENDA ITEM 1.15 |  |

Introduction

This input contribution proposes revisions to the working document towards the preliminary draft CPM text for WRC-27 agenda item 1.15, as contained in Annex 2 of the Working Party 7B Chair’s Report (Document 7B/192). The attachment provides updates to section 4/1.15/3 with additional summary and analysis of the results of ITU-R studies for WRC-27 agenda item 1.15, adds Methods A1, B1, C1, D1, E1, F1, G1, H1 and K1 in sections 4/1/15/4, and example regulatory text in 4/1.15/5. Additionally, the results of consultation between WP 5D, WP 4C and WP 7B regarding frequency overlaps between AIs 1.7, 1.11, 1.13 and AI 1.15 is included in section 4/1.15/3.

Attachment:  1

| Radiocommunication Study Groups |  |
| --- | --- |
| Source: Annex 2 to Document 7B/192 Reference: Resolution 680 (WRC-23) Subject: WRC-27 agenda item 1.15 | Annex XX to Document 7B/ZZZ-E |
|  | XX March 2026 |
|  | English only |
| Annex 2 to Working Party 7B Chair’s Report |  |
| Working document towards preliminary draft CPM text for WRC-27 agenda item 1.15 |  |

CHAPTER 4

Science issues

(Agenda items 1.15, 1.16, 1.17, 1.18, 1.19)

Agenda item 1.15

(WP 7B / WP 3J, WP 4A, WP 4C, WP 5A, WP 5B, WP 5C, WP 5D, WP 7A, WP 7C, WP 7D)

1.15	to consider studies on frequency-related matters, including possible new or modified space research service (space-to-space) allocations, for future development of communications on the lunar surface and between lunar orbit and the lunar surface, in accordance with Resolution 680 (WRC-23);

Resolution 680 (WRC-23) – Studies on frequency-related matters, including possible new or modified space research service (space-to-space) allocations, for future development of communications on the lunar surface and between lunar orbit and the lunar surface

# 4/1.15/1	Executive summary

[Text of the executive summary, not more than half a page of text to describe briefly the purpose of the agenda item, summarize the results of the studies carried out and, most importantly, provide a brief description of the method(s) identified that may satisfy the agenda item]

# 4/1.15/2	Background

[Text of the background, not more than half a page of text to provide general information in a concise manner, in order to describe the rationale of the agenda items (or issue(s))]

WRC-27 agenda item 1.15 focuses on the identification of new or modified frequency bands for the Space Research Service (SRS) (space-to-space) for future development of communications on the lunar surface and between lunar orbit and lunar surface, as outlined in Resolution 680 (WRC-23). It calls for studies addressing technical, operational, and regulatory considerations in the frequency bands 390-406.1 MHz, 420-430 MHz, and 440-450 MHz, limited to outside the shielded zone of the Moon (SZM), as well as in the bands 2 400-2 690 MHz, 3 500-3 800 MHz, 5 150-5 570 MHz, 5 570-5 725 MHz, 5 775-5 925 MHz, 7 190-7 235 MHz, 8 450-8 500 MHz, and 25.25-28.35 GHz. These studies aim to support the growing global interest in lunar exploration for conducting scientific discovery and space exploration activities in lunar orbit and on the lunar surface.

The goal is to address the lunar spectrum needs while ensuring that the SRS does not interfere with radiocommunication services, as specified in recognizing g) to n) of Resolution 680 (WRC-23), and with the RAS on the Earth and in the SZM in the same, adjacent, or nearby bands. This aims to assist countries in developing communication systems on the Moon and in its vicinity, promote technological advancements, and ensure compatibility with existing services.

# 4/1.15/3	Summary and analysis of the results of ITU-R studies

Sharing and compatibility studies were carried out using technical and operational characteristics of lunar SRS stations given in Report ITU-R SA.2553 and characteristics from contributing WPs responsible for incumbent services identified in Resolution 680 (WRC-23) recognizings g) to n). The summary and analysis of results of these studies are provided below in 4/1.15/3.2.

Additional studies were carried out among Working Party 7B (WP 7B), WP 5D and WP 4C concerning the frequency overlap of the band 7 190-7 235 MHz between WRC-27 agenda item 1.15 and the frequency bands under consideration by WP 5D under AI 1.7, and the bands 2 483.5-2 500 MHz   and 2 500-2 570 MHz/2 620-2 690 MHz by WP 4C under AIs 1.11 and 1.13, respectively.  Based on the results of these studies, owing to the large positive margins against the respective protection criteria, WP 7B and WP 5D jointly concluded that WRC-27 agenda item 1.15 and WRC-27 agenda item 1.7 can be treated independently.  Similarly, based on the results of studies, owing to the large positive margins against the respective protection criteria, WP 7B and WP 4C jointly concluded that WRC-27 agenda item 1.15 and WRC-27 agenda items 1.11 and 1.13 can be treated independently.

## 4/1.15/3.1	Relevant ITU-R Recommendations and Reports

A list of the relevant ITU-R Recommendations and Reports are as follows:

Recommendations: M.2101, P.525, M.1184, P.372, F.758, F.1567, F.699, SA.509, S.672, M.1462, M.2134, M.2046, [TBD]

Reports: SA.2553, F.2108, F.2439, SA.[LUNAR_1.15_STUDIES], [TBD]

## 4/1.15/3.2	Sharing and compatibility studies

The results of the sharing and compatibility studies in the frequency ranges identified by resolves 1 of Resolution 680 (WRC-23) for incumbent services identified in recognizings g) thru n) are provided in the following subsections.

### 4/1.15/3.2.1	Frequency range 390-406.1 MHz (outside SZM)

Within the frequency range 390-406.1 MHz, sharing and compatibility studies to ensure protection of FS, MS, and RAS are specified in Resolution 680 (WRC-23). Additional sharing and compatibility studies with MSS (space-to-Earth) and MSS (Earth-to-space) can be found in Annex 1 of the working document towards a preliminary draft new Report ITU-R SA.[LUNAR_1.15_STUDIES].

The characteristics of the lunar SRS stations in the band 390-406.1 MHz are provided in Table 3.2-4 of Report ITU-R SA.2553. The corresponding characteristics for the lunar orbiting stations are provided in Section 3.3 of the same Report. Note that under Resolution 680 (WRC-23), the lunar SRS stations (surface and orbit) may only operate in the 390-406.1 MHz band outside the SZM.

The characteristics of the FS and MS in the band 390-406.1 MHz are documented in Annex 1 of the working document towards a preliminary draft new Report ITU-R SA.[LUNAR_1.15_STUDIES].

#### 4/1.15/3.2.1.1	Lunar SRS sharing with the fixed service

The results of one sharing study show that with the single entry static worst-case analysis, there is a margin of at least 40.3 dB to satisfy the FS protection criteria.   The result also implies that aggregating over 10 700 simultaneously transmitting worst case towards the Earth interfering transmitters would still meet the FS protection criteria, which far exceeds the expected active number of lunar transmitters in this band. Thus, the results of this static worst-case study suggest that sharing between SRS and terrestrial FS operating in the 390-406 MHz band (co-channel) is feasible.

#### 4/1.15/3.2.1.2	Lunar SRS sharing with the mobile service

The results of one sharing study show that with the single entry static worst-case analysis, there is a margin of at least 36.3 dB to satisfy the mobile service protection criteria for Public Protection and Disaster Relief (PPDR) applications.   The result also implies that aggregating over 4 265 simultaneously transmitting worst case towards the Earth interfering transmitters would still meet the MS protection criteria, which far exceeds the expected active number of lunar transmitters in this band. Thus, the results of this static worst-case study suggest that sharing between SRS and terrestrial MS, including PPDR applications, operating in the 390-406 MHz band (co-channel) is feasible.

#### 4/1.15/3.2.1.3	Lunar SRS compatibility with the radioastronomy service

[TBD]

### 4/1.15/3.2.2	Frequency range 420-430 MHz (outside SZM)

Within the frequency range 420-430 MHz, sharing and compatibility studies to ensure protection of FS, MS (except aeronautical mobile), and RLS are specified in Resolution 680 (WRC-23).

The characteristics of the lunar SRS stations in the band 420-430 MHz are provided in Table 3.2-4 of Report ITU-R SA.2553. The corresponding characteristics for the lunar orbiting stations are provided in Section 3.3 of the same Report. Note that under Resolution 680 (WRC-23), the lunar SRS stations (surface and orbit) may only operate in the 420-430 MHz band outside the SZM.

The characteristics of the FS, MS (except aeronautical mobile), and RLS in the band 420-430 MHz are documented in Annex 2 of the working document towards a preliminary draft new Report ITU-R SA.[LUNAR_1.15_STUDIES].

#### 4/1.15/3.2.2.1	Lunar SRS sharing with the fixed service

The results of one sharing study show that with the single entry static worst-case analysis, there is a margin of at least 29.3 dB to satisfy the FS protection criteria.  The result also implies that aggregating over 850 worst case interfering transmitters would still meet the FS protection criteria, which far exceeds the expected number of lunar transmitters in this band. The results of this study suggest that sharing between SRS and ground-based FS operating in the 420-430 MHz band (co-channel) is feasible.

A second sharing study shows that, using the interference-to-noise ratio metric in a single entry static worst-case analysis, the FS protection criteria is met with a margin of 29.3 dB or more. The results of this study suggest that protection of FS from SRS in the 420-430 MHz band is ensured.

#### 4/1.15/3.2.2.2	Lunar SRS sharing with the mobile (except aeronautical mobile) service

The results of one sharing study show that with the single entry static worst-case analysis, there is a margin of at least 35.3 dB to satisfy the mobile service protection criteria for Public Protection and Disaster Relief (PPDR) applications.   The result also implies that aggregating over 3 388 simultaneously transmitting worst case towards the Earth interfering transmitters would still meet the MS protection criteria, which far exceeds the expected active number of lunar transmitters in this band. Thus, the results of this static worst-case study suggest that sharing between SRS and terrestrial MS, including PPDR applications, operating in the 420-430 MHz band (co-channel) is feasible.

#### 4/1.15/3.2.2.3	Lunar SRS sharing with the radiolocation service

The results of one sharing study suggest that protection of radars operating in the radiolocation service in the 420-430 MHz band (co-channel) from SRS interference is ensured. Furthermore, the interference-to-noise ratio metric shows that in the static worst-case analysis for a single interferer, there is a margin of 10.4 dB or more to satisfy the radiolocation service protection criteria.

### 4/1.15/3.2.3	Frequency range 440-450 MHz (outside SZM)

Within the frequency range 440-450 MHz, sharing and compatibility studies to ensure protection of the FS, MS (except aeronautical mobile), and RLS are specified in Resolution 680 (WRC-23).

The characteristics of the lunar surface stations in the band 440-450 MHz are provided in Table 3.2-4 of Report ITU-R SA.2553. The corresponding characteristics for the lunar orbiting stations are provided in Section 3.3 of the same Report. Note that under Resolution 680 (WRC-23), the lunar SRS stations (surface and orbit) may only operate in the 440-450 MHz band outside the SZM.

The characteristics of the FS, MS, and MSS in the band 440-450 MHz are documented in Annex 3 of the working document towards a preliminary draft new Report ITU-R SA.[LUNAR_1.15_STUDIES].

#### 4/1.15/3.2.3.1	Lunar SRS sharing with the fixed service

The results of one sharing study show that with the single entry static worst-case analysis, there is a margin of at least 30.6 dB to satisfy the FS protection criteria. This result also implies that aggregating over 1 100 worst case interfering transmitters would still meet the FS protection criteria, which far exceeds the expected number of lunar transmitters in this band. The study results suggest that sharing between lunar SRS and terrestrial FS operating in the 440-450 MHz band (co-channel) is feasible.

The results of a second sharing study show that using the interference-to-noise ratio metric in the single entry static worst-case analysis, the FS protection criteria is met with a margin of 27.9 dB or more. The results of this study suggest that protection of FS from SRS in the 440-450 MHz band is ensured.

#### 4/1.15/3.2.3.2	Lunar SRS sharing with the mobile (except aeronautical mobile) service

The results of one sharing study show that with the single entry static worst-case analysis, there is a margin of at least 36.3 dB to satisfy the mobile service protection criteria for Public Protection and Disaster Relief (PPDR) applications.   The result also implies that aggregating over 4 265 simultaneously transmitting worst case towards the Earth interfering transmitters would still meet the MS protection criteria, which far exceeds the expected active number of lunar transmitters in this band. Thus, the results of this static worst-case study suggest that sharing between SRS and terrestrial MS, including PPDR applications, operating in the 440-450 MHz band (co-channel) is feasible.

#### 4/1.15/3.2.3.3	Lunar SRS sharing with the radiolocation service

The results of one sharing study show that using the interference-to-noise ratio metric in the static worst-case analysis for a single potential interferer, there is a positive margin of 9.8 dB or more with respect to the radiolocation service protection criteria. The results of this study suggest that protection of radars operating in the radiolocation service in the 440-450 MHz band (co-channel) from SRS lunar surface transmitters interference is ensured.

### 4/1.15/3.2.4	Frequency range 2 400-2 690 MHz

Within the frequency range 2 400-2 690 MHz, sharing and compatibility studies to ensure protection of the FS, MS, MS (identified for IMT), MS (identified for HIBS), RLS, BSS, and RAS are specified in Resolution 680 (WRC-23).  Additional sharing and compatibility studies with MSS and ARNS can be found in Annex 4 of the working document towards a preliminary draft new Report ITU-R SA.[LUNAR_1.15_STUDIES].

The characteristics of the lunar surface stations in the band 2 400-2 690 MHz are provided in Table 3.2-4 of Report ITU-R SA.2553. The corresponding characteristics for the lunar orbiting stations are provided in Section 3.3 of the same Report.

#### 4/1.15/3.2.4.1	Lunar SRS sharing with the fixed service

The results of one sharing study show that using the interference-to-noise ratio metric in the single entry static worst-case analysis, there is a margin of 18.9 dB or more to satisfy the FS protection criteria.

The results of a second sharing study show that using the interference-to-noise ratio metric in the single entry static worst-case analysis, the FS protection criteria is met with a margin of 26.0 dB or more. The results of this study suggest that protection of FS from SRS in the 2 400-2 690 MHz band is ensured.

#### 4/1.15/3.2.4.2	Lunar SRS sharing with the mobile service

[TBD]

#### 4/1.15/3.2.4.3	Lunar SRS sharing with the mobile service operating as a terrestrial component of IMT

The results of one sharing study show that using the interference-to-noise ratio metric in the single entry static worst-case analysis, the IMT base station protection criteria is met with a margin of 21.2 dB or more and the IMT user equipment (UE) protection criteria is met with a margin of 54.4 dB or more. For the aggregate worst-case analysis, the interference margin is 12.2 dB for the IMT base station and 45.4 dB for the IMT UE. The results of this study suggest that protection of terrestrial IMT systems from SRS in the 2 500-2 690 MHz band is ensured.

The results of a second sharing study show that the single entry worst case I/N for the IMT base station and user equipment receivers are -27.3 dB and -64.5 dB, respectively.  For the aggregate worst case interference assuming 30 lunar surface transmitters, the I/N for the IMT base station and user equipment receivers are -12.5 dB and -49.7 dB, respectively.  Considering the IMT protection criterion of −6 dB I/N, the results show a sufficient positive margin across all four scenarios.

#### 4/1.15/3.2.4.4	Lunar SRS sharing with high-altitude platform stations as IMT base stations (HIBS)

The results of one study suggest that sharing between SRS and HIBS in the 2500-2690 MHz band (co-channel), is feasible. The interference-to-noise ratio metric shows that in the single entry static worst-case analysis, there is a margin of 33.4 dB or more to satisfy the HIBS protection criteria. This implies that the aggregate interference from 2187 worst case lunar transmitters would not exceed the HIBS protection criterion in this band.

#### 4/1.15/3.2.4.5	Lunar SRS sharing with the broadcast satellite service

WP 4A has expressed the view that sharing studies to ensure protection of BSS are not required in the 2 500 – 2 690 MHz band, provided that possible future allocations to space research service (space-to-space) in the 2 500 – 2 690 MHz band under WRC-27 agenda item 1.15 are limited to Lunar surface-to-surface communications, as specified in Report ITU-R SA.2553.

#### 4/1.15/3.2.4.6	Lunar SRS sharing with the radiolocation service

[TBD]

#### 4/1.15/3.2.4.7	Lunar SRS compatibility with the radioastronomy service

The results of one study, considering single-entry and aggregated worst-case assumptions, indicate that for a lunar surface transmitter in the 2500–2690 MHz band, operating in the space research service (SRS), compatibility with the radio astronomy service (RAS) in the adjacent 2690–2700 MHz band, operating within the Shielded Zone of the Moon (SZM), is satisfied when there is a minimum separation distance of 65.6 km between the systems.

### 4/1.15/3.2.5	Frequency range 3 500-3 800 MHz

Within the frequency range 3 500-3 800 MHz, sharing and compatibility studies to ensure protection of the FS, MS (except aeronautical mobile), MS (identified for IMT), and FSS (space-to-Earth) are specified in Resolution 680 (WRC-23).  Additional sharing and compatibility studies with RLS can be found in Annex 5 of the working document towards a preliminary draft new Report ITU-R SA.[LUNAR_1.15_STUDIES].

The characteristics of the lunar surface stations in the band 3 500-3 800 MHz are provided in Table 3.2-4 of Report ITU-R SA.2553.

#### 4/1.15/3.2.5.1	Lunar SRS sharing with the fixed service

The results of one sharing study show that using the interference-to-noise ratio metric in the dynamic analysis including single potential interferers, there is a margin of 66 dB or more to the FS long term (20% of the time) protection criteria. The results of this study suggest that protection of ground-based FS from SRS operating in the 3 500-3 800 MHz band (co-channel) is ensured.

The results of a second sharing study show that using the interference-to-noise ratio metric in the single entry static worst-case analysis, the FS protection criteria for 256 QAM, QPSK and 512 QAM fixed systems are met.

#### 4/1.15/3.2.5.2	Lunar SRS sharing with the mobile service (except aeronautical mobile)

[TBD]

#### 4/1.15/3.2.5.3	Lunar SRS sharing with the mobile service operating as a terrestrial component of IMT

The results of one sharing study show that using the interference-to-noise (I/N) ratio metric in the single entry static worst-case analysis, there is a margin of 20.9 dB or more to satisfy the IMT base station protection criteria and 55.3 dB margin for the IMT user equipment. For the aggregate interference analysis, the interference margin is 11.9 dB for the IMT base station and 46.3 dB for the IMT user equipment.

The results of a second sharing study show that the single entry worst case I/N for the IMT base station and user equipment receivers are -27.2 dB and -65.4 dB, respectively.  For the aggregate worst case interference assuming 30 lunar surface transmitters, the I/N for the IMT base station and user equipment receivers are -12.4 dB and -50.6 dB, respectively.  Considering the IMT protection criterion of −6 dB I/N, the results show a sufficient positive margin across all four scenarios.

#### 4/1.15/3.2.5.4	Lunar SRS sharing with the fixed satellite service (space-to-Earth)

The results of one study suggests that sharing between Fixed Satellite Service (FSS) (space-to-Earth) and Space Research Service (SRS) on the lunar surface operating in the 3500–3800 MHz band (co-channel) is feasible. Based on the interference-to-noise ratio (I/N) metric and the dynamic analysis taking into account antenna pointing, the FSS protection criteria were met in all assessed cases, with margins of at least 67.3 dB for the long-term (20% of the time) and 27.6 dB for the short-term (0.005% of the time) protection criteria.

### 4/1.15/3.2.6	Frequency range 5 150-5 570 MHz

Within the frequency range 5 150-5 570 MHz, sharing and compatibility studies to ensure protection of the MS (except aeronautical mobile), RLS, ARNS, AMS, FS, SRS (active), RNS, and MRNS are specified in Resolution 680 (WRC-23).

The characteristics of the lunar surface stations in the band 5 150-5 570 MHz are provided in Table 3.2-4 of Report ITU-R SA.2553.

#### 4/1.15/3.2.6.1	Lunar SRS sharing with the mobile (except aeronautical mobile) service

[TBD]

## 4/1.15/3.2.6.2	Lunar SRS sharing with the radiolocation service

The results of one sharing study suggest that sharing between lunar SRS and Earth-based radars operating in the radiolocation service in the 5 150-5 570 MHz band (co-channel) is feasible. The interference-to-noise ratio metric shows that in the single entry static worst-case analysis, there is a margin of 29 dB or more to satisfy the radiolocation service protection criteria. That implies the aggregate interference from 794 lunar surface transmitters would still meet the RLS protection criteria in this band.

## 4/1.15/3.2.6.3	Lunar SRS sharing with the aeronautical radionavigation service

The results of one sharing study suggest that sharing between lunar SRS and radars operating in the aeronautical radionavigation service in the 5 150-5 570 MHz band (co-channel) is feasible. The interference-to-noise ratio metric in the single entry static worst-case analysis shows a margin of 49.1 dB or more to satisfy the aeronautical radionavigation service protection criteria. This implies that the worst-case aggregate interference from 81283 lunar transmitters would still meet the ARNS protection criteria.

## 4/1.15/3.2.6.4	Lunar SRS sharing with the aeronautical mobile service (AMS)

The results of one sharing study suggest that sharing between SRS and aeronautical mobile service (AMS) operating in the 5 150-5 570 MHz band (co-channel) is feasible. The interference-to-noise ratio metric shows that in the single entry static worst-case analysis, there is a margin of 46.6 dB or more to satisfy the AMS protection criteria. This implies that even the worst-case aggregate interference from 45708 lunar surface transmitters would not exceed the AMS protection criterion in this band.

## 4/1.15/3.2.6.5	Lunar SRS sharing with the fixed service

The results of one sharing study suggest that sharing between SRS and fixed service operating in the 5 150-5 570 MHz band (co-channel) is feasible. The interference-to-noise ratio metric shows that in the single entry static worst-case analysis, there is a margin of 71 dB or more to satisfy the FS protection criteria. This implies that even the worst-case aggregate interference from 12.5 million lunar surface transmitters would not exceed the FS protection criterion in this band.

## 4/1.15/3.2.6.6	Lunar SRS sharing with the space research (active) service

[TBD]

## 4/1.15/3.2.6.7	Lunar SRS sharing with the radionavigation service

The results of one sharing study suggest that sharing between lunar SRS and radars operating in the radionavigation service in the 5 150-5 570 MHz band (co-channel) is feasible. The interference-to-noise ratio metric in the single entry static worst-case analysis shows a margin of 26.2 dB or more to satisfy the radionavigation service protection criteria. This implies that the worst-case aggregate interference from 416 lunar transmitters would still meet the radionavigation service protection criteria.

## 4/1.15/3.2.6.8	Lunar SRS sharing with the maritime radionavigation service

The results of one sharing study suggest that sharing between lunar SRS and radars operating in the maritime radionavigation service in the 5 150-5 570 MHz band (co-channel) is feasible. The interference-to-noise ratio metric in the single entry static worst-case analysis shows a margin of 58.3 dB or more to satisfy the maritime radionavigation service protection criteria. This implies that the worst-case aggregate interference from 676000 lunar transmitters would still meet the MRNS protection criteria.

### 4/1.15/3.2.7	Frequency range 5 570-5 725 MHz

Within the frequency range 5 570-5 725 MHz, sharing and compatibility studies to ensure protection of the RLS, FS, MS, MS (except aeronautical mobile), ARNS, and MRNS are specified in Resolution 680 (WRC-23).

The characteristics of the lunar surface stations in the band 5 570-5 725 MHz are provided in Table 3.2-4 of Report ITU-R SA.2553.

## 4/1.15/3.2.7.1	Lunar SRS sharing with the radiolocation service

The results of one sharing study suggest that sharing between SRS and radars operating in the radiolocation service in the 5 570-5 725 MHz band (co-channel) is feasible. The interference-to-noise ratio metric shows that in the single entry static worst-case analysis, there is a margin of 29.3 dB or more to satisfy the radiolocation service protection criteria. This implies that the worst-case aggregate interference from 851 lunar surface transmitters would still meet the RLS protection criteria in this band.

## 4/1.15/3.2.7.2	Lunar SRS sharing with the fixed service

The results of one sharing study suggest that sharing between SRS and fixed service operating in in the 5 570-5 725 MHz band (co-channel) is feasible. The interference-to-noise ratio metric shows that in the single entry static worst-case analysis, there is a margin of 33.1 dB or more to satisfy the radiolocation service protection criteria. This implies that the worst-case aggregate interference from 2401 lunar surface transmitters would still meet the RLS protection criteria in this band.

## 4/1.15/3.2.7.3	Lunar SRS sharing with the mobile service

[TBD]

## 4/1.15/3.2.7.4	Lunar SRS sharing with the mobile (except aeronautical mobile) service

[TBD]

## 4/1.15/3.2.7.5	Lunar SRS sharing with the aeronautical radionavigation service

The results of one sharing study suggest that sharing between lunar SRS and radars operating in the aeronautical radionavigation service in the 5 570-5 725 MHz band (co-channel) is feasible. The interference-to-noise ratio metric in the single entry static worst-case analysis shows a margin of 42 dB or more to satisfy the aeronautical radionavigation service protection criteria. This implies that the worst-case aggregate interference from 15848 lunar transmitters would still meet the ARNS protection criteria.

## 4/1.15/3.2.7.6	Lunar SRS sharing with the maritime radionavigation service

The results of one sharing study suggest that sharing between lunar SRS and radars operating in the maritime radionavigation service in the 5 570-5 725 MHz band (co-channel) is feasible. The interference-to-noise ratio metric in the single entry static worst-case analysis shows a margin of 40.5 dB or more to satisfy the maritime radionavigation service protection criteria. This implies that the worst-case aggregate interference from 11220 lunar transmitters would still meet the maritime radionavigation service protection criteria.

### 4/1.15/3.2.8	Frequency range 5 775-5 925 MHz

Within the frequency range 5 775-5 925 MHz, sharing and compatibility studies to ensure protection of the RLS, FS, FSS (E-s), and MS are specified in Resolution 680 (WRC-23).

The characteristics of the lunar surface stations in the band 5 775-5 925 MHz are provided in Table 3.2-4 of Report ITU-R SA.2553.

## 4/1.15/3.2.8.1	Lunar SRS sharing with the radiolocation service

The results of one sharing study suggest that sharing between SRS and radars operating in the radiolocation service in the 5 775-5 925 MHz band (co-channel) is feasible. The interference-to-noise ratio metric shows that in the single entry static worst-case analysis, there is a margin of 29.7 dB or more to satisfy the radiolocation service protection criteria. This implies that the worst-case aggregate interference from 933 lunar surface transmitters would still meet the RLS protection criteria in this band.

## 4/1.15/3.2.8.2	Lunar SRS sharing with the fixed service

The results of one sharing study suggest that sharing between SRS and terrestrial FS operating in the 5 775-5 925 MHz band (co-channel) is feasible. The interference-to-noise ratio metric shows that even in the static worst-case analysis, including single and aggregated potential interferers, there is a margin of 31.8 dB or more to satisfy the FS protection criteria. This implies that the worst-case aggregate interference from even 1513 lunar surface transmitters would not exceed the FS protection criterion.

## 4/1.15/3.2.8.3	Lunar SRS sharing with the fixed satellite service (Earth-to-space)

WP 4A has expressed the view that sharing studies to ensure protection of FSS are not required in the 5 775 – 5 925 MHz band, provided that possible future allocations to space research service (space-to-space) in the 5 775 – 5 925 MHz band under WRC-27 agenda item 1.15 are limited to Lunar surface-to-surface communications, as specified in Report ITU-R SA.2553.

## 4/1.15/3.2.8.4	Lunar SRS sharing with the mobile service

[TBD]

### 4/1.15/3.2.9	Frequency range 7 190-7 235 MHz

Within the frequency range 7 190-7 235 MHz, sharing and compatibility studies to ensure protection of the FS, MS, and SRS (E-s) are specified in Resolution 680 (WRC-23).

The characteristics of the lunar orbiting stations in the band 7 190-7 235 MHz are provided in Section 3.4.2 of Report ITU-R SA.2553.

## 4/1.15/3.2.9.1	Lunar SRS sharing with the fixed service

The assessment in one sharing study of the power flux-density at the Earth’s surface produced by emissions from a lunar data relay satellite (LDRS) space station, operating forward inter-orbit links in the 7 190-7 235 MHz band, demonstrate that existing power-flux-density limits in Table 21-4 of the RR, applicable to the frequency band 7 250-7 900 MHz, would be met. It is assumed that no additional conditions would be required to demonstrate compatibility of the SRS (space-to-space) with terrestrial services in the frequency band 7 190-7 235 MHz.

## 4/1.15/3.2.9.2	Lunar SRS sharing with the mobile service

The assessment in one sharing study of the power flux-density at the Earth’s surface produced by emissions from a lunar data relay satellite space station, operating forward inter-orbit links in the 7 190-7 235 MHz band, demonstrate that existing power-flux-density limits in Table 21-4 of the RR, applicable to the frequency band 7 250-7 900 MHz, would be met. It is assumed that no additional conditions would be required to demonstrate compatibility of the SRS (space-to-space) with terrestrial services in the frequency band 7 190-7 235 MHz.

## 4/1.15/3.2.9.3	Lunar SRS sharing with the space research service (Earth-to-space)

[TBD]

### 4/1.15/3.2.10	Frequency range 8 450-8 500 MHz

Within the frequency range 8 450-8 500 MHz, sharing and compatibility studies to ensure protection of the FS, MS (except aeronautical mobile), and SRS (s-E) are specified in Resolution 680 (WRC-23).

The characteristics of the lunar orbiting stations in the band 8 450-8 500 MHz are provided in Section 3.4.2 of Report ITU-R SA.2553.

## 4/1.15/3.2.10.1	Lunar SRS sharing with the fixed service

The results of one sharing study show that using the interference-to-noise ratio metric in the single entry static worst-case analysis, there is a margin 6.9 dB or more to satisfy the FS protection criteria. That also implies that aggregated worst-case interfering transmitters would still meet the FS protection criteria. Thus the results of this static worst-case study suggest that sharing between SRS and terrestrial FS operating in the 8 450-8 500 MHz band (co-channel) is ensured.

The assessment in a second sharing study of the power flux-density at the Earth’s surface produced by emissions from a lunar data relay satellite (LDRS) space station, operating return inter-orbit links in the 8 450 - 8 500 MHz band, demonstrate that existing power-flux-density limits in Table 21-4 of the RR, applicable to the frequency band 8 450-8 500 MHz, would be met. It is assumed that no additional conditions would be required to demonstrate compatibility of the SRS (space-to-space) with terrestrial services in the frequency band 8 450-8 500 MHz.

## 4/1.15/3.2.10.2	Lunar SRS sharing with the mobile (except aeronautical mobile) service

The assessment in one sharing study of the power flux-density at the Earth’s surface produced by emissions from a lunar data relay satellite space station, operating return inter-orbit links in the 8 450-8 500 MHz band, demonstrate that existing power-flux-density limits in Table 21-4 of the RR, applicable to the frequency band 8 450-8 500 MHz, would be met. It is assumed that no additional conditions would be required to demonstrate compatibility of the SRS (space-to-space) with terrestrial services in the frequency band 8 450-8 500 MHz.

## 4/1.15/3.2.10.3	Lunar SRS sharing with the space research service (space-to-Earth)

[TBD]

### 4/1.15/3.2.11	Frequency range 25.25-28.35 GHz

Within the frequency range 25.25-28.35 GHz, sharing and compatibility studies to ensure protection of the FS, FS (identified for HAPS), MS, and ISS are specified in Resolution 680 (WRC-23).

The characteristics of the lunar surface stations in the band 25.25-28.35 GHz are provided in Table 3.2-4 of the Report ITU-R SA.2553.

## 4/1.15/3.2.11.1	Lunar SRS sharing with the fixed service

The results of one sharing study show that using the interference-to-noise ratio metric in the static worst-case analysis, for single-entry interferers, there is a margin of 20.5 dB to satisfy the FS protection criteria.  The results of this study suggest that protection of ground-based FS from SRS operating in the 27.5-28.35 GHz band (co-channel) is ensured.

The results of a second study show that using the interference-to-noise ratio metric in the single entry static worst-case analysis, the FS protection criteria is met with a margin of 18.4 dB or more for all situations. The results of this study suggest that protection of FS from SRS in the 27.5-28.35 GHz band is ensured.

## 4/1.15/3.2.11.2	Lunar SRS sharing with fixed services using high altitude platform stations (HAPS)

The results of one sharing study show that using the interference-to-noise ratio metric in the static worst-case analysis for a single lunar surface interferer, the FS (HAPS) protection criteria is met with a margin of 11.6 dB or more. The results of this study suggest that protection of FS using HAPS ground-based system operating in the 27.5 – 28.35 GHz band (co-channel) from SRS is ensured.

## 4/1.15/3.2.11.3	Lunar SRS sharing with the mobile service

The results of one sharing study show that, using the interference-to-noise ratio metric in the static worst-case analysis for a single lunar potential interferer, the MS protection criteria for the base station is met with a margin of 46 dB or more, and the MS protection criteria for the mobile station is met with a margin of 52 dB or more.  The results of this study suggest that protection of mobile services, including base stations and user equipment in the 27.5-28.35 GHz band (co-channel) from SRS is ensured.

## 4/1.15/3.2.11.4	Lunar SRS sharing with the intersatellite service

[TBD]

# 4/1.15/4	Methods to satisfy the agenda item

[This section should contain the brief description of the Method or Methods to satisfy the agenda item as per section A2.4 of Annex 2 to Resolution ITU-R 2-9]

## 4/1.15/4.1	Issue A: Frequency band 390-406.1 MHz

[Brief text describing Issue A]

### 4/1.15/4.1.1	Method A1: Add SRS (space-to-space) primary allocation

This method proposes to add an allocation to the space research service (space-to-space) on a primary basis in the frequency band 390-406.1 MHz to the Table of Allocations in Article 5 of the Radio Regulations.  A new footnote is appended to the allocation which limits the use of the allocation to the vicinity of the Moon and outside the shielded zone of the Moon, with conditions contained in a draft new WRC-27 resolution.  Resolution 680 (WRC-23) is updated to reflection completion of the lunar SRS (s-s) sharing studies, and to focus on future spectrum needs for lunar communications and systems beyond space research.

## 4/1.15/4.2	Issue B: Frequency band 420-430 MHz

[Brief text describing Issue B]

### 4/1.15/4.2.1	Method B1: Add SRS (space-to-space) primary allocation

This method proposes to add an allocation to the space research service (space-to-space) on a primary basis in the frequency band 420-430 MHz to the Table of Allocations in Article 5 of the Radio Regulations.  A new footnote is appended to the allocation which limits the use of the allocation to the vicinity of the Moon and outside the shielded zone of the Moon, with conditions contained in a draft new WRC-27 resolution.  Resolution 680 (WRC-23) is updated to reflection completion of the lunar SRS (s-s) sharing studies, and to focus on future spectrum needs for lunar communications and systems beyond space research.

## 4/1.15/4.3	Issue C: Frequency band 440-450 MHz

[Brief text describing Issue C]

### 4/1.15/4.3.1	Method C1: Add SRS (space-to-space) primary allocation

This method proposes to add an allocation to the space research service (space-to-space) on a primary basis in the frequency band 440-450 MHz to the Table of Allocations in Article 5 of the Radio Regulations.  A new footnote is appended to the allocation which limits the use of the allocation to the vicinity of the Moon and outside the shielded zone of the Moon, with conditions contained in a draft new WRC-27 resolution.  Resolution 680 (WRC-23) is updated to reflection completion of the lunar SRS (s-s) sharing studies, and to focus on future spectrum needs for lunar communications and systems beyond space research.

## 4/1.15/4.4	Issue D: Frequency band 2 400-2 690 MHz

[Brief text describing Issue D]

### 4/1.15/4.4.1	Method D1: Add SRS (space-to-space) primary allocation

This method proposes to add an allocation to the space research service (space-to-space) on a primary basis in the frequency band 2 400-2 690 MHz to the Table of Allocations in Article 5 of the Radio Regulations.   A new footnote is appended to the allocation which limits the use of the allocation to the vicinity of the Moon, with conditions contained in a draft new WRC-27 resolution.  Resolution 680 (WRC-23) is updated to reflection completion of the lunar SRS (s-s) sharing studies, and to focus on future spectrum needs for lunar communications and systems beyond space research.

## 4/1.15/4.5	Issue E: Frequency band 3 500-3 800 MHz

[Brief text describing Issue E]

### 4/1.15/4.5.1	Method E1: Add SRS (space-to-space) primary allocation

This method proposes to add an allocation to the space research service (space-to-space) on a primary basis in the frequency band 3 500-3 800 MHz to the Table of Allocations in Article 5 of the Radio Regulations.   A new footnote is appended to the allocation which limits the use of the allocation to the vicinity of the Moon, with conditions contained in a draft new WRC-27 resolution. Resolution 680 (WRC-23) is updated to reflection completion of the lunar SRS (s-s) sharing studies, and to focus on future spectrum needs for lunar communications and systems beyond space research.

## 4/1.15/4.6	Issue F: Frequency band 5 150-5 570 MHz

[Brief text describing Issue F]

### 4/1.15/4.6.1	Method F1: Add SRS (space-to-space) primary allocation

This method proposes to add an allocation to the space research service (space-to-space) on a primary basis in the frequency band 5 150-5 570 MHz to the Table of Allocations in Article 5 of the Radio Regulations.   A new footnote is appended to the allocation which limits the use of the allocation to the vicinity of the Moon, with conditions contained in a draft new WRC-27 resolution.  Resolution 680 (WRC-23) is updated to reflection completion of the lunar SRS (s-s) sharing studies, and to focus on future spectrum needs for lunar communications and systems beyond space research.

Footnote 5.447D is modified to indicate that allocation of the 5 250-5 255 MHz band to the space research service on a primary basis is limited to active spaceborne sensors and space-to-space radiocommunications in the vicinity of the Moon; other uses of the 5 250-5 255 MHz band by the space research service are on a secondary basis.

## 4/1.15/4.7	Issue G: Frequency band 5 570-5 725 MHz

[Brief text describing Issue G]

### 4/1.15/4.7.1	Method G1: Add SRS (space-to-space) primary allocation

This method proposes to add an allocation to the space research service (space-to-space) on a primary basis in the frequency band 5 570-5 725 MHz to the Table of Allocations in Article 5 of the Radio Regulations.  A new footnote is appended to the allocation which limits use of the allocation to the vicinity of the Moon, with conditions contained in a draft new WRC-27 resolution. Resolution 680 (WRC-23) is updated to reflection completion of the lunar SRS (s-s) sharing studies, and to focus on future spectrum needs for lunar communications and systems beyond space research.

## 4/1.15/4.8	Issue H: Frequency band 5 775-5 925 MHz

[Brief text describing Issue H]

### 4/1.15/4.8.1	Method H1: Add SRS (space-to-space) primary allocation

This method proposes to add an allocation to the space research service (space-to-space) on a primary basis in the frequency band 5 775-5 925 MHz to the Table of Allocations in Article 5 of the Radio Regulations.  A new footnote is appended to the allocation which limits use of the allocation to the vicinity of the Moon, with conditions contained in a draft new WRC-27 resolution. Resolution 680 (WRC-23) is updated to reflection completion of the lunar SRS (s-s) sharing studies, and to focus on future spectrum needs for lunar communications and systems beyond space research.

## 4/1.15/4.9	Issue I: Frequency band 7 190-7 235 MHz

[Brief text describing Issue I]

### 4/1.15/4.9.1	Method I1: [title of Method I1, if any]

[Text describing the first method to satisfy Issue I]

### 4/1.15/4.9.2	Method I2: [title of Method I2, if any]

[Text describing the second method to satisfy Issue I]

[Additional sections with text describing other methods to satisfy Issue I, if any]

## 4/1.15/4.10		Issue J: Frequency band 8 450-8 500 MHz

[Brief text describing Issue J]

### 4/1.15/4.10.1	Method J1: [title of Method J1, if any]

[Text describing the first method to satisfy Issue J]

### 4/1.15/4.10.2	Method J2: [title of Method J2, if any]

[Text describing the second method to satisfy Issue J]

[Additional sections with text describing other methods to satisfy Issue J, if any]

### 4/1.15/4.11	Issue K: Frequency band 25.25-28.35 GHz

[Brief text describing Issue K]

### 4/1.15/4.11.1	Method K1: Add SRS (space-to-space) primary allocation

This method proposes to add an allocation to the space research service (space-to-space) on a primary basis in the frequency band 27.5-28.35 GHz to the Table of Allocations in Article 5 of the Radio Regulations.  A new footnote is appended to the allocation which limits use of the allocation to the vicinity of the Moon, with conditions contained in a draft new WRC-27 resolution. Resolution 680 (WRC-23) is updated to reflection completion of the lunar SRS (s-s) sharing studies, and to focus on future spectrum needs for lunar communications and systems beyond space research.

Space research applications can utilize the existing Inter-Satellite Service allocation in the 25.25 – 27.5 GHz band per RR No. 5.536.

4/1.15/5	Regulatory and procedural considerations

Editor’s Note:  Changes below to modify the table of frequency allocations and add footnotes are all new.  Track changes are shown on existing text of the Radio Regulations.

## 4/1.15/5.1	For Issues A, B, and C: Frequency bands 390-406.1 MHz, 420-430 MHz, and 440-450 MHz

### 4/1.15/5.1.1	For Methods A1, B1, and C1

ARTICLE 5

Frequency allocations

Section IV – Table of Frequency Allocations
(See No. 2.1)

MOD

335.4-410 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| … |  |  |
| 390-399.9 FIXED MOBILE SPACE RESEARCH (space-to-space) 5.A115-1A 5.254 |  |  |
| 399.9-400.05 MOBILE-SATELLITE (Earth-to-space) 5.209 5.220 5.260A 5.260B SPACE RESEARCH (space-to-space) 5.A115-1A |  |  |
| 400.05-400.15 STANDARD FREQUENCY AND TIME SIGNAL- SATELLITE (400.1 MHz) SPACE RESEARCH (space-to-space) 5.A115-1A 5.261 5.262 |  |  |
| 400.15-401 METEOROLOGICAL AIDS METEOROLOGICAL-SATELLITE (space-to-Earth) MOBILE-SATELLITE (space-to-Earth) 5.208A 5.208B 5.209 SPACE RESEARCH (space-to-Earth) (space-to-space) 5.263 5.A115-1A Space operation (space-to-Earth) 5.262 5.264 |  |  |
| 401-402 METEOROLOGICAL AIDS SPACE OPERATION (space-to-Earth) EARTH EXPLORATION-SATELLITE (Earth-to-space) METEOROLOGICAL-SATELLITE (Earth-to-space) SPACE RESEARCH (space-to-space) 5.A115-1A Fixed Mobile except aeronautical mobile 5.264A 5.264B |  |  |
| 402-403 METEOROLOGICAL AIDS EARTH EXPLORATION-SATELLITE (Earth-to-space) METEOROLOGICAL-SATELLITE (Earth-to-space) SPACE RESEARCH (space-to-space) 5.A115-1A Fixed Mobile except aeronautical mobile 5.264A 5.264B |  |  |
| 403-406 METEOROLOGICAL AIDS SPACE RESEARCH (space-to-space) 5.A115-1A Fixed Mobile except aeronautical mobile 5.265 |  |  |
| 406-406.1 MOBILE-SATELLITE (Earth-to-space) SPACE RESEARCH (space-to-space) 5.A115-1A 5.265 5.266 5.267 |  |  |
| … |  |  |

410-460 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| … |  |  |
| 420-430 FIXED MOBILE except aeronautical mobile SPACE RESEARCH (space-to-space) 5.A115-1A Radiolocation 5.269 5.270 5.271 |  |  |
| … |  |  |
| 440-450 FIXED MOBILE except aeronautical mobile SPACE RESEARCH (space-to-space) 5.A115-1A Radiolocation 5.269 5.285 5.270 5.271 5.284 5.286 |  |  |
| … |  |  |

ADD

5.A115-1A		The use of the frequency bands 390-406.1 MHz, 420-430 MHz, and 440-450 MHz by stations in the space research service (space-to-space) is limited to the vicinity of the Moon outside the shielded zone of the Moon and shall be in accordance with Resolution aaa (WRC-27).      (WRC-27)

## 4/1.15/5.2	For Issues D, E, F, G, H, and K: Frequency bands 2 400-2 690 MHz, 3 500-3 800 MHz, 5 150-5 570 MHz, 5 570-5 725 MHz, 5 775-5 925 MHz, and 25.25-28.35 GHz

### 4/1.15/5.2.1	For Methods D1, E1, F1, G1, H1, and K1

ARTICLE 5

Frequency allocations

Section IV – Table of Frequency Allocations
(See No. 2.1)

MOD

2 170-2 520 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| … |  |  |
| 2 300-2 400 FIXED MOBILE 5.384A Amateur Radiolocation | 2 300-2 400 FIXED MOBILE 5.384A RADIOLOCATION Amateur |  |
| 5.150 5.282 5.395 | 5.150 5.282 5.393 5.394 |  |
| 2 400-2 450 FIXED MOBILE 5.384A SPACE RESEARCH (space-to-space) 5.A115-1B Amateur Radiolocation 5.150 5.282 5.395 | 2 400-2 450 FIXED MOBILE 5.384A RADIOLOCATION SPACE RESEARCH (space-to-space) 5.A115-1B Amateur 5.150 5.282 5.393 5.394 |  |
| 2 450-2 483.5 FIXED MOBILE SPACE RESEARCH (space-to-space) 5.A115-1B Radiolocation 5.150 | 2 450-2 483.5 FIXED MOBILE RADIOLOCATION SPACE RESEARCH (space-to-space) 5.A115-1B 5.150 |  |
| 2 483.5-2 500 FIXED MOBILE MOBILE-SATELLITE (space-to-Earth) 5.351A RADIODETERMINATION- SATELLITE (space-to-Earth) 5.398 SPACE RESEARCH (space-to-space) 5.A115-1B Radiolocation 5.398A 5.150 5.368 5.372A 5.399 5.401 5.402 | 2 483.5-2 500 FIXED MOBILE MOBILE-SATELLITE (space-to-Earth) 5.351A RADIOLOCATION RADIODETERMINATION- SATELLITE (space-to-Earth) 5.398 SPACE RESEARCH (space-to-space) 5.A115-1B 5.150 5.368 5.372A 5.402 | 2 483.5-2 500 FIXED MOBILE MOBILE-SATELLITE (space-to-Earth) 5.351A RADIOLOCATION RADIODETERMINATION- SATELLITE (space-to-Earth) 5.398 SPACE RESEARCH (space-to-space) 5.A115-1B 5.150 5.368 5.372A 5.401 5.402 |
| 2 500-2 520 FIXED 5.410 MOBILE except aeronautical mobile 5.384A 5.409A SPACE RESEARCH (space-to-space) 5.A115-1B 5.412 | 2 500-2 520 FIXED 5.410 FIXED-SATELLITE (space-to-Earth) 5.415 MOBILE except aeronautical mobile 5.384A 5.409A SPACE RESEARCH (space-to-space) 5.A115-1B | 2 500-2 520 FIXED 5.410 FIXED-SATELLITE (space-to-Earth) 5.415 MOBILE except aeronautical mobile 5.384A 5.409A MOBILE-SATELLITE (space-to-Earth) 5.351A 5.407 5.414 5.414A SPACE RESEARCH (space-to-space) 5.A115-1B 5.404 |

2 520-2 700 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 2 520-2 655 FIXED 5.410 MOBILE except aeronautical mobile 5.384A 5.409A BROADCASTING-SATELLITE 5.413 5.416 SPACE RESEARCH (space-to-space) 5.A115-1B 5.339 5.412 5.418B 5.418C | 2 520-2 655 FIXED 5.410 FIXED-SATELLITE (space-to-Earth) 5.415 MOBILE except aeronautical mobile 5.384A 5.409A BROADCASTING-SATELLITE 5.413 5.416 SPACE RESEARCH (space-to-space) 5.A115-1B 5.339 5.418B 5.418C | 2 520-2 535 FIXED 5.410 FIXED-SATELLITE (space-to-Earth) 5.415 MOBILE except aeronautical mobile 5.384A 5.409A BROADCASTING-SATELLITE 5.413 5.416 SPACE RESEARCH (space-to-space) 5.A115-1B 5.403 5.414A 5.415A |
|  |  | 2 535-2 655 FIXED 5.410 MOBILE except aeronautical mobile 5.384A 5.409A BROADCASTING-SATELLITE 5.413 5.416 SPACE RESEARCH (space-to-space) 5.A115-1B |
|  |  | 5.339 5.418 5.418A 5.418B 5.418C |
| 2 655-2 670 FIXED 5.410 MOBILE except aeronautical mobile 5.384A 5.409A BROADCASTING-SATELLITE 5.208B 5.413 5.416 SPACE RESEARCH (space-to-space) 5.A115-1B Earth exploration-satellite (passive) Radio astronomy Space research (passive) 5.149 5.412 | 2 655-2 670 FIXED 5.410 FIXED-SATELLITE (Earth-to-space) (space-to-Earth) 5.415 MOBILE except aeronautical mobile 5.384A 5.409A BROADCASTING-SATELLITE 5.413 5.416 SPACE RESEARCH (space-to-space) 5.A115-1B Earth exploration-satellite (passive) Radio astronomy Space research (passive) 5.149 5.208B | 2 655-2 670 FIXED 5.410 FIXED-SATELLITE (Earth-to-space) 5.415 MOBILE except aeronautical mobile 5.384A BROADCASTING-SATELLITE 5.208B 5.413 5.416 SPACE RESEARCH (space-to-space) 5.A115-1B Earth exploration-satellite (passive) Radio astronomy Space research (passive) 5.149 5.420 |
| 2 670-2 690 FIXED 5.410 MOBILE except aeronautical mobile 5.384A 5.409A SPACE RESEARCH (space-to-space) 5.A115-1B Earth exploration-satellite (passive) Radio astronomy Space research (passive) | 2 670-2 690 FIXED 5.410 FIXED-SATELLITE (Earth-to-space) (space-to-Earth) 5.208B 5.415 MOBILE except aeronautical mobile 5.384A 5.409A SPACE RESEARCH (space-to-space) 5.A115-1B Earth exploration-satellite (passive) Radio astronomy Space research (passive) | 2 670-2 690 FIXED 5.410 FIXED-SATELLITE (Earth-to-space) 5.415 MOBILE except aeronautical mobile 5.384A MOBILE-SATELLITE (Earth-to-space) 5.351A 5.419 SPACE RESEARCH (space-to-space) 5.A115-1B Earth exploration-satellite (passive) Radio astronomy Space research (passive) |
| 5.149 5.412 | 5.149 | 5.149 |
| … |  |  |

2 700-3 600 MHz

| Allocation to services |  |  |  |  |
| --- | --- | --- | --- | --- |
| Region 1 | Region 2 |  | Region 3 |  |
| … |  |  |  |  |
| 3 400-3 500 FIXED FIXED-SATELLITE (space-to-Earth) MOBILE except aeronautical mobile 5.430A Radiolocation 5.431 |  | 3 400-3 500 FIXED FIXED-SATELLITE (space-to-Earth) MOBILE except aeronautical mobile 5.431A 5.431B Amateur Radiolocation 5.433 5.282 |  | 3 400-3 500 FIXED FIXED-SATELLITE (space-to-Earth) Amateur Mobile 5.432 5.432B Radiolocation 5.433 5.282 5.432A |  |
| 3 500-3 600 FIXED FIXED-SATELLITE (space-to-Earth) MOBILE except aeronautical mobile 5.430A SPACE RESEARCH (space-to-space) 5.A115-1B Radiolocation |  | 3 500-3 600 FIXED FIXED-SATELLITE (space-to-Earth) MOBILE except aeronautical mobile 5.431B SPACE RESEARCH (space-to-space) 5.A115-1B Radiolocation 5.433 |  | 3 500-3 600 FIXED FIXED-SATELLITE (space-to-Earth) MOBILE except aeronautical mobile 5.433A SPACE RESEARCH (space-to-space) 5.A115-1B Radiolocation 5.433 |  |

3 600-4 800 MHz

| Allocation to services |  |  |  |
| --- | --- | --- | --- |
| Region 1 | Region 2 | Region 3 |  |
| 3 600-3 800 FIXED FIXED-SATELLITE (space-to-Earth) MOBILE except aeronautical mobile 5.433B 5.434A 5.434B 5.435A SPACE RESEARCH (space-to-space) 5.A115-1B | 3 600-3 700 FIXED FIXED-SATELLITE (space-to-Earth) MOBILE except aeronautical mobile 5.434 SPACE RESEARCH (space-to-space) 5.A115-1B Radiolocation 5.433 | 3 600-3 700 FIXED FIXED-SATELLITE (space-to-Earth) MOBILE except aeronautical mobile SPACE RESEARCH (space-to-space) 5.A115-1B Radiolocation 5.435 |  |
|  | 3 700-3 800 FIXED FIXED-SATELLITE (space-to-Earth) MOBILE except aeronautical mobile 5.435B SPACE RESEARCH (space-to-space) 5.A115-1B |  |  |
| 3 800-4 200 FIXED FIXED-SATELLITE (space-to-Earth) Mobile | 3 800-4 200 FIXED FIXED-SATELLITE (space-to-Earth) MOBILE except aeronautical mobile 5.435B |  |
| ... |  |  |  |

4 800-5 250 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| … |  |  |
| 5 150-5 250 FIXED-SATELLITE (Earth-to-space) 5.447A MOBILE except aeronautical mobile 5.446A 5.446B AERONAUTICAL RADIONAVIGATION SPACE RESEARCH (space-to-space) 5.A115-1B 5.446 5.446C 5.446D 5.447 5.447B 5.447C |  |  |

5 250-5 570 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 5 250-5 255 EARTH EXPLORATION-SATELLITE (active) MOBILE except aeronautical mobile 5.446A 5.447F RADIOLOCATION SPACE RESEARCH MOD 5.447D 5.A115-1B 5.447E 5.448 5.448A |  |  |
| 5 255-5 350 EARTH EXPLORATION-SATELLITE (active) MOBILE except aeronautical mobile 5.446A 5.447F RADIOLOCATION SPACE RESEARCH (active) SPACE RESEARCH (space-to-space) 5.A115-1B 5.447E 5.448 5.448A |  |  |
| 5 350-5 460 EARTH EXPLORATION-SATELLITE (active) 5.448B RADIOLOCATION 5.448D AERONAUTICAL RADIONAVIGATION 5.449 SPACE RESEARCH (active) 5.448C SPACE RESEARCH (space-to-space) 5.A115-1B |  |  |
| 5 460-5 470 EARTH EXPLORATION-SATELLITE (active) RADIOLOCATION 5.448D RADIONAVIGATION 5.449 SPACE RESEARCH (active) SPACE RESEARCH (space-to-space) 5.A115-1B 5.448B |  |  |
| 5 470-5 570 EARTH EXPLORATION-SATELLITE (active) MOBILE except aeronautical mobile 5.446A 5.450A RADIOLOCATION 5.450B MARITIME RADIONAVIGATION SPACE RESEARCH (active) SPACE RESEARCH (space-to-space) 5.A115-1B 5.448B 5.450 5.451 |  |  |

5 570-6 700 MHz

| Allocation to services |  |  |  |
| --- | --- | --- | --- |
| Region 1 | Region 2 | Region 3 |  |
| 5 570-5 650 MOBILE except aeronautical mobile 5.446A 5.450A RADIOLOCATION 5.450B MARITIME RADIONAVIGATION SPACE RESEARCH (space-to-space) 5.A115-1B 5.450 5.451 5.452 |  |  |  |
| 5 650-5 725 MOBILE except aeronautical mobile 5.446A 5.450A RADIOLOCATION SPACE RESEARCH (space-to-space) 5.A115-1B Amateur Space research (deep space) 5.454 5.282 5.451 5.453 5.455 |  |  |  |
| 5 725-5 775 FIXED-SATELLITE (Earth-to-space) RADIOLOCATION Amateur | 5 725-5 775 RADIOLOCATION Amateur |  |  |
| 5.150 5.451 5.453 5.455 | 5.150 5.453 5.455 |  |  |
| 5 775-5 830 FIXED-SATELLITE (Earth-to-space) RADIOLOCATION SPACE RESEARCH (space-to-space) 5.A115-1B Amateur | 5 775-5 830 RADIOLOCATION SPACE RESEARCH (space-to-space) 5.A115-1B Amateur |  |  |
| 5.150 5.451 5.453 5.455 | 5.150 5.453 5.455 |  |  |
| 5 830-5 850 FIXED-SATELLITE (Earth-to-space) RADIOLOCATION SPACE RESEARCH (space-to-space) 5.A115-1B Amateur Amateur-satellite (space-to-Earth) | 5 830-5 850 RADIOLOCATION SPACE RESEARCH (space-to-space) 5.A115-1B Amateur Amateur-satellite (space-to-Earth) |  |  |
| 5.150 5.451 5.453 5.455 | 5.150 5.453 5.455 |  |  |
| 5 850-5 925 FIXED FIXED-SATELLITE (Earth-to-space) MOBILE SPACE RESEARCH (space-to-space) 5.A115-1B | 5 850-5 925 FIXED FIXED-SATELLITE (Earth-to-space) MOBILE SPACE RESEARCH (space-to-space) 5.A115-1B Amateur Radiolocation | 5 850-5 925 FIXED FIXED-SATELLITE (Earth-to-space) SPACE RESEARCH (space-to-space) 5.A115-1B MOBILE Radiolocation |  |
| 5.150 | 5.150 | 5.150 |  |
| … |  |  |

24.75-29.9 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| ... |  |  |
| 27.5-28.35 FIXED 5.537A FIXED-SATELLITE (Earth-to-space) 5.484A 5.516B 5.517A 5.517B 5.539 INTER-SATELLITE 5.521A MOBILE SPACE RESEARCH (space-to-space) 5.A115-1B 5.538 5.540 |  |  |
| 28.35-5-28.5 FIXED 5.537A FIXED-SATELLITE (Earth-to-space) 5.484A 5.516B 5.517A 5.517B 5.539 INTER-SATELLITE 5.521A MOBILE 5.538 5.540 |  |  |
| … |  |  |

MOD

5.447D	The allocation of the band 5 250-5 255 MHz to the space research service on a primary basis is limited to active spaceborne sensors, and for space-to-space radiocommunications in accordance with No. 5.A115-1B. Other uses of the band by the space research service are on a secondary basis.     (WRC-27)

ADD

5.A115-1B		The use of the frequency bands 2 400 – 2 690 MHz, 3 500 – 3 800 MHz,  5 150 – 5 570 MHz, 5 570 – 5 725 MHz, 5 775 – 5 925 MHz, and 27.5 – 28.35 GHz by stations in the space research service (space-to-space) is limited to the vicinity of the Moon and shall be in accordance with Resolution [USA-XXXaaa (WRC-27)].      (WRC-27)

## 4/1.15/5.3	For Issue I: Frequency band 7 190-7 235 MHz

### 4/1.15/5.3.1	For Method I1: [title of Method I1]

[Example(s) of regulatory text for the first method to satisfy Issue I]

### 4/1.15/5.3.2	For Method I2: [title of Method I2]

[Example(s) of regulatory text for the second method to satisfy Issue I]

[Additional sections with example(s) of regulatory text for the other methods to satisfy Issue I, if any]

## 4/1.15/5.4		For Issue J: Frequency band 8 450-8 500 MHz

### 4/1.15/5.4.1	For Method J1: [title of Method J1]

[Example(s) of regulatory text for the first method to satisfy Issue J]

### 4/1.15/5.4.2	For Method J2: [title of Method J2]

[Example(s) of regulatory text for the second method to satisfy Issue J]

[Additional sections with example(s) of regulatory text for the other methods to satisfy Issue J, if any]

ADD DRAFT NEW RESOLUTION [USA-XXX (WRC-27)]

Reasons:  This new resolution implements the decision of WRC-27 in the making of new SRS (s-s) allocations in the bands identified in the title of this resolution.

DRAFT NEW RESOLUTION [USA-XXX (WRC-27)]

Use of the bands 390-406.1 MHz, 420-430 MHz, 440-450 MHz, 2 400-2 690 MHz, 3 500-3 800 MHz, 5 150-5 570 MHz, 5 570-5 725 MHz, 5 775-5 925 MHz, and 27.5-28.35 GHz by stations in the space research service (space-to-space) in the vicinity of the Moon

The World Radiocommunication Conference (Shanghai, 2027)

considering

- that allocation in frequency bands 390-406.1 MHz, 420-430 MHz, 440-450 MHz, 2 400-2 690 MHz, 3 500-3 800 MHz, 5 150-5 570 MHz, 5 570-5 725 MHz, 5 775-5 925 MHz, and 27.5-28.35 GHz to the space research service (space-to-space) on a primary basis is intended to support communications on the lunar surface and between lunar orbit and the lunar surface;

- that Report ITU-R SA.2553 contains the technical and operational characteristics for space research systems in the vicinity of the Moon;

- that results of ITU Radiocommunication Sector studies indicate that sharing of these frequency bands with terrestrial and active space services in the vicinity of the Earth is feasible given the Earth-Moon distances involved;

recognizing

a)	that the shielded zone of the Moon comprises the area of the Moon’s surface and an adjacent volume of space which are shielded from emissions originating within a distance of 100 000 km from the centre of the Earth (see No. 22.22.1);

- that No. 22.25 contains the provision for protection of radio astronomy observations and other passive services in the SZM by agreement between concerned administrations in frequency bands in which emissions are not prohibited by No. 22.22-22.24;

resolves

- that the use of the frequency bands 390-406.1 MHz, 420-430 MHz, and 440-450 MHz by stations in the space research service (space-to-space) be limited to space stations outside the SZM;

- that the use of frequency bands 390-406 MHz and 440-450 MHz by stations in the space research service (space-to-space) are limited to transmissions between a space station in orbit of the Moon and a space station on or near the surface of the Moon;

- that the use of the frequency bands 406-406.1 MHz by stations in the space research service (space-to-space) are limited to transmissions from a space station operating in the vicinity of the Moon for distress and emergency purposes;

- that the use of the frequency bands 420-430 MHz, 2 400-2 483.5 MHz, 2 500-2 690 MHz, 3 500-3 800 MHz, 5 150-5 570 MHz, 5 570-5 725 MHz, 5 775-5 925 MHz, and 27.5-28.35 GHz by stations in the space research service (space-to-space) are limited to transmissions between space stations separated by less than 20 km in the vicinity of the Moon.

5	that the use of the frequency bands 2 483.5-2 500 MHz by stations in the space research service (space-to-space) are limited to transmissions from a space station in orbit around the Moon to enable positioning, navigation and timing accuracy;

instructs the Director of the Radiocommunication Bureau

to take all necessary measures to implement this Resolution.

MOD Resolution 680 (WRC-23)

Reasons:	Completing the mandate and international agreement in RES 680 (WRC-23) invites the ITU Radiocommunication Sector 1 and 2 in the 2028-2031 study cycle to enable lunar activities beyond space science for the benefit of all mankind.  The modification focuses on future spectrum needs for lunar communications and systems beyond space research.

RESOLUTION 680 (WRC-23)

Future spectrum needs for lunar communications and systems beyond space research

The World Radiocommunication Conference (Shanghai, 2027),

considering

a)	that lunar scientific and exploration activities can advance the development of potential future space activities beyond space research, which may in the future include other relevant radiocommunication services for lunar communications;

b)	that space communication infrastructure and technologies for human presence and lunar outposts will mature through near-term scientific discovery and space exploration activities under the WRC-27 lunar vicinity SRS (s-s) allocations;

- that the scientific lunar-based operational experiences enable necessary improvements including enhancing sustainability, cost effectiveness, human wellness for future extension for activities beyond space research;

- that continued search for new knowledge beyond the Earth is an innate nature of humanity;

noting

a)	that Section V of Article 22 addresses protection of radio astronomy and passive services in the SZM;

b)	that Recommendation ITU-R RA.479-5 relates to the protection of frequencies for radioastronomical measurements in the SZM, with a view to preserving the unique radioastronomical capabilities in this zone;

c)	that the impact of unintended electromagnetic radiation from electrical and electronic systems into radio astronomy receivers should be assessed (see Question ITU-R 243/1);

recognizing

a)	that studies on future spectrum needs for lunar communications and systems, beyond those identified in Resolution 680 (WRC-23) resolves to invite the ITU Radiocommunication Sector to complete in time for the 2027 world radiocommunication conference 1, were initated in the 2023-2027 study cycle within WP 7B;

b)	that frequencies for communications between the Earth and the Moon are provided through the existing allocations to the SRS;

c)	that frequencies for communications between satellites orbiting the Moon can operate in existing frequency allocations to the SRS (space-to-space) and the inter-satellite service;

d)	that existing allocations to the amateur radio service have also been used for communications between the Earth and the Moon, and Earth-to-Earth via passive reflection from the Moon;

e)	that dedicated frequencies in the lunar vicinity for local communications between systems operating on the lunar surface and between systems in lunar orbit and systems on the lunar surface in the allocations made by the WRC-27 to the SRS (space-to-space) are available for near-term lunar vicinity exploration activities;

--

-

resolves to invite the ITU Radiocommunication Sector

--

1	to study spectrum needs for lunar communications and systems, taking into account considering a) and based on reference scenarios in Preliminary Draft New Report ITU-R SA.[FUTURE LUNAR COMMUNICATION AND SYSTEMS];

2	to study whether spectrum needs from resolves to invite the ITU Radiocommunication Sector 1 can be accommodated within existing space radiocommunication services and whether the regulatory provisions described in the Radio Regulations are sufficient,

3	to prepare one or more reports containing the results of studies mentioned in resolves 1 and 2 above,

invites administrations

to participate in the studies by submitting contributions to the ITU Radiocommunication Sector,

instructs the Director of the Radiocommunication Bureau

to report to WRC-31 on the progress of this resolution.

-