| U.S. Radiocommunication Sector FACT SHEET |  |  |  |
| --- | --- | --- | --- |
| Study Group: WP7B |  | Document No: US7B_27_040_NC |  |
| Reference: Annex 2 to the September 2025 WP7B Chairman’s Report Doc 7B/192, Resolution 680 (WRC-23), WRC-27 Agenda Item 1.15 |  | Date: February 6, 2026 |  |
| Document Title: Working document towards PDN Report ITU-R SA.[LUNAR_1.15_STUDIES] |  |  |  |
| Authors | Telephone |  | E-Mail |
| Dennis Lee, NASA/JPL Cathy Sham, NASA/JSC Scott Kotler, Lockheed Martin Luciano Alexandre | 818-354-6908 281-222-1117 703-789-3923 818-393-0663 |  | dennis.k.lee@jpl.nasa.gov catherine.c.sham@nasa.gov scott.kotler@LMCO.com luciano.alexandre@jpl.nasa.gov |
| Purpose/Objective: The purpose of this contribution is to further progress the working document in WP7B with sharing and compatibility studies per Res. 680 (WRC-23) resolves to invite… in time for 2027 World Radio Conference 1, 2 and 4 |  |  |  |
| Abstract: Under Agenda item 1.15 (WRC-27), possible new or modified space research service (space-to-space) allocations, in the frequency ranges identified in Resolution 680 (WRC-23) resolves to invite 1, are under study to ensure sharing and compatibility between SRS links on the lunar surface/between lunar orbit and lunar surface and incumbent services in Resolution 680 (WRC-23) recognizings g) to n) and RAS in adjacent and nearby bands. At the September 2025 WP 7B meeting, updates to the AI 1.15 sharing and compatibility studies with terrestrial services in the frequency ranges identified by RES 680 (WRC-23) were introduced. This contribution will provide additional studies with the goal of completing work on the working document towards a Preliminary Draft New Report SA.[LUNAR 1.15 STUDIES] at the March 2026 WP 7B meeting. |  |  |  |
| Fact Sheet Preparer: Dennis Lee, NASA/JPL |  |  |  |

| Radiocommunication Study Groups |  |
| --- | --- |
| Source: Document 7B/TEMP/78 Reference: Resolution 680 (WRC-23) Subject: WRC-27 agenda item 1.15 | Annex 1 to Document 7B/192-E |
|  | 25 September 2025 |
|  | English only |
| Annex 1 to Working Party 7B Chair’s Report |  |
| Working document towards a preliminary draft new Report ITU-R SA.[LUNAR 1.15 STUDIES] |  |
| Sharing studies of space research systems for lunar operations under WRC-27 agenda item 1.15. |

(202X)

Scope

Keywords

Glossary / Abbreviations

Related ITU Recommendations and Reports

# 1	Introduction

This document provides the sharing studies and spectrum needs related to systems in the space research service planned for operations on the lunar surface, or systems in lunar orbit communicating with systems on the lunar surface, in the following frequency ranges or portions thereof:

–	390-406.1 MHz, 420-430 MHz and 440-450 MHz, limited to outside the SZM;

–	2 400-2 690 MHz, 3 500-3 800 MHz, 5 150-5 570 MHz, 5 570-5 725 MHz, 5 775-5 925 MHz, 7 190-7 235 MHz, 8 450-8 500 MHz and 25.25-28.35 GHz.

And taking into account protection of incumbent services in Resolution 680 (WRC-23) recognizing g) to n) and RAS on the Earth and in the shielded zone of the Moon (SZM) in the same, adjacent or nearby bands.

The objective of this document is to respond to resolves to invite the ITU Radiocommunication Sector to complete in time for the 2027 world radiocommunication conference 1) and 4) of Resolution 680 (WRC-23) under WRC-27 agenda item 1.15.

# 2	Spectrum needs

Figure 2.1 shows a conceptual diagram of the envisioned elements [1] used for space research operations on the lunar surface, and the associated communication network architecture in the vicinity of the Moon. The communication links between the lunar elements will be used to support scientific discovery and human/robotic exploration of the Moon, and may leverage technology and frequency ranges originally developed for wireless communication networks on Earth. A detailed description of the technical and operating characteristics of systems in the space research service planned for operations on the lunar surface, or systems in lunar orbit communicating with systems on the lunar surface, are provided in Report ITU-R SA.2553. This section provides a description of the amount of spectrum needed within each frequency range needed to support the envisioned concept of operations as described in the aforementioned Report and summarized below.

Figure 2.1

Envisioned Lunar Surface Communication Architecture

In this lunar communication network architecture, it is assumed that the initial human exploration activities will be centred around the lunar landers, and eventually around permanent habitation modules. The lunar landers will be considered as a stationary surface platform for the duration of the landed mission phase, which may last up to several months. There will likely be multiple landers present on the lunar surface at any one time, some which may be crewed and others which are for robotic exploration only. There may also be multiple habitation modules, each with their own local area communication networks and high-rate point-to-point links between habitation modules.

The lander and the habitation module are expected to provide a wireless local area network (WLAN) service for crew and rovers in the near vicinity of the lander (< 300 meters). The data rate for each device is expected to be between 1-30 Mbps including file transfers and high-definition video, with up to 15-20 connected devices per WLAN. The frequency ranges for this application are expected to in the 2 400-2 483.5 MHz and 5 150-5 725 MHz bands using Wi-Fi technology. In addition, lunar terrain vehicles and pressurized rovers may also provide WLAN coverage in their immediate vicinity for crewed exploration of the Moon away from the lander/habitation modules.

For longer range connectivity around the lander and habitation modules, it is expected that network coverage will be provided by technology similar to 3GPP developed for terrestrial networks. The data rates per lunar device is expected to between 3-12 Mbps for voice, data, and video. Initial deployments on the Moon will likely involve a base station installed on the lunar lander supporting EVA communications for up to 6 lunar elements (crew and/or rovers) in a 2 km radius around the lander. This can be supported using a FDD network in the 2 500-2 690 MHz band, assuming a 50 MHz guard band between the FDD uplink and downlink bands. For crew contingency walk-back scenarios where coverage is limited or the range is large (up to 10 km), the data rate using this network may limited to 100 kbps.

Eventual deployment around the habitation module will involve multiple base stations for a larger coverage area and more lunar elements (up to approximately 30-50 devices). These networks may utilize the 3 500-3 800 MHz frequency range in order to provide higher data rates (up to 100 Mbps per device) and increased network capacity. The lunar terrain vehicle and pressurized rover may also carry a base station to provide connectivity during long range crewed exploration missions away from the lander/habitation module.

During crewed exploration missions, short range crew suit-to-suit communications up to 200 meters distance is need for voice and data. The data rate for this application is expected to be around 100 kbps and will use the 420-430 MHz frequency range. For higher rate suit-to-suit communications involving high-definition video (~12 Mbps), technology based on the Sidelink device-to-device communications in the 5 855-5 925 frequency band may also be used. Furthermore, the astronauts will carry a distress beacon in the 406-406.1 MHz band. This beacon will be used to notify a lunar orbiting satellite in the event of an emergency. The crew EVA suits and rovers will also carry devices to receive signals transmitted by lunar orbiting satellites for position determination and time synchronization in the 2 483.5-2 500 MHz band. Outside of the SZM, relay communications between elements on the lunar surface and lunar orbiting satellites may also take place in the 390-405 MHz and 440-450 MHz bands.

For high-rate point-to-point links between the habitation module and remote experimental platforms involving science observations and/or data collection, the 27.5-28.35 GHz frequency range is expected be used. These links may involve data rates up to 1 Gbps using highly directional antennas. This frequency band may also be used for high-rate communications between two habitation modules, or for relay communications between a lander and a habitation module.

Table 2.1 shows a calculation of the spectrum needs for different envisioned lunar SRS applications.

Table 2.1

Envisaged Concept of Operations of Spectrum Use for SRS Systems in the Lunar Environment

| Frequency Band | Lunar Surface to Lunar Surface | Lunar Orbit to Lunar Surface | Lunar Surface to Lunar Orbit |
| --- | --- | --- | --- |
| 390-405 MHz* |  | X |  |
| 406-406.1 MHz* |  |  | X |
| 420-430 MHz* | X |  |  |
| 440-450 MHz* |  |  | X |
| 2 400-2 483.5 MHz | X |  |  |
| 2 483.5-2 500 MHz |  | X |  |
| 2 500-2 690 MHz | X |  |  |
| 3 500-3 800 MHz | X |  |  |
| 5 150-5 570 MHz | X |  |  |
| 5 570-5 725 MHz | X |  |  |
| 5 775-5 855 MHz | X |  |  |
| 5 855-5 925 MHz | X |  |  |
| 7 190-7 235 MHz |  | X |  |
| 8 450-8 500 MHz |  |  | X |
| 27.5-28.35 GHz | X |  |  |
| * Outside of the Shielded Zone of the Moon. |  |  |  |

Table 2.2

Lunar SRS Spectrum Needs

|  | EVA Comms | HAB module | Point-to-point Surface Comms | Orbiter relay comms (proximity link) | Remote Science Instruments | Exploration Vehicle (LTV, pressurized rover) |
| --- | --- | --- | --- | --- | --- | --- |
| Data Type | Voice, Data, Video | High-rate data transfer, file upload, high-definition video | Backhaul data | Relay data | Scientific measurement data | Voice, Data, Video |
| Data Rate (Mbps) | 12 | 30 | 1000 | 100 | 30 | 30 |
| # of Devices/ Links per Site | 6-12 | TBD | TBD | 6 (forward and return) | 20 | 4 |
| Total capacity (Mbps) | 144 | TBD | TBD | 600 | 600 | 120 |
| Spectral efficiency (bits/s/Hz) | 1 | TBD | TBD | 2 | 2 | 1 |
| Spectrum needs (MHz) | 144 | TBD | TBD | 300 | 300 | 120 |
| Total Spectrum needs (MHz)(1) | TBD |  |  |  |  |  |
| (1) Not including bandwidth for backup communications and guard bands |  |  |  |  |  |  |

# 3	Services identified for sharing and compatibility studies with space research systems for lunar communications

This section provides a summary of the services identified for sharing and compatibility studies with lunar surface SRS systems, or systems in lunar orbit communicating with systems on the lunar surface, in the frequencies ranges listed for study in Resolution 680 (WRC-23). The following tables provide a summary of the services identified for sharing and compatibility studies in each frequency range, along with the regions and relevant footnotes from Article 5 in the Radio Regulations.

The technical and operating characteristics of systems in the space research service planned for operations on the lunar surface, or systems in lunar orbit communicating with systems on the lunar surface, are provided in Report ITU-R SA.2553. All the lunar communications described in this report are between or among types of space stations on the lunar surface or in lunar orbit.

## 3.1	390-406.1 MHz

TABLE 3.1.1

Services identified for sharing studies with lunar communications (390-406.1 MHz)

| Service | Frequencies |  | Footnotes |
| --- | --- | --- | --- |
| FIXED | 390.0-399.9 MHz |  |  |
|  | 400.15-401 MHz |  | 5.262 |
| MOBILE | 390.0-399.9 MHz |  |  |
|  | 400.15-401 MHz |  | 5.262 |
| STANDARD FREQUENCY AND TIME SIGNAL-SATELLITE | 400.05-400.15 MHz |  | 5.261, 5.262 |
| MOBILE-SATELLITE (Earth-to-space) | 399.9-400.05 MHz |  | 5.209, 5.220, 5.260A, 5.260B |
|  | 406.00-406.10 MHz |  | 5.265, 5.266, 5.267 |
| MOBILE-SATELLITE (space-to-Earth) | 400.15-401.00 MHz |  | 5.208A, 5.208B, 5.209 |
| Fixed(1) | 401-406 MHz |  |  |
| (1) Service allocated on a secondary basis as per Radio Regulations but identified for study by WP 5C. |  |  |

## 3.2	420-430 MHz

TABLE 3.2.1

Services identified for sharing studies with lunar communications (420-430 MHz)

| Service | Frequencies | Footnotes |
| --- | --- | --- |
| FIXED | 420-430 MHz |  |
| MOBILE except aeronautical mobile | 420-430 MHz |  |
| RADIOLOCATION | 420-430 MHz | 5.269 |

## 3.3	440-450 MHz

TABLE 3.3.1

Services identified for sharing studies with lunar communications (440-450 MHz)

| Service | Frequencies | Footnotes |
| --- | --- | --- |
| FIXED | 440-450 MHz |  |
| MOBILE except aeronautical mobile | 440-450 MHz |  |
| RADIOLOCATION | 440-450 MHz | 5.269, 5.285 |

## 3.4	2 400-2 690 MHz

TABLE 3.4.1

Services identified for sharing studies with lunar communications (2 400-2 690 MHz)

| Service | Frequencies | Footnotes |
| --- | --- | --- |
| FIXED | 2 400-2 690 MHz |  |
| MOBILE | 2 400-2500 MHz |  |
| RADIOLOCATION | 2 450-2 500 MHz |  |
| RADIODETERMINATION-SATELLITE (space-to-Earth) | 2 483.5-2 500 MHz | 5.398, 5.399 (Region 1), 5.401 (Regions 1&3), 5.402 |
| MOBILE-SATELLITE (space-to-Earth) | 2 483.5-2 500 MHz | 5.351A, 5.401 (Regions 1&3), 5.402 |
|  | 2 500-2 520 MHz |  |
| MOBILE-SATELLITE (Earth-to-space) | 2 655-2 670 MHz | 5.420 |
|  | 2 670-2 690 MHz | 5.351A |
| FIXED-SATELLITE (space-to-Earth) | 2 500-2 535 MHz | 5.415 |
|  | 2 535-2 655 MHz | 5.415 |
|  | 2 655-2 690 MHz | 5.415 |
| FIXED-SATELLITE (Earth-to-space) | 2 655-2 690 MHz | 5.415 |
| MOBILE except aeronautical mobile | 2 500-2 690 MHz | 5.384A, 5.409A (Regions 1&2) |
| MOBILE (identified for IMT) | 2 500-2 690 MHz | 5.384A |
| MOBILE (identified for HIBS) | 2 500-2 690 MHz | 5.409A |
| BROADCASTING-SATELLITE | 2 520-2 655 MHz | 5.413, 5.416, 5.418B, 5.418C |
|  | 2 520-2 535 MHz | 5.413, 5.416 |
|  | 2 535-2 655 MHz | 5.413, 5.416, 5.418, 5.418A, 5.418B, 5.418C |
|  | 2 655-2 670 MHz | 5.413, 5.416 |
| RADIOASTRONOMY | 2 690-2 700 MHz |  |
| AERONAUTICAL RADIONAVIGATION (2) | 2 700-2 900 MHz | 5.423 |
| NOTE: The frequency range from 2 400 to 2 500 MHz is also designated for industrial, scientific and medical (ISM) applications according RR No. 5.150. Radiocommunication services operating within these bands must accept harmful interference which may be caused by these applications. ISM equipment operating in these bands is subject to the provisions of No. 15.13. (1) Adjacent band compatibility analysis (2) Service identified for compatibility analysis by WP 5B. |  |  |

## 3.5	3 500-3 800 MHz

TABLE 3.5.1

Services identified for sharing studies with lunar communications (3 500-3 800 MHz)

| Service | Frequencies | Footnotes |
| --- | --- | --- |
| FIXED | 3 500-3 800 MHz |  |
| FIXED SATELLITE (space-to-Earth) | 3 500-3 800 MHz |  |
| MOBILE except aeronautical mobile | 3 500-3 600 MHz | 5.430A (Region 1), 5.431B (Region 2), 5.433A (Region 3) |
|  | 3 600-3 800 MHz | 5.434A (Region 1), 5.435A (Region 1), 5.433B (Region 1), 5.434B (Region 1), 5.434 (Region 2) |
| MOBILE (identified for IMT) | 3 500-3 600 MHz | 5.430A, 5.431B, 5.433A |
|  | 3 600-3 700 MHz | 5.434 |
|  | 3 700-3 800 MHz | 5.434B |
|  | 3 700-3 800 MHz | 5.435B |
| RADIOLOCATION (Regions 2 and 3) | 3 500-3 600 MHz | 5.433 |

## 3.6	5 150-5 570 MHz

TABLE 3.6.1

Services identified for sharing studies with lunar communications (5 150-5 570 MHz)

| Service | Frequencies | Footnotes |
| --- | --- | --- |
| FIXED SATELLITE (Earth-to-space) | 5 150-5 250 MHz | 5.447A, 5.447C |
| FIXED SATELLITE (1) (space-to-Earth) | 5 150-5 216 MHz | 5.447B |
| MOBILE except aeronautical mobile | 5 150-5 250 MHz | 5.446A, 5.446B, 5.447 |
|  | 5 250-5 350 MHz | 5.446A, 5.447F |
|  | 5 470-5 570 MHz | 5.446A, 5.450A |
| AERONAUTICAL MOBILE | 5 150 -5 250 MHz | 5.446C, 5.446D |
| AERONAUTICAL RADIONAVIGATION | 5 150-5 250 MHz |  |
|  | 5 350-5 460 MHz | 5.449 |
|  | 5 470-5 650 MHz | 5.450 |
| FIXED | 5 250-5 350 MHz | 5.447E |
| RADIOLOCATION | 5 250-5 350 MHz | 5.448A |
|  | 5 350-5 470 MHz | 5.448D |
|  | 5 470-5 570 MHz | 5.450B |
| SPACE RESEARCH (active) | 5 250-5 255 MHz | 5.447D |
|  | 5 255-5 350 MHz | 5.448A |
|  | 5 350-5 460 MHz | 5.448C |
|  | 5 460-5 570 MHz | 5.448B |
| RADIONAVIGATION | 5 250-5 350 MHz | 5.448 |
|  | 5 460-5 470 MHz | 5.448B, 5.449 |
| MARITIME RADIONAVIGATION | 5 470-5 570 MHz | 5.448B |
| (1) This allocation is limited to feeder links of non-geostationary-satellite systems in the mobile satellite service and is subject to provisions of No. 9.11A. |  |  |

## 3.7	5 570-5 725 MHz

TABLE 3.7.1

Services identified for sharing studies with lunar communications (5 570-5 725 MHz)

| Service | Frequencies | Footnotes |
| --- | --- | --- |
| RADIOLOCATION | 5 570-5 650 MHz | 5.450B |
|  | 5 650-5 725 MHz |  |
| FIXED | 5 650-5 725 MHz | 5.453, 5.455 |
| MOBILE | 5 650-5 725 MHz | 5.453 |
| MOBILE except aeronautical mobile | 5 570-5 725 MHz | 5.450A |

## 3.8	5 775-5 925 MHz

TABLE 3.8.1

Services identified for sharing studies with lunar communications (5 775-5 925 MHz)

| Service | Frequencies | Footnotes |
| --- | --- | --- |
| FIXED SATELLITE (Earth-to-space) | 5 775-5 850 MHz |  |
|  | 5 850-5 925 MHz |  |
| RADIOLOCATION | 5 775-5 850 MHz |  |
| FIXED | 5 775-5 850 MHz | 5.453 |
|  | 5 850-5 925 MHz |  |
| MOBILE | 5 775-5 850 MHz | 5.453 |
| NOTE: The frequency range from 5 725 to 5 875 MHz is also designated for industrial, scientific and medical (ISM) applications according RR No. 5.415A. Radiocommunication services operating within these bands must accept harmful interference which may be caused by these applications. ISM equipment operating in these bands is subject to the provisions of No. 15.13. |  |  |

## 3.9	7 190-7 235 MHz

TABLE 3.9.1

Services identified for sharing studies with lunar communications (7 190-7 235 MHz)

| Service | Frequencies | Footnotes |
| --- | --- | --- |
| FIXED | 7 190-7 235 MHz |  |
| MOBILE | 7 190-7 235 MHz |  |
| SPACE RESEARCH (Earth-to-space) | 7 190-7 235 MHz | 5.460, 5.459 |

## 3.10	8 400-8 500 MHz

TABLE 3.10.1

Services identified for sharing studies with lunar communications (8 450-8 500 MHz)

| Service | Frequencies | Footnotes |
| --- | --- | --- |
| FIXED | 8 450-8 500 MHz |  |
| MOBILE except aeronautical mobile | 8 450-8 500 MHz |  |
| SPACE RESEARCH (space-to-Earth) | 8 450-8 500 MHz |  |

## 3.11	25.25-28.35 GHz

TABLE 3.11.1

Services identified for sharing studies with lunar communications (25.25-28.35 GHz)

| Service | Frequencies | Footnotes |
| --- | --- | --- |
| FIXED | 25.25-27 GHz | 5.534A |
|  | 27-27.5 GHz | 5.534A (Regions 2&3) |
|  | 27.5-28.35 GHz | 5.537A |
| FIXED (identified for HAPS) | 25.25-27.5 GHz | 5.534A |
| INTER SATELLITE | 25.25-27.5 GHz | 5.536, 5.537 (Regions 2&3) |
|  | 27.5-28.35 GHz (1) | 5.521A |
| MOBILE | 25.25-27.5 GHz | 5.532AB |
|  | 27.5-28.35 GHz |  |
| FIXED SATELLITE (Earth-to-space) | 27-27.5 GHz |  |
|  | 27.5-28.35 GHz | 5.484A, 5.516B, 5.517A, 5.539, 5.538, 5.540, 5.517B |
| (1) New intersatellite service allocation from WRC-23. |  |  |

# 4	Technical and operational characteristics for sharing and compatibility studies

Technical and operational characteristics to be considered in the sharing and compatibility studies are presented in the table below, as provided by WP 7B and the contributing groups to WRC-27 agenda item 1.15.

| Working Party 7B/ | Contributing Group | Services/Parameters |
| --- | --- | --- |
| Report ITU-R SA.2553 | WP 7B | Technical and operational characteristics for space research systems in the vicinity of the Moon |
| 58, 119 | WP 3J | Propagation models |
| 200 | WP 4A | Fixed-satellite service Broadcasting-satellite service |
| 41, 106, 196 | WP 4C | Mobile-satellite service |
| 115 | WP 5A | Land mobile service Amateur and Amateur-satellite service |
| 49, 114 | WP 5B | Aeronautical mobile service Radiodetermination service |
| 56, 201 | WP 5C | Fixed service |
| 64, 105 | WP 5D | International Mobile Telecommunications (IMT) |
| 95 | WP 7A | Standard Frequency and Time Signal |
| − | WP 7B | Space research service Earth exploration-satellite service |
| 103 | WP 7C | Space research service (active) Earth exploration-satellite service (passive) |
| 101, 193 | WP 7D | Radio astronomy service |

# 5	Sharing and compatibility studies

The sharing and compatibility studies are presented in the Annexes to this document.

Annex 1:	Sharing and compatibility studies of SRS for lunar operations in the frequency band 390-406.1 MHz

Annex 2:	Sharing and compatibility studies of SRS or lunar operations in the frequency band 420-430 MHz

Annex 3:	Sharing and compatibility studies of SRS for lunar operations in the frequency band 440-450 MHz

Annex 4:	Sharing and compatibility studies of SRS for lunar operations in the frequency band 2 400-2 690 MHz

Annex 5:	Sharing and compatibility studies of SRS for lunar operations in the frequency band 3 500-3 800 MHz

Annex 6:	Sharing and compatibility studies of SRS for lunar operations in the frequency band 5 150-5 570 MHz

Annex 7:	Sharing and compatibility studies of SRS for lunar operations in the frequency band 5 570-5 725 MHz

Annex 8:	Sharing and compatibility studies of SRS for lunar operations in the frequency band 5 775-5 925 MHz

Annex 9:	Sharing and compatibility studies of SRS for lunar operations in the frequency band 7 190-7 235 MHz

Annex 10:	Sharing and compatibility studies of SRS for lunar operations in the frequency band 8 450-8 500 MHz

Annex 11:	Sharing and compatibility studies of SRS LDRS (Lunar Data Relay Satellite) system operations in the frequency bands 7 190-7 235 MHz and 8 450-8 500 MHz

Annex 12:	Sharing and compatibility studies of SRS for lunar operations in the frequency band 25.25-28.35 GHz

ANNEX 1

Sharing and compatibility studies of SRS for lunar operations 
in the frequency band 390-406.1 MHz

## A1.1	Sharing and compatibility of fixed service and SRS for lunar operations in the frequency band 390-406.1 MHz

### A1.1.1	Study A [USA]

#### A1.1.1.1 	Technical and operational characteristics of SRS operating in the 390-406 MHz frequency range

The technical and operational characteristics for SRS systems are based on those available in the Report ITU-R SA.2553, including Tables 3.3-2 and 3.3-4, as summarized in Table A1.1.1.1.

Table A1.1.1.1

Technical Characteristics of the SRS Systems

| Station | Lunar Orbit Space Station Transmitter |
| --- | --- |
| Frequency range | 390-406 MHz |
| Beam Type | Fixed |
| Polarization | CP |
| Peak Gain (dBi) | 0 |
| Max. EIRP Density (dBW/Hz) | ‒63 |
| Max. EIRP (dBW) | 0 |
| Orbital Characteristics or Antenna Height | Apolune: 7 000-11 000 km Perilune: 600-2 700 km |
| Channel BW (MHz) | 2 |

#### A1.1.1.2 	Technical and operational characteristics of fixed service operating in the 390-406 MHz frequency range

Document 7B/56 provides relevant technical information to support studies between SRS and Fixed Services (FS) under agenda item 1.15. The characteristics for FS for point-to-point (PP) systems are based on the information contained in the Report ITU-R F.2108, Table 1, as summarized in Table A1.1.1.2.

Table A1.1.1.2

Characteristics of the FS (PP) systems

| Parameter | Value |
| --- | --- |
| Frequency range | 340-470 MHz |
| Modulation format | GMSK |
| Receiver noise bandwidth | 0.6 MHz |
| Antenna gain | 12 dBi |
| Feeder loss | 4.4 dB |
| Receiver thermal noise (Nrx) | −146.5 dBW |
| Protection criteria (I/N) | −6 dB |

#### A1.1.1.3 	Propagation model

This sharing study includes the lunar SRS system in orbit and on the surface of the Moon, and fixed services operating on Earth's surface. Therefore, Working Party 3J in Document 7B/58 recommends using the propagation model based on Recommendation ITU-R P.525 for this scenario.

#### A1.1.1.4 	Methodology

The methodology employed in this study is based on a static analysis of both single and aggregated worst-case interference. It evaluates whether, even under conditions of perfect antenna coupling between the systems, at the minimum distance between the stations on Earth and the Moon, and in the absence of antenna pointing errors and losses such as depolarization and clutter, the interference levels comply with the established ITU-R service protection criteria.

#### A1.1.1.5 	Study results

The study was conducted in accordance with the scenario outlined in Figure A1.1.1.1. For the worst-case analysis, the following assumptions were considered:

–	Main beam antenna coupling between the potentially interfering lunar SRS transmitters and the FS victim receiver;

–	Free space propagation loss assuming the minimum distance between the stations on the Earth and the Moon (355,000 km), while atmospheric, depolarization and clutter losses are not considered;

–	In the present study, it is assumed that both systems operate at the same central frequency. A bandwidth correction factor (CF) is applied when the bandwidth of the potential interferer exceeds the bandwidth of the victim receiver under analysis. If the bandwidth of the interferer is equal to or smaller than that of the victim receiver, the bandwidth correction factor is 0:

CF = 10 * log10bandwidthvictimbandwidthinterferer

–	The aggregate interference-to-noise ratio (I/N) is calculated considering Lunar SRS transmitters pointing towards the FS victim receiver using the following equation:

INTotal= 1n (EIRPnθ,φ+CF-FSPL-Floss+Grxθ,φ)-Nrx

where n is the number of lunar surface transmitters;  EIRPnθ,φ is the equivalent isotropic radiated power of each lunar transmitter; FSPL is the free space path loss calculated using Recommendation ITU-R P.525; Floss is the FS feeder loss; (Grx)θ,φ is the FS victim receiver maximum antenna gain; and Nrx is the thermal noise power in the FS victim receiver bandwidth; CF is the frequency-dependent rejection based on the transmit and receiver bandwidths;

The single and aggregate I/N results are compared to the FS protection criteria (I/N ≤ −6 dB) to verify whether the FS protection is met

Figure A1.1.1.1

SRS–FS Scenario

Table A1.1.1.3

SRS - FS co-channel results

| SRS Station | SRS centre frequency | Analysis | I/N result | Margin to the FS protection criteria |
| --- | --- | --- | --- | --- |
| Lunar Orbit Transmitter | 391 MHz | Single Entry | −46.3 dB | 40.3 dB |

#### A1.1.1.6 	Summary and analysis of the results of Study A

The interference-to-noise ratio metric shows that even in the single entry static worst-case analysis, there is a margin of at 40.3 dB or more to satisfy the FS protection criteria. That also implies that aggregating over 10 700 simultaneously transmitting worst case towards the Earth interfering transmitters would still meet the FS protection criteria, which far exceeds the expected active number of lunar transmitters in this band. Thus, the results of this static worst-case study suggest that sharing between SRS and terrestrial FS operating in the 390-406 MHz band (co-channel) is feasible.

### A1.1.2	Study B

### A1.1.3	Study C

## A1.2	Sharing studies between mobile satellite service (MSS) and SRS operating in the frequency band 390-406.1 MHz

### A1.2.1	Study A [USA]

#### A1.2.1.1	Technical and operational characteristics of SRS operating in the 390-406 MHz frequency range

The technical and operational characteristics for SRS systems are based on those available in the Report ITU-R SA.2553, including Table 3.3-4, as summarized in Table A1.2.1.1.

Table A1.2.1.1

Technical Characteristics of the SRS Systems

| Station | Lunar Orbit Space Station Transmitter |
| --- | --- |
| Frequency range | 390-406 MHz |
| Beam Type | Fixed |
| Polarization | CP |
| Peak Gain (dBi) | 0 |
| Max. EIRP Density (dBW/Hz) | ‒63 |
| Max. EIRP (dBW) | 0 |
| Orbital Characteristics | Apolune: 7 000-11 000 km Perilune: 600-2 700 km |
| Channel BW (MHz) | 2 |

#### A1.2.1.2	Technical and operational characteristics of mobile satellite service (MSS) operating in the 390-406.1 MHz frequency range

Document 7B/41 provides relevant technical information to support studies between SRS and mobile satellite service (MSS) under Agenda Item 1.15. The technical characteristics and protection criteria used in this study were extracted from Recommendation ITU-R M.2046 for non-geostationary (non-GSO) MSS systems (Earth-to-space) operating in the 399.9-400.05 MHz band, and from Recommendation ITU-R M.1184 for MSS (space-to-Earth) systems operating in the 400.15-401 MHz band, as summarized in Table A1.2.1.2.

In the absence of protection criteria defined in ITU-R Recommendations, Document 7B/41 recommends using a ΔT/T criterion of 6%. This criterion is currently found in Table 5-1 of RR Appendix 5 and serves as the coordination criterion between GSO MSS networks. Additionally, according to Document 7B/106, there is no available characteristics or protection criteria available for MSS systems in the 406-406.1 MHz band, so no particular study for this frequency range was conducted.

Table A1.2.1.2

Characteristics of the MSS Systems

| Parameter | MSS (Earth-to-space) | MSS (space-to-Earth) |
| --- | --- | --- |
| Frequency range | 399-400.05 MHz | 400.6-400.9 MHz |
| System | non-GSO (ARGOS 4) | non-GSO |
| System noise temperature (K) / Receiver noise figure (dB) | 1214 K | 3.8 dB* |
| Antenna gain | 3.85 dBi | 7 dBi |
| Receiver bandwidth | 1.6 kHz | 300 kHz |
| Protection criteria | spfd ≤ –197.9 dB(W/(m2 · Hz)) | I/N ≤ -12.2 dB |
| * Assumed parameter (no available receiver noise figure value) |  |  |

#### A1.2.1.3	Propagation model

This sharing study includes the lunar SRS system both in orbit and on the surface of the Moon, as well as MSS systems orbiting the Earth and operating on its surface. Therefore, Working Party 3J in Document 7B/58 recommends using the propagation model based on Recommendation ITU-R P.525 for those scenarios.

#### A1.2.1.4	Methodology

The methodology employed in this study is based on a static analysis of both single and aggregated worst-case interference. It evaluates whether, even under conditions of perfect antenna coupling between the systems, at the minimum distance between the stations on Earth and the Moon, and in the absence of antenna pointing errors and losses such as depolarization and clutter, the interference levels comply with the established ITU-R service protection criteria.

#### A1.2.1.5	Study results

The study was conducted in accordance with the scenarios outlined in Figure A1.2.1.1. For the worst-case analysis, the following assumptions were considered:

–	Main beam antenna coupling between the potentially interfering lunar SRS transmitters and the MSS victim receiver.

–	For the MSS (space-to-Earth), the free space propagation loss assumes a minimum distance of 355,000 km between the stations on Earth and the Moon, as presented in Figure A.1.2.1.1 (b). Atmospheric, depolarization, and clutter losses are not considered.

–	In the present study, it is assumed that both systems operate at the same central frequency. A bandwidth correction factor (CF) is applied when the bandwidth of the potential interferer exceeds the bandwidth of the victim receiver under analysis. If the bandwidth of the interferer is equal to or smaller than that of the victim receiver, the bandwidth correction factor is 0.

CF = 10 * log10bandwidthvictimbandwidthinterferer

–	The aggregate interference-to-noise ratio (I/N) is calculated considering Lunar SRS transmitters pointing towards the MSS victim receiver using the following equation:

INTotal= 1n (EIRPnθ,φ+CF-FSPL+Grxθ,φ)-Nrx

where n is the number of lunar surface transmitters;  EIRPnθ,φ is the equivalent isotropic radiated power of each lunar transmitter; FSPL is the free space path loss calculated using Recommendation ITU-R P.525; (Grx)θ,φ is the MSS victim receiver maximum antenna gain; and Nrx is the thermal noise power in the MSS victim receiver bandwidth

–	The receiver thermal noise power is calculated from the receiver noise figure (NF) using the formula provided in Recommendation ITU-R P.372-17.

–	For the MSS (Earth-to-space), considering a satellite orbiting the Earth, the free space propagation loss assumes a non-GSO altitude of 650 km. This results in a minimum distance of 354,350 km between the stations on Earth and the Moon, as presented in Figure A.1.2.1.1(a)

–	For the MSS (Earth-to-space), the aggregate interference spectral flux density (SFD) to the MSS space receiver is calculated as shown below, considering Lunar SRS potential interferers

ITotal= 1n (EIRPnθ,φ+CF-FSPL+(Grx)θ,φ)

SFDTotal= 1n10*log1010Itotal10Antennaeffective area*Bandwidth

–	The single and aggregate interference results are compared to the MSS protection criteria to verify whether the service protection criteria is met

Figure A1.2.1.1

SRS–MSS Scenarios: (a) MSS (Earth-to-space); (b) MSS (space-to-Earth)

(a)

(b)

Table A1.2.1.3

SRS – MSS (Earth-to-space) co-channel results

| SRS Station | SRS centre frequency | Analysis | SFD result dB(W/m2*Hz) | Margin to the MSS protection criteria |
| --- | --- | --- | --- | --- |
| Lunar Orbit Space Station Transmitter | 400 MHz | Single Entry | −244.9 | 47.0 dB |

Table A1.2.1.4

SRS – MSS (space-to-Earth) co-channel results

| SRS Station | SRS centre frequency | Analysis | I/N result | Margin to the MSS protection criteria |
| --- | --- | --- | --- | --- |
| Lunar Orbit Space Station Transmitter | 400.8 MHz | Single Entry | −51.2 | 39.0 dB |

#### A1.2.1.6	Summary and analysis of the results of Study A

The interference metrics show that even in the single entry static worst-case analysis, there is a margin of 39 dB or more to satisfy the MSS (s-E) protection criteria. That also implies that aggregating over 7900 worst case interfering transmitters would still meet the MSS protection criteria, which far exceeds the expected number of lunar transmitters in this band. Thus, the results of this study suggest that sharing between SRS Lunar orbit transmitters and MSS (s-E) systems operating in the 390-406.1 MHz band (co-channel) is feasible.

For MSS (E-s), the interference metrics show that even in the single entry static worst-case analysis, there is a margin of 47 dB to satisfy the protection criteria. That also implies that aggregating over 50,000 worst case interfering transmitters would still meet the MSS protection criteria, which far exceeds the expected number of lunar transmitters in this band. Thus, the results of this study suggest that the protection of MSS (E-s) systems from SRS Lunar orbit systems interference is ensured.

### A1.2.2	Study B [CAN]

Document 7B/188 provides a study of interference from lunar surface emergency beacons to COSPAS-SARSAT satellite receivers in the 406-406.1 MHz band. Using the COSPAS-SARSAT protection criteria in Table 4, Annex 11 of Rec. ITU-R M.1478-3, the calculated maximum allowable EIRP spectral density for lunar surface transmitters in the 406-406.1 MHz band is -24.4 dBW/Hz for wideband emissions (≥ 40 Hz), and 11.4 dBW/Hz for narrowband emissions (< 40 Hz).  Based on the analysis, emergency beacons on the surface of the Moon that are operated close to an astronaut’s body will meet the protection criteria for COSPAS-SARSAT instruments orbiting Earth by a margin of 7.1 dB for narrowband emissions and a margin of 12.9 dB for wideband emissions.

{Editor’s note: the results in this study are based on a revised version of 7B/188 that was supplied by Canada during the Sep 2025 meeting. Canada indicated that they plan to make a contribution to the first meeting in 2026 to update this current section with the details of their revised study document}

## A1.3	Sharing studies between mobile service (MS) and SRS operating in the frequency band 390-406.1 MHz

### A1.3.1	Study A [USA]

#### A1.3.1.1	Technical and operational characteristics of SRS operating in the 390-406 MHz frequency range

The technical and operational characteristics for SRS systems are based on those available in the Report ITU-R SA.2553, including Table 3.3-4, as summarized in Table A1.3.1.1.

Table A1.3.1.1

Technical Characteristics of the SRS Systems

| Station | Lunar Orbit Space Station Transmitter |
| --- | --- |
| Frequency range | 390-406 MHz |
| Beam Type | Fixed |
| Polarization | CP |
| Peak Gain (dBi) | 0 |
| Max. EIRP Density (dBW/Hz) | ‒63 |
| Max. EIRP (dBW) | 0 |
| Orbital Characteristics | Apolune: 7 000-11 000 km Perilune: 600-2 700 km |
| Channel BW (MHz) | 2 |

#### A1.3.1.2	Technical and operational characteristics of mobile service operating in the 390-406.1 MHz frequency range

Document 7B/115 provides relevant technical information to support studies between SRS and mobile service under Agenda Item 1.15. The technical characteristics and protection criteria used in this study were extracted from Recommendation ITU-R M.1808 (System A in Table 1B) for conventional and trunked land mobile systems in the 335.4 – 399.9 MHz band.  For public protection and disaster relief (PPDR) applications in this band, an I/N of -10 dB is used.

Table A1.3.1.2

Characteristics of the MS Systems

| Parameter | Mobile service receiver (Digital System A) |  |
| --- | --- | --- |
| Frequency range | 335.4-399.9 MHz |  |
| Receiver noise figure (dB) | 5 to 12 dB |  |
| Antenna gain | 0 to 11 dBd |  |
| Receiver bandwidth | 25 to 1250 kHz |  |
| Protection criteria (I/N) | -10 dB (PPDR applications) |  |

#### A1.3.1.3	Propagation model

This sharing study includes the lunar SRS system in orbit and on the surface of the Moon, and mobile services (conventional and trunked land mobile) operating on Earth's surface. Therefore, Working Party 3J in Document 7B/58 recommends using the propagation model based on Recommendation ITU-R P.525 for this scenario.

#### A1.3.1.4	Methodology

The methodology employed in this study is based on a static analysis of both single and aggregated worst-case interference. It evaluates whether, even under conditions of perfect antenna coupling between the systems, at the minimum distance between the stations on Earth and the Moon, and in the absence of antenna pointing errors and losses such as depolarization and clutter, the interference levels comply with the established ITU-R service protection criteria.

#### A1.3.1.5	Study results

The study was conducted in accordance with the scenarios outlined in Figure A1.2.1.1. For the worst-case analysis, the following assumptions were considered:

–	Main beam antenna coupling between the potentially interfering lunar SRS transmitters and the MS victim receiver.

–	Free space propagation loss assuming the minimum distance between the stations on the Earth and the Moon (355,000 km), while atmospheric, depolarization and clutter losses are not considered;

–	In the present study, it is assumed that both systems operate at the same central frequency. A bandwidth correction factor (CF) is applied when the bandwidth of the potential interferer exceeds the bandwidth of the victim receiver under analysis. If the bandwidth of the interferer is equal to or smaller than that of the victim receiver, the bandwidth correction factor is 0:

CF = 10 * log10bandwidthvictimbandwidthinterferer

–	The aggregate interference-to-noise ratio (I/N) is calculated considering Lunar SRS transmitters pointing towards the MS victim receiver using the following equation:

INTotal= 1n (EIRPnθ,φ+CF-FSPL+Grxθ,φ)-Nrx

where n is the number of lunar surface transmitters;  EIRPnθ,φ is the equivalent isotropic radiated power of each lunar transmitter; FSPL is the free space path loss calculated using Recommendation ITU-R P.525; (Grx)θ,φ is the MS victim receiver maximum antenna gain; and Nrx is the thermal noise power in the MS victim receiver bandwidth; CF is the frequency-dependent rejection based on the transmit and receiver bandwidths;

–	The receiver thermal noise power is calculated from the receiver noise figure (NF) using the formula provided in Recommendation ITU-R P.372-17.

–	The single and aggregate interference results are compared to the MS / PPDR protection criteria to verify whether the service protection criteria is met

Figure A1.2.1.1

SRS–MS Scenario

(a)

(b)

Table A1.3.1.3

SRS – MS co-channel results

| SRS Station | SRS centre frequency | Analysis | I/N result | Margin to the MS/PPDR protection criteria |
| --- | --- | --- | --- | --- |
| Lunar Orbit Space Station Transmitter | 395 MHz | Single Entry | -46.3 dB | 36.3 dB |

#### A1.3.1.6 	Summary and analysis of the results of Study A

#### The interference-to-noise ratio metric shows that even in the single entry static worst-case analysis, there is a margin of 36.3 dB or more to satisfy the protection criteria for mobile service PPDR applications. This also implies that aggregating over 4 265 simultaneously transmitting worst case towards the Earth interfering transmitters would still meet the MS/PPDR protection criteria, which far exceeds the expected active number of lunar transmitters in this band. Thus, the results of this static worst-case study suggest that sharing between SRS and terrestrial MS operating in the 390-406 MHz band (co-channel) is feasible.

## A1.4	Compatibility studies between SRS operating in the frequency band 406-406.1 MHz and radio astronomy service in the adjacent 406.1-410 MHz band

### A1.4.1	Study A [USA]

#### A1.4.1.1	Technical and operational characteristics of SRS operating in the 406-406.1 MHz frequency range

The technical and operational characteristics for SRS systems are based on those available in the Report ITU-R SA.2553, including Table 3.3-4, as summarized in Table A1.3.1.1.  The unwanted emissions level is based on the spurious domain limit given in Recommendation ITU-R SM.329-13.

Table A1.4.1.1

Technical Characteristics of the SRS Systems

| Station | Lunar Surface Transmitter |
| --- | --- |
| Frequency range | 406-406.1 MHz |
| Beam Type | Fixed |
| Polarization | CP |
| Peak Gain (dBi) | 0 |
| Conducted unwanted emission level (dBW) in 406.1-410 MHz | -43 |
| Transmit antenna pattern | ND |
| Channel BW (MHz) | 0.05 |

#### A1.4.1.2	Technical and operational characteristics of radio astronomy service operating in the 406.1-410 MHz frequency range

Document 7B/101 provides relevant technical information to support studies between SRS and radio astronomy service under Agenda Item 1.15. The technical characteristics and protection criteria used in this study were extracted from Recommendation ITU-R RA.749.

Table A1.4.1.2

Characteristics of the RAS Systems

| Parameter | RAS receiver |
| --- | --- |
| Frequency range | 406.1-410 MHz |
| Antenna Gain (sidelobe) | 0 dBi |
| Receiver noise temperature (K) | 60 |
| Threshold interference level (input to RAS receiver) | -203 dBW |
| Receiver Bandwidth | 3.9 MHz |

#### A1.4.1.3	Propagation model

This sharing study includes the lunar SRS transmitter operating on the surface of the Moon, and a radio astronomy receiver operating on Earth's surface. Working Party 3J in Document 7B/58 recommends using the propagation model based on Recommendation ITU-R P.525 for this scenario.

#### A1.4.1.4	Methodology

The methodology employed in this study is based on a static analysis of both single and aggregated worst-case interference. It evaluates whether, even under conditions of perfect antenna coupling between the systems, at the minimum distance between the stations on Earth and the Moon, and in the absence of antenna pointing errors and losses such as depolarization and clutter, the interference levels comply with the established ITU-R service protection criteria.

#### A1.4.1.5	Study results

The study was conducted in accordance with the scenarios outlined in Figure A1.4.1.1. For the worst-case analysis, the following assumptions were considered:

–	Antenna coupling between the potentially interfering lunar SRS transmitter main lobe and the victim RAS receiver antenna sidelobe.

–	Free space propagation loss assuming the minimum distance between the stations on the Earth and the Moon (355,000 km), while atmospheric, depolarization and clutter losses are not considered;

–	In the present study, it is assumed that both systems operate at the same central frequency. A bandwidth correction factor (CF) is applied when the bandwidth of the potential interferer exceeds the bandwidth of the victim receiver under analysis. If the bandwidth of the interferer is equal to or smaller than that of the victim receiver, the bandwidth correction factor is 0:

CF = 10 * log10bandwidthvictimbandwidthinterferer

–	The single and aggregate interference results are compared to the RAS protection criteria to verify whether the service protection criteria is met

Figure A1.4.1.1

SRS–RAS Scenario

(a)

(b)

Table A1.4.1.3

SRS – RAS co-channel results

| SRS Station | SRS centre frequency | Analysis | Received interference power level | Margin to the RAS protection criteria |
| --- | --- | --- | --- | --- |
| Lunar Surface Transmitter | 406.05 MHz | Single Entry | -238.7 dBW | 35.7 dB |

#### A1.4.1.6 	Summary and analysis of the results of Study A

The single entry static worst-case analysis shows that the unwanted emissions from a lunar SRS transmitter in the 406-406.1 MHz band will satisfy the protection criteria for a radio astronomy receiver operating in the adjacent 406.1-410 MHz band on Earth with a margin of 35.7 dB. The results of this static worst-case study suggest that lunar SRS in the 406-406.1 MHz band is compatible with terrestrial RAS operating in the adjacent 406.1-410 MHz band.

## A1.5	Sharing studies between standard frequency and time signal-satellite service  (SFTSS) and SRS operating in the frequency band 390-406.1 MHz

### A1.5.1	Study A [USA]

#### A1.5.1.1	Technical and operational characteristics of SRS operating in the 390-406 MHz frequency range

The technical and operational characteristics for SRS systems are based on those available in the Report ITU-R SA.2553, including Table 3.3-4, as summarized in Table A1.5.1.1.

Table A1.5.1.1

Technical Characteristics of the SRS Systems

| Station | Lunar Orbit Space Station Transmitter |
| --- | --- |
| Frequency range | 390-406 MHz |
| Beam Type | Fixed |
| Polarization | CP |
| Peak Gain (dBi) | 0 |
| Max. EIRP Density (dBW/Hz) | ‒63 |
| Max. EIRP (dBW) | 0 |
| Orbital Characteristics | Apolune: 7 000-11 000 km Perilune: 600-2 700 km |
| Channel BW (MHz) | 2 |

#### A1.5.1.2	Technical and operational characteristics of standard frequency and time signal-satellite service  in the 390-406.1 MHz frequency range

In Document 7B/95, WP 7A noted that 400.05-400.15 MHz is allocated to the SFTSS service on a primary basis and suggests that this frequency range not be considered for additional SRS allocations in WP 7B’s studies.  No characteristics for the SFTSS systems in the 400.05-400.15 MHz band have been provided by WP 7A.

#### A1.5.1.3 	Summary and analysis of the results of Study A

## Since no characteristics or protection criteria of the SFTSS systems in the 400.05-400.15 MHz band have been provided by WP 7A, it was not possible to determine if SFTSS sharing with lunar SRS is feasible in this band.

## A1.[X]	Summary and analysis of the results of studies

[Note: The summary should offer an objective overview, encompassing the results of sharing and compatibility studies, alongside any pertinent information applicable for future comparisons among studies. Additionally, it should address any mitigation techniques or proposals employed to safeguard services operating within the same band or adjacent bands, where applicable, to SRS lunar systems.]

ANNEX 2

Sharing and compatibility studies of SRS for lunar operations
in the frequency band 420-430 MHz

## A2.1	Sharing studies between fixed service and SRS for lunar operations in the frequency band 420-430 MHz

## A2.1.1	Study A [USA]

#### A2.1.1.1	Technical and operational characteristics of SRS operating in the 420-430 MHz frequency range

The technical and operational characteristics for SRS systems are based on those available in the Report ITU-R SA.2553, including Table 3.2-4, as summarized in Table A2.1.1.1.

Table A2.1.1.1

Technical Characteristics of the SRS Systems

| Station | Lunar Surface-to-Surface Transmitter |
| --- | --- |
| Frequency range | 420 – 430 MHz |
| Polarization | Vertical |
| Peak Gain (dBi) | 0.0 |
| EIRP Density (dBW/MHz) | ‒0.4 |
| Max. EIRP (dBW) | 2.6 |
| Channel BW (MHz) | 2 |

#### A2.1.1.2	Technical and operational characteristics of fixed service operating in the 420-430 MHz frequency range

Document 7B/56 provides relevant technical information to support studies between SRS and Fixed Services (FS) under agenda item 1.15. The characteristics for fixed service (FS) for point-to-point (PP) systems are based on the information contained in Recommendation ITU-R F.758-8, Table 17, as summarized in Table A2.1.1.2.

Table A2.1.1.2

Characteristics of the FS (PP) systems

| Parameter | Value |
| --- | --- |
| Frequency range | 406.1- 450 MHz |
| Reference | Recommendation ITU-R F.1567 |
| Modulation format | 32-QAM |
| Receiver noise bandwidth | 3.5 MHz |
| Antenna height | 30 m |
| Antenna pattern | Recommendation ITU-R F.699 |
| Antenna gain | 25 dBi |
| Feeder loss | 2 dB |
| Receiver noise figure | 3.5 dB |
| Protection criteria (I/N) | ‒6 dB |

#### A2.1.1.3	Propagation model

This sharing study includes the lunar SRS system on the surface of the Moon, and fixed services operating on Earth's surface. Therefore, Working Party 3J in Document 7B/58 recommends using the propagation model based on Recommendation ITU-R P.525 for this scenario.

#### A2.1.1.4	Methodology

The methodology employed in this study is based on a static analysis of both single and aggregated worst-case interference. It evaluates whether, even under conditions of perfect antenna coupling between the systems, at the minimum distance between the stations on Earth and the Moon, and in the absence of antenna pointing errors and losses such as depolarization and clutter, the interference levels comply with the established ITU-R service protection criteria.

#### A2.1.1.5	Study results

The study was conducted in accordance with the scenario outlined in Figure A2.1.1.1. For the worst-case analysis, the following assumptions were considered:

–	Main beam antenna coupling between the potentially interfering lunar SRS transmitters and the FS victim receiver.

–	Free space propagation loss assuming the minimum distance between the stations on the Earth and the Moon (355,000 km), while atmospheric, depolarization and clutter losses are not considered.

–	The aggregate interference-to-noise ratio (I/N) is calculated considering Lunar SRS transmitters pointing towards the FS victim receiver using the following equation:

INTotal= 1n (EIRPnθ,φ-FSPL-Floss+Grxθ,φ)-Nrx

where n is the number of lunar surface transmitters;  EIRPnθ,φ is the equivalent isotropic radiated power of each lunar transmitter; FSPL is the free space path loss calculated using Recommendation ITU-R P.525; Floss is the FS feeder loss; (Grx)θ,φ is the FS victim receiver maximum antenna gain; and Nrx is the thermal noise power in the FS victim receiver bandwidth.

–	The receiver thermal noise power is calculated from the receiver noise figure (NF) using the formula provided in Recommendation ITU-R P.372-17.

–	The single and aggregate I/N results are compared to the FS protection criteria (I/N ≤ −6 dB) to verify whether the FS protection is met.

Figure A2.1.1.1

SRS–FS Scenario

Table A2.1.1.3

SRS - FS co-channel results

| SRS Station | SRS centre frequency | Analysis | I/N result | Margin to the FS protection criteria |
| --- | --- | --- | --- | --- |
| Lunar Surface Transmitter | 425 MHz | Single Entry | –35.3 dB | 29.3 dB |

#### A2.1.1.6	Summary and analysis of the results of Study A

The interference-to-noise ratio metric shows that in the single entry static worst-case analysis, there is a margin of 29.3 dB or more to satisfy the FS protection criteria. That also implies that aggregating over 850 worst case interfering transmitters would still meet the FS protection criteria, which far exceeds the expected number of lunar transmitters in this band. The results of this study suggest that sharing between SRS and ground-based FS operating in the 420-430 MHz band (co-channel) is feasible.

### A2.1.2	Study B [D]

#### A2.1.2.1 	Technical and operational characteristics of SRS operating in the 420-430 MHz frequency range

The technical and operational characteristics for SRS systems are based on those available in Report ITU-R SA.2553, including Table 3.2-4, as summarized in Table A2.1.2.1.

Table A2.1.2.1

Technical Characteristics of the SRS Systems

| Station | Lunar Surface-to-Surface Transmitter |
| --- | --- |
| Frequency range | 420-430 MHz |
| Polarization | Vertical |
| Peak Gain (dBi) | 0.0 |
| EIRP Density (dBW/MHz) | ‒0.4 |
| Max. EIRP (dBW) | 2.6 |
| Channel BW (MHz) | 2 |

#### A2.1.2.2 	Technical and operational characteristics of fixed service operating in the 420– 430 MHz frequency range

The characteristics for fixed service (FS) for point-to-point (PP) systems are based on the information contained in Recommendation ITU-R F.758-8, Table 17, as summarized in Table A2.1.2.2.

Table A2.1.2.2

Characteristics of the FS (PP) systems

| Parameter | Value | Value |
| --- | --- | --- |
| Frequency range | 406.1-450 MHz | 406.1-450 MHz |
| Reference | Recommendation ITU-R F.1567 | Recommendation ITU-R F.1567 |
| Modulation format | QPSK | 32-QAM |
| Receiver noise bandwidth (MHz) | 0.05, 0.1, 0.15, 0.2, 0.25, 0.3, 0.5, 0.6, 0.75, 1.0, 1.75, 3.5 | 0.05, 0.1, 0.15, 0.2, 0.25, 0.3, 0.5, 0.6, 0.75, 1.0, 1.75, 3.5 |
| Antenna pattern | Recommendation ITU-R F.699 | Recommendation ITU-R F.699 |
| Antenna gain | 25 dBi | 25 dBi |
| Feeder loss | 2 dB | 2 dB |
| Receiver noise figure | 5 dB | 3.5 dB |
| Protection criteria (I/N) | ‒6 dB | ‒6 dB |

#### A2.1.2.3 	Propagation model

This sharing study includes the lunar SRS system on the surface of the Moon, and fixed services operating on Earth's surface. Working Party 3J in Document 7B/58 recommends using the propagation model based on Recommendation ITU-R P.525 for this scenario.

#### A2.1.2.4 	Methodology

The methodology employed in this study is based on a static analysis of single-entry worst-case interference. It evaluates whether, even under conditions of perfect antenna coupling between the systems, at the minimum distance between the stations on Earth and the Moon, and in the absence of antenna pointing errors and losses such as depolarization and clutter, the interference levels comply with the established ITU-R service protection criteria.

#### A2.1.2.5 	Study results

The study was conducted in accordance with the scenario outlined in Figure A2.1.2.1. For the worst-case analysis, the following assumptions were considered:

–	Main beam antenna coupling between the potentially interfering lunar SRS transmitters and the FS victim receiver.

–	Free space propagation loss assuming the minimum distance between the stations on the Earth and the Moon (355,000 km), while atmospheric, depolarization and clutter losses are not considered.

–	The aggregate interference-to-noise ratio (I/N) is calculated considering Lunar SRS transmitters pointing towards the FS victim receiver using the following equation:

INTotal= 1n (EIRPnθ,φ-FSPL-Floss+Grxθ,φ)-Nrx

where n is the number of lunar surface transmitters;  EIRPnθ,φ is the equivalent isotropic radiated power of each lunar transmitter; FSPL is the free space path loss calculated using Recommendation ITU-R P.525; Floss is the FS feeder loss; (Grx)θ,φ is the FS victim receiver maximum antenna gain; and Nrx is the thermal noise power in the FS victim receiver bandwidth.

–	The receiver thermal noise power is calculated from the receiver noise figure (NF) using the formula provided in Recommendation ITU-R P.372-17.

–	The single and aggregate I/N results are compared to the FS protection criteria (I/N ≤ −6 dB) to verify whether the FS protection is met.

Figure A2.1.2.1

SRS–FS Scenario

Table A2.1.2.3

SRS - FS co-channel results

|  |  | FS QPSK |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| bandwidth | MHz | 0.05 | 0.1 | 0.15 | 0.2 | 0.25 | 0.3 | 0.5 | 0.6 | 0.75 | 1 | 1.75 | 3.5 |
| Receiver noise power | dBW | -152 | -149 | -147.2 | -146 | -145 | -144.2 | -142 | -141 | -140.2 | -139 | -136.5 | -133.5 |
| Single Interferer | dBW | -186.4 | -183.4 | -181.6 | -180.4 | -179.4 | -178.6 | -176.4 | -175.6 | -174.6 | -173.4 | -171.0 | -170.4 |
| I/N result | dB | -34.4 | -34.4 | -34.4 | -34.4 | -34.4 | -34.4 | -34.4 | -34.4 | -34.4 | -34.4 | -34.4 | -36.8 |
| Margin to the FS protection criterion | dB | 28.4 | 28.4 | 28.4 | 28.4 | 28.4 | 28.4 | 28.4 | 28.4 | 28.4 | 28.4 | 28.4 | 30.8 |
|  |  | FS 32-QAM |  |  |  |  |  |  |  |  |  |  |  |
| bandwidth | MHz | 0.05 | 0.1 | 0.15 | 0.2 | 0.25 | 0.3 | 0.5 | 0.6 | 0.75 | 1 | 1.75 | 3.5 |
| Receiver noise power | dBW | -153.5 | -150.5 | -148.7 | -147.5 | -146.5 | -145.7 | -143.5 | -142.7 | -141.7 | -140.5 | -138.0 | -135.0 |
| Single Interferer | dBW | -186.4 | -183.4 | -181.6 | -180.4 | -179.4 | -178.6 | -176.4 | -175.6 | -174.6 | -173.4 | -171.0 | -170.4 |
| I/N result | dB | -32.9 | -32.9 | -32.9 | -32.9 | -32.9 | -32.9 | -32.9 | -32.9 | -32.9 | -32.9 | -32.9 | -35.3 |
| Margin to the FS protection criterion | dB | 26.9 | 26.9 | 26.9 | 26.9 | 26.9 | 26.9 | 26.9 | 26.9 | 26.9 | 26.9 | 26.9 | 29.3 |

#### A2.1.2.6 	Summary and analysis of the results of Study B

The interference-to-noise ratio metric shows that in the single entry static worst-case analysis, the FS protection criteria is met with a margin of 29.3 dB or more. The results of this study suggest that protection of FS from SRS in the 420-430 MHz band is ensured.

### A2.1.3	Study C

## A2.2	Sharing studies between radiolocation service and SRS operating in the frequency band 420-430 MHz

### A2.2.1	Study A [USA]

#### A2.2.1.1	Technical and operational characteristics of SRS operating in the 420-430 MHz frequency range

The technical and operational characteristics for SRS systems are based on those available in the Report ITU-R SA.2553, including Table 3.2-4, as summarized in Table A2.2.1.1.

Table A2.2.1.1

Technical Characteristics of the SRS Systems

| Station | Lunar Surface-to-Surface Transmitter |
| --- | --- |
| Frequency range | 420-430 MHz |
| Polarization | Vertical |
| Peak Gain (dBi) | 0.0 |
| EIRP Density (dBW/MHz) | ‒0.4 |
| Max. EIRP (dBW) | 2.6 |
| Channel BW (MHz) | 2 |

#### A2.2.1.2	Technical and operational characteristics of radiolocation service operating in the 420-430 MHz frequency range

Document 7B/114 provides relevant technical information to support studies between SRS and radars operating in the radiolocation service under Agenda Item 1.15. The characteristics for radars systems are based on the Recommendation ITU-R M.1462, Tables 1, 3 and 4, as summarized in Table A2.2.1.2.

Table A2.2.1.2

Characteristics of the radar systems

| Parameter | Ground | Airborne | Shipborne |
| --- | --- | --- | --- |
| Frequency range | 420-450 MHz |  |  |
| Polarization | Circular | Horizontal | - |
| Platform height/altitude | 15 m | Ascent, descent and operational operations. Ceiling is around 9 km | - |
| Antenna gain | 40 dBi | 22 dBi | 30 dBi |
| Antenna scan | Sector ±60° azimuth scan, with rotating or random modes 3-85° elevation | ±60° elevation (mechanically positioned or electronically scanned); 3600 azimuth at 3-7 rpm | 3600 azimuth |
| Receiver noise figure | 4 dB | 5 dB | 4 dB |
| Receiver IF bandwidth | 2 MHz | 1 MHz | 2 MHz |
| Protection criteria (I/N) | −6 dB |  |  |

#### A2.2.1.3	Propagation model

This sharing study includes the lunar SRS system on the surface of the Moon, and radiolocation service operating on Earth's surface. Working Party 3J in Document 7B/58 recommends using the propagation model based on Recommendation ITU-R P.525 for this scenario.

#### A2.2.1.4	Methodology

The methodology employed in this study is based on a static analysis of both single and aggregated worst-case interference. It evaluates whether, even under conditions of perfect antenna coupling between the systems, at the minimum distance between the stations on Earth and the Moon, and in the absence of antenna pointing errors and losses such as depolarization and clutter, the interference levels comply with the established ITU-R service protection criteria.

#### A2.2.1.5	Study results

The study was conducted in accordance with the scenarios outlined in Figure A2.2.1.1. For the worst-case analysis, the following assumptions were considered:

–	Main beam antenna coupling between the potentially interfering lunar SRS transmitters and the radar victim receiver.

–	Free space propagation loss assuming the minimum distance between the stations on the Earth and the Moon (355,000 km for ground and shipborne radars and 354,991 km for airborne), while atmospheric, depolarization and clutter losses are not considered.

–	In the present study, it is assumed that both systems operate at the same central frequency. A bandwidth correction factor (CF) is applied when the bandwidth of the potential interferer exceeds the bandwidth of the victim receiver under analysis. If the bandwidth of the interferer is equal to or smaller than that of the victim receiver, the bandwidth correction factor is 0:

CF = 10 * log10bandwidthvictimbandwidthinterferer

–	The aggregate interference-to-noise ratio (I/N) is calculated considering Lunar SRS transmitters pointing towards the radar victim receiver using the following equation:

INTotal= 1n (EIRPnθ,φ+CF-FSPL+Grxθ,φ)-Nrx

where n is the number of lunar surface transmitters;  EIRPnθ,φ is the equivalent isotropic radiated power of each lunar transmitter; FSPL is the free space path loss calculated using Recommendation ITU-R P.525; (Grx)θ,φ is the radar victim receiver maximum antenna gain; and Nrx is the thermal noise power in the radar victim receiver bandwidth.

–	The receiver thermal noise power is calculated from the receiver noise figure (NF) using the formula provided in Recommendation ITU-R P.372-17.

–	The single and aggregate I/N results are compared to the radar protection criteria (I/N ≤ −6 dB) to verify whether the radiolocation service protection is met.

Figure A2.2.1.1

SRS–RLS Scenarios: (a) Ground and shipborne; (b) Airborne

(a)

(b)

Table A2.2.1.3

SRS – RLS co-channel results

| SRS Station | SRS centre frequency | Radar | Analysis | I/N result | Margin to the radar protection criteria |
| --- | --- | --- | --- | --- | --- |
| Lunar Surface Transmitter | 425 MHz | Ground | Single Entry | −16.4 dB | 10.4 dB |
| Lunar Surface Transmitter | 425 MHz | Airborne | Single Entry | –35.4 dB | 29.4 dB |
| Lunar Surface Transmitter | 425 MHz | Shipborne | Single Entry | –26.4 dB | 20.4 dB |

#### A2.2.1.6	Summary and analysis of the results of Study A

The results of this study suggest that protection of radars operating in the radiolocation service in the 420-430 MHz band (co-channel) from SRS interference is ensured. Furthermore, the interference-to-noise ratio metric shows that in the static worst-case analysis for a single interferer, there is a margin of 10.4 dB or more to satisfy the radiolocation service protection criteria.

## A2.3	Sharing studies between mobile service (MS) and SRS operating in the frequency band 420-430 MHz

### A2.3.1	Study A [USA]

#### A2.3.1.1	Technical and operational characteristics of SRS operating in the 420-430 MHz frequency range

The technical and operational characteristics for SRS systems are based on those available in the Report ITU-R SA.2553, including Table 3.2-4, as summarized in Table A2.3.1.1.

Table A2.3.1.1

Technical Characteristics of the SRS Systems

| Station | Lunar Surface-to-Surface Transmitter |
| --- | --- |
| Frequency range | 420 – 430 MHz |
| Polarization | Vertical |
| Peak Gain (dBi) | 0.0 |
| EIRP Density (dBW/MHz) | ‒0.4 |
| Max. EIRP (dBW) | 2.6 |
| Channel BW (MHz) | 2 |

#### A2.3.1.2	Technical and operational characteristics of mobile service operating in the 420-430 MHz frequency range

Document 7B/115 provides relevant technical information to support studies between SRS and mobile service under Agenda Item 1.15. The technical characteristics and protection criteria used in this study were extracted from Recommendation ITU-R M.1808 (System A in Table 1C) for conventional and trunked land mobile systems in the 406.1 – 470 MHz band.  For public protection and disaster relief (PPDR) applications in this band, an I/N of -10 dB is used.

Table A2.3.1.2

Characteristics of the MS Systems

| Parameter | Mobile service receiver (System A) |
| --- | --- |
| Frequency range | 406.1-470 MHz |
| Receiver noise figure (dB) | 6 to 12 dB |
| Antenna gain | 0 to 11 dBd |
| Receiver bandwidth | 5.5 kHz |
| Protection criteria (I/N) | -10 dB (PPDR applications) |

#### A2.3.1.3	Propagation model

This sharing study includes the lunar SRS system on the surface of the Moon and mobile services (conventional and trunked land mobile) operating on Earth's surface. Therefore, Working Party 3J in Document 7B/58 recommends using the propagation model based on Recommendation ITU-R P.525 for this scenario.

#### A2.3.1.4	Methodology

The methodology employed in this study is based on a static analysis of both single and aggregated worst-case interference. It evaluates whether, even under conditions of perfect antenna coupling between the systems, at the minimum distance between the stations on Earth and the Moon, and in the absence of antenna pointing errors and losses such as depolarization and clutter, the interference levels comply with the established ITU-R service protection criteria.

#### A2.3.1.5	Study results

The study was conducted in accordance with the scenarios outlined in Figure A2.3.1.1. For the worst-case analysis, the following assumptions were considered:

–	Main beam antenna coupling between the potentially interfering lunar SRS transmitters and the MS victim receiver.

–	Free space propagation loss assuming the minimum distance between the stations on the Earth and the Moon (355,000 km), while atmospheric, depolarization and clutter losses are not considered;

–	In the present study, it is assumed that both systems operate at the same central frequency. A bandwidth correction factor (CF) is applied when the bandwidth of the potential interferer exceeds the bandwidth of the victim receiver under analysis. If the bandwidth of the interferer is equal to or smaller than that of the victim receiver, the bandwidth correction factor is 0:

CF = 10 * log10bandwidthvictimbandwidthinterferer

–	The aggregate interference-to-noise ratio (I/N) is calculated considering Lunar SRS transmitters pointing towards the MS victim receiver using the following equation:

INTotal= 1n (EIRPnθ,φ+CF-FSPL+Grxθ,φ)-Nrx

where n is the number of lunar surface transmitters;  EIRPnθ,φ is the equivalent isotropic radiated power of each lunar transmitter; FSPL is the free space path loss calculated using Recommendation ITU-R P.525; (Grx)θ,φ is the MS victim receiver maximum antenna gain; and Nrx is the thermal noise power in the MS victim receiver bandwidth; CF is the frequency-dependent rejection based on the transmit and receiver bandwidths;

–	The receiver thermal noise power is calculated from the receiver noise figure (NF) using the formula provided in Recommendation ITU-R P.372-17.

–	The single and aggregate interference results are compared to the MS / PPDR protection criteria to verify whether the service protection criteria is met

Figure A2.3.1.1

SRS–MS Scenario

(a)

(b)

Table A2.3.1.3

SRS – MS co-channel results

| SRS Station | SRS centre frequency | Analysis | I/N result | Margin to the MS/PPDR protection criteria |
| --- | --- | --- | --- | --- |
| Lunar Orbit Space Station Transmitter | 425 MHz | Single Entry | -45.3 dB | 35.3 dB |

#### A2.3.1.6 	Summary and analysis of the results of Study A

The interference-to-noise ratio metric shows that even in the single entry static worst-case analysis, there is a margin of 35.3 dB or more to satisfy the protection criteria for mobile service PPDR applications. This also implies that aggregating over 3 388 simultaneously transmitting worst case towards the Earth interfering transmitters would still meet the mobile service PPDR protection criteria, which far exceeds the expected active number of lunar transmitters in this band. Thus, the results of this static worst-case study suggest that sharing between SRS and terrestrial MS operating in the 420-430 MHz band (co-channel) is feasible.

## A2.[X]	Summary and analysis of the results of studies

[Note: The summary should offer an objective overview, encompassing the results of sharing and compatibility studies, alongside any pertinent information applicable for future comparisons among studies. Additionally, it should address any mitigation techniques or proposals employed to safeguard services operating within the same band or adjacent bands, where applicable, to SRS lunar systems.]

ANNEX 3

Sharing and compatibility studies of SRS for lunar operations
in the frequency band 440-450 MHz

## A3.1	Sharing studies between fixed services and SRS operating in the frequency band 440-450 MHz

### A3.1.1	Study A [USA]

#### A3.1.1.1	Technical and operational characteristics of SRS operating in the 440-450 MHz frequency range

The technical and operational characteristics for SRS systems are based on those available in the Report ITU-R SA.2553, including Table 3.2-4, as summarized in Table A3.1.1.1.

Table A3.1.1.1

Technical Characteristics of the SRS Systems

| Station | Lunar Surface Transmitter |
| --- | --- |
| Frequency range | 440-450 MHz |
| Beam Type | Fixed |
| Polarization | Vertical |
| Peak Gain (dBi) | 0.0 |
| Max. EIRP Density (dBW/Hz) | ‒61 |
| Max. EIRP (dBW) | 2 |
| Antenna Height | < 5 m |
| Channel BW (MHz) | 2 |

#### A3.1.1.2	Technical and operational characteristics of fixed service operating in the 440– 450 MHz frequency range

Document 7B/56 provides relevant technical information to support studies between SRS and Fixed Services (FS) under agenda item 1.15. The characteristics for fixed service (FS) for point-to-point (PP) systems are based on the information contained in Recommendation ITU-R F.758-8, Table 17, as summarized in Table A3.1.1.2.

Table A3.1.1.2

Characteristics of the FS (PP) systems

| Parameter | Value |
| --- | --- |
| Frequency range | 406.1-450 MHz |
| Reference | Recommendation ITU-R F.1567 |
| Modulation format | 32-QAM |
| Receiver noise bandwidth | 3.5 MHz |
| Antenna height | 30 m |
| Antenna pattern | Recommendation ITU-R F.699 |
| Antenna gain | 25 dBi |
| Feeder loss | 2 dB |
| Receiver noise figure | 3.5 dB |
| Protection criteria (I/N) | ‒6 dB |

#### A3.1.1.3	Propagation model

This sharing study includes the lunar SRS system on the surface of the Moon and fixed services operating on Earth's surface. Working Party 3J in Document 7B/58 recommends using the propagation model based on Recommendation ITU-R P.525 for this scenario.

#### A3.1.1.4	Methodology

The methodology employed in this study is based on a static analysis of both single and aggregated worst-case interference. It evaluates whether, even under conditions of perfect antenna coupling between the systems, at the minimum distance between the stations on Earth and the Moon, and in the absence of antenna pointing errors and losses such as depolarization and clutter, the interference levels comply with the established ITU-R service protection criteria.

#### A3.1.1.5	Study Results

The study was conducted in accordance with the scenario outlined in Figure A3.1.1.1. For the worst-case analysis, the following assumptions were considered:

–	Main beam antenna coupling between the potentially interfering lunar SRS transmitters and the FS victim receiver.

–	Free space propagation loss assuming the minimum distance between the stations on the Earth and the Moon (355,000 km), while atmospheric, depolarization and clutter losses are not considered.

–	The aggregate interference-to-noise ratio (I/N) is calculated considering Lunar SRS transmitters pointing towards the FS victim receiver using the following equation:

INTotal= 1n (EIRPnθ,φ-FSPL-Floss+Grxθ,φ)-Nrx

where n is the number of lunar surface transmitters;  EIRPnθ,φ is the equivalent isotropic radiated power of each lunar transmitter; FSPL is the free space path loss calculated using Recommendation ITU-R P.525; Floss is the FS feeder loss; (Grx)θ,φ is the FS victim receiver maximum antenna gain; and Nrx is the thermal noise power in the FS victim receiver bandwidth.

–	The receiver thermal noise power is calculated from the receiver noise figure (NF) using the formula provided in Recommendation ITU-R P.372-17.

–	The single and aggregate I/N results are compared to the FS protection criteria (I/N ≤ −6 dB) to verify whether the FS protection is met.

Figure A3.1.1.1

SRS – FS Scenario

Table A3.1.1.3

SRS - FS co-channel results

| SRS Station | SRS centre frequency | Analysis | I/N result | Margin to the FS protection criteria |
| --- | --- | --- | --- | --- |
| Lunar Surface Transmitter | 425 MHz | Single Entry | −36.3 dB | 30.6 dB |

#### A3.1.1.6	Summary and analysis of the results of Study A

The results of this study suggest that sharing between SRS and ground-based FS operating in the 440-450 MHz band (co-channel) is feasible. The interference-to-noise ratio metric shows that even in the single entry static worst-case analysis, there is a margin of 30.6 dB or more to satisfy the FS protection criteria. That also implies that aggregating over 1100 worst case interfering transmitters would still meet the FS protection criteria, which far exceeds the expected number of lunar transmitters in this band.

### A3.1.2	Study B [D]

#### A3.1.2.1 	Technical and operational characteristics of SRS operating in the 440-450 MHz frequency range

The technical and operational characteristics for SRS systems are based on those available in Report ITU-R SA.2553, including Table 3.2-4, as summarized in Table A3.1.2.1.

Table A3.1.2.1

Technical Characteristics of the SRS Systems

| Station | Lunar Surface-to-Surface Transmitter |
| --- | --- |
| Frequency range | 440-450 MHz |
| Beam Type | Fixed |
| Polarization | Vertical |
| Peak Gain (dBi) | 0.0 |
| Max. EIRP Density (dBW/Hz) | ‒61 |
| Max. EIRP (dBW) | 2 |
| Antenna Height | < 5 m |
| Channel BW (MHz) | 2 |

#### A3.1.2.2 	Technical and operational characteristics of fixed service operating in the 440–450 MHz frequency range

The characteristics for fixed service (FS) for point-to-point (PP) systems are based on the information contained in Recommendation ITU-R F.758-8, Table 17, as summarized in Table A3.1.2.2.

Table A3.1.2.2

Characteristics of the FS (PP) systems

| Parameter | Value | Value |
| --- | --- | --- |
| Frequency range | 406.1-450 MHz | 406.1-450 MHz |
| Reference | Recommendation ITU-R F.1567 | Recommendation ITU-R F.1567 |
| Modulation format | QPSK | 32-QAM |
| Receiver noise bandwidth (MHz) | 0.05, 0.1, 0.15, 0.2, 0.25, 0.3, 0.5, 0.6, 0.75, 1.0, 1.75, 3.5 | 0.05, 0.1, 0.15, 0.2, 0.25, 0.3, 0.5, 0.6, 0.75, 1.0, 1.75, 3.5 |
| Antenna pattern | Recommendation ITU-R F.699 | Recommendation ITU-R F.699 |
| Antenna gain | 25 dBi | 25 dBi |
| Feeder loss | 2 dB | 2 dB |
| Receiver noise figure | 5 dB | 3.5 dB |
| Protection criteria (I/N) | ‒6 dB | ‒6 dB |

#### A3.1.2.3 	Propagation model

This sharing study includes the lunar SRS system on the surface of the Moon and fixed services operating on Earth's surface. Working Party 3J in Document 7B/58 recommends using the propagation model based on Recommendation ITU-R P.525 for this scenario.

#### A3.1.2.4 	Methodology

The methodology employed in this study is based on a static analysis of single-entry worst-case interference. It evaluates whether, even under conditions of perfect antenna coupling between the systems, at the minimum distance between the stations on Earth and the Moon, and in the absence of antenna pointing errors and losses such as depolarization and clutter, the interference levels comply with the established ITU-R service protection criteria.

#### A3.1.2.5 	Study Results

The study was conducted in accordance with the scenario outlined in Figure A3.1.2.1. For the worst-case analysis, the following assumptions were considered:

–	Main beam antenna coupling between the potentially interfering lunar SRS transmitters and the FS victim receiver.

–	Free space propagation loss assuming the minimum distance between the stations on the Earth and the Moon (355,000 km), while atmospheric, depolarization and clutter losses are not considered.

–	The aggregate interference-to-noise ratio (I/N) is calculated considering Lunar SRS transmitters pointing towards the FS victim receiver using the following equation:

INTotal= 1n (EIRPnθ,φ-FSPL-Floss+Grxθ,φ)-Nrx

where n is the number of lunar surface transmitters;  EIRPnθ,φ is the equivalent isotropic radiated power of each lunar transmitter; FSPL is the free space path loss calculated using Recommendation ITU-R P.525; Floss is the FS feeder loss; (Grx)θ,φ is the FS victim receiver maximum antenna gain; and Nrx is the thermal noise power in the FS victim receiver bandwidth.

–	The receiver thermal noise power is calculated from the receiver noise figure (NF) using the formula provided in Recommendation ITU-R P.372-17.

–	The single and aggregate I/N results are compared to the FS protection criteria (I/N ≤ −6 dB) to verify whether the FS protection is met.

Figure A3.1.2.1

SRS – FS Scenario

Table A3.1.2.3

SRS - FS co-channel results

|  |  | FS QPSK |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| bandwidth | MHz | 0.05 | 0.1 | 0.15 | 0.2 | 0.25 | 0.3 | 0.5 | 0.6 | 0.75 | 1 | 1.75 | 3.5 |
| Receiver noise power | dBW | -152 | -149 | -147.2 | -146 | -145 | -144.2 | -142 | -141 | -140.2 | -139 | -136.5 | -133.5 |
| Single Interferer | dBW | -187.4 | -184.4 | -182.6 | -181.4 | -180.4 | -179.6 | -177.4 | -176.6 | -175.6 | -174.4 | -172.0 | -171.4 |
| I/N result | dB | -35.4 | -35.4 | -35.4 | -35.4 | -35.4 | -35.4 | -35.4 | -35.4 | -35.4 | -35.4 | -35.4 | -37.8 |
| Margin to the FS protection criterion | dB | 29.4 | 29.4 | 29.4 | 29.4 | 29.4 | 29.4 | 29.4 | 29.4 | 29.4 | 29.4 | 29.4 | 31.8 |
|  |  | FS 32-QAM |  |  |  |  |  |  |  |  |  |  |  |
| bandwidth | MHz | 0.05 | 0.1 | 0.15 | 0.2 | 0.25 | 0.3 | 0.5 | 0.6 | 0.75 | 1 | 1.75 | 3.5 |
| Receiver noise power | dBW | -153.5 | -150.5 | -148.7 | -147.5 | -146.5 | -145.7 | -143.5 | -142.7 | -141.7 | -140.5 | -138.0 | -135.0 |
| Single Interferer | dBW | -187.4 | -184.4 | -182.6 | -181.4 | -180.4 | -179.6 | -177.4 | -176.6 | -175.6 | -174.4 | -172.0 | -171.4 |
| I/N result | dB | -33.9 | -33.9 | -33.9 | -33.9 | -33.9 | -33.9 | -33.9 | -33.9 | -33.9 | -33.9 | -33.9 | -36.3 |
| Margin to the FS protection criterion | dB | 27.9 | 27.9 | 27.9 | 27.9 | 27.9 | 27.9 | 27.9 | 27.9 | 27.9 | 27.9 | 27.9 | 30.3 |

#### A3.1.2.6 	Summary and analysis of the results of Study B

The interference-to-noise ratio metric shows that in the single entry static worst-case analysis, the FS protection criteria is met with a margin of 27.9 dB or more. The results of this study suggest that protection of FS from SRS in the 440-450 MHz band is ensured.

### A3.1.3	Study C

## A3.2	Sharing studies between radiolocation service and SRS operating in the frequency band 440-450 MHz

### A3.2.1	Study A [USA]

#### A3.2.1.1	Technical and operational characteristics of SRS operating in the 440-450 MHz frequency range

The technical and operational characteristics for SRS systems are based on those available in the Report ITU-R SA.2553, including Table 3.2-4, as summarized in Table A3.2.1.1.

Table A3.2.1.1

Technical Characteristics of the SRS Systems

| Station | Lunar Surface Transmitter |
| --- | --- |
| Frequency range | 440-450 MHz |
| Beam Type | Fixed |
| Polarization | Vertical |
| Peak Gain (dBi) | 0.0 |
| Max. EIRP Density (dBW/Hz) | ‒61 |
| Max. EIRP (dBW) | 2 |
| Antenna Height | < 5 m |
| Channel BW (MHz) | 2 |

#### A3.2.1.2	Technical and operational characteristics of radiolocation service operating in the 440-450 MHz frequency range

Document 7B/114 provides relevant technical information to support studies between SRS and radars operating in the radiolocation service (RLS) under agenda item 1.15. The characteristics for radars systems are based on the Recommendation ITU-R M.1462-1, Tables 1, 3 and 4, as summarized in Table A3.2.1.2.

Table A3.2.1.2

Characteristics of the radar systems

| Parameter | Ground | Airborne | Shipborne |
| --- | --- | --- | --- |
| Frequency range | 420- 450 MHz |  |  |
| Polarization | Circular | Horizontal | - |
| Platform height/altitude | 15 m | Ascent, descent and operational operations. Ceiling is around 9 km | - |
| Antenna gain | 40 dBi | 22 dBi | 30 dBi |
| Antenna scan | Sector ±60° azimuth scan, with rotating or random modes 3-85° elevation | ±60° elevation (mechanically positioned or electronically scanned); 3600 azimuth at 3-7 rpm | 3600 azimuth |
| Receiver noise figure | 2.5 dB | 5 dB | 4.95 dB (based on receiver noise level -136 dBW) |
| Receiver IF bandwidth | 2 MHz | 1 MHz | 2 MHz |
| Protection criteria (I/N) | ‒6 dB |  |  |

#### A3.2.1.3	Propagation model

This sharing study includes the lunar SRS system on the surface of the Moon, and radiolocation service operating on Earth's surface. Working Party 3J in Document 7B/58 recommends using the propagation model based on Recommendation ITU-R P.525 for this scenario.

#### A3.2.1.4	Methodology

The methodology employed in this study is based on a static analysis of both single and aggregated worst-case interference. It evaluates whether, even under conditions of perfect antenna coupling between the systems, at the minimum distance between the stations on Earth and the Moon, and in the absence of antenna pointing errors and losses such as depolarization and clutter, the interference levels comply with the established ITU-R service protection criteria.

#### A3.2.1.5	Study results

The study was conducted in accordance with the scenarios outlined in Figure A3.2.1.1. For the worst-case analysis, the following assumptions were considered:

–	Main beam antenna coupling between the potentially interfering lunar SRS transmitters and the radar victim receiver.

–	Free space propagation loss assuming the minimum distance between the stations on the Earth and the Moon (355,000 km for ground and shipborne radars and 354,991 km for airborne), while atmospheric, depolarization and clutter losses are not considered.

–	In the present study, it is assumed that both systems operate at the same central frequency. A bandwidth correction factor (CF) is applied when the bandwidth of the potential interferer exceeds the bandwidth of the victim receiver under analysis. If the bandwidth of the interferer is equal to or smaller than that of the victim receiver, the bandwidth correction factor is 0:

CF = 10 * log10bandwidthvictimbandwidthinterferer

–	The aggregate interference-to-noise ratio (I/N) is calculated considering three Lunar SRS transmitters pointing towards the radar victim receiver using the following equation:

INTotal= 1n (EIRPnθ,φ+CF-FSPL+Grxθ,φ)-Nrx

where n is the number of lunar surface transmitters;  EIRPnθ,φ is the equivalent isotropic radiated power of each lunar transmitter; FSPL is the free space path loss calculated using Rec. ITU-R P.525; (Grx)θ,φ is the radar victim receiver maximum antenna gain; and Nrx is the thermal noise power in the radar victim receiver bandwidth.

–	The receiver thermal noise power is calculated from the receiver noise figure (NF) using the formula provided in Recommendation ITU-R P.372-17.

–	The single and aggregate I/N results are compared to the radar protection criteria (I/N ≤ −6 dB) to verify whether the radiolocation service protection is met.

Figure A3.2.1.1

SRS–RLS Scenarios: (a) Ground and shipborne; (b) Airborne

(a)

(b)

Table A3.2.1.3

SRS – RLS co-channel results

| SRS Station | SRS centre frequency | Radar | Analysis | I/N result | Margin to the radar protection criteria |
| --- | --- | --- | --- | --- | --- |
| Lunar Surface Transmitter | 445 MHz | Ground | Single Entry | −15.8 dB | 9.8 dB |
| Lunar Surface Transmitter | 445 MHz | Airborne | Single Entry | −36.3 dB | 30.3 dB |
| Lunar Surface Transmitter | 445 MHz | Shipborne | Single Entry | −28.3 dB | 22.3 dB |

#### A3.2.1.6	Summary and analysis of the results of Study A

The interference-to-noise ratio metric shows that in the static worst-case analysis for a single potential interferer, there is a positive margin of 9.8 dB or more with respect to the radiolocation service protection criteria. The results of this study suggest that protection of radars operating in the radiolocation service in the 440-450 MHz band (co-channel) from SRS lunar surface transmitters interference is ensured.

## A3.3	Sharing studies between mobile service (MS) and SRS operating in the frequency band 440-450 MHz

### A3.3.1	Study A [USA]

#### A3.3.1.1	Technical and operational characteristics of SRS operating in the 440-450 MHz frequency range

The technical and operational characteristics for SRS systems are based on those available in the Report ITU-R SA.2553, including Table 3.2-4, as summarized in Table A3.3.1.1.

Table A3.3.1.1

Technical Characteristics of the SRS Systems

| Station | Lunar Surface Transmitter |
| --- | --- |
| Frequency range | 440-450 MHz |
| Beam Type | Fixed |
| Polarization | Vertical |
| Peak Gain (dBi) | 0.0 |
| Max. EIRP Density (dBW/Hz) | ‒61 |
| Max. EIRP (dBW) | 2 |
| Antenna Height | < 5 m |
| Channel BW (MHz) | 2 |

#### A3.3.1.2	Technical and operational characteristics of mobile service operating in the 440-450 MHz frequency range

Document 7B/115 provides relevant technical information to support studies between SRS and mobile service under Agenda Item 1.15. The technical characteristics and protection criteria used in this study were extracted from Recommendation ITU-R M.1808 (System A in Table 1C) for conventional and trunked land mobile systems in the 406.1 – 470 MHz band.  For public protection and disaster relief (PPDR) applications in this band, an I/N of -10 dB is used.

Table A3.3.1.2

Characteristics of the MS Systems

| Parameter | Mobile service receiver (System A) |
| --- | --- |
| Frequency range | 406.1-470 MHz |
| Receiver noise figure (dB) | 6 to 12 dB |
| Antenna gain | 0 to 11 dBd |
| Receiver bandwidth | 5.5 kHz |
| Protection criteria (I/N) | -10 dB (PPDR applications) |

#### A3.3.1.3	Propagation model

This sharing study includes the lunar SRS system on the surface of the Moon and mobile services (conventional and trunked land mobile) operating on Earth's surface. Therefore, Working Party 3J in Document 7B/58 recommends using the propagation model based on Recommendation ITU-R P.525 for this scenario.

#### A3.3.1.4	Methodology

The methodology employed in this study is based on a static analysis of both single and aggregated worst-case interference. It evaluates whether, even under conditions of perfect antenna coupling between the systems, at the minimum distance between the stations on Earth and the Moon, and in the absence of antenna pointing errors and losses such as depolarization and clutter, the interference levels comply with the established ITU-R service protection criteria.

#### A3.3.1.5	Study results

The study was conducted in accordance with the scenarios outlined in Figure A3.3.1.1. For the worst-case analysis, the following assumptions were considered:

–	Main beam antenna coupling between the potentially interfering lunar SRS transmitters and the MS victim receiver.

–	Free space propagation loss assuming the minimum distance between the stations on the Earth and the Moon (355,000 km), while atmospheric, depolarization and clutter losses are not considered;

–	In the present study, it is assumed that both systems operate at the same central frequency. A bandwidth correction factor (CF) is applied when the bandwidth of the potential interferer exceeds the bandwidth of the victim receiver under analysis. If the bandwidth of the interferer is equal to or smaller than that of the victim receiver, the bandwidth correction factor is 0:

CF = 10 * log10bandwidthvictimbandwidthinterferer

–	The aggregate interference-to-noise ratio (I/N) is calculated considering Lunar SRS transmitters pointing towards the MS victim receiver using the following equation:

INTotal= 1n (EIRPnθ,φ+CF-FSPL+Grxθ,φ)-Nrx

where n is the number of lunar surface transmitters;  EIRPnθ,φ is the equivalent isotropic radiated power of each lunar transmitter; FSPL is the free space path loss calculated using Recommendation ITU-R P.525; (Grx)θ,φ is the MS victim receiver maximum antenna gain; and Nrx is the thermal noise power in the MS victim receiver bandwidth; CF is the frequency-dependent rejection based on the transmit and receiver bandwidths;

–	The receiver thermal noise power is calculated from the receiver noise figure (NF) using the formula provided in Recommendation ITU-R P.372-17.

–	The single and aggregate interference results are compared to the MS / PPDR protection criteria to verify whether the service protection criteria is met

Figure A3.3.1.1

SRS–MS Scenario

(a)

(b)

Table A3.3.1.3

SRS – MS co-channel results

| SRS Station | SRS centre frequency | Analysis | I/N result | Margin to the MS/PPDR protection criteria |
| --- | --- | --- | --- | --- |
| Lunar Orbit Space Station Transmitter | 445 MHz | Single Entry | -46.3 dB | 36.3 dB |

#### A3.3.1.6 	Summary and analysis of the results of Study A

The interference-to-noise ratio metric shows that even in the single entry static worst-case analysis, there is a margin of 36.3 dB or more to satisfy the protection criteria for mobile service PPDR applications. This also implies that aggregating over 4 265 simultaneously transmitting worst case towards the Earth interfering transmitters would still meet the mobile service PPDR protection criteria, which far exceeds the expected active number of lunar transmitters in this band. Thus, the results of this static worst-case study suggest that sharing between SRS and terrestrial MS operating in the 440-450 MHz band (co-channel) is feasible.

## A3.[X]	Summary and analysis of the results of studies

[Note: The summary should offer an objective overview, encompassing the results of sharing and compatibility studies, alongside any pertinent information applicable for future comparisons among studies. Additionally, it should address any mitigation techniques or proposals employed to safeguard services operating within the same band or adjacent bands, where applicable, to SRS lunar systems.]

ANNEX 4

Sharing and compatibility studies of SRS for lunar operations
 in the frequency band 2 400-2 690 MHz

[Note: Insert relevant sharing and compatibility studies for each of the identified service.]

## A4.1	Sharing studies between fixed services and SRS for lunar operations in the frequency band 2 400-2 690 MHz

### A4.1.1	Study A [USA]

#### A4.1.1.1	Technical and operational characteristics of SRS operating in the 2 400-2 690 MHz frequency range

The technical and operational characteristics for SRS systems are based on those available in the Report ITU-R SA.2553, including Table 3.2-4, as summarized in Table A4.1.1.1.

Table A4.1.1.1

Technical Characteristics of the SRS Systems

| Station | Lunar Surface-to-Surface Transmitter |  |
| --- | --- | --- |
| Frequency range | 2 400-2 483.5 MHz | 2 500-2 690 MHz |
| Polarization | Vertical | Linear ±45º |
| Peak Gain (dBi) | 7 | 16 |
| EIRP Density (dBW/MHz) | 19 | 19 |
| Max. EIRP (dBW) | 6 | 29 |
| Channel BW (MHz) | 20 | 10 |

#### A4.1.1.2	Technical and operational characteristics of fixed service operating in the 2 400-2 690 MHz frequency range

Document 7B/56 provides relevant technical information to support studies between SRS and Fixed Services (FS) under agenda item 1.15. The characteristics for fixed service (FS) for point-to-point (PP) and point-to-multipoint (PmP) systems are based on the information contained in the draft revision of Recommendation ITU-R F.758-7, Tables 17 and 20, as summarized in Table A4.1.1.2.

Table A4.1.1.2

Characteristics of the FS systems

| Parameter | Point-to-point (PP) | Point-to-multipoint (PmP) |
| --- | --- | --- |
| Frequency range | 2290-2670 MHz | 1350-2690 MHz |
| Reference | Recommendation ITU-R F.1243 | Recommendation ITU-R F.701 |
| Modulation format | MSK | QPSK |
| Receiver noise bandwidth | 14 MHz | 3.5 MHz |
| Antenna height | 50 m | 30 m |
| Antenna pattern | Recommendation ITU-R F.699 | Recommendation ITU-R F.699 |
| Antenna gain | 25 dBi | 27 dBi |
| Feeder loss | 4 dB | 0 dB |
| Receiver noise figure | 4 dB | 3.5 dB |
| Protection criteria (I/N) | −6 dB |  |

#### A4.1.1.3	Propagation model

This study includes the lunar SRS system orbiting the Moon and fixed services operating on Earth's surface. Working Party 3J in Document 7B/58 recommends using the propagation model based on Recommendation ITU-R P.525 for this scenario.

#### A4.1.1.4	Methodology

The methodology employed in this study is based on a static analysis of both single and aggregated worst-case interference. It evaluates whether, even under conditions of perfect antenna coupling between the systems, at the minimum distance between the stations on Earth and the Moon, and in the absence of antenna pointing errors and losses such as depolarization and clutter, the interference levels comply with the established ITU-R service protection criteria.

#### A4.1.1.5	Study Results

The study was conducted in accordance with the scenario outlined in Figure A4.1.1.1. For the worst-case analysis, the following assumptions were considered:

–	Main beam antenna coupling between the potentially interfering lunar SRS transmitters and the FS victim receiver.

–	Free space propagation loss assuming the minimum distance between the stations on the Earth and the Moon (355,000 km), while atmospheric, depolarization and clutter losses are not considered.

–	In the present study, it is assumed that both systems operate at the same central frequency. A bandwidth correction factor (CF) is applied when the bandwidth of the potential interferer exceeds the bandwidth of the victim receiver under analysis. If the bandwidth of the interferer is equal to or smaller than that of the victim receiver, the bandwidth correction factor is 0:

CF = 10 * log10bandwidthvictimbandwidthinterferer

–	The aggregate interference-to-noise ratio (I/N) is calculated considering Lunar SRS transmitters pointing towards the FS victim receiver using the following equation:

INTotal= 1n (EIRPnθ,φ+CF-FSPL-Floss+Grxθ,φ)-Nrx

where n is the number of lunar surface transmitters;  EIRPnθ,φ is the equivalent isotropic radiated power of each lunar transmitter; FSPL is the free space path loss calculated using Recommendation ITU-R P.525; Floss is the FS feeder loss; (Grx)θ,φ is the FS victim receiver maximum antenna gain; and Nrx is the thermal noise power in the FS victim receiver bandwidth.

–	The receiver thermal noise power is calculated from the receiver noise figure (NF) using the formula provided in Recommendation ITU-R P.372-17.

–	The single and aggregate I/N results are compared to the FS protection criteria (I/N ≤ −6 dB) to verify whether the FS protection is met.

Figure A4.1.1.1

SRS–FS Scenario

Table A4.1.1.3

SRS – FS (PP) co-channel results

| SRS Station | SRS centre frequency | Analysis | I/N result | Margin to the FS protection criteria |
| --- | --- | --- | --- | --- |
| Lunar Surface Transmitter | 2 410 MHz | Single Entry | –57.0 dB | 51.0 dB |
| Lunar Surface Transmitter | 2 510 MHz | Single Entry | –32.8 dB | 26.8 dB |

Table A4.1.1.4

SRS – FS (PmP) co-channel results

| SRS Station | SRS centre frequency | Analysis | I/N result | Margin to the FS protection criteria |
| --- | --- | --- | --- | --- |
| Lunar Surface Transmitter | 2 410 MHz | Single Entry | –50.5 dB | 44.5 dB |
| Lunar Surface Transmitter | 2 510 MHz | Single Entry | –24.9 dB | 18.9 dB |

#### A4.1.1.6	Conclusion of Study A

The interference-to-noise ratio metric shows that in the single entry static worst-case analysis, there is a margin of 18.9 or more to satisfy the FS protection criteria.

[USA Note: An updated analysis will include the aggregate impact of lunar emissions.]

### A4.1.2	Study B [D]

#### A4.1.2.1 	Technical and operational characteristics of SRS operating in the 2 400-2 690 MHz frequency range

The technical and operational characteristics for SRS systems are based on those available in Report ITU-R SA.2553, including Table 3.2-4, as summarized in Table A4.1.2.1.

Table A4.1.2.1

Technical Characteristics of the SRS Systems

| Station | Lunar Surface-to-Surface Transmitter |  |
| --- | --- | --- |
| Frequency range | 2 400-2 483.5 MHz | 2 500-2 690 MHz |
| Polarization | Vertical | Linear ±45º |
| Peak Gain (dBi) | 7 | 16 |
| EIRP Density (dBW/MHz) | 19 | 19 |
| Max. EIRP (dBW) | 6 | 29 |
| Channel BW (MHz) | 20 | 10 |

#### A4.1.2.2 	Technical and operational characteristics of fixed service operating in the 2 400-2 690 MHz frequency range

The characteristics for fixed service (FS) for point-to-point (PP) and point-to-multipoint (PmP) systems are based on the information contained in Recommendation ITU-R F.758-8, Tables 17 and 20, as summarized in Table A4.1.2.2.

Table A4.1.2.2

Characteristics of the FS systems

| Parameter | Point-to-point (PP) | Point-to-multipoint (PmP) |
| --- | --- | --- |
| Frequency range | 2 290-2 670 MHz | 1 350-2 690 MHz |
| Reference | Recommendation ITU-R F.1243 | Recommendation ITU-R F.701 |
| Modulation format | MSK | QPSK |
| Receiver noise bandwidth (MHz) | 0.25, 0.5, 1.0, 1.75, 2.0, 2.5, 3.5, 7.0, 14.0 | 2.0, 3.5 |
| Antenna pattern | Recommendation ITU-R F.699 | Recommendation ITU-R F.699 |
| Antenna gain (dBi) | 25 | 17, 27 |
| Feeder loss | 4 dB | 0 dB |
| Receiver noise figure | 4 dB | 3.5 dB |
| Protection criteria (I/N) | ‒6 dB |  |

#### A4.1.2.3 	Propagation model

This study includes the lunar SRS system orbiting the Moon and fixed services operating on Earth's surface. Working Party 3J in Document 7B/58 recommends using the propagation model based on Recommendation ITU-R P.525 for this scenario.

#### A4.1.2.4 	Methodology

The methodology employed in this study is based on a static analysis of single-entry worst-case interference. It evaluates whether, even under conditions of perfect antenna coupling between the systems, at the minimum distance between the stations on Earth and the Moon, and in the absence of antenna pointing errors and losses such as depolarization and clutter, the interference levels comply with the established ITU-R service protection criteria.

#### A4.1.2.5 	Study Results

The study was conducted in accordance with the scenario outlined in Figure A4.1.2.1. For the worst-case analysis, the following assumptions were considered:

–	Main beam antenna coupling between the potentially interfering lunar SRS transmitters and the FS victim receiver.

–	Free space propagation loss assuming the minimum distance between the stations on the Earth and the Moon (355,000 km), while atmospheric, depolarization and clutter losses are not considered.

–	In the present study, it is assumed that both systems operate at the same central frequency. A bandwidth correction factor (CF) is applied when the bandwidth of the potential interferer exceeds the bandwidth of the victim receiver under analysis. If the bandwidth of the interferer is equal to or smaller than that of the victim receiver, the bandwidth correction factor is 0:

CF = 10 * log10bandwidthvictimbandwidthinterferer

–	The aggregate interference-to-noise ratio (I/N) is calculated considering Lunar SRS transmitters pointing towards the FS victim receiver using the following equation:

INTotal= 1n (EIRPnθ,φ+CF-FSPL-Floss+Grxθ,φ)-Nrx

where n is the number of lunar surface transmitters;  EIRPnθ,φ is the equivalent isotropic radiated power of each lunar transmitter; FSPL is the free space path loss calculated using Recommendation ITU-R P.525; Floss is the FS feeder loss; (Grx)θ,φ is the FS victim receiver maximum antenna gain; and Nrx is the thermal noise power in the FS victim receiver bandwidth.

–	The receiver thermal noise power is calculated from the receiver noise figure (NF) using the formula provided in Recommendation ITU-R P.372-17.

–	The single and aggregate I/N results are compared to the FS protection criteria (I/N ≤ −6 dB) to verify whether the FS protection is met.

Figure A4.1.2.1

SRS–FS Scenario

Table A4.1.2.3

SRS – FS (PP) co-channel results (2 410 MHz)

|  |  | FS MSK |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| bandwidth | MHz | 0.25 | 0.5 | 1.0 | 1.75 | 2.0 | 2.5 | 3.5 | 7.0 | 14.0 |
| Receiver noise power | dBW | -146 | -143 | -140 | -137.5 | -137 | -136 | -134.5 | -131.5 | -128.5 |
| Single Interferer | dBW | -190.1 | -187.1 | -184.1 | -181.6 | -181.2 | -180.1 | -178.6 | -175.6 | -172.6 |
| I/N result | dB | -44.1 | -44.1 | -44.1 | -44.1 | -44.1 | -44.1 | -44.1 | -44.1 | -44.1 |
| Margin to the FS protection criterion | dB | 38.1 | 38.1 | 38.1 | 38.1 | 38.1 | 38.1 | 38.1 | 38.1 | 38.1 |

Table A4.1.2.4

SRS – FS (PmP) co-channel results (2 410 MHz)

|  |  | FS QPSK |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Victim antenna gain | dBi | 17 | 17 | 27 | 27 |
| bandwidth | MHz | 2 | 3.5 | 2 | 3.5 |
| Receiver noise power | dBW | -137.5 | -135.0 | -137.5 | -135.0 |
| Single Interferer | dBW | -185.0 | -182.6 | -175.0 | -172.6 |
| I/N result | dB | -47.6 | -47.6 | -37.6 | -37.6 |
| Margin to the FS protection criterion | dB | 41.6 | 41.6 | 31.6 | 31.6 |

Table A4.1.2.5

SRS – FS (PP) co-channel results (2 510 MHz)

|  |  | FS MSK |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| bandwidth | MHZ | 0.25 | 0.5 | 1.0 | 1.75 | 2.0 | 2.5 | 3.5 | 7.0 | 14.0 |
| Receiver noise power | dBW | -146 | -143 | -140 | -137.5 | -137 | -136 | -134.5 | -131.5 | -128.5 |
| Single Interferer | dBW | -184.5 | -181.4 | -178.4 | -176.0 | -175.4 | -174.4 | -173.0 | -170.0 | -167.0 |
| I/N result | dB | -38.5 | -38.5 | -38.5 | -38.5 | -38.5 | -38.5 | -38.5 | -38.5 | -38.5 |
| Margin to the FS protection criterion | dB | 32.5 | 32.5 | 32.5 | 32.5 | 32.5 | 32.5 | 32.5 | 32.5 | 32.5 |

Table A4.1.2.6

SRS – FS (PmP) co-channel results (2 510 MHz)

|  |  | FS QPSK |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Victim antenna gain | dBi | 17 | 17 | 27 | 27 |
| bandwidth | MHz | 2 | 3.5 | 2 | 3.5 |
| Receiver noise power | dBW | -137.5 | -135.0 | -137.5 | -135.0 |
| Single Interferer | dBW | -179.4 | -177.0 | -169.4 | -177.0 |
| I/N result | dB | -42.0 | -42.0 | -32.0 | -32.0 |
| Margin to the FS protection criterion | dB | 36.0 | 36.0 | 26.0 | 26.0 |

#### A4.1.2.6 Summary and analysis of results of Study B

The interference-to-noise ratio metric shows that in the single entry static worst-case analysis, the FS protection criteria is met with a margin of 26.0 dB or more. The results of this study suggest that protection of FS from SRS in the 2 400-2 690 MHz band is ensured.

### A4.1.3	Study C

#### A4.2	Sharing studies between mobile service operating as a terrestrial component of IMT and SRS operating in the frequency band 2 400-2 690 MHz

### A4.2.1	Study A [USA]

#### A4.2.1.1	Technical and operational characteristics of SRS operating in the 2 500-2 690 MHz frequency range

The technical and operational characteristics for SRS systems are based on those available in the Report ITU-R SA.2553, including table 3.2-4, as summarized in Table A4.2.1.1.

Table A4.2.1.1

Technical Characteristics of the SRS Systems

| Station | Lunar Surface-to-Surface Transmitter |
| --- | --- |
| Frequency range | 2 500-2 690 MHz |
| Polarization | Linear ±45º |
| Peak Gain (dBi) | 16 |
| EIRP Density (dBW/MHz) | 19 |
| Max. EIRP (dBW) | 29 |
| Channel BW (MHz) | 10 |

#### A4.2.1.2	Technical and operational characteristics of mobile service operating as a terrestrial component of IMT in the 2 500-2 690 MHz frequency range

Document 7B/105 provides relevant technical information to support studies between SRS and mobile services operating as a terrestrial component of IMT, under agenda item 1.15. The characteristics and protection criteria for terrestrial component of IMT are based on the information contained in the Annex 4.4 to Document 5D/716, as summarized in Tables A4.2.1.2 and A4.2.1.3.

Table A4.2.1.2

IMT Base Station (BS) parameters for bands between 2 and 3 GHz

|  | Rural/ Urban/suburban macro |
| --- | --- |
| Antenna height | 30 m rural /20 m urban /25 m suburban |
| AAS Antenna gain | 26.2 dBi |
| AAS BS antenna polarization | Linear ±45 degrees |
| Channel bandwidth | 10 MHz |
| AAS BS Noise Figure | 5 dB |

Table A4.2.1.3

IMT User Equipment (UE) parameters for bands between 2 and 3 GHz

|  | Rural/Urban/suburban macro |
| --- | --- |
| UE height | 1.5 m |
| Typical antenna gain | −3 dBi |
| Body loss | 4 dB |
| UE Noise Figure | 9 dB |

For this study, the AAS rural/urban/suburban macro scenario represents the worst case, as the base station antenna gain is higher than in non-AAS case.

#### A4.2.1.3	Propagation model

This sharing study includes the lunar SRS system on the surface of the Moon, and mobile services operating on Earth's surface. Working Party 3J in Document 7B/58 recommends using the propagation model based on Recommendation ITU-R P.525 for this scenario.

#### A4.2.1.4	Methodology

The methodology employed in this study is based on a static analysis of both single and aggregated worst-case interference. It evaluates whether, even under conditions of perfect antenna coupling between the systems, at the minimum distance between the stations on Earth and the Moon, and in the absence of antenna pointing errors and losses such as depolarization, body, building and clutter, the interference levels comply with the established ITU-R service protection criteria.

The study also considered the IMT network in TDD to examine the lower frequencies for both base stations and user equipment. However, no TDD factor was applied in this analysis to compute the interference received by the base station and user equipment 100% of the time, in order to verify compliance with the established ITU-R service protection criteria, even in the worst-case scenario.

#### A4.2.1.5	Study results

The study was conducted in accordance with the scenario outlined in Figure A4.2.1.1. For the worst-case analysis, the following assumptions were considered:

–	Main beam antenna coupling between the potentially interfering lunar SRS transmitters and the IMT victim receiver.

–	Free space propagation loss assuming the minimum distance between the stations on the Earth and the Moon (355,000 km), while atmospheric, depolarization and clutter losses are not considered.

–	The aggregate interference-to-noise ratio (I/N) is calculated considering Lunar SRS transmitters pointing towards the IMT victim receiver using the following equation:

INTotal= 1n (EIRPnθ,φ-FSPL+Grxθ,φ)-Nrx

where n is the number of lunar surface transmitters;  EIRPnθ,φ is the equivalent isotropic radiated power of each lunar transmitter; FSPL is the free space path loss calculated using Rec. ITU-R P.525; (Grx)θ,φ is the IMT victim receiver maximum antenna gain; and Nrx is the thermal noise power in the IMT victim receiver bandwidth.

–	The receiver thermal noise power is calculated from the receiver noise figure (NF) using the formula provided in Recommendation ITU-R P.372-17.

–	The worst case aggregate static interference is calculated based on the lunar surface deployment characteristics provided in Report ITU-R SA.2553. Notably the lunar surface deployment in the 2 500–2 690 MHz band assumes 1-3 LCT base stations per cluster with up to 10 clusters located on the Moon and a network activity factor of 50%. Assuming that roughly half of the clusters on the Moon will be visible to Earth, then the aggregate static interference calculation assumes that up to 8 LCT base station transmitters could be active simultaneously.

–	The single and aggregate I/N results are compared to the IMT protection criteria (I/N ≤ −6 dB) to verify whether the IMT protection is met.

Figure A4.2.1.1

SRS–IMT Scenario

Table A4.2.1.4

SRS – IMT co-channel results

| SRS Station | SRS centre frequency | IMT Station | Analysis | I/N result | Margin to the IMT protection criteria |
| --- | --- | --- | --- | --- | --- |
| Lunar Surface Transmitter | 2 510 MHz | Base Station | Single Entry | –27.2 dB | 21.2 dB |
|  |  |  | Aggregate | −18.2 dB | 12. 2 dB |
| Lunar Surface Transmitter | 2 510 MHz | User Equipment | Single Entry | –60.4 dB | 54.4 dB |
|  |  |  | Aggregate | −51.4 dB | 45.4 dB |

#### A4.2.1.6	Summary and analysis of the results of Study A

The interference-to-noise ratio metric shows that in the single entry static worst-case analysis, the IMT Base Station protection criteria is met with a margin of 21.2 dB or more and the IMT User Equipment (UE) protection criteria is met with a margin of 54.4 dB or more. For the aggregate worst-case analysis, the interference margin is 12.2 dB for the IMT base station and 45.4 dB for the IMT UE. The results of this study suggest that protection of terrestrial IMT systems from SRS in the 2 500-2 690 MHz band is ensured.

### A4.2.2	Study B [KOR]

#### A4.2.2.1 	Methodology

A worst-case static analysis was performed for both single and aggregate interference originating from the lunar surface. The worst-case assumptions include: maximum EIRP at the lunar surface directed toward Earth, free-space path loss calculated for the shortest distance between the Moon’s and Earth’s surfaces, no polarization loss, receivers at the Earth pointed toward the Moon, etc.

The worst-case interference-to-noise ratio (I/N) that terrestrial IMT systems could experience from transmissions on the lunar surface was calculated as follows:

IN dB=A-B+C-D

where A is the maximum (aggregate) EIRP from the interferer at the lunar surface, B is the minimum propagation loss from the lunar surface to the Earth, and C and D are the receive antenna gain and the noise power of the terrestrial IMT system. The values for A, B, C, and D under this interference scenario are derived in the following sub-sections.

For the study, a carrier frequency of 2 545 MHz (centre of the spectrum) and a channel bandwidth of 10 MHz for the interferer were assumed.

#### A4.2.2.2 	Technical and operational characteristics of SRS operating in the 2500-2690 MHz frequency range – A in the equation above

The interferer for this scenario is the lunar surface-to-surface transmitter, and its characteristics can be found at Table 3.2-4 in report ITU-R SA.2553, and part of the table is shown below.

Table A4.2.2.1

Part of Table 3.2.4 Lunar surface-to-surface transmit characteristics by frequency bands from Report ITU-R SA.2553 - Technical and operational characteristics for space research systems in the vicinity of the Moon

|  | 2 500-2 690 MHz |  |
| --- | --- | --- |
| Max EIRP (dBW) | 29 | -2 |
| Channel BW (MHz) | 10 | 0.128 |
| EIRP Density dB(W/MHz) | 19 | 7 |
| Deployment characteristics Coverage area Deployment density Activity factor (average) | Wide area (10 km radius) 1-3 LCTs and 1-6 EVAs per cell 20%, 50% | Small area (1 km radius) 1 Lander, 1 Rover 50% |

According to the table above, A=29 dBW can be considered as the maximum EIRP from a single interferer at the lunar surface considering the wide area deployment. It is also noted in the report ITU-R SA.2553 that the number of active clusters at any given time is no more than 10, while the deployment densities and activity factors of LCTs and EVAs are given in the table above. Based on these numbers, 30 can be estimated as the maximum number of simultaneous transmissions from the lunar surface. For the aggregate case this gives A=29 dBW+10log30 dB=43 dBW maximum EIRP from the lunar surface.

#### A4.2.2.3 	Propagation loss – B in the equation above

Working Party 3J in Document 7B/58 recommends using the propagation model based on Recommendation ITU-R P.525 for this scenario. Given the minimum distance between the stations on the Earth and the Moon (355,000 km), the free-space basic transmission loss can be calculated according to (6) of Recommendation ITU-R P.525 as

Lbf=32.4+20×logf+20×logd    (dB)

where f=2 545 (MHz)  and d=355,000 (km). Using these values, B=211.5 dB can be determined.

#### A4.2.2.4 	Technical and operational characteristics of mobile service operating as a terrestrial component of IMT in the 2500-2690 MHz frequency range – C in the equation above

The characteristics and protection criteria for terrestrial component of IMT are based on the information contained in the Annex 4.4 to Document 5D/716. Table 9 and 5-2 provide the receive antenna parameters of AAS antenna used at the terrestrial IMT base station and UE, respectively.

Part of Table 9, where the IMT base station receive antenna characteristics can be found is summarized as follows.

Table A4.2.2.2

Part of Table 9 Beamforming antenna characteristics for IMT in 1 710-4 990 MHz
 from Annex 4.4 to Document 5D/716

|  |  | Rural macro | Suburban macro | Urban macro | Urban small cell (outdoor)/Micro cell | Indoor (small cell) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Base station antenna characteristics |  |  |  |  |  |
| 1.2 | Element gain (dBi) (Note 1) | 6.4 | 6.4 | 6.4 | 6.4 | N/A |
| 1.6 | Antenna array configuration (Row × Column) (Note 2) | 4 × 8 elements | 4 × 8 elements | 4 × 8 elements | 8 × 8 elements | N/A |
| 1.7a | Number of element rows in sub-array, Msub | 3 | 3 | 3 | N/A | N/A |
| Note 1: The element gain in row 1.2 includes the loss given in row 1.8 and is per polarization. This means that this parameter in row 1.8 is not needed for the calculation of the BS composite antenna gain and e.i.r.p. Note 2: For the small/micro cell case, 8 × 8 means there are 8 vertical and 8 horizontal radiating elements. For the extended AAS model case, 4 × 8 means there are 4 vertical and 8 horizontal radiating sub-arrays. |  |  |  |  |  |  |

For rural macro, suburban macro, and urban macro deployment scenario, it can be assumed that an AAS in an IMT base station has 4×8×3 =96 antenna elements and each antenna element has a gain of 6.4 dBi. This leads to the receive antenna gain of IMT base station  CBS=6.4+10×log96=26.2 dBi.

Part of Table 5-2 where the IMT UE receive antenna characteristics can be found is summarized as follows.

TABLE A4.2.2.3

Part of Table 5-2 UE parameters for bands between 1 and 3 GHz from Annex 4.4 to Document 5D/716

|  | Rural macro | Urban/suburban macro | Small cell (outdoor)/Micro cell | Indoor (small cell) |
| --- | --- | --- | --- | --- |
| User terminal characteristics |  |  |  |  |
| Typical antenna gain for user terminals | −3 dBi | −3 dBi | −3 dBi | −3 dBi |
| Body loss | 4 dB | 4 dB | 4 dB | 4 dB |

Taking the table above into consideration, the receive antenna gain of IMT UE is led to

CUE=-3 dBi-4 dB=-7 dBi.

#### A4.2.2.5 	Noise characteristics of IMT systems – D in the equation above

According to Recommendation ITU-R P.372-17, the receiver thermal noise power is calculated as,

D=NF+kTb.

Noise figure (NF) is 5 dB and 9 dB for AAS IMT base station and IMT UE, respectively, according to Table 1 of Annex 4.4 to Document 5D/716. 5 dB noise figure for AAS IMT base station is the smallest (therefore the worst-case assumption) noise figure among several deployment scenarios. k is the Boltzmann’s constant of 1.38 ×10-23 J/K, T0 is the reference temperature (K) taken as 290 K, and B is the noise power bandwidth of the receiving system (Hz). As we assume the channel bandwidth of the interferer as 10 MHz, which is b=107. Taking these into consideration,

DBS=5+10 ×log1.38×10-23×290×107= -129 dBW

for IMT base station and DUE=-125 dBW for IMT UE.

#### A4.2.2.6 	Study results and conclusions

Based on the above results – A, B, C and D calculated in the previous sub-sections, I/N for IMT base station is calculated as

IN=29-211.5+26.2+129= -27.3 dB

for a single interferer and -12.5 dB for maximum aggregate interferers. Similarly, I/N for IMT UE is calculated as

IN=29-211.5-7+125= -64.5 dB

for a single interferer and -49.7 dB for maximum aggregate interferers.

Table A4.2.2.4

SRS – IMT co-channel results

| SRS Station | SRS centre frequency/ Channel Bandwidth | IMT Station on the Earth | Number of Interferers on the Lunar Surface | I/N result |
| --- | --- | --- | --- | --- |
| Lunar Surface Transmitter | 2 545 MHz/10 MHz | Base Station | 1 | –27.3 dB |
|  |  | Base Station | 30 | –12.5 dB |
|  |  | User Equipment | 1 | –64.5 dB |
|  |  | User Equipment | 30 | –49.7 dB |

Considering the IMT protection criterion of −6 dB I/N, the results show a sufficient positive margin across all four scenarios. It should be noted that the AAS receive antenna gain of the IMT base station (26.2 dBi) assumes beamforming toward the intended transmitter. Since the intended transmitter is located on the Earth’s surface, the AAS receive antenna gain in the direction of a lunar interferer would be significantly lower—by at least 20 dB. This reduction provides sufficient I/N margin to account for aggregate interference originating from the lunar surface.

### A4.2.3	Study C

## A4.3	Sharing studies between mobile satellite service (MSS) and SRS operating in the frequency band 2 483.5-2 500 MHz

### A4.3.1	Study A [USA]

#### A4.3.1.1	Technical and operational characteristics of SRS operating in the 2483.5-2500 MHz frequency range

The technical and operational characteristics for SRS systems are based on those available in the Report ITU-R SA.2553, including Table 3.3-4, as summarized in Table A4.3.1.1.

Table A4.3.1.1

Technical Characteristics of the SRS Systems

| Station | Lunar Orbit Space Station Transmitter |
| --- | --- |
| Frequency range | 2 483.5-2 500 MHz |
| Polarization | CP |
| Peak Gain (dBi) | 16 |
| Max EIRP Density (dBW/Hz) | −48 |
| Max. EIRP (dBW) | 24 |
| Orbital Characteristics Apolune (km) Perilune (km) | 7 000-11 000 600-2 700 |
| Channel BW (MHz) | 16 |

#### A4.3.1.2	Technical and operational characteristics of Mobile Satellite Service (MSS) operating in the 2 483.5-2 500 MHz frequency range

Document 7B/106 provides relevant technical information to support studies between SRS and Mobile Satellite Service (MSS) (space-to-Earth) under agenda item 1.15, with the characteristics and protection criteria summarized in Table A4.3.1.2.

Table A4.3.1.2

MSS (space-to-Earth) receiver characteristics in 2 483.5-2 500 MHz

|  | System 1 (HIBLEO-X) | System 2 |
| --- | --- | --- |
| Height of receivers | 1.5 m | 1.5 m |
| Centre frequency | 2 484.39 MHz | 2 485.75 MHz |
| Bandwidth | 1.23 MHz | 4.5 MHz |
| Terminal noise temperature | N = −113.9 dBm/MHz | N = −100.9 dBm/MHz |
| Antenna gain | 2.4 dBi | −1 dBi |
| Protection criteria (I/N) | −10 dB | −10 dB |
| Location | Suburban / Rural | Suburban / Rural |

#### A4.3.1.3	Propagation model

This sharing study includes the lunar SRS system in orbit of the Moon, and MSS operating on Earth's surface. Working Party 3J in Document 7B/58 recommends using the propagation model based on Recommendation ITU-R P.525 for this scenario.

#### A4.3.1.4	Methodology

The methodology employed in this study is based on a static analysis of both single and aggregated worst-case interference. It evaluates whether, even under conditions of perfect antenna coupling between the systems, at the minimum distance between the stations on Earth and the Moon, and in the absence of antenna pointing errors and losses such as depolarization and clutter, the interference levels comply with the established ITU-R service protection criteria.

#### A4.3.1.5	Study results

The study was conducted in accordance with the scenario outlined in Figure A4.3.1.1. For the worst-case analysis, the following assumptions were considered:

–	Main beam antenna coupling between the potentially interfering lunar SRS transmitters and the MSS victim receiver.

–	Free space propagation loss assuming the minimum distance between the stations on the Earth and the Moon (355,000 km), while atmospheric, depolarization and clutter losses are not considered.

–	In the present study, it is assumed that both systems operate at the same central frequency. A bandwidth correction factor (CF) is applied when the bandwidth of the potential interferer exceeds the bandwidth of the victim receiver under analysis. If the bandwidth of the interferer is equal to or smaller than that of the victim receiver, the bandwidth correction factor is 0:

CF = 10 * log10bandwidthvictimbandwidthinterferer

–	The aggregate interference-to-noise ratio (I/N) is calculated considering Lunar SRS transmitters pointing towards the MSS victim receiver using the following equation:

INTotal= 1n (EIRPnθ,φ+CF-FSPL+Grxθ,φ)-Nrx

where n is the number of lunar surface transmitters;  EIRPnθ,φ is the equivalent isotropic radiated power of each lunar transmitter; FSPL is the free space path loss calculated using Rec. ITU-R P.525; (Grx)θ,φ is the MSS victim receiver maximum antenna gain; and Nrx is the thermal noise power in the MSS victim receiver bandwidth

–	The receiver thermal noise power is calculated from the receiver noise figure (NF) using the formula provided in Recommendation ITU-R P.372-17.

–	The single and aggregate I/N results are compared to the MSS protection criteria (I/N ≤ −10 dB) to verify whether the MSS protection is met.

Figure A4.3.1.1

SRS–MSS (space-to-Earth) Scenario

Table A4.3.1.3

SRS – MSS (space-to-Earth) co-channel results

| SRS Station | SRS centre frequency | MSS Station | Analysis | I/N result | Margin to the MSS protection criteria |
| --- | --- | --- | --- | --- | --- |
| Lunar Orbit Space Transmitter | 2 491.75 MHz | System 1 | Single Entry | –53.0 dB | 43.0 dB |
| Lunar Orbit Space Transmitter | 2 491.75 MHz | System 2 | Single Entry | –69.4 dB | 59.4 dB |

#### A4.3.1.6	Summary and analysis of the results of Study A

The results of this study suggest that sharing between SRS and MSS (space-to-Earth) operating in the 2 483.5-2 500 MHz band (co-channel), is feasible. The interference-to-noise ratio metric shows that in the single entry static worst-case analysis, there is a margin of 43 dB or more to satisfy the MSS protection criteria. This implies that the aggregate interference from 19 952 lunar transmitters would not exceed the MSS protection criterion in this band.

## A4.4	Sharing studies between high-altitude platform stations as IMT base stations (HIBS) and SRS operating in the frequency band 2 500-2 690 MHz

### A4.4.1	Study A [USA]

#### A4.4.1.1	Technical and operational characteristics of SRS operating in the 2 500-2 690 MHz frequency range

The technical and operational characteristics for SRS systems are based on those available in the new report ITU-R SA.2553, including Table 3.2-4, as summarized in Table A4.4.1.1.

Table A4.4.1.1

Technical Characteristics of the SRS Systems

| Station | Lunar Surface-to-Surface Transmitter |
| --- | --- |
| Frequency range | 2 500-2 690 MHz |
| Polarization | Linear ±45º |
| Peak Gain (dBi) | 16 |
| EIRP Density (dBW/MHz) | 19 |
| Max. EIRP (dBW) | 29 |
| Channel BW (MHz) | 10 |

#### A4.4.1.2	Technical and operational characteristics of high-altitude platform stations as IMT base stations (HIBS) in the 2500-2690 MHz frequency range

Document 7B/64 provides the characteristics and protection criteria to support studies between SRS and HIBS under agenda item 1.15. The technical and operational characteristics are summarized in Table A4.4.1.2.

Table A4.4.1.2

Specification and deployment related parameters of HIBS

| Parameter | Band 3 (2 500-2 690 MHz) |
| --- | --- |
| Duplex method and transmission direction | FDD/TDD (as per frequency arrangements in Recommendation ITU-R M.1036) |
| Channel bandwidth (MHz) | 20 MHz |
| HIBS Platform Altitude | 18-25 km |
| Number of cells/HIBS | 7 |
| Antenna gain | 14 dBi (1st layer cell), 17 dBi (2nd layer cell) |
| Noise Figure | 5 dB |
| Protection criteria (I/N) | −6 dB |

#### A4.4.1.3	Propagation model

This sharing study includes the lunar SRS system on the surface of the Moon, and HIBS operating on Earth's surface. Working Party 3J in Document 7B/58 recommends using the propagation model based on Recommendation ITU-R P.525 for this scenario.

#### A4.4.1.4	Methodology

The methodology employed in this study is based on a static analysis of both single and aggregated worst-case interference. It evaluates whether, even under conditions of perfect antenna coupling between the systems, at the minimum distance between the stations on Earth and the Moon, and in the absence of antenna pointing errors and losses such as depolarization and atmospheric, the interference levels comply with the established ITU-R service protection criteria.

#### A4.4.1.5	Study results

The study was conducted in accordance with the scenario outlined in Figure A4.4.1.1. For the worst-case analysis, the following assumptions were considered:

–	Main beam antenna coupling between the potentially interfering lunar SRS transmitters and the HIBS victim receiver.

–	For the HIBS victim receiver, the highest antenna gain of 17 dBi was considered, which corresponds to the second-layer cell.

–	Free space propagation loss assuming the minimum distance between the stations on the Earth and the Moon (354,975 km), while atmospheric and depolarization and are not considered.

–	The aggregate interference-to-noise ratio (I/N) is calculated considering Lunar SRS transmitters pointing towards the HIBS victim receiver using the following equation:

INTotal= 1n (EIRPnθ,φ-FSPL+Grxθ,φ)-Nrx

where n is the number of lunar surface transmitters;  EIRPnθ,φ is the equivalent isotropic radiated power of each lunar transmitter; FSPL is the free space path loss calculated using Rec. ITU-R P.525; (Grx)θ,φ is the HIBS victim receiver maximum antenna gain; and Nrx is the thermal noise power in the HIBS victim receiver bandwidth.

–	The receiver thermal noise power is calculated from the receiver noise figure (NF) using the formula provided in Recommendation ITU-R P.372-17.

–	The single and aggregate I/N results are compared to the HIBS protection criteria (I/N ≤ −6 dB) to verify whether the HIBS protection is met.

Figure A4.4.1.1

SRS–HIBS Scenario

Table A4.4.1.3

SRS – HIBS co-channel results

| SRS Station | SRS centre frequency | Analysis | I/N result | Margin to the HIBS protection criteria |
| --- | --- | --- | --- | --- |
| Lunar Surface Transmitter | 2520 MHz | Single Entry | −39.4 dB | 33.4 dB |

#### A4.4.1.6	Summary and analysis of the results of Study A

The results of this study suggest that sharing between SRS and HIBS in the 2500-2690 MHz band (co-channel), is feasible. The interference-to-noise ratio metric shows that in the single entry static worst-case analysis, there is a margin of 33.4 dB or more to satisfy the HIBS protection criteria. This implies that the aggregate interference from 2187 worst case lunar transmitters would not exceed the HIBS protection criterion in this band.

## A4.5	Compatibility studies between aeronautical radionavigation service (ARNS) operating in the frequency band 2 700-2900 MHz and SRS operating in the frequency band 2 500-2 690 MHz

### A4.5.1	Study A

This study shows the static worst case interference analysis for adjacent band compatibility between ARNS in 2 700 – 2 900 MHz and lunar surface SRS in the frequency band 2 500-2 690 MHz.

#### A4.5.1.1	Lunar SRS Unwanted Emissions

Due to the 10 MHz frequency separation between the ARNS and lunar SRS bands, the spurious emissions limit for 3GPP stations is assumed. Thus, the unwanted emissions level for the lunar SRS transmitter in 2 700 – 2 900 MHz used in this analysis is −13 dBm/MHz.

#### A4.5.1.2	ARNS radar characteristics

The characteristics of different types of ARNS radars in 2 700 – 2 900 is found in Recommendation ITU-R M.1464-2, and a summary of some of the relevant parameters is given in the table below. A protection criteria of -10 dB I/N for the ARNS radars is used in this analysis.

Table A4.5.1.1

Characteristics of ARNS radars in frequency band 2 700 – 2 900 MHz

|  | Radar A | Radar B | Radar C | Radar D | Radar E | Radar F |
| --- | --- | --- | --- | --- | --- | --- |
| Transmit power into antenna (kW) | 1400 | 1320 | 25 | 450 | 22 | 70 |
| Antenna main beam gain (dBi) | 33.5 | 33.5 | 34 | 32.8 | 34.3 | 33.5 |
| Receiver bandwidth (MHz) | 13 | 0.7 | 1.1 |  | 1.2 | 4 |
| Receiver noise figure (dB) | 4 | 4 | 3.3 | 2.7 | 2.1 | 2 |

#### A4.5.1.3	Study results

A summary of the I/N results for the ARNS receiver using worst-case static analysis is given in Table A4.5.1.2 below. Due to the Moon-Earth distance and the spurious emissions limit for the lunar SRS transmitter, the calculated interference is significantly lower than the protection criterion of −10 dB I/N.

TABLE A4.5.1.2

Summary of Lunar SRS-ARNS interference results in 2700-2900 MHz

|  | Radar A | Radar B | Radar C | Radar D | Radar E | Radar F |
| --- | --- | --- | --- | --- | --- | --- |
| I/N (dB) | −66.5 | −66.5 | −66.3 | −65.9 | −63.8 | −64.5 |
| Protection Criteria (dB) | −10 | −10 | −10 | −10 | −10 | −10 |
| Interference Margin (dB) | 56.5 | 56.5 | 56.3 | 55.9 | 53.8 | 54.5 |

### A4.5.1.4	Summary and analysis of the results of Study A

The results of this study suggest that lunar SRS operating in the 2 500 – 2 690 MHz band and ARNS in the near adjacent 2 700 - 2 900 MHz band are compatible.  The interference-to-noise ratio metric shows that in the single entry static worst-case analysis, there is a margin of 53.8 dB or more to satisfy the ARNS protection criteria.

## A4.6	Sharing studies between fixed satellite service (FSS) and SRS operating in the frequency band 2 500-2 690 MHz

### A4.6.1	Study A [USA]

In Document 7B/200, WP 4A expressed the view that sharing studies to ensure protection of FSS are not required in the 2 500 – 2 690 MHz band, provided that possible future allocations to space research service (space-to-space) in the 2 500 – 2 690 MHz band under WRC-27 agenda item 1.15 are limited to Lunar surface-to-surface communications, as specified in Report ITU-R SA.2553.

## A4.6	Sharing studies between broadcast satellite service (BSS) and SRS operating in the frequency band 2 500-2 690 MHz

### A4.6.1	Study A [USA]

In Document 7B/200, WP 4A expressed the view that sharing studies to ensure protection of BSS are not required in the 2 500 – 2 690 MHz band, provided that possible future allocations to space research service (space-to-space) in the 2 500 – 2 690 MHz band under WRC-27 agenda item 1.15 are limited to Lunar surface-to-surface communications, as specified in Report ITU-R SA.2553.

## A4.7.1 	Sharing studies between radiodetermination satellite service (RDSS) and SRS operating in the frequency band 2 483.5-2 500 MHz

### A4.7.1	Study A [USA]

#### A4.7.1.1	Technical and operational characteristics of SRS operating in the 2483.5-2500 MHz frequency range

The technical and operational characteristics for SRS systems are based on those available in the Report ITU-R SA.2553, including Table 3.3-4, as summarized in Table A4.7.1.1.

Table A4.7.1.1

Technical Characteristics of the SRS Systems

| Station | Lunar Orbit Space Station Transmitter |
| --- | --- |
| Frequency range | 2 483.5-2 500 MHz |
| Polarization | CP |
| Peak Gain (dBi) | 16 |
| Max EIRP Density (dBW/Hz) | −48 |
| Max. EIRP (dBW) | 24 |
| Orbital Characteristics Apolune (km) Perilune (km) | 7 000-11 000 600-2 700 |
| Channel BW (MHz) | 16 |

#### A4.7.1.2	Technical and operational characteristics of radiodetermination satellite service (RDSS) operating in the 2 483.5-2 500 MHz frequency range

Document 7B/196 provides guidance from WP 4C to support studies between SRS and RDSS under agenda item 1.15.  Specifically, WP 4C noted that there are currently no ITU-R Recommendations regarding the technical and operational parameters, nor protection criteria, for RDSS systems in the 2 483.5-2 500 MHz band.  For purposes of this study, the characteristics and interference threshold level of a representative RDSS system used in this study are summarized in Table A4.7.1.2.

Table A4.7.1.2

RDSS (space-to-Earth) receiver characteristics in 2 483.5-2 500 MHz

| Station |  | RDSS receiver |
| --- | --- | --- |
| Centre frequency (MHz) |  | 2491.75 |
| Bandwidth (MHz) |  | 16.5 |
| Receiver noise figure |  | 2 dB |
| Receive antenna gain |  | 2.6 dBi |
| Interference threshold (I/N) |  | -6 dB |

#### A4.7.1.3	Propagation model

This sharing study includes the lunar SRS system in orbit of the Moon, and RDSS operating on Earth's surface. Working Party 3J in Document 7B/58 recommends using the propagation model based on Recommendation ITU-R P.525 for this scenario.

#### A4.7.1.4	Methodology

The methodology employed in this study is based on a static analysis of both single and aggregated worst-case interference. It evaluates whether, even under conditions of perfect antenna coupling between the systems, at the minimum distance between the stations on Earth and the Moon, and in the absence of antenna pointing errors and losses such as depolarization and clutter, the interference levels comply with the established ITU-R service protection criteria.

#### A4.7.1.5	Study results

The study was conducted in accordance with the scenario outlined in Figure A4.7.1.1. For the worst-case analysis, the following assumptions were considered:

–	Main beam antenna coupling between the potentially interfering lunar SRS transmitters and the RDSS victim receiver.

–	Free space propagation loss assuming the minimum distance between the stations on the Earth and the Moon (355,000 km), while atmospheric, depolarization and clutter losses are not considered.

–	In the present study, it is assumed that both systems operate at the same central frequency. A bandwidth correction factor (CF) is applied when the bandwidth of the potential interferer exceeds the bandwidth of the victim receiver under analysis. If the bandwidth of the interferer is equal to or smaller than that of the victim receiver, the bandwidth correction factor is 0:

CF = 10 * log10bandwidthvictimbandwidthinterferer

–	The aggregate interference-to-noise ratio (I/N) is calculated considering Lunar SRS transmitters pointing towards the RDSS victim receiver using the following equation:

INTotal= 1n (EIRPnθ,φ+CF-FSPL+Grxθ,φ)-Nrx

where n is the number of lunar surface transmitters;  EIRPnθ,φ is the equivalent isotropic radiated power of each lunar transmitter; FSPL is the free space path loss calculated using Rec. ITU-R P.525; (Grx)θ,φ is the RDSS victim receiver maximum antenna gain; and Nrx is the thermal noise power in the RDSS victim receiver bandwidth

–	The receiver thermal noise power is calculated from the receiver noise figure (NF) using the formula provided in Recommendation ITU-R P.372-17.

Figure A4.7.1.1

SRS–RDSS (space-to-Earth) Scenario

Table A4.7.1.3

SRS – RDSS (space-to-Earth) co-channel results

| SRS Station | SRS centre frequency |  | Analysis |  | I/N result |  | Margin to the -6 dB I/N threshold |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Lunar Orbit Space Transmitter | 2 491.75 MHz |  | Single Entry |  | -54.7 dB |  | 48.7 dB |  |

#### A4.7.1.6	Summary and analysis of the results of Study A

The results of this study suggest that sharing between SRS and RDSS in the 2483.5-2500 MHz band (co-channel), is feasible. The interference-to-noise ratio metric shows that in the single entry static worst-case analysis, there is a margin of 48.7 dB or more to satisfy the -6 dB I/N interference threshold. This implies that the aggregate interference from 74131 worst case lunar transmitters would not exceed the -6 dB I/N threshold in this band.

## A4.8	Sharing and compatibility of the radiolocation service and SRS for lunar operations in the frequency band 2 400-2 483.5 MHz

### A4.8.1	Study A [USA]

#### A4.8.1.2	Technical and operational characteristics of SRS operating in the 2 400 - 2 483.5 MHz frequency range

The technical and operational characteristics for SRS systems are based on those available in the Report ITU-R SA.2553, as summarized below.

Table A4.8.1.1

Technical Characteristics of the Lunar SRS Systems

| Station | Lunar Surface-to-Surface Transmitter |
| --- | --- |
| Frequency range | 2 400 – 2 483.5 MHz |
| Polarization | Vertical |
| Peak Gain (dBi) | 7 |
| EIRP Density (dBW/MHz) | -7 |
| Max. EIRP (dBW) | 6 |
| Channel BW (MHz) | 20 MHz |

#### A4.8.1.2	Technical and operational characteristics of radiolocation service operating in the 2 400-2 500 MHz frequency range

[TBD]

#### A4.8.1.3	Propagation model

This analysis uses the propagation model based on Recommendation ITU-R P.525 for this scenario, as advised by Working Party 3J in Document 7B/58.

#### A4.8.1.4	Methodology

The methodology employed in this study is based on a static analysis of both single and aggregated worst-case interference. It evaluates whether, even under conditions of perfect antenna coupling between the systems, at the minimum distance between the stations on Earth and the Moon, and in the absence of antenna pointing errors and losses such as depolarization and clutter, the interference levels comply with the established ITU-R service protection criteria.

#### A4.8.1.5	Study Results

For the worst-case analysis, the following assumptions were considered:

–	Main beam antenna coupling between the potentially interfering lunar SRS transmitters and the radar victim receiver.

–	Free space propagation loss assuming the minimum distance between the stations on the Earth and the Moon (355,000 km for ground and shipborne radars and 354,991 km for airborne), while atmospheric, depolarization and clutter losses are not considered.

–	In the present study, it is assumed that both systems operate at the same central frequency. A bandwidth correction factor (CF) is applied when the bandwidth of the potential interferer exceeds the bandwidth of the victim receiver under analysis. If the bandwidth of the interferer is equal to or smaller than that of the victim receiver, the bandwidth correction factor is 0:

CF = 10 * log10bandwidthvictimbandwidthinterferer

–	The interference-to-noise ratio (I/N) is calculated considering Lunar SRS transmitter pointing towards the radar victim receiver using the following equation:

INTotal= 1n (EIRPnθ,φ+CF-FSPL+Grxθ,φ)-Nrx

where n is the number of lunar surface transmitters;  EIRPnθ,φ is the equivalent isotropic radiated power of each lunar transmitter; FSPL is the free space path loss calculated using Rec. ITU-R P.525; (Grx)θ,φ is the radar victim receiver maximum antenna gain; and Nrx is the thermal noise power in the radar victim receiver bandwidth.

–	The receiver thermal noise power is calculated from the receiver noise figure (NF) using the formula provided in Recommendation ITU-R P.372-17.

–	The single I/N result is compared to the radar protection criteria (I/N ≤ 6 dB) to verify whether the radiolocation service protection is met.

Figure A4.8.1.1

SRS–RLS Scenarios

(a)

SRS–RLS Scenario

Table A4.8.1.2

SRS – RLS co-channel results

| SRS Station | SRS centre frequency | Radar | Analysis | I/N result | Margin to the RLS protection criteria |
| --- | --- | --- | --- | --- | --- |
| Lunar Surface Transmitter | 2450 MHz | Land-based | Single Entry | TBD | TBD |

#### A4.8.1.6	Summary and analysis of the results of Study A

TBD

## A4.8.1 	Compatibility studies between SRS operating in the frequency band 2 500-2 690 MHz and radio astronomy service in the adjacent frequency band 2 690-2 700 MHz

### A4.8.1	Study A [USA]

### This study examines compatibility between lunar SRS transmitters in the frequency band 2 500-2 690 MHz and radio astronomy receivers located on Earth operating in the adjacent frequency band 2 690-2 700 MHz.

#### A4.8.1.1	Technical and operational characteristics of SRS operating in the 2 500-2 690 MHz frequency range

The technical and operational characteristics for SRS systems are based on those available in the Report ITU-R SA.2553, including Table 3.3-4, as summarized in Table A4.8.1.1.  The SRS unwanted emissions used in this study are based on the unwanted emission levels for 3GPP base stations, as shown in Figure A4.6.1.1.

Table A4.8.1.1

Technical Characteristics of the SRS Systems

| Station | Lunar Surface-to-Surface Transmitter |
| --- | --- |
| Frequency range | 2 500-2 690 MHz |
| Polarization | Linear ±45º |
| Peak Gain (dBi) | 16 |
| Conducted unwanted emission level in 2 690-2 700 MHz | –21.54 dBW/10 MHz |

#### A4.8.1.2	Technical and operational characteristics of radio astronomy service operating in the 2 690-2 700 MHz frequency range

Document 7B/101 provides relevant technical information to support studies between SRS and radio astronomy service under Agenda Item 1.15. The technical characteristics and protection criteria used in this study were extracted from Recommendation ITU-R RA.769.

Table A4.8.1.2

Characteristics of the RAS Systems

| Parameter | RAS receiver |
| --- | --- |
| Frequency range | 2 690 – 2 700 MHz |
| Antenna gain (sidelobe) | 0 dBi |
| Receiver noise temperature (K) | 10 |
| Threshold interference level (input power to RAS receiver) | -207 dBW |
| Receiver bandwidth (MHz) | 10 |

#### A4.8.1.3	Propagation model

This sharing study includes the lunar SRS transmitter operating on the surface of the Moon, and a radio astronomy receiver operating on Earth's surface. Working Party 3J in Document 7B/58 recommends using the propagation model based on Recommendation ITU-R P.525 for this scenario.

#### A4.8.1.4	Methodology

The methodology employed in this study is based on a static analysis of both single and aggregated worst-case interference. It evaluates whether, even under conditions of perfect antenna coupling between the systems, at the minimum distance between the stations on Earth and the Moon, and in the absence of antenna pointing errors and losses such as depolarization and clutter, the interference levels comply with the established ITU-R service protection criteria.

#### A4.8.1.5	Study results

The study was conducted in accordance with the scenarios outlined in Figure A4.8.1.1. For the worst-case analysis, the following assumptions were considered:

–	Antenna coupling between the potentially interfering lunar SRS transmitter main beam and the victim RAS receiver antenna sidelobe.

–	Free space propagation loss assuming the minimum distance between the stations on the Earth and the Moon (355,000 km), while atmospheric, depolarization and clutter losses are not considered;

–	In the present study, it is assumed that both systems operate at the same central frequency. A bandwidth correction factor (CF) is applied when the bandwidth of the potential interferer exceeds the bandwidth of the victim receiver under analysis. If the bandwidth of the interferer is equal to or smaller than that of the victim receiver, the bandwidth correction factor is 0:

CF = 10 * log10bandwidthvictimbandwidthinterferer

–	The single and aggregate interference results are compared to the RAS protection criteria to verify whether the service protection criteria is met

Figure A4.8.1.1

SRS–RAS Scenario

(a)

Table A4.8.1.3

SRS – RAS co-channel results

| SRS Station | SRS centre frequency | Analysis | Received interference power level | Margin to the RAS protection criteria |
| --- | --- | --- | --- | --- |
| Lunar Surface Transmitter | 2685 MHz | Single Entry | -233.6 dBW | 26.6 dB |

#### A4.8.1.6 	Summary and analysis of the results of Study A

[TBD]

## A4.9	Compatibility studies between the radio astronomy service (RAS) operating within the Shielded Zone of the Moon (SZM) in the 2690–2700 MHz band and SRS operating in the frequency band 2500–2690 MHz

### A4.9.1	Study A

This study presents a static analysis of single-entry and aggregated worst-case interference for adjacent-band compatibility between a RAS station operating within the Shielded Zone of the Moon (SZM) in the 2 690–2 700 MHz band and a lunar surface SRS station operating in the 2 500–2 690 MHz band.

#### A4.9.1.1	Technical and operational characteristics of SRS operating in the 2 500-2 690 MHz frequency range

The technical and operational characteristics of SRS systems operating on the lunar surface are based on the information available in ITU-R Report SA.2553-0, including Table 6, and are summarized in Table A4.9.1.1.

Table A4.9.1.1

Technical Characteristics of SRS Systems Operating on the Lunar Surface

| Station | Lunar Surface-to-Surface Transmitter |
| --- | --- |
| Frequency range | 2 500-2 690 MHz |
| Polarization | Linear ±45º |
| Peak Gain (dBi) | 16 |
| Max. EIRP (dBW) | 29 |
| Channel BW (MHz) | 10 |

The SRS unwanted emissions used in this study are based on the unwanted emission levels for 3GPP base stations. Figure A4.9.1.1 illustrates the SRS transmitter spectrum mask, considering the operational centre frequency of 2 695 MHz with a bandwidth of 10 MHz, adjacent to the RAS-allocated band from 2 690 to 2 700 MHz.

Figure A4.9.1.1

SRS Transmit Spectrum Mask

#### A4.9.1.2	Technical and operational characteristics of radio astronomy service operating in the 2 690-2 700 MHz frequency range

The characteristics for RAS and the protection criteria are based on Document 7B/193, which defines the threshold levels of interference to radio astronomy in the SZM. This document provides the equations used to calculate the applicable threshold values needed to protect radio astronomy observations in the SZM. The antenna gain used to assess potential interference into a RAS station is based on Recommendation ITU-R RA.769-2, Section 1.3, Antenna response pattern. The relevant characteristics are summarized in Table A4.9.1.2.

TABLE A4.9.1.2

Characteristics of the RAS

| Parameter | Value |
| --- | --- |
| Band of operation | 2 690-2 700 MHz |
| Bandwidth | 10 MHz |
| Antenna height | 20 m |
| Antenna pattern | Isotropic |
| Antenna gain (side-lobe) | 0 dBi |
| Antenna noise temperature | 5.12 K |
| Receiver noise temperature | 10 K |
| Protection criteria (input power) | −222 dBW |

#### A4.9.1.3	Propagation model

This compatibility study considers the lunar SRS system deployed on the surface of the Moon and the RAS operating within the SZM, both situated at the lunar surface. The analysis employs the propagation model described in Recommendation ITU-R P.2170-0.

#### A4.9.1.4	Methodology

The methodology employed in this study is based on a static analysis of both single-entry and aggregated worst-case interference. It assesses the minimum required separation distance between systems to ensure that the interference levels comply with the RAS protection criteria, even under conditions of perfect antenna coupling between the systems and without considering antenna-pointing errors or losses such as depolarization.

#### A4.9.1.5	Study results

For the worst-case interference analysis, the following assumptions were considered:

–	The analysis was performed using the maximum SRS transmitter antenna gain.

–	The propagation model from Rec. ITU-R P.2170-0 was applied, considering the input parameters listed in Table A4.9.1.3.

–	The unwanted emission level of the lunar transmitter is determined from its spectrum mask and integrated over the RAS bandwidth to calculate the total conducted power entering the RAS adjacent channel. The resulting total conducted unwanted emission per transmitter is –21.54 dBW/10 MHz.

–	The aggregate interference level (I) is calculated considering 3 Lunar SRS transmitters pointing towards the RAS victim receiver using the following equation:

ITotal= 1n(Unwanted emissionsn+GSRS Txθ,φ-Path loss+GRAS rxθ,φ)

where n is the number of lunar surface transmitters; Unwanted emissionsn is the conducted transmitted unwanted emissions in the RAS 10 MHz bandwidth of each lunar transmitter; GSRS Txθ,φ is the SRS lunar transmitter maximum antenna gain (peak), Path loss is the propagation loss calculated using Rec. ITU-R P.2170-0, and (Grx)θ,φ is the RAS victim receiver antenna gain.

–	For the aggregated worst case, all three transmitters were assumed to vary in azimuth while maintaining the same distance between each transmitter and the RAS station, with each transmitter pointed toward the RAS victim receiver.

–	The single and aggregate computed interference received level results are compared to the RAS protection criteria to verify the required minimum separation distance between both systems to ensure the interference level below the RAS protection criteria.

TABLE A4.9.1.3

Parameters used in the propagation model based on Rec. ITU-R P.2170-0

| Parameter | Value |
| --- | --- |
| Lunar SRS transmitter height | 50 m |
| RAS station receiver height | 20 m |
| Lunar SRS transmitter position | Fixed |
| RAS station receiver position | Fixed |
| Lunar terrain irregularity | 0 m (smooth) |
| Lunar terrain permittivity | 2 |
| Lunar terrain conductivity | 0.005 S/m |

Considering the evaluated scenarios, the required propagation losses to meet the RAS protection criteria in the SZM are 216.46 dB and 221.23 dB for single-entry and aggregated analyses, respectively. Table A4.9.1.4 presents the corresponding minimum separation distances between the systems to ensure compatibility.

Table A4.9.1.4

SRS - RAS adjacent-channel results

| SRS Station | SRS centre frequency | RAS frequency range | Analysis | Required separation distance |
| --- | --- | --- | --- | --- |
| Lunar Surface Transmitter | 2685 MHz | 2690-2700 MHz | Single Entry | 62.6 km |
| Lunar Surface Transmitter | 2685 MHz | 2690-2700 MHz | Aggregate | 65.6 km |

### A4.9.1.6	Summary and analysis of the results of Study A

The results of the study, considering single-entry and aggregated worst-case assumptions, indicate that for a lunar surface transmitter in the 2500–2690 MHz band, operating in the space research service (SRS), compatibility with the radio astronomy service (RAS) in the adjacent 2690–2700 MHz band, operating within the Shielded Zone of the Moon (SZM), is satisfied when there is a minimum separation distance of 65.6 km between the systems.

[USA Note:  Further studies of in-band sharing with radioastronomy service in the SZM are needed]

## A4.[X]	Summary and analysis of the results of studies

[Note: The summary should offer an objective overview, encompassing the results of sharing and compatibility studies, alongside any pertinent information applicable for future comparisons among studies. Additionally, it should address any mitigation techniques or proposals employed to safeguard services operating within the same band or adjacent bands, where applicable, to SRS lunar systems.]

ANNEX 5

Sharing and compatibility studies of SRS for lunar operations
in the frequency band 3 500-3 800 MHz

## A5.1	Sharing and compatibility of fixed service and SRS for lunar operations in the frequency band 3 500-3 800 MHz

[Note: Insert relevant sharing and compatibility studies for each of the identified service.]

### A5.1.1	Study A [USA]

#### A5.1.1.1	Technical and operational characteristics of SRS operating in the 3500-3800 MHz frequency range

The technical and operational characteristics for SRS systems are based on those available in the Report ITU-R SA.2553, including table 3.2-4, as summarized in Table A5.1.1.1.

Table A5.1.1.1

Technical Characteristics of the SRS Systems

| Station | Lunar Surface-to-Surface Transmitter |
| --- | --- |
| Frequency range | 3500 – 3800 MHz |
| Polarization | Linear ±45º |
| Peak Gain (dBi) | 26.2 |
| EIRP Density (dBW/MHz) | 22.3 |
| Max. EIRP (dBW) | 42.3 |
| Channel BW (MHz) | 100 MHz |

#### A5.1.1.2	Technical and operational characteristics of fixed service operating in the 3500– 3800 MHz frequency range

Document 7B/56 provides relevant technical information to support studies between SRS and Fixed Services (FS) under agenda item 1.15. The characteristics for fixed service (FS) for point-to-point (PP) systems are based on the information contained in the draft revision of Recommendation ITU-R F.758-7, Table 18, as summarized in Table A5.1.1.2.

Table A5.1.1.2

Characteristics of the FS (PP) systems

| Parameter | Value |
| --- | --- |
| Frequency range | 3600 - 4200 MHz |
| Reference | Recommendation ITU-R F.635 |
| Modulation format | 64 QAM |
| Receiver noise bandwidth | 10 MHz |
| Antenna height | 30 m |
| Antenna pattern | Recommendation ITU-R F.699 |
| Antenna gain | 42 dBi |
| Feeder loss | 0 dB |
| Receiver noise figure | 3 dB |
| Long Term Protection criteria (I/N) | −20 dB @ 20% percent of time (Recommendation ITU-R F.758) |

#### A5.1.1.3	Propagation model

This sharing study includes the lunar SRS system on the surface of the Moon, and fixed services operating on Earth's surface. Working Party 3J in Document 7B/58 recommends using the propagation model based on Recommendation ITU-R P.525 for this scenario.

#### A5.1.1.4	Methodology

The methodology employed in this study is based on a dynamic analysis of single-entry interference. This analysis investigates the communication link between the SRS lunar transmitter, which may potentially cause interference in a FS PP receiver on Earth, and a receiver that could be an astronaut, lunar terrain vehicle, or lander. The SRS antenna pointing is varied across different elevation angles and azimuths, considering the location of the lunar receiver relative to the SRS lunar transmitter. The SRS lunar transmitter is positioned at the Moon's South Pole, a potential candidate location for NASA's Artemis Mission.

The analysis incorporates deployment considerations based on Recommendation ITU-R F.2086 for FS. The simulation accounts for the orbits of the Earth and Moon.

Considering orbital dynamics and system deployment characteristics, the simulation varies several parameters to compute the interference received by the FS receiver. The simulation spans a 5-year period, with a step size of 10 seconds, accounting for FS visibility to the Moon transmitter and potential antenna pointing configurations based on orbital dynamics. The complementary cumulative distribution function (CCDF) of interference over noise ratio (INR) is computed and compared with the protection criteria specified in ITU-R for FS. It is important to note that the simulation does not account for Earth or Moon terrain obstructions or clutter losses.

#### A5.1.1.5	Study results

The study was conducted based on the assumptions below:

–	The position of the SRS Lunar transmitter is at a latitude of -84.201560 and a longitude of -60.699890, as shown in Figure A5.1.1.1, while the lunar receiver is positioned within a 360-degree azimuth range, at a maximum distance of 6 km from the transmitter.

–	The FS transmitter is located at a latitude of 34.039810 and a longitude of -117.772100, and the FS receiver is located at a latitude of 34.201010 and a longitude of -118.170010 as shown in Figure A5.1.1.1.

–	The FS link distance is 40.5 km, the FS antenna height is 39 m, and the elevation angle is - 0.2 degrees, all within the range for FS deployment scenarios as specified in Recommendation ITU-R F.2086.

–	The FS antenna pattern is based on Recommendation ITU-R F.699, while the SRS Lunar transmitter antenna pattern is based on Radio Regulations Appendix 8, Annex 3.

–	The SRS antenna elevation varies from 0 to -30 degrees, and the azimuth spans from 0 to 360 degrees, following the position of the lunar receiver. The antenna patterns are presented in Figure A5.1.1.2.

–	In the present study, it is assumed that both systems operate at the same central frequency. A bandwidth correction factor (CF) is applied when the bandwidth of the potential interferer exceeds the bandwidth of the victim receiver under analysis. If the bandwidth of the interferer is equal to or smaller than that of the victim receiver, the bandwidth correction factor is 0:

CF = 10 * log10bandwidthvictimbandwidthinterferer

–	The interference-to-noise ratio (I/N) is calculated the following equation:

INTotal= 1n (EIRPnθ,φ+CF-FSPL-Floss+Grxθ,φ)-Nrx

where n is the number of lunar surface transmitters;  EIRPnθ,φ is the equivalent isotropic radiated power of each lunar transmitter towards the FS receiver; FSPL is the free space path loss calculated using Rec. ITU-R P.525; Floss is the FS feeder loss; (Grx)θ,φ is the FS victim receiver antenna gain in relation to the interferer; and Nrx is the thermal noise power in the FS victim receiver bandwidth.

–	The receiver thermal noise power is calculated from the receiver noise figure (NF) using the following equation:

Nrx = 10 log10 [k  (290  10(NF/10))  B]

where k is Boltzmann’s constant; B is the receiver bandwidth, and NF is the receiver noise figure in dB.

–	The I/N results are compared to the FS protection criteria (I/N ≤ 10 dB) to verify whether the FS protection is met.

Figure A5.1.1.1

SRS Transmitter- FS receiver positions

Figure A5.1.1.2

Antenna patterns: (a) SRS; (b) FS

| (a) | (b) |
| --- | --- |

The study results are presented in the Complementary Cumulative Distribution Function (CCDF), which illustrates the probability that the interference-to-noise ratio (I/N) exceeds the FS protection criteria, as shown in Figure A5.1.1.3. The maximum I/N value obtained was -16.9 dB, which occurs less than 0.0001% of the time.

Figure A5.1.1.3

CCDF of I/N

#### A5.1.1.6	Summary and analysis of the results of Study A

In this study the interference-to-noise ratio metric shows that, in the dynamic analysis including single potential interferers, there is a margin of 66 dB or more to the FS long term (20% of the time) protection criteria. The results of this study suggest that protection of ground-based FS from SRS operating in the 3500-3800 MHz band (co-channel) is ensured.

### A5.1.2	Study B [D]

#### A5.1.2.1 	Technical and operational characteristics of SRS operating in the 3 500-3 800 MHz frequency range

The technical and operational characteristics for SRS systems are based on those available in Report ITU-R SA.2553, including Table 3.2-4, as summarized in Table A5.1.2.1.

Table A5.1.2.1

Technical Characteristics of the SRS Systems

| Station | Lunar Surface-to-Surface Transmitter |
| --- | --- |
| Frequency range | 3 500-3 800 MHz |
| Polarization | Linear ±45º |
| Peak Gain (dBi) | 26.2 |
| EIRP Density (dBW/MHz) | 22.3 |
| Max. EIRP (dBW) | 42.3 |
| Channel BW (MHz) | 100 MHz |

#### A5.1.2.2 	Technical and operational characteristics of fixed service operating in the 3 500-3 800 MHz frequency range

#### The characteristics for fixed service (FS) for point-to-point (PP) systems are based on the information contained in Document 7B/56, Table 18, as summarized in Table A5.1.2.2.

Table A5.1.2.2

Characteristics of the FS (PP) systems

| Parameter | Value | Value | Value | Value |
| --- | --- | --- | --- | --- |
| Frequency range | 3 600-4 200 MHz | 3 600-4 200 MHz | 3 600-4 200 MHz | 3 600-4 200 MHz |
| Reference | Recommendation ITU-R F.635 | Recommendation ITU-R F.635 | Recommendation ITU-R F.635 | Recommendation ITU-R F.635 |
| Modulation format | 64 QAM | 512 QAM | QPSK | 256 QAM |
| Receiver noise bandwidth (MHz) | 10, 30, 40, 60, 80, 90 | 10, 30, 40, 60, 80, 90 | 28, 29 | 9, 10. 13, 20, 28, 29, 40, 60, 80 |
| Antenna pattern | Recommendation ITU-R F.699 | Recommendation ITU-R F.699 | Recommendation ITU-R F.699 | Recommendation ITU-R F.699 |
| Antenna gain | 42 dBi | 40 dBi | 37 dBi | 22,5 dBi |
| Feeder loss | 0 dB | 0 dB | 0 dB | 0 dB |
| Receiver noise figure | 3 dB | 2 dB | 4 dB | 6,5 dB |
| Protection criteria (I/N) | ‒10 dB | ‒10 dB | ‒10 dB | ‒10 dB |

#### A5.1.2.3 	Propagation model

This study includes the lunar SRS system orbiting the Moon and fixed services operating on Earth's surface. Working Party 3J in Document 7B/58 recommends using the propagation model based on Recommendation ITU-R P.525 for this scenario.

#### A5.1.2.4 	Methodology

The methodology employed in this study is based on a static analysis of single-entry worst-case interference. It evaluates whether, even under conditions of perfect antenna coupling between the systems, at the minimum distance between the stations on Earth and the Moon, and in the absence of antenna pointing errors and losses such as depolarization and clutter, the interference levels comply with the established ITU-R service protection criteria.

#### A5.1.2.5 	Study Results

The study was conducted in accordance with the scenario outlined in Figure A5.1.2.1. For the worst-case analysis, the following assumptions were considered:

–	Main beam antenna coupling between the potentially interfering lunar SRS transmitters and the FS victim receiver.

–	Free space propagation loss assuming the minimum distance between the stations on the Earth and the Moon (355,000 km), while atmospheric, depolarization and clutter losses are not considered.

–	In the present study, it is assumed that both systems operate at the same central frequency. A bandwidth correction factor (CF) is applied when the bandwidth of the potential interferer exceeds the bandwidth of the victim receiver under analysis. If the bandwidth of the interferer is equal to or smaller than that of the victim receiver, the bandwidth correction factor is 0:

CF = 10 * log10bandwidthvictimbandwidthinterferer

–	The aggregate interference-to-noise ratio (I/N) is calculated considering Lunar SRS transmitters pointing towards the FS victim receiver using the following equation:

INTotal= 1n (EIRPnθ,φ+CF-FSPL-Floss+Grxθ,φ)-Nrx

where n is the number of lunar surface transmitters;  EIRPnθ,φ is the equivalent isotropic radiated power of each lunar transmitter; FSPL is the free space path loss calculated using Recommendation ITU-R P.525; Floss is the FS feeder loss; (Grx)θ,φ is the FS victim receiver maximum antenna gain; and Nrx is the thermal noise power in the FS victim receiver bandwidth.

–	The receiver thermal noise power is calculated from the receiver noise figure (NF) using the formula provided in Recommendation ITU-R P.372-17.

–	The single and aggregate I/N results are compared to the FS protection criteria (I/N ≤ −6 dB) to verify whether the FS protection is met.

Figure A5.1.2.1

SRS–FS Scenario

Table A5.1.2.3

SRS – FS (PP) co-channel results

|  |  | FS 64 QAM |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| bandwidth | MHz | 10 | 30 | 40 | 60 | 80 | 90 |
| Receiver noise power | dBW | -131.0 | -126.2 | -125.0 | -123.2 | -121.9 | -121.4 |
| Single Interferer | dBW | -140.5 | -135.7 | -134.4 | -132.7 | -131.4 | -130.9 |
| I/N result | dB | -9.5 | -9.5 | -9.5 | -9.5 | -9.5 | -9.5 |
| Margin to the FS protection criterion | dB | -0.5 | -0.5 | -0.5 | -0.5 | -0.5 | -0.5 |

Table A5.1.2.4

SRS – FS (PP) co-channel results

|  |  | FS 512 QAM |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| bandwidth | MHz | 10 | 30 | 40 | 60 | 80 | 90 |
| Receiver noise power | dBW | -132.0 | -127.2 | -126.0 | -124.2 | -122.9 | -122.4 |
| Single Interferer | dBW | -142.5 | -137.7 | -136.4 | -134.7 | -133.4 | -132.9 |
| I/N result | dB | -10.5 | -10.5 | -10.5 | -10.5 | -10.5 | -10.5 |
| Margin to the FS protection criterion | dB | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 |

Table A5.1.2.5

SRS – FS (PP) co-channel results

|  |  | FS QPSK |  |
| --- | --- | --- | --- |
| bandwidth | MHz | 28 | 29 |
| Receiver noise power | dBW | -125.5 | -125.5 |
| Single Interferer | dBW | -141.0 | -141.0 |
| I/N result | dB | -15.5 | -15.5 |
| Margin to the FS protection criterion | dB | 5.5 | 5.5 |

Table A5.1.2.6

SRS – FS (PP) co-channel results

|  |  | FS 256 QAM |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| bandwidth | MHz | 9 | 10 | 13 | 20 | 28 | 29 | 40 | 60 | 80 |
| Receiver noise power | dBW | -127.9 | -127.5 | -126.3 | -124.5 | -123.0 | -122.9 | -121.5 | -119.7 | -118.4 |
| Single Interferer | dBW | -160.4 | -160.0 | -158.8 | -157.0 | -155.5 | -155.3 | -153.9 | -152.2 | -150.9 |
| I/N result | dB | -32.5 | -32.5 | -32.5 | -32.5 | -32.5 | -32.5 | -32.5 | -32.5 | -32.5 |
| Margin to the FS protection criterion | dB | 22.5 | 22.5 | 22.5 | 22.5 | 22.5 | 22.5 | 22.5 | 22.5 | 22.5 |

#### A5.1.2.6 	Summary and analysis of the results of Study B

The interference-to-noise ratio metric shows that in the single entry static worst-case analysis, the FS protection criteria for 256 QAM, QPSK and 512 QAM fixed systems are met.

### Further studies are required to identify the impact of the SRS systems on fixed systems under more realistic conditions, e.g. realistic antenna pointings.

### A5.1.3	Study C

## A5.2	Sharing studies between mobile service operating as a terrestrial component of IMT and SRS operating in the frequency band 3 500-3 800 MHz

### A5.2.1	Study A [USA]

#### A5.2.1.1	Technical and operational characteristics of SRS operating in the 3500-3800 MHz frequency range

The technical and operational characteristics for SRS systems are based on those available in the Report ITU-R SA.2553, including Table 3.2-4, as summarized in Table A5.2.1.1.

Table A5.2.1.1

Technical Characteristics of the SRS Systems

| Station | Lunar Surface-to-Surface Transmitter |
| --- | --- |
| Frequency range | 3 500-3 800 MHz |
| Polarization | Linear ±45º |
| Peak Gain (dBi) | 26.2 |
| EIRP Density (dBW/MHz) | 22.3 |
| Max. EIRP (dBW) | 42.3 |
| Channel BW (MHz) | 100 MHz |

#### A5.2.1.2	Technical and operational characteristics of mobile service operating as a terrestrial component of IMT in the 3500-3800 MHz frequency range

Document 7B/105 provides relevant technical information to support studies between SRS and mobile services operating as a terrestrial component of IMT, under agenda item 1.15. The characteristics and protection criteria for terrestrial component of IMT are based on the information contained in the Annex 4.4 to Document 5D/716, as summarized in Tables A5.2.1.2 and A5.2.1.3.

Table A5.2.1.2

IMT Base Station (BS) parameters for bands between 3 and 6 GHz

|  | Rural/Urban/suburban macro |
| --- | --- |
| Antenna height | 35 m rural /20 m urban /25 m suburban |
| AAS Antenna gain | 26.2 dBi |
| AAS BS antenna polarization | Linear±45 degrees |
| Channel bandwidth | 100 MHz |
| AAS BS Noise Figure | 5 dB |

Table A5.2.1.3

IMT User Equipment (UE) parameters for bands between 2 and 3 GHz

|  | Rural/Urban/suburban macro |
| --- | --- |
| UE height | 1.5 m |
| Typical antenna gain | −4 dBi |
| Body loss | 4 dB |
| UE Noise Figure | 9 dB |

For this study, the AAS rural/urban/suburban macro scenario represents the worst case, as the base station antenna gain is higher than in non-AAS case.

#### A5.2.1.3	Propagation model

This sharing study includes the lunar SRS system on the surface of the Moon, and mobile services operating on Earth's surface. Working Party 3J in Document 7B/58 recommends using the propagation model based on Recommendation ITU-R P.525 for this scenario.

#### A5.2.1.4	Methodology

The methodology employed in this study is based on a static analysis of both single and aggregated worst-case interference. It evaluates whether, even under conditions of perfect antenna coupling between the systems, at the minimum distance between the stations on Earth and the Moon, and in the absence of antenna pointing errors and losses such as depolarization, body, building and clutter, the interference levels comply with the established ITU-R service protection criteria.

The TDD factor was not applied in this analysis to calculate the interference received by the base station and user equipment 100% of the time, in order to verify compliance with the established ITU-R service protection criteria, even in the worst-case scenario.

#### A5.2.1.5	Study results

The study was conducted in accordance with the scenario outlined in Figure A5.2.1.1. For the worst-case analysis, the following assumptions were considered:

–	Main beam antenna coupling between the potentially interfering lunar SRS transmitters and the IMT victim receiver.

–	Free space propagation loss assuming the minimum distance between the stations on the Earth and the Moon (355,000 km), while atmospheric, depolarization and clutter losses are not considered.

–	The aggregate interference-to-noise ratio (I/N) is calculated considering Lunar SRS transmitters pointing towards the IMT victim receiver using the following equation:

INTotal= 1n (EIRPnθ,φ-FSPL+Grxθ,φ)-Nrx

where n is the number of lunar surface transmitters;  EIRPnθ,φ is the equivalent isotropic radiated power of each lunar transmitter; FSPL is the free space path loss calculated using Rec. ITU-R P.525; (Grx)θ,φ is the IMT victim receiver maximum antenna gain; and Nrx is the thermal noise power in the IMT victim receiver bandwidth.

–	The receiver thermal noise power is calculated from the receiver noise figure (NF) using the formula provided in Recommendation ITU-R P.372-17.

–	The worst-case aggregate static interference is calculated based on the lunar surface deployment characteristics provided in Report ITU-R SA.2553. Notably the lunar surface deployment in the 3 500 – 3 800 MHz band assumes 1-3 LCT base stations per cluster with up to 10 clusters located on the Moon and a network activity factor of 50%. Assuming that roughly half of the clusters on the Moon will be visible to Earth, then the aggregate static interference calculation assumes that up to 8 LCT base station transmitters could be active simultaneously.

–	The single and aggregate I/N results are compared to the IMT protection criteria (I/N ≤ −6 dB) to verify whether the IMT protection is met.

Figure A5.2.1.1

SRS–IMT Scenario

Table A5.2.1.4

SRS – IMT co-channel results

| SRS Station | SRS centre frequency | IMT Station | Analysis | I/N result | Margin to the IMT protection criteria |
| --- | --- | --- | --- | --- | --- |
| Lunar Surface Transmitter | 3550 MHz | Base Station | Single Entry | −26.9 dB | 20.9 dB |
|  |  |  | Aggregate | −17.9 dB | 11.9 dB |
| Lunar Surface Transmitter | 3550 MHz | User Equipment | Single Entry | −61.3 dB | 55.3 dB |
|  |  |  | Aggregate | −52.3 dB | 46.3 dB |

#### A5.2.1.6	Summary and analysis of the results of Study A

The interference-to-noise ratio metric shows that in the single entry static worst-case analysis, there is a margin of 20.9 dB or more to satisfy the IMT base station protection criteria and 55.3 dB margin for the IMT user equipment. For the aggregate interference analysis, the interference margin is 11.9 dB for the IMT base station and 46.3 dB for the IMT user equipment.

### A5.2.2	Study B [KOR]

#### A5.2.2.1 	Methodology

A worst-case static analysis for both single and aggregate interference originating from the lunar surface was performed. The worst-case assumptions include: maximum EIRP at the lunar surface directed toward Earth, free-space path loss calculated for the shortest distance between the Moon’s and Earth’s surfaces, no polarization loss, receivers at the Earth pointed toward the Moon, etc.

The worst-case interference-to-noise ratio (I/N) that terrestrial IMT systems could experience from transmissions on the lunar surface was calculated as follows:

IN dB=A-B+C-D

where A is the maximum (aggregate) EIRP from the interferer at the lunar surface, B is the minimum propagation loss from the lunar surface to the Earth, and C and D are the receive antenna gain and the noise power of the terrestrial IMT system. The values for A, B, C, and D under this interference scenario are derived in the following sub-sections.

For the study, a carrier frequency of 3 650 MHz (centre of the spectrum) and a channel bandwidth of 100 MHz for the interferer was assumed.

#### A5.2.2.2 	Technical and operational characteristics of SRS operating in the 3500-3800 MHz frequency range – A in the equation above

The interferer for this scenario is the lunar surface-to-surface transmitter, and its characteristics can be found at Table 3.2-4 of report ITU-R SA.2553, and part of the table is shown below.

Table A5.2.2.1

Part of Table 3.2.4 Lunar surface-to-surface transmit characteristics by frequency bands from Report ITU-R SA.2553 - Technical and operational characteristics for space research systems in the vicinity of the Moon

|  |  | 3 500-3 800 MHz |  |
| --- | --- | --- | --- |
| Max EIRP (dBW) |  | 42.3 |  |
| Channel BW (MHz) |  | 100 |  |
| EIRP Density dB(W/MHz) |  | 22.3 |  |
| Deployment characteristics Coverage area Deployment density Activity factor (average) |  | Wide area (10 km radius) 1-3 LCTs and 1-6 EVAs per cell 20%, 50% |  |

According to the table above, we can consider A=42.3 dBW as the maximum EIRP from a single interferer at the lunar surface considering the wide area deployment. It is also noted in the new report ITU-R SA.2553 that the number of active clusters at any given time is no more than 10, while the deployment densities and activity factors of LCTs and EVAs are given in the table above. Based on these numbers, 30 can be estimated as the maximum number of simultaneous transmissions from the lunar surface. This gives A=42.3 dBW+10log30 dB=57.1 dBW would be the maximum aggregate EIRP from the lunar surface.

#### A5.2.2.3 	Propagation loss – B in the equation above

Working Party 3J in Document 7B/58 recommends using the propagation model based on Recommendation ITU-R P.525 for this scenario. Given the minimum distance between the stations on the Earth and the Moon (355,000 km), the free-space basic transmission loss can be calculated according to (6) of Recommendation ITU-R P.525 as:

Lbf=32.4+20×logf+20×logd    (dB)

where f=3 650 (MHz)  and d=355,000 (km). Using these values, B=214.7 dB can be determined.

#### A5.2.2.4 	Technical and operational characteristics of mobile service operating as a terrestrial component of IMT in the 3500-3800 frequency range – C in the equation above

The characteristics and protection criteria for terrestrial component of IMT are based on the information contained in the Annex 4.4 to Document 5D/716. Table 9 and 6-2 provide the receive antenna parameters of AAS antenna used at the terrestrial IMT base station and UE, respectively.

Part of Table 9, where the IMT base station receive antenna characteristics can be found is summarized as follows.

Table A5.2.2.2

Part of Table 9 Beamforming antenna characteristics for IMT in 1 710-4 990 MHz
 from Annex 4.4 to Document 5D/716

|  |  | Rural macro | Suburban macro | Urban macro | Urban small cell (outdoor)/Micro cell | Indoor (small cell) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Base station antenna characteristics |  |  |  |  |  |
| 1.2 | Element gain (dBi) (Note 1) | 6.4 | 6.4 | 6.4 | 6.4 | N/A |
| 1.6 | Antenna array configuration (Row × Column) (Note 2) | 4 × 8 elements | 4 × 8 elements | 4 × 8 elements | 8 × 8 elements | N/A |
| 1.7a | Number of element rows in sub-array, Msub | 3 | 3 | 3 | N/A | N/A |
| Note 1: The element gain in row 1.2 includes the loss given in row 1.8 and is per polarization. This means that this parameter in row 1.8 is not needed for the calculation of the BS composite antenna gain and e.i.r.p. Note 2: For the small/micro cell case, 8 × 8 means there are 8 vertical and 8 horizontal radiating elements. For the extended AAS model case, 4 × 8 means there are 4 vertical and 8 horizontal radiating sub-arrays. |  |  |  |  |  |  |

For rural macro, suburban macro, and urban macro deployment scenario, it can be assumed that an AAS in an IMT base station has 4×8×3 =96 antenna elements and each antenna element has a gain of 6.4 dBi. This leads to the receive antenna gain of IMT base station  CBS=6.4+10×log96=26.2 dBi.

Part of Table 6-2 where the IMT UE receive antenna characteristics can be found is summarized as follows.

TABLE A5.2.2.3

Part of Table 6-2 UE parameters for bands between 3 and 6 GHz from Annex 4.4 to Document 5D/716

|  | Rural macro | Urban/suburban macro | Small cell (outdoor)/Micro cell | Indoor (small cell) |  |
| --- | --- | --- | --- | --- | --- |
| User terminal characteristics |  |  |  |  |  |
| Typical antenna gain for user terminals | −4 dBi | −4 dBi | −4 dBi |  | −4 dBi |
| Body loss | 4 dB | 4 dB | 4 dB |  | 4 dB |

Taking the table above into consideration, the receive antenna gain of IMT UE is led to  CUE=-4 dBi-4 dB=-8 dBi.

#### A5.2.2.5 	Noise characteristics of IMT systems – D in the equation above

According to Recommendation ITU-R P.372-17, the receiver thermal noise power is calculated as,

D=NF+kTb.

Noise figure (NF) is 5 dB and 9 dB for AAS IMT base station and IMT UE, respectively, according to Table 1 of Annex 4.4 to Document 5D/716. 5 dB noise figure for AAS IMT base station is the smallest (therefore the worst-case assumption) noise figure among several deployment scenarios. k is the Boltzmann’s constant of 1.38 ×10-23 J/K, T0 is the reference temperature (K) taken as 290 K, and B is the noise power bandwidth of the receiving system (Hz). As we assume the channel bandwidth of the interferer as 100 MHz, which is b=108. Taking these into consideration,

DBS=5+10 ×log1.38×10-23×290×108= -119.0 dBW

for IMT base station and DUE=-115.0 dBW for IMT UE.

#### A5.2.2.6 	Study results and conclusions

Based on the above results – A, B, C and D calculated in the previous sub-sections, I/N for IMT base station is calculated as

IN=42.3-214.7+26.2+119.0= -27.2 dB

for a single interferer and -12.4 dB for maximum aggregate interferers. Similarly, I/N for IMT UE is calculated as

IN=42.3-214.7-8+115.0= -65.4 dB

for a single interferer and -50.6 dB for maximum aggregate interferers.

Table A5.2.2.4

SRS – IMT co-channel results

| SRS Station | SRS centre frequency/ Channel Bandwidth | IMT Station on the Earth | Number of Interferers in Lunar Surface | I/N result |
| --- | --- | --- | --- | --- |
| Lunar Surface Transmitter | 3 650 MHz/100 MHz | Base Station | 1 | –27.2 dB |
|  |  | Base Station | 30 | –12.4 dB |
|  |  | User Equipment | 1 | –65.4 dB |
|  |  | User Equipment | 30 | –50.6 dB |

Considering the IMT protection criterion of −6 dB I/N, the results show a sufficient positive margin across all four scenarios. It should be noted that the AAS receive antenna gain of the IMT base station (26.2 dBi) assumes beamforming toward the intended transmitter. Since the intended transmitter is located on the Earth’s surface, the AAS receive antenna gain in the direction of a lunar interferer would be significantly lower—by at least 20 dB. This reduction provides sufficient I/N margin to account for aggregate interference originating from the lunar surface.

### A5.2.3	Study C

## A5.3	Sharing and compatibility of the radiolocation service and SRS for lunar operations in the frequency band 3 500-3 600 MHz

### A5.3.1	Study A [USA]

This section shows the static worst-case interference analysis between RLS and lunar surface SRS in the frequency band 3500-3800 MHz.

#### A5.3.1.1	Technical and operational characteristics of SRS operating in the 3500-3800 MHz frequency range

The technical and operational characteristics for SRS systems are based on those available in the Report ITU-R SA.2553, as summarized below.

Table A5.3.1

Technical Characteristics of the Lunar SRS Systems

| Station | Lunar Surface-to-Surface Transmitter |
| --- | --- |
| Frequency range | 3 500 – 3 800 MHz |
| Polarization | Linear ±45º / CP |
| Peak Gain (dBi) | 26.2 |
| EIRP Density (dBW/MHz) | 22.3 |
| Max. EIRP (dBW) | 42.3 |
| Channel BW (MHz) | 100 MHz |

#### A5.3.1.2	Technical and operational characteristics of radiolocation service operating in the 3500-3800 MHz frequency range

The characteristics and protection criteria for radars operating in the radiolocation service (RLS) are based on Recommendation ITU-R M.1465-4 and summarized below.

Table A5.3.2

Characteristics of the radar systems

| Parameter | Land-based | Shipborne | Airborne |
| --- | --- | --- | --- |
| Tuning range | 3.1-3.7 GHz | 2.9-3.7 GHz | 3.1-3.7 GHz |
| Antenna gain | 40 dBi | 42 dBi | 40 dBi |
| Receiver noise figure | 4 dB | 5 dB | 3 dB |
| Receiver IF bandwidth | 0.67 MHz | 10 MHz | 1 MHz |
| Protection criteria (I/N) | −6 dB |  |  |

#### A5.3.1.3	Propagation model

This analysis uses the propagation model based on Recommendation ITU-R P.525 for this scenario, as advised by Working Party 3J in Document 7B/58.

#### A5.3.1.4	Methodology

The methodology employed in this study is based on a static analysis of both single and aggregated worst-case interference. It evaluates whether, even under conditions of perfect antenna coupling between the systems, at the minimum distance between the stations on Earth and the Moon, and in the absence of antenna pointing errors and losses such as depolarization and clutter, the interference levels comply with the established ITU-R service protection criteria.

#### A5.3.1.5	Study results

For the worst-case analysis, the following assumptions were considered:

–	Main beam antenna coupling between the potentially interfering lunar SRS transmitters and the radar victim receiver.

–	Free space propagation loss assuming the minimum distance between the stations on the Earth and the Moon (355,000 km for ground and shipborne radars and 354,991 km for airborne), while atmospheric, depolarization and clutter losses are not considered.

–	In the present study, it is assumed that both systems operate at the same central frequency. A bandwidth correction factor (CF) is applied when the bandwidth of the potential interferer exceeds the bandwidth of the victim receiver under analysis. If the bandwidth of the interferer is equal to or smaller than that of the victim receiver, the bandwidth correction factor is 0:

CF = 10 * log10bandwidthvictimbandwidthinterferer

–	The interference-to-noise ratio (I/N) is calculated considering Lunar SRS transmitter pointing towards the radar victim receiver using the following equation:

INTotal= 1n (EIRPnθ,φ+CF-FSPL+Grxθ,φ)-Nrx

where n is the number of lunar surface transmitters;  EIRPnθ,φ is the equivalent isotropic radiated power of each lunar transmitter; FSPL is the free space path loss calculated using Rec. ITU-R P.525; (Grx)θ,φ is the radar victim receiver maximum antenna gain; and Nrx is the thermal noise power in the radar victim receiver bandwidth.

–	The receiver thermal noise power is calculated from the receiver noise figure (NF) using the formula provided in Recommendation ITU-R P.372-17.

–	The single I/N result is compared to the radar protection criteria (I/N ≤ 6 dB) to verify whether the radiolocation service protection is met.

Figure A5.3.1

SRS–RLS Scenarios: (a) Land-based and shipborne; (b) Airborne

(a)

(b)

SRS–RLS Scenario

Table A5.3.3

SRS – RLS co-channel results

| SRS Station | SRS centre frequency | Radar | Analysis | I/N result | Margin to the RLS protection criteria |
| --- | --- | --- | --- | --- | --- |
| Lunar Surface Transmitter | 3550 MHz | Land-based | Single Entry | -12.1 dB | 6.1 dB |
| Lunar Surface Transmitter | 3550 MHz | Shipborne | Single Entry | -11.1 dB | 5.1 dB |
| Lunar Surface Transmitter | 3550 MHz | Airborne | Single Entry | -11.1 dB | 5.1 dB |

#### A5.3.1.6	Summary and analysis of the results of Study A

The results of this study suggest that sharing between SRS and RLS in the 3500-3800 MHz frequency range is feasible. The interference-to-noise ratio metric shows that in the single entry static worst-case analysis, there is a margin of 5.1 dB or more to satisfy the RLS protection criteria.

## A5.4	Sharing and compatibility of fixed satellite service (space-to-Earth) and SRS for lunar operations in the frequency band 3 500-3 800 MHz

### A5.4.1	Study A [USA]

#### A5.4.1.1	Technical and operational characteristics of SRS operating in the 3500-3800 MHz frequency range

The technical and operational characteristics for SRS systems are based on those available in the Report ITU-R SA.2553, including Table 6, as summarized in Table A5.1.1.1.

Table A5.4.1.1

Technical Characteristics of the SRS Systems

| Station | Lunar Surface-to-Surface Transmitter |
| --- | --- |
| Frequency range | 3500 – 3800 MHz |
| Polarization | Linear ±45º |
| Peak Gain (dBi) | 26.2 |
| EIRP Density (dBW/MHz) | 22.3 |
| Max. EIRP (dBW) | 42.3 |
| Channel BW (MHz) | 100 MHz |

#### A5.4.1.2	Technical and operational characteristics of fixed satellite service operating in the 3500– 3800 MHz frequency range

Document 7B/200 provides relevant technical information to support studies between SRS and fixed satellite service (FS) under agenda item 1.15. The characteristics and protection criteria for FSS earth stations used in this study are provided below.

FSS earth station parameters in 3 500 – 3 800 MHz Used in This Study

| Parameter | Value |
| --- | --- |
| Satellite antenna coverage | Regional/global |
| Earth station sample location | 34°12'3.64"N, 118°10'12.04"W / (USA) |
| FSS earth station antenna diameter, m | 4.2, 32 |
| Earth station antenna reference pattern | Recommendation ITU-R S.465 |
| Receiver noise bandwidth | 72 MHz |
| Receiving system noise temperature | 70 K |
| Elevation angles, degrees | 50, 300, 500 |
| Protection criteria | I/N ≤−10.5 dB for 20% of time; I/N ≤−1.3 dB for 0.005% of time |

#### A5.4.1.3	Propagation model

This sharing study includes the lunar surface SRS system, and FSS operating on Earth's surface. Working Party 3J in Document 7B/58 recommends using the propagation model based on Recommendation ITU-R P.525 for this scenario.

#### A5.4.1.4	Methodology

The methodology employed in this study is based on a dynamic analysis of single-entry interference. It evaluates potential interference to an FSS earth station receiver from a communications link between a SRS lunar surface transmitter and a lunar surface receiver, which may be an astronaut, a lunar terrain vehicle, or a lander on the lunar surface. The pointing of the SRS antenna is varied across different elevation angles and azimuths, taking into account the location of the lunar receiver relative to the SRS transmitter.

The SRS lunar transmitter is assumed to be located at the Moon’s South Pole. The simulation incorporates the orbital motions of the Earth and Moon. Based on orbital dynamics and system deployment characteristics, several parameters are varied to compute the interference received by the FSS Earth station when pointed toward its satellite, considering different simulated positions, three elevation angles, and two antenna diameters.

The simulation spans a five-year period with a step size of 10 seconds, accounting for the FSS earth station’s visibility to the Moon transmitter and possible antenna pointing configurations dictated by orbital dynamics. The complementary cumulative distribution function (CCDF) of the interference-to-noise ratio (INR) is calculated and compared with the FSS long and short-term protection criteria. It should be noted that the simulation does not account for terrain obstructions or clutter losses on either the Earth or the Moon.

#### A5.4.1.5	Study results

The study was conducted in accordance with the scenario outlined in Figure A5.3.1.1. For the worst-case analysis, the following assumptions were considered:

–	The position of the SRS Lunar transmitter is at a latitude of -84.201560 and a longitude of -60.699890 on the Moon, as shown in Figure A5.1.1.1, while the lunar receiver is positioned within a 360-degree azimuth range, at a maximum distance of 6 km from the transmitter.

–	The FSS earth station receiver is located at a latitude of 34.201010 and a longitude of -118.170010 as shown in Figure A5.4.1.1.

–	The FSS antenna height is 30 m. Two antenna diameters, 4.5 m and 32 m, were considered, with the antenna pattern based on Recommendation ITU-R S.465. An example of the 32 m-diameter antenna pattern is shown in Figure A5.4.1.2(b).

–	Three FSS Earth station elevation angles of 5°, 30°, and 50° were assessed, corresponding to the FSS antennas pointing toward their respective satellite orbital locations.

–	The SRS antenna elevation varies from 0° to –30°, and the azimuth spans 0° to 360°, following the position of the lunar receiver. The antenna pattern for the Lunar SRS transmitter is shown in Figure A5.4.1.2(a).

–	Free space propagation loss assuming the minimum distance between the stations on the Earth and the Moon (355,000 km), while atmospheric, depolarization and clutter losses are not considered.

–	In the present study, it is assumed that both systems operate at the same central frequency (3650 MHz). A bandwidth correction factor (CF) is applied when the bandwidth of the potential interferer exceeds the bandwidth of the victim receiver under analysis. If the bandwidth of the interferer is equal to or smaller than that of the victim receiver, the bandwidth correction factor is 0:

CF = 10 * log10bandwidthvictimbandwidthinterferer

–	The interference-to-noise ratio (I/N) is calculated using the following equation:

INTotal= 1n (EIRPnθ,φ+CF-FSPL+Grxθ,φ)-Nrx

where n is the number of lunar surface transmitters;  EIRPnθ,φ is the equivalent isotropic radiated power of the lunar transmitter; FSPL is the free space path loss calculated using Rec. ITU-R P.525; (Grx)θ,φ is the FSS victim receiver antenna gain in the direction of the interferer; and Nrx is the thermal noise power in the FSS victim receiver bandwidth

–	The receiver thermal noise power is calculated from the receiver noise figure (NF) using the formula provided in Recommendation ITU-R P.372-17.

–	The single and aggregate I/N results are compared to the FSS long and short-term protection criteria to verify whether the FSS protection is met.

Figure A5.4.1.1

SRS Transmitter- FSS receiver positions

The study results are presented as a Complementary Cumulative Distribution Function (CCDF), showing the probability that the interference-to-noise ratio (I/N) exceeds the FSS protection criteria, as shown in Figure A5.4.1.3. All assessed cases met the FSS protection criteria, with margins of 67.3 dB for the long-term and 27.6 dB for the short-term criterion.

Figure A5.4.1.2

CCDF of I/N

#### A5.4.1.6	Summary and analysis of the results of Study A

The results of this study suggest that sharing between Fixed Satellite Service (FSS) (space-to-Earth) and Space Research Service (SRS) on the lunar surface operating in the 3500–3800 MHz band (co-channel) is feasible. Based on the interference-to-noise ratio (I/N) metric and the dynamic analysis taking into account antenna pointing, the FSS protection criteria were met in all assessed cases, with margins of at least 67.3 dB for the long-term (20% of the time) and 27.6 dB for the short-term (0.005% of the time) protection criteria.

## A5.[X]	Summary and analysis of the results of studies

[Note: The summary should offer an objective overview, encompassing the results of sharing and compatibility studies, alongside any pertinent information applicable for future comparisons among studies. Additionally, it should address any mitigation techniques or proposals employed to safeguard services operating within the same band or adjacent bands, where applicable, to SRS lunar systems.]

ANNEX 6

Sharing and compatibility studies of SRS for lunar operations 
in the frequency band 5 150-5 570 MHz

## A6.1	Sharing and compatibility of radiolocation service (RLS) and SRS operating in the frequency band 5 150-5 570 MHz

[Note: Insert relevant sharing and compatibility studies for each of the identified service.]

### A6.1.1	Study A [USA]

#### A6.1.1.1	Technical and operational characteristics of SRS operating in the 5 150-5 570 MHz frequency range

The technical and operational characteristics for SRS systems are based on those available in the Report ITU-R SA.2553, including Table 3.2-4, as summarized in Table A6.1.1.1.

Table A6.1.1.1

Technical Characteristics of the SRS Systems

| Station | Lunar Surface-to-Surface Transmitter |
| --- | --- |
| Frequency range | 5 150-5 725 MHz |
| Polarization | Vertical |
| Peak Gain (dBi) | 6 |
| EIRP Density (dBW/MHz) | −10 |
| Max. EIRP (dBW) | 6 |
| Channel BW (MHz) | 40 MHz |

#### A6.1.1.2	Technical and operational characteristics of radiolocation service (RLS) operating in the 5 250-5 570 MHz frequency range

The characteristics and protection criteria for radars operating in the radiolocation service are based on the Recommendation ITU-R M.1638, Table 3, as summarized in Table A6.1.1.2.

Table A6.1.1.2

Characteristics of the radar systems

| Parameter | Ground | Airborne | Shipborne |
| --- | --- | --- | --- |
| Radar | L | S | Q |
| Frequency range | 5 350-5 850 MHz | 5 250-5 725 MHz | 5 450-5 825 MHz |
| Polarization | Vertical/left- hand circular | Circular | Horizontal |
| Platform height/altitude | 20 m | 9000 m | 40 m |
| Antenna gain | 54 dBi | 40 dBi | 30 dBi |
| Antenna horizontal scan type | N/A (Tracking) | Continuous | 30-270 Sector |
| Receiver noise figure | 5 dB | 3.5 dB | 10 dB |
| Receiver IF bandwidth | 0.25 MHz | 1 MHz | 1 MHz |
| Protection criteria (I/N) | ‒6 dB |  |  |

#### A6.1.1.3	Propagation model

This sharing study includes the lunar SRS system on the surface of the Moon, and radiolocation service operating on Earth's surface. Working Party 3J in Document 7B/58 recommends using the propagation model based on Recommendation ITU-R P.525 for this scenario.

#### A6.1.1.4	Methodology

The methodology employed in this study is based on a static analysis of both single and aggregated worst-case interference. It evaluates whether, even under conditions of perfect antenna coupling between the systems, at the minimum distance between the stations on Earth and the Moon, and in the absence of antenna pointing errors and losses such as depolarization and clutter, the interference levels comply with the established ITU-R service protection criteria.

#### A6.1.1.5	Study results

The study was conducted in accordance with the scenarios outlined in Figure A6.1.1.1. For the worst-case analysis, the following assumptions were considered:

–	Main beam antenna coupling between the potentially interfering lunar SRS transmitters and the radar victim receiver.

–	Free space propagation loss assuming the minimum distance between the stations on the Earth and the Moon (355,000 km for ground and shipborne radars and 354,991 km for airborne), while atmospheric, depolarization and clutter losses are not considered.

–	In the present study, it is assumed that both systems operate at the same central frequency. A bandwidth correction factor (CF) is applied when the bandwidth of the potential interferer exceeds the bandwidth of the victim receiver under analysis. If the bandwidth of the interferer is equal to or smaller than that of the victim receiver, the bandwidth correction factor is 0:

CF = 10 * log10bandwidthvictimbandwidthinterferer

–	The aggregate interference-to-noise ratio (I/N) is calculated considering Lunar SRS transmitters pointing towards the radar victim receiver using the following equation:

INTotal= 1n (EIRPnθ,φ+CF-FSPL+Grxθ,φ)-Nrx

where n is the number of lunar surface transmitters;  EIRPnθ,φ is the equivalent isotropic radiated power of each lunar transmitter; FSPL is the free space path loss calculated using Rec. ITU-R P.525; (Grx)θ,φ is the radar victim receiver maximum antenna gain; and Nrx is the thermal noise power in the radar victim receiver bandwidth.

–	The receiver thermal noise power is calculated from the receiver noise figure (NF) using the formula provided in Recommendation ITU-R P.372-17.

–	The single and aggregate I/N results are compared to the radar protection criteria (I/N ≤ -6 dB) to verify whether the radiolocation service protection is met.

Figure A6.1.1.1

SRS–RLS Scenarios: (a) Ground and shipborne; (b) Airborne.

(a)

(b)

Table A6.1.1.3

SRS – RLS co-channel results

| SRS Station | SRS centre frequency | Radar | Analysis | I/N result | Margin to the radar protection criteria |
| --- | --- | --- | --- | --- | --- |
| Lunar Surface Transmitter | 5 370 MHz | Ground | Single Entry | –35.0 dB | 29.0 dB |
| Lunar Surface Transmitter | 5 370 MHz | Airborne | Single Entry | –47.5 dB | 41.5 dB |
| Lunar Surface Transmitter | 5 470 MHz | Shipborne | Single Entry | –64.2 dB | 58.2 dB |

#### A6.1.1.6	Summary and analysis of the results of Study A

The results of this study suggest that sharing between lunar SRS and Earth-based radars operating in the radiolocation service in the 5 150-5 570 MHz band (co-channel) is feasible. The interference-to-noise ratio metric shows that in the single entry static worst-case analysis, there is a margin of 29 dB or more to satisfy the radiolocation service protection criteria. That implies the aggregate interference from 794 lunar surface transmitters would still meet the RLS protection criteria in this band.

### A6.1.2	Study B

### A6.1.3	Study C

## A6.2	Sharing studies between aeronautical radionavigation service (ARNS) and SRS operating in the frequency band 5 150-5 570 MHz

### A6.2.1	Study A [USA]

#### A6.2.1.1	Technical and operational characteristics of SRS operating in the 5 150-5 570 MHz frequency range

The technical and operational characteristics for SRS systems are based on those available in the Report ITU-R SA.2553, including Table 3.2-4, as summarized in Table A6.2.1.1.

Table A6.2.1.1

Technical Characteristics of the SRS Systems

| Station | Lunar Surface-to-Surface Transmitter |
| --- | --- |
| Frequency range | 5 150-5 725 MHz |
| Polarization | Linear ±45º |
| Peak Gain (dBi) | 6 |
| EIRP Density (dBW/MHz) | −10 |
| Max. EIRP (dBW) | 6 |
| Channel BW (MHz) | 40 MHz |

#### A6.2.1.2	Technical and operational characteristics of aeronautical radionavigation service (ARNS) operating in the 5 250-5 570 MHz frequency range

The characteristics and protection criteria for radars operating in the aeronautical radionavigation service (ARNS) are based on the Recommendation ITU-R M.1638, Table 2, as summarized in Table A6.2.1.2.

Table A6.2.1.2

Characteristics of the radar system

| Parameter | Airborne |
| --- | --- |
| Radar | D |
| Frequency range | 5 440 MHz |
| Polarization | Horizontal |
| Platform height/altitude | 9000 m (Aircraft altitude) |
| Antenna gain | 34 dBi |
| Antenna horizontal scan type | Continuous |
| Receiver noise figure | 5 dB |
| Receiver IF bandwidth | 1 MHz |
| Protection criteria (I/N) | −6 dB |

#### A6.2.1.3	Propagation model

This sharing study includes the lunar SRS system on the surface of the Moon, and radiolocation service operating on Earth's surface. Working Party 3J in Document 7B/58 recommends using the propagation model based on Recommendation ITU-R P.525 for this scenario.

#### A6.2.1.4	Methodology

The methodology employed in this study is based on a static analysis of both single and aggregated worst-case interference. It evaluates whether, even under conditions of perfect antenna coupling between the systems, at the minimum distance between the stations on Earth and the Moon, and in the absence of antenna pointing errors and losses such as depolarization and clutter, the interference levels comply with the established ITU-R service protection criteria.

#### A6.2.1.5	Study results

The study was conducted in accordance with the scenario outlined in Figure A6.2.1.1. For the worst-case analysis, the following assumptions were considered:

–	Main beam antenna coupling between the potentially interfering lunar SRS transmitters and the radar victim receiver.

–	Free space propagation loss assuming the minimum distance between the stations on the Earth and the Moon (354,991 km), considering the aircraft altitude, while atmospheric, depolarization and clutter losses are not considered.

–	In the present study, it is assumed that both systems operate at the same central frequency. A bandwidth correction factor (CF) is applied when the bandwidth of the potential interferer exceeds the bandwidth of the victim receiver under analysis. If the bandwidth of the interferer is equal to or smaller than that of the victim receiver, the bandwidth correction factor is 0:

CF = 10 * log10bandwidthvictimbandwidthinterferer

–	The aggregate interference-to-noise ratio (I/N) is calculated considering Lunar SRS transmitters pointing towards the radar victim receiver using the following equation:

INTotal= 1n (EIRPnθ,φ+CF-FSPL+Grxθ,φ)-Nrx

where n is the number of lunar surface transmitters;  EIRPnθ,φ is the equivalent isotropic radiated power of each lunar transmitter; FSPL is the free space path loss calculated using Rec. ITU-R P.525; (Grx)θ,φ is the radar victim receiver maximum antenna gain; and Nrx is the thermal noise power in the radar victim receiver bandwidth.

–	The receiver thermal noise power is calculated from the receiver noise figure (NF) using the formula provided in Recommendation ITU-R P.372-17.

–	The single and aggregate I/N results are compared to the radar protection criteria (I/N ≤ −6 dB) to verify whether the radiolocation service protection is met.

Figure A6.2.1.1

SRS–ARNS Scenario

Table A6.2.1.3

SRS – ARNS co-channel results

| SRS Station | SRS centre frequency | Radar | Analysis | I/N result | Margin to the radar protection criteria |
| --- | --- | --- | --- | --- | --- |
| Lunar Surface Transmitter | 5440 MHz | Airborne | Single Entry | −55.1 dB | 49.1 dB |

#### A6.2.1.6	Summary and analysis of the results of Study A

The results of this study suggest that sharing between lunar SRS and radars operating in the aeronautical radionavigation service in the 5 150-5 570 MHz band (co-channel) is feasible. The interference-to-noise ratio metric in the single entry static worst-case analysis shows a margin of 49.1 dB or more to satisfy the aeronautical radionavigation service protection criteria. This implies that the worst-case aggregate interference from 81283 lunar transmitters would still meet the ARNS protection criteria.

## A6.3	Sharing studies between aeronautical mobile service (AMS) and SRS operating in the frequency band 5 150-5 570 MHz

### A6.3.1	Study A [USA]

#### A6.3.1.1	Technical and operational characteristics of SRS operating in the 5 150-5 570 MHz frequency range

The technical and operational characteristics for SRS systems are based on those available in the Report ITU-R SA.2553, including Table 3.2-4, as summarized in Table A6.3.1.1.

Table A6.3.1.1

Technical Characteristics of the SRS Systems

| Station | Lunar Surface-to-Surface Transmitter |
| --- | --- |
| Frequency range | 5 150-5 725 MHz |
| Polarization | Linear ±45º |
| Peak Gain (dBi) | 6 |
| EIRP Density (dBW/MHz) | −10 |
| Max. EIRP (dBW) | 6 |
| Channel BW (MHz) | 40 MHz |

#### A6.3.1.2	Technical and operational characteristics of aeronautical mobile service (AMS) operating in the 5 150-5 250 MHz frequency range

The characteristics and protection criteria for AMS are based on the Recommendation ITU-R M.2122, Table 1, as summarized in Table A6.3.1.2.

Table A6.3.1.2

Characteristics of the AMS ground system

| Parameter | Aeronautical mobile telemetry system |
| --- | --- |
| Frequency range | 5 150 – 5 160 MHz |
| Channel bandwidth | 8 MHz |
| Receiver antenna gain | 40 dBi |
| Platform height | 40 m |
| Receiver noise figure | 9 dB |
| Protection criteria (I/N) | −6 dB |

#### A6.3.1.3	Propagation model

This sharing study includes the lunar SRS system on the surface of the Moon, and radiolocation service operating on Earth's surface. Working Party 3J in Document 7B/58 recommends using the propagation model based on Recommendation ITU-R P.525 for this scenario.

#### A6.3.1.4	Methodology

The methodology employed in this study is based on a static analysis of both single and aggregated worst-case interference. It evaluates whether, even under conditions of perfect antenna coupling between the systems, at the minimum distance between the stations on Earth and the Moon, and in the absence of antenna pointing errors and losses such as depolarization and clutter, the interference levels comply with the established ITU-R service protection criteria.

#### A6.3.1.5 	Study results

The study was conducted in accordance with the scenario outlined in Figure A6.3.1.1. For the worst-case analysis, the following assumptions were considered:

–	Main beam antenna coupling between the potentially interfering lunar SRS transmitters and the AMS victim receiver.

–	Free space propagation loss assuming the minimum distance between the stations on the Earth and the Moon (355,000 km), while atmospheric, depolarization and clutter losses are not considered.

–	In the present study, it is assumed that both systems operate at the same central frequency. A bandwidth correction factor (CF) is applied when the bandwidth of the potential interferer exceeds the bandwidth of the victim receiver under analysis. If the bandwidth of the interferer is equal to or smaller than that of the victim receiver, the bandwidth correction factor is 0:

CF = 10 * log10bandwidthvictimbandwidthinterferer

–	The aggregate interference-to-noise ratio (I/N) is calculated considering Lunar SRS transmitters pointing towards the radar victim receiver using the following equation:

INTotal= 1n (EIRPnθ,φ+CF-FSPL+Grxθ,φ)-Nrx

where n is the number of lunar surface transmitters;  EIRPnθ,φ is the equivalent isotropic radiated power of each lunar transmitter; FSPL is the free space path loss calculated using Rec. ITU-R P.525; (Grx)θ,φ is the AMS victim receiver maximum antenna gain; and Nrx is the thermal noise power in the AMS victim receiver bandwidth.

–	The receiver thermal noise power is calculated from the receiver noise figure (NF) using the formula provided in Recommendation ITU-R P.372-17.

–	The single and aggregate I/N results are compared to the radar protection criteria (I/N ≤ −6 dB) to verify whether the AMS protection is met.

Figure A6.3.1.1

SRS–AMS Scenario

Table A6.3.1.3

SRS – AMS co-channel results

| SRS Station | AMS centre frequency | AMS | Analysis | I/N result | Margin to the AMS protection criteria |
| --- | --- | --- | --- | --- | --- |
| Lunar Surface Transmitter | 5155 MHz | Ground | Single Entry | −52.6 dB | 46.6 dB |

#### A6.3.1.6	Summary and analysis of the results of Study A

The results of this study suggest that sharing between SRS and aeronautical mobile service (AMS) operating in the 5 150-5 570 MHz band (co-channel) is feasible. The interference-to-noise ratio metric shows that in the single entry static worst-case analysis, there is a margin of 46.6 dB or more to satisfy the AMS protection criteria. This implies that even the worst-case aggregate interference from 45708 lunar surface transmitters would not exceed the AMS protection criterion in this band.

## A6.4	Sharing studies between fixed satellite service (FSS) and SRS operating in the frequency band 5 150-5 570 MHz

### A6.4.1	Study A [USA]

In Document 7B/200, WP 4A expressed the view that sharing studies to ensure protection of FSS are not required in the 5 150 – 5 570 MHz frequency range, provided that possible future allocations to space research service (space-to-space) in the 5 150 – 5 570 MHz band under WRC-27 agenda item 1.15 are limited to Lunar surface-to-surface communications, as specified in Report ITU-R SA.2553.

## A6.5	Sharing studies between mobile service (MS) and SRS operating in the frequency band 5 150-5 570 MHz

### A6.5.1	Study A [USA]

#### A6.5.1.1	Technical and operational characteristics of SRS operating in the 5 150-5 570 MHz frequency range

The technical and operational characteristics for SRS systems are based on those available in the Report ITU-R SA.2553, including Table 3.2-4, as summarized in Table A6.5.1.1.

Table A6.5.1.1

Technical Characteristics of the SRS Systems

| Station | Lunar Surface Transmitter |
| --- | --- |
| Frequency range | 5 150-5 725 MHz |
| Polarization | Vertical |
| Peak Gain (dBi) | 6 |
| Max. EIRP Density (dBW/MHz) | ‒10 |
| Max. EIRP (dBW) | 6 |
| Channel BW (MHz) | 40 |

#### A6.5.1.2	Technical and operational characteristics of mobile service operating in the 5 150 -5 570 MHz frequency range

Document 7B/115 provides relevant technical information to support studies between SRS and mobile service under Agenda Item 1.15.  The documents referenced by WP 5A do not contain characteristics for the mobile service in the 5 150-5 570 MHz frequency range; however, the mobile service characteristics from Table 2b of Report ITU-R M.2116-2 for the 3.4-4.2 GHz band have been applied for this study.

Table A2.3.1.2

Characteristics of the MS Systems

| Parameter | Mobile service receiver (fixed outdoor) |
| --- | --- |
| Frequency range | 5 150 – 5 350 MHz |
| Receiver noise figure (dB) | 8 |
| Antenna gain | 17 dBi |
| Channel bandwidth | 7 MHz |
| Protection criteria (I/N) | -6 dB |

#### A6.5.1.3	Propagation model

This sharing study includes the lunar SRS system on the surface of the Moon and mobile services (conventional and trunked land mobile) operating on Earth's surface. Therefore, Working Party 3J in Document 7B/58 recommends using the propagation model based on Recommendation ITU-R P.525 for this scenario.

#### A6.5.1.4	Methodology

The methodology employed in this study is based on a static analysis of both single and aggregated worst-case interference. It evaluates whether, even under conditions of perfect antenna coupling between the systems, at the minimum distance between the stations on Earth and the Moon, and in the absence of antenna pointing errors and losses such as depolarization and clutter, the interference levels comply with the established ITU-R service protection criteria.

#### A6.5.1.5	Study results

The study was conducted in accordance with the scenarios outlined in Figure A6.5.1.1. For the worst-case analysis, the following assumptions were considered:

–	Main beam antenna coupling between the potentially interfering lunar SRS transmitters and the MS victim receiver.

–	Free space propagation loss assuming the minimum distance between the stations on the Earth and the Moon (355,000 km), while atmospheric, depolarization and clutter losses are not considered;

–	In the present study, it is assumed that both systems operate at the same central frequency. A bandwidth correction factor (CF) is applied when the bandwidth of the potential interferer exceeds the bandwidth of the victim receiver under analysis. If the bandwidth of the interferer is equal to or smaller than that of the victim receiver, the bandwidth correction factor is 0:

CF = 10 * log10bandwidthvictimbandwidthinterferer

–	The aggregate interference-to-noise ratio (I/N) is calculated considering Lunar SRS transmitters pointing towards the MS victim receiver using the following equation:

INTotal= 1n (EIRPnθ,φ+CF-FSPL+Grxθ,φ)-Nrx

where n is the number of lunar surface transmitters;  EIRPnθ,φ is the equivalent isotropic radiated power of each lunar transmitter; FSPL is the free space path loss calculated using Recommendation ITU-R P.525; (Grx)θ,φ is the MS victim receiver maximum antenna gain; and Nrx is the thermal noise power in the MS victim receiver bandwidth; CF is the frequency-dependent rejection based on the transmit and receiver bandwidths;

–	The receiver thermal noise power is calculated from the receiver noise figure (NF) using the formula provided in Recommendation ITU-R P.372-17.

–	The single and aggregate interference results are compared to the mobile service protection criteria to verify whether the service protection criteria is met

Figure A6.6.1.1

SRS–MS Scenario

(a)

(b)

Table A3.3.1.3

SRS – MS co-channel results

| SRS Station | SRS centre frequency | Analysis | I/N result | Margin to the MS protection criteria |
| --- | --- | --- | --- | --- |
| Lunar Surface Transmitter | 5 250 MHz | Single Entry | -74.9 dB | 68.9 dB |

#### A6.5.1.6 	Summary and analysis of the results of Study A

The results of this study suggest that sharing between lunar SRS operating in the 5 150-5 570 MHz band and the mobile service (co-channel) is feasible. The interference-to-noise ratio metric shows that in the single entry static worst-case analysis, there is a margin of 68.9 dB or more to satisfy the MS protection criteria. This implies that even the worst-case aggregate interference from 7.76 million lunar surface transmitters would not exceed the MS protection criterion in this band.

## A6.6	Sharing studies between fixed service (FS) and SRS operating in the frequency band 5 150-5 570 MHz

### A6.6.1	Study A [USA]

#### A6.6.1.1	Technical and operational characteristics of SRS operating in the 5 150-5 570 MHz frequency range

The technical and operational characteristics for SRS systems are based on those available in the Report ITU-R SA.2553, including Table 3.2-4, as summarized in Table A6.6.1.1.

Table A6.6.1.1

Technical Characteristics of the SRS Systems

| Station | Lunar Surface Transmitter |
| --- | --- |
| Frequency range | 5 150-5 725 MHz |
| Polarization | Vertical |
| Peak Gain (dBi) | 6 |
| Max. EIRP Density (dBW/MHz) | ‒10 |
| Max. EIRP (dBW) | 6 |
| Channel BW (MHz) | 40 |

#### A6.6.1.2	Technical and operational characteristics of fixed service operating in the 5 150 -5 570 MHz frequency range

Document 7B/56 provides relevant technical information to support studies between SRS and the fixed service under Agenda Item 1.15.  The technical characteristics for fixed service systems (point-to-multipoint) in the 5 250-5 350  MHz band are provided in Recommendation ITU-R F.1613, as shown below in Table A6.6.1.2.

Table A6.6.1.2

Characteristics of the FS Systems

| Parameter | Fixed service receiver (remote station) |
| --- | --- |
| Frequency range | 5 250 – 5 350 MHz |
| Receiver noise figure (dB) | 8 |
| Antenna gain | 15 dBi |
| Receiver bandwidth | 20 |
| Protection criteria (I/N) | -6 dB |

#### A6.6.1.3	Propagation model

This sharing study includes the lunar SRS system on the surface of the Moon and fixed service systems operating on Earth's surface. Therefore, Working Party 3J in Document 7B/58 recommends using the propagation model based on Recommendation ITU-R P.525 for this scenario.

#### A6.6.1.4	Methodology

The methodology employed in this study is based on a static analysis of both single and aggregated worst-case interference. It evaluates whether, even under conditions of perfect antenna coupling between the systems, at the minimum distance between the stations on Earth and the Moon, and in the absence of antenna pointing errors and losses such as depolarization and clutter, the interference levels comply with the established ITU-R service protection criteria.

#### A6.6.1.5	Study results

The study was conducted in accordance with the scenarios outlined in Figure A6.6.1.1. For the worst-case analysis, the following assumptions were considered:

–	Main beam antenna coupling between the potentially interfering lunar SRS transmitters and the FS victim receiver.

–	Free space propagation loss assuming the minimum distance between the stations on the Earth and the Moon (355,000 km), while atmospheric, depolarization and clutter losses are not considered;

–	In the present study, it is assumed that both systems operate at the same central frequency. A bandwidth correction factor (CF) is applied when the bandwidth of the potential interferer exceeds the bandwidth of the victim receiver under analysis. If the bandwidth of the interferer is equal to or smaller than that of the victim receiver, the bandwidth correction factor is 0:

CF = 10 * log10bandwidthvictimbandwidthinterferer

–	The aggregate interference-to-noise ratio (I/N) is calculated considering Lunar SRS transmitters pointing towards the FS victim receiver using the following equation:

INTotal= 1n (EIRPnθ,φ+CF-FSPL+Grxθ,φ)-Nrx

where n is the number of lunar surface transmitters;  EIRPnθ,φ is the equivalent isotropic radiated power of each lunar transmitter; FSPL is the free space path loss calculated using Recommendation ITU-R P.525; (Grx)θ,φ is the FS victim receiver maximum antenna gain; and Nrx is the thermal noise power in the FS victim receiver bandwidth; CF is the frequency-dependent rejection based on the transmit and receiver bandwidths;

–	The receiver thermal noise power is calculated from the receiver noise figure (NF) using the formula provided in Recommendation ITU-R P.372-17.

–	The single and aggregate interference results are compared to the fixed service protection criteria to verify whether the service protection criteria is met

Figure A3.3.1.1

SRS–FS Scenario

(a)

(b)

Table A6.6.1.3

SRS – FS co-channel results

| SRS Station | SRS centre frequency | Analysis | I/N result | Margin to the FS protection criteria |
| --- | --- | --- | --- | --- |
| Lunar Surface Transmitter | 5 300 MHz | Single Entry | -77.0 dB | 71 dB |

#### A6.6.1.6 	Summary and analysis of the results of Study A

The results of this study suggest that sharing between lunar SRS and FS operating in the 5 150-5 570 MHz band (co-channel) is feasible. The interference-to-noise ratio metric in the single entry static worst-case analysis shows a margin of 71 dB to satisfy the fixed service protection criteria. This implies that the worst-case aggregate interference from 12.5 million lunar transmitters would still meet the FS protection criteria.

## A6.7	Sharing studies between SRS (active) and SRS operating in the frequency band 5 150-5 570 MHz

### A6.7.1	Study A [USA]

#### A6.7.1.1	Technical and operational characteristics of SRS operating in the 5 150-5 570 MHz frequency range

The technical and operational characteristics for SRS systems operating in the vicinity of the Moon are based on those available in the Report ITU-R SA.2553, including Table 3.2-4, as summarized in Table A6.7.1.1.

Table A6.7.1.1

Technical Characteristics of the SRS Systems

| Station | Lunar Surface Transmitter |
| --- | --- |
| Frequency range | 5 150-5 725 MHz |
| Polarization | Vertical |
| Peak Gain (dBi) | 6 |
| Max. EIRP Density (dBW/MHz) | ‒10 |
| Max. EIRP (dBW) | 6 |
| Channel BW (MHz) | 40 |

#### A6.7.1.2	Technical and operational characteristics of SRS (active) operating in the 5 150 -5 570 MHz frequency range

Document 7B/103 provides relevant technical information to support studies between SRS (active) and SRS under Agenda Item 1.15.  The technical characteristics used in this analysis are shown below in Table A6.7.1.2 based on a spaceborne SAR sensor (SAR-D1 in the WP 7C liaison statement) operating in this frequency band.

Table A6.7.1.2

Characteristics of the SRS (active) Systems

| Parameter | SRS (active) sensor |
| --- | --- |
| Center frequency | 5405 MHz |
| Orbit altitude (above Earth) | 693 km |
| Receiver noise figure (dB) | 3.2 |
| Peak receive antenna gain | 43.5 to 44.8 dBi |
| Receiver bandwidth | 100 MHz |
| Protection criteria (I/N) | -6 dB |

#### A6.7.1.3	Propagation model

This sharing study includes the lunar SRS system on the surface of the Moon and a spaceborne SRS (active) system in Earth orbit. Therefore, Working Party 3J in Document 7B/58 recommends using the propagation model based on Recommendation ITU-R P.525 for this scenario.

#### A6.7.1.4	Methodology

The methodology employed in this study is based on a static analysis of both single and aggregated worst-case interference. It evaluates whether, even under conditions of perfect antenna coupling between the systems, at the minimum distance between the stations on Earth and the Moon, and in the absence of antenna pointing errors and losses such as depolarization and clutter, the interference levels comply with the established ITU-R service protection criteria.

#### A6.7.1.5	Study results

The study was conducted in accordance with the scenarios outlined in Figure A6.7.1.1. For the worst-case analysis, the following assumptions were considered:

–	Main beam antenna coupling between the potentially interfering lunar SRS transmitters and the SRS (active) victim sensor.

–	Free space propagation loss assuming the minimum distance between the stations in Earth orbit and on the Moon (354,307 km), while atmospheric, depolarization and clutter losses are not considered;

–	In the present study, it is assumed that both systems operate at the same central frequency. A bandwidth correction factor (CF) is applied when the bandwidth of the potential interferer exceeds the bandwidth of the victim receiver under analysis. If the bandwidth of the interferer is equal to or smaller than that of the victim receiver, the bandwidth correction factor is 0:

CF = 10 * log10bandwidthvictimbandwidthinterferer

–	The aggregate interference-to-noise ratio (I/N) is calculated considering Lunar SRS transmitters pointing towards the SRS (active) victim sensor using the following equation:

INTotal= 1n (EIRPnθ,φ+CF-FSPL+Grxθ,φ)-Nrx

where n is the number of lunar surface transmitters;  EIRPnθ,φ is the equivalent isotropic radiated power of each lunar transmitter; FSPL is the free space path loss calculated using Recommendation ITU-R P.525; (Grx)θ,φ is the SRS (active) victim receiver maximum antenna gain; and Nrx is the thermal noise power in the SRS (active) victim sensor bandwidth; CF is the frequency-dependent rejection based on the transmit and receiver bandwidths;

–	The receiver thermal noise power is calculated from the receiver noise figure (NF) using the formula provided in Recommendation ITU-R P.372-17.

–	The single and aggregate interference results are compared to the SRS (active) protection criteria to verify whether the service protection criteria is met

Figure A6.7.1.1

SRS–SRS (active) Scenario

Table A6.7.1.3

SRS – SRS (active) co-channel results

| SRS Station | SRS centre frequency | Analysis | I/N result | Margin to the FS protection criteria |
| --- | --- | --- | --- | --- |
| Lunar Surface Transmitter | 5 405 MHz | Single Entry | -46.5 dB | 40.5 dB |

#### A6.7.1.6 	Summary and analysis of the results of Study A

The results of this study suggest that sharing between lunar SRS operating in the 5 150-5 570 MHz band and SRS (active) in low earth orbit is feasible. The interference-to-noise ratio metric in the single entry static worst-case analysis shows a margin of 40.5 dB to satisfy the SRS (active) protection criteria.

## A6.8	Sharing studies between maritime radionavigation service (MRNS) and SRS operating in the frequency band 5 150-5 570 MHz

### A6.8.1	Study A [USA]

#### A6.8.1.1	Technical and operational characteristics of SRS operating in the 5 150-5 570 MHz frequency range

The technical and operational characteristics for SRS systems are based on those available in the Report ITU-R SA.2553, including Table 3.2-4, as summarized in Table A6.8.1.1.

Table A6.8.1.1

Technical Characteristics of the SRS Systems

| Station | Lunar Surface-to-Surface Transmitter |
| --- | --- |
| Frequency range | 5 150-5 725 MHz |
| Polarization | Linear ±45º |
| Peak Gain (dBi) | 6 |
| EIRP Density (dBW/MHz) | −10 |
| Max. EIRP (dBW) | 6 |
| Channel BW (MHz) | 40 MHz |

#### A6.8.1.2	Technical and operational characteristics of maritime radionavigation service operating in the 5 150-5 570 MHz frequency range

The characteristics and protection criteria for radars operating in the maritime radionavigation service are based on the Recommendation ITU-R M.1638, Table 2, as summarized in Table A6.8.1.2.

Table A6.8.1.2

Characteristics of the MRNS radar system

| Parameter | Shipborne (Radar 7) |
| --- | --- |
| Frequency | 5 500 MHz |
| Polarization | Horizontal |
| Antenna gain | 30 dBi |
| Receiver noise figure | 10 dB |
| Receiver IF bandwidth | 1.2 MHz, 10 MHz |
| Protection criteria (I/N) | −6 dB |

#### A6.8.1.3	Propagation model

This sharing study includes the lunar SRS system on the surface of the Moon and shipborne radars operating in the maritime radionavigation service on the Earth's surface. Working Party 3J in Document 7B/58 recommends using the propagation model based on Recommendation ITU-R P.525 for this scenario.

#### A6.8.1.4	Methodology

The methodology employed in this study is based on a static analysis of both single and aggregated worst-case interference. It evaluates whether, even under conditions of perfect antenna coupling between the systems, at the minimum distance between the stations on Earth and the Moon, and in the absence of antenna pointing errors and losses such as depolarization and clutter, the interference levels comply with the established ITU-R service protection criteria.

#### A6.8.1.5	Study results

The study was conducted in accordance with the scenario outlined in Figure A6.8.1.1. For the worst-case analysis, the following assumptions were considered:

–	Main beam antenna coupling between the potentially interfering lunar SRS transmitters and the radar victim receiver.

–	Free space propagation loss assuming the minimum distance between the stations on the Earth and the Moon (355,000 km), while atmospheric, depolarization and clutter losses are not considered.

–	In the present study, it is assumed that both systems operate at the same central frequency. A bandwidth correction factor (CF) is applied when the bandwidth of the potential interferer exceeds the bandwidth of the victim receiver under analysis. If the bandwidth of the interferer is equal to or smaller than that of the victim receiver, the bandwidth correction factor is 0:

CF = 10 * log10bandwidthvictimbandwidthinterferer

–	The aggregate interference-to-noise ratio (I/N) is calculated considering Lunar SRS transmitters pointing towards the radar victim receiver using the following equation:

INTotal= 1n (EIRPnθ,φ+CF-FSPL+Grxθ,φ)-Nrx

where n is the number of lunar surface transmitters;  EIRPnθ,φ is the equivalent isotropic radiated power of each lunar transmitter; FSPL is the free space path loss calculated using Rec. ITU-R P.525; (Grx)θ,φ is the radar victim receiver maximum antenna gain; and Nrx is the thermal noise power in the radar victim receiver bandwidth.

–	The receiver thermal noise power is calculated from the receiver noise figure (NF) using the formula provided in Recommendation ITU-R P.372-17.

–	The single and aggregate I/N results are compared to the radar protection criteria (I/N ≤ −6 dB) to verify whether the maritime radionavigation service protection is met.

Figure A6.8.1.1

SRS–MRNS Scenario

Table A6.8.1.3

SRS – MRNS co-channel results

| SRS Station | SRS centre frequency | Radar | Analysis | I/N result | Margin to the radar protection criteria |
| --- | --- | --- | --- | --- | --- |
| Lunar Surface Transmitter | 5 500 MHz | Shipborne | Single Entry | −64.3 dB | 58.3 dB |

#### A6.8.1.6	Summary and analysis of the results of Study A

The results of this study suggest that sharing between lunar SRS and radars operating in the maritime radionavigation service in the 5 150-5 570 MHz band (co-channel) is feasible. The interference-to-noise ratio metric in the single entry static worst-case analysis shows a margin of 58.3 dB or more to satisfy the maritime radionavigation service protection criteria. This implies that the worst-case aggregate interference from 676 000 lunar transmitters would still meet the MRNS protection criteria.

## A6.9	Sharing studies between radionavigation service (RNS) and SRS operating in the frequency band 5 150-5 570 MHz

### A6.9.1	Study A [USA]

#### A6.9.1.1	Technical and operational characteristics of SRS operating in the 5 150-5 570 MHz frequency range

The technical and operational characteristics for SRS systems are based on those available in the Report ITU-R SA.2553, including Table 3.2-4, as summarized in Table A6.8.1.1.

Table A6.9.1.1

Technical Characteristics of the SRS Systems

| Station | Lunar Surface-to-Surface Transmitter |
| --- | --- |
| Frequency range | 5 150-5 725 MHz |
| Polarization | Linear ±45º |
| Peak Gain (dBi) | 6 |
| EIRP Density (dBW/MHz) | −10 |
| Max. EIRP (dBW) | 6 |
| Channel BW (MHz) | 40 MHz |

#### A6.9.1.2	Technical and operational characteristics of radionavigation service operating in the 5 150-5 570 MHz frequency range

The characteristics and protection criteria for radars operating in the radionavigation service are based on the Recommendation ITU-R M.1638, Table 2, as summarized in Table A6.9.1.2.

Table A6.9.1.2

Characteristics of the RNS radar system

| Parameter | Radionavigation radar (Radar 10A) |
| --- | --- |
| Frequency | 5 465 MHz |
| Polarization | Horizontal |
| Antenna gain | 33 dBi (<55) |
| Receiver noise figure | 3 dB |
| Receiver IF bandwidth | 11 MHz |
| Protection criteria (I/N) | −6 dB |

#### A6.9.1.3	Propagation model

This sharing study includes the lunar SRS system on the surface of the Moon and radars operating in the radionavigation service on the Earth's surface. Working Party 3J in Document 7B/58 recommends using the propagation model based on Recommendation ITU-R P.525 for this scenario.

#### A6.9.1.4	Methodology

The methodology employed in this study is based on a static analysis of both single and aggregated worst-case interference. It evaluates whether, even under conditions of perfect antenna coupling between the systems, at the minimum distance between the stations on Earth and the Moon, and in the absence of antenna pointing errors and losses such as depolarization and clutter, the interference levels comply with the established ITU-R service protection criteria.

#### A6.9.1.5	Study results

The study was conducted in accordance with the scenario outlined in Figure A6.9.1.1. For the worst-case analysis, the following assumptions were considered:

–	Main beam antenna coupling between the potentially interfering lunar SRS transmitters and the radar victim receiver.

–	Free space propagation loss assuming the minimum distance between the stations on the Earth and the Moon (355,000 km), while atmospheric, depolarization and clutter losses are not considered.

–	In the present study, it is assumed that both systems operate at the same central frequency. A bandwidth correction factor (CF) is applied when the bandwidth of the potential interferer exceeds the bandwidth of the victim receiver under analysis. If the bandwidth of the interferer is equal to or smaller than that of the victim receiver, the bandwidth correction factor is 0:

CF = 10 * log10bandwidthvictimbandwidthinterferer

–	The aggregate interference-to-noise ratio (I/N) is calculated considering Lunar SRS transmitters pointing towards the radar victim receiver using the following equation:

INTotal= 1n (EIRPnθ,φ+CF-FSPL+Grxθ,φ)-Nrx

where n is the number of lunar surface transmitters;  EIRPnθ,φ is the equivalent isotropic radiated power of each lunar transmitter; FSPL is the free space path loss calculated using Rec. ITU-R P.525; (Grx)θ,φ is the radar victim receiver maximum antenna gain; and Nrx is the thermal noise power in the radar victim receiver bandwidth.

–	The receiver thermal noise power is calculated from the receiver noise figure (NF) using the formula provided in Recommendation ITU-R P.372-17.

–	The single and aggregate I/N results are compared to the radar protection criteria (I/N ≤ −6 dB) to verify whether the radionavigation service protection is met.

Figure A6.9.1.1

SRS–RNS Scenario

Table A6.9.1.3

SRS – RNS co-channel results

| SRS Station | SRS centre frequency | Radar | Analysis | I/N result | Margin to the RNS protection criteria |
| --- | --- | --- | --- | --- | --- |
| Lunar Surface Transmitter | 5 500 MHz | Shipborne | Single Entry | −32.3 dB | 26.2 dB |

#### A6.9.1.6	Summary and analysis of the results of Study A

The results of this study suggest that sharing between lunar SRS and radars operating in the radionavigation service in the 5 150-5 570 MHz band (co-channel) is feasible. The interference-to-noise ratio metric in the single entry static worst-case analysis shows a margin of 26.2 dB or more to satisfy the radionavigation service protection criteria. This implies that the worst-case aggregate interference from 416 lunar transmitters would still meet the RNS protection criteria.

## A6.[X]	Summary and analysis of the results of studies

[Note: The summary should offer an objective overview, encompassing the results of sharing and compatibility studies, alongside any pertinent information applicable for future comparisons among studies. Additionally, it should address any mitigation techniques or proposals employed to safeguard services operating within the same band or adjacent bands, where applicable, to SRS lunar systems.]

ANNEX 7

Sharing and compatibility studies of SRS for lunar operations
 in the frequency band 5 570-5 725 MHz

## A7.1	Sharing and compatibility of radiolocation service (RLS) and SRS operating in the frequency band 5 570-5 725 MHz

[Note: Insert relevant sharing and compatibility studies for each of the identified service.]

### A7.1.1	Study A [USA]

#### A7.1.1.1 	Technical and operational characteristics of SRS operating in the 5 570-5 725 MHz frequency range

The technical and operational characteristics for SRS systems are based on those available in the Report ITU-R SA.2553, including Table 3.2-4, as summarized in Table A7.1.1.1.

Table A7.1.1.1

Technical Characteristics of the SRS Systems

| Station | Lunar Surface-to-Surface Transmitter |
| --- | --- |
| Frequency range | 5 150-5 725 MHz |
| Polarization | Linear ±45º |
| Peak Gain (dBi) | 6 |
| EIRP Density (dBW/MHz) | -10 |
| Max. EIRP (dBW) | 6 |
| Channel BW (MHz) | 40 MHz |

#### A7.1.1.2	Technical and operational characteristics of radiolocation service (RLS) operating in the 5 570-5 725 MHz frequency range

The characteristics and protection criteria for radars operating in the radiolocation service are based on the Recommendation ITU-R M.1638, Table 3, as summarized in Table A7.1.1.2.

Table A7.1.1.2

Characteristics of the radar systems

| Parameter | Ground | Airborne | Shipborne |
| --- | --- | --- | --- |
| Radar | L | S | Q |
| Frequency range | 5 350-5 850 MHz | 5 250-5 725 MHz | 5 450-5 825 MHz |
| Polarization | Vertical/left- hand circular | Circular | Horizontal |
| Platform height/altitude | 20 m | 9000 m | 40 m |
| Antenna gain | 54 dBi | 40 dBi | 30 dBi |
| Antenna horizontal scan type | N/A (Tracking) | Continuous | 30-270 Sector |
| Receiver noise figure | 5 dB | 3.5 dB | 10 dB |
| Receiver IF bandwidth | 0.25 MHz | 1 MHz | 1 MHz |
| Protection criteria (I/N) | ‒6 dB |  |  |

#### A7.1.1.3	Propagation model

This sharing study includes the lunar SRS system on the surface of the Moon, and radiolocation service operating on Earth's surface. Working Party 3J in Document 7B/58 recommends using the propagation model based on Recommendation ITU-R P.525 for this scenario.

#### A7.1.1.4	Methodology

The methodology employed in this study is based on a static analysis of both single and aggregated worst-case interference. It evaluates whether, even under conditions of perfect antenna coupling between the systems, at the minimum distance between the stations on Earth and the Moon, and in the absence of antenna pointing errors and losses such as depolarization and clutter, the interference levels comply with the established ITU-R service protection criteria.

#### A7.1.1.5	Study results

The study was conducted in accordance with the scenarios outlined in Figures A7.1.1.1 and A.7.1.1.2. For the worst-case analysis, the following assumptions were considered:

–	Main beam antenna coupling between the potentially interfering lunar SRS transmitters and the radar victim receiver.

–	Free space propagation loss assuming the minimum distance between the stations on the Earth and the Moon (355,000 km for ground and shipborne scenarios and 354,991 km for airborne), while atmospheric, depolarization and clutter losses are not considered.

–	In the present study, it is assumed that both systems operate at the same central frequency. A bandwidth correction factor (CF) is applied when the bandwidth of the potential interferer exceeds the bandwidth of the victim receiver under analysis. If the bandwidth of the interferer is equal to or smaller than that of the victim receiver, the bandwidth correction factor is 0:

CF = 10 * log10bandwidthvictimbandwidthinterferer

–	The aggregate interference-to-noise ratio (I/N) is calculated considering Lunar SRS transmitters pointing towards the radar victim receiver using the following equation:

INTotal= 1n (EIRPnθ,φ+CF-FSPL+Grxθ,φ)-Nrx

where n is the number of lunar surface transmitters;  EIRPnθ,φ is the equivalent isotropic radiated power of each lunar transmitter; FSPL is the free space path loss calculated using Rec. ITU-R P.525; (Grx)θ,φ is the radar victim receiver maximum antenna gain; and Nrx is the thermal noise power in the radar victim receiver bandwidth.

–	The receiver thermal noise power is calculated from the receiver noise figure (NF) using the formula provided in Recommendation ITU-R P.372-17.

–	The single and aggregate I/N results are compared to the radar protection criteria (I/N ≤ −6 dB) to verify whether the radiolocation service protection is met.

Figure A7.1.1.1

SRS–RLS Scenarios: (a) Ground and shipborne; (b) Airborne

(a)

(b)

Table A7.1.1.3

SRS – RLS co-channel results

| SRS Station | SRS centre frequency | Radar | Analysis | I/N result | Margin to the radar protection criteria |
| --- | --- | --- | --- | --- | --- |
| Lunar Surface Transmitter | 5590 MHz | Ground | Single Entry | −35.3 dB | 29.3 dB |
| Lunar Surface Transmitter | 5590 MHz | Airborne | Single Entry | −47.8 dB | 41.8 dB |
| Lunar Surface Transmitter | 5590 MHz | Shipborne | Single Entry | −64.3 dB | 58.3 dB |

#### A7.1.1.6	Summary and analysis of the results of Study A

The results of this study suggest that sharing between SRS and radars operating in the radiolocation service in the 5 570-5 725 MHz band (co-channel) is feasible. The interference-to-noise ratio metric shows that in the single entry static worst-case analysis, there is a margin of 29.3 dB or more to satisfy the radiolocation service protection criteria. This implies that the worst-case aggregate interference from 851 lunar surface transmitters would still meet the RLS protection criteria in this band.

### A7.1.2	Study B

### A7.1.3	Study C

## A7.2	Sharing studies between fixed service and SRS for lunar operations in the frequency band 5 570-5 725 MHz

### A7.2.1	Study A [USA]

#### A7.2.1.1	Technical and operational characteristics of SRS operating in the 5 570-5 725 MHz frequency range

The technical and operational characteristics for SRS systems are based on those available in the Report ITU-R SA.2553, including Table 3.2-4, as summarized in Table A8.2.1.1.

Table A7.2.1.1

Technical Characteristics of the SRS Systems

| Station | Lunar Surface-to-Surface Transmitter |
| --- | --- |
| Frequency range | 5 570-5 725 MHz |
| Polarization | Linear ±45º |
| Peak Gain (dBi) | 6 |
| EIRP Density (dBW/MHz) | −10 |
| Max. EIRP (dBW) | 6 |
| Channel BW (MHz) | 40 MHz |

#### A7.2.1.2	Technical and operational characteristics of fixed service operating in the 5 570-5 725 MHz frequency range

Document 7B/201 provides relevant technical information to support studies between SRS and Fixed Services (FS) under agenda item 1.15. As noted by WP 5C, the characteristics for fixed service (FS) in the 5 925 – 6 425 MHz band in Recommendation ITU-R F.758 can be used for the purposes of these sharing studies in the frequency band 5 650 – 5 725 MHz.

Table A7.2.1.2

Characteristics of the FS (PP) systems

| Parameter | Value |  |
| --- | --- | --- |
| Frequency range | 5 650 – 5 725 MHz |  |
| Modulation |  | 64-QAM |  | 128-QAM |  |
| Receiver noise bandwidth |  | 40 MHz |  | 29.65 MHz |  |
| Antenna gain |  | 38.1 – 45.0 dBi |  | 38.7 – 46.6 dBi |  |
| Feeder loss |  | 2.5 – 5.6 dB |  | 1.1 – 3 dB |  |
| Receiver noise power density typical |  | -139 dBW/MHz |  | -140 dBW/MHz |  |
| Protection criteria (I/N) |  | -10 dB |  | -10 dB |  |

#### A7.2.1.3	Propagation model

This sharing study includes the lunar SRS system on the surface of the Moon, and fixed services operating on Earth's surface. Working Party 3J in Document 7B/58 recommends using the propagation model based on Recommendation ITU-R P.525 for this scenario.

#### A7.2.1.4	Methodology

The methodology employed in this study is based on a static analysis of both single and aggregated worst-case interference. It evaluates whether, even under conditions of perfect antenna coupling between the systems, at the minimum distance between the stations on Earth and the Moon, and in the absence of antenna pointing errors and losses such as depolarization and clutter, the interference levels comply with the established ITU-R service protection criteria.

#### A7.2.1.5	Study results

The study was conducted in accordance with the scenario outlined in Figure A7.2.1.1. For the worst-case analysis, the following assumptions were considered:

–	Main beam antenna coupling between the potentially interfering lunar SRS transmitters and the FS victim receiver.

–	Free space propagation loss assuming the minimum distance between the stations on the Earth and the Moon (355,000 km), while atmospheric, depolarization and clutter losses are not considered.

–	In the present study, it is assumed that both systems operate at the same central frequency. A bandwidth correction factor (CF) is applied when the bandwidth of the potential interferer exceeds the bandwidth of the victim receiver under analysis. If the bandwidth of the interferer is equal to or smaller than that of the victim receiver, the bandwidth correction factor is 0:

CF = 10 * log10bandwidthvictimbandwidthinterferer

–	The aggregate interference-to-noise ratio (I/N) is calculated considering Lunar SRS transmitters pointing towards the FS victim receiver using the following equation:

INTotal= 1n (EIRPnθ,φ+CF-FSPL-Floss+Grxθ,φ)-Nrx

where n is the number of lunar surface transmitters;  EIRPnθ,φ is the equivalent isotropic radiated power of each lunar transmitter; FSPL is the free space path loss calculated using Rec. ITU-R P.525; Floss is the FS feeder loss; (Grx)θ,φ is the FS victim receiver maximum antenna gain; and Nrx is the thermal noise power in the FS victim receiver bandwidth.

–	The single and aggregate I/N results are compared to the FS protection criteria (I/N ≤ −10 dB) to verify whether the FS protection is met.

Figure A7.2.1.1

SRS–FS Scenario

Table A7.2.1.3

SRS - FS co-channel results

| SRS Station | SRS centre frequency | Analysis | I/N result | Margin to the FS protection criteria |
| --- | --- | --- | --- | --- |
| Lunar Surface Transmitter | 5687.5 MHz | Single Entry | -43.1 dB | 33.1 dB |

#### A7.2.1.6	Summary and analysis of the results of Study A

The interference-to-noise ratio metric shows that even in the single entry static worst-case analysis, there is a margin of at 33.1 dB or more to satisfy the protection criteria for FS applications. This also implies that aggregating over 2401 simultaneously transmitting worst case towards the Earth interfering transmitters would still meet the FS protection criteria. Thus, the results of this static worst-case study suggest that sharing between SRS and terrestrial FS operating in the 5 570-5 725 MHz band (co-channel) is feasible.

## A7.3	Sharing studies between mobile service (MS) and SRS operating in the frequency band 5 570-5 725 MHz

### A7.3.1	Study A [USA]

#### A7.3.1.1	Technical and operational characteristics of SRS operating in the 5 570-5 725 MHz frequency range

The technical and operational characteristics for SRS systems are based on those available in the Report ITU-R SA.2553, including Table 3.2-4, as summarized in Table A7.3.1.1.

Table A7.3.1.1

Technical Characteristics of the SRS Systems

| Station | Lunar Surface Transmitter |
| --- | --- |
| Frequency range | 5 150-5 725 MHz |
| Polarization | Vertical |
| Peak Gain (dBi) | 6 |
| Max. EIRP Density (dBW/MHz) | ‒10 |
| Max. EIRP (dBW) | 6 |
| Channel BW (MHz) | 40 |

#### A7.3.1.2	Technical and operational characteristics of mobile service operating in the 5 570 -5 725 MHz frequency range

Document 7B/115 provides relevant technical information to support studies between SRS and mobile service under Agenda Item 1.15.   The documents referenced by WP 5A do not contain characteristics for the mobile service in the 5 570-5 725 MHz frequency range; however, the mobile service characteristics from Table 2b of Report ITU-R M.2116-2 for the 3.4-4.2 GHz band have been applied for this study.

Table A7.3.1.2

Characteristics of the MS Systems

| Parameter | Mobile service receiver |
| --- | --- |
| Frequency range | 5 650 – 5 850 MHz |
| Receiver noise figure (dB) | 8 |
| Antenna gain | 17 dBi |
| Receiver bandwidth | 7 MHz |
| Protection criteria (I/N) | -6 dB |

#### A7.3.1.3	Propagation model

This sharing study includes the lunar SRS system on the surface of the Moon and mobile services  operating on Earth's surface. Therefore, Working Party 3J in Document 7B/58 recommends using the propagation model based on Recommendation ITU-R P.525 for this scenario.

#### A7.3.1.4	Methodology

The methodology employed in this study is based on a static analysis of both single and aggregated worst-case interference. It evaluates whether, even under conditions of perfect antenna coupling between the systems, at the minimum distance between the stations on Earth and the Moon, and in the absence of antenna pointing errors and losses such as depolarization and clutter, the interference levels comply with the established ITU-R service protection criteria.

#### A7.3.1.5	Study results

The study was conducted in accordance with the scenarios outlined in Figure A6.5.1.1. For the worst-case analysis, the following assumptions were considered:

–	Main beam antenna coupling between the potentially interfering lunar SRS transmitters and the MS victim receiver.

–	Free space propagation loss assuming the minimum distance between the stations on the Earth and the Moon (355,000 km), while atmospheric, depolarization and clutter losses are not considered;

–	In the present study, it is assumed that both systems operate at the same central frequency. A bandwidth correction factor (CF) is applied when the bandwidth of the potential interferer exceeds the bandwidth of the victim receiver under analysis. If the bandwidth of the interferer is equal to or smaller than that of the victim receiver, the bandwidth correction factor is 0:

CF = 10 * log10bandwidthvictimbandwidthinterferer

–	The aggregate interference-to-noise ratio (I/N) is calculated considering Lunar SRS transmitters pointing towards the MS victim receiver using the following equation:

INTotal= 1n (EIRPnθ,φ+CF-FSPL+Grxθ,φ)-Nrx

where n is the number of lunar surface transmitters;  EIRPnθ,φ is the equivalent isotropic radiated power of each lunar transmitter; FSPL is the free space path loss calculated using Recommendation ITU-R P.525; (Grx)θ,φ is the MS victim receiver maximum antenna gain; and Nrx is the thermal noise power in the MS victim receiver bandwidth; CF is the frequency-dependent rejection based on the transmit and receiver bandwidths;

–	The receiver thermal noise power is calculated from the receiver noise figure (NF) using the formula provided in Recommendation ITU-R P.372-17.

–	The single and aggregate interference results are compared to the mobile service protection criteria to verify whether the service protection criteria is met

Figure A7.3.1.1

SRS–MS Scenario

(a)

Table A7.3.1.3

SRS – MS co-channel results

| SRS Station | SRS centre frequency | Analysis | I/N result | Margin to the MS protection criteria |
| --- | --- | --- | --- | --- |
| Lunar Surface Transmitter | 5 700 MHz | Single Entry | -75.6 dB | 69.9 dB |

#### A7.3.1.6 	Summary and analysis of the results of Study A

The results of this study suggest that sharing between lunar SRS operating in the 5 570-5 725 MHz band and the mobile service (co-channel) is feasible. The interference-to-noise ratio metric shows that in the single entry static worst-case analysis, there is a margin of 69.6 dB or more to satisfy the MS protection criteria. This implies that even the worst-case aggregate interference from 9.12 million lunar surface transmitters would not exceed the MS protection criterion in this band.

## A7.4	Sharing studies between aeronautical radionavigation service (ARNS) and SRS operating in the frequency band 5 570-5 725 MHz

### A7.4.1	Study A [USA]

#### A7.4.1.1	Technical and operational characteristics of SRS operating in the 5 570-5 725 MHz frequency range

The technical and operational characteristics for SRS systems are based on those available in the Report ITU-R SA.2553, including Table 3.2-4, as summarized in Table A7.4.1.1.

Table A7.4.1.1

Technical Characteristics of the SRS Systems

| Station | Lunar Surface-to-Surface Transmitter |
| --- | --- |
| Frequency range | 5 150-5 725 MHz |
| Polarization | Linear ±45º |
| Peak Gain (dBi) | 6 |
| EIRP Density (dBW/MHz) | −10 |
| Max. EIRP (dBW) | 6 |
| Channel BW (MHz) | 40 MHz |

#### A7.4.1.2	Technical and operational characteristics of aeronautical radionavigation service (ARNS) operating in the 5 570-5 725 MHz frequency range

The characteristics and protection criteria for radars operating in the aeronautical radionavigation service (ARNS) are based on the Recommendation ITU-R M.1638, Table 2, as summarized in Table A7.4.1.2.

Table A7.4.1.2

Characteristics of the ARNS radar system

| Parameter | Airborne (Radar 9) |
| --- | --- |
| Center frequency | 5 600 MHz |
| Polarization | Circular |
| Platform height/altitude | 9000 m (Aircraft altitude) |
| Antenna gain | 30-40 dBi |
| Antenna horizontal scan type | Continuous |
| Receiver noise figure | 3.5 dB |
| Receiver IF bandwidth | 1 MHz |
| Protection criteria (I/N) | −6 dB |

#### A7.4.1.3	Propagation model

This sharing study includes the lunar SRS system on the surface of the Moon and ARNS operating on an airborne platform above the Earth's surface. Working Party 3J in Document 7B/58 recommends using the propagation model based on Recommendation ITU-R P.525 for this scenario.

#### A7.4.1.4	Methodology

The methodology employed in this study is based on a static analysis of both single and aggregated worst-case interference. It evaluates whether, even under conditions of perfect antenna coupling between the systems, at the minimum distance between the stations on Earth and the Moon, and in the absence of antenna pointing errors and losses such as depolarization and clutter, the interference levels comply with the established ITU-R service protection criteria.

#### A7.4.1.5	Study results

The study was conducted in accordance with the scenario outlined in Figure A7.4.1.1. For the worst-case analysis, the following assumptions were considered:

–	Main beam antenna coupling between the potentially interfering lunar SRS transmitters and the radar victim receiver.

–	Free space propagation loss assuming the minimum distance between the stations on the Earth and the Moon (354,991 km), considering the aircraft altitude, while atmospheric, depolarization and clutter losses are not considered.

–	In the present study, it is assumed that both systems operate at the same central frequency. A bandwidth correction factor (CF) is applied when the bandwidth of the potential interferer exceeds the bandwidth of the victim receiver under analysis. If the bandwidth of the interferer is equal to or smaller than that of the victim receiver, the bandwidth correction factor is 0:

CF = 10 * log10bandwidthvictimbandwidthinterferer

–	The aggregate interference-to-noise ratio (I/N) is calculated considering Lunar SRS transmitters pointing towards the radar victim receiver using the following equation:

INTotal= 1n (EIRPnθ,φ+CF-FSPL+Grxθ,φ)-Nrx

where n is the number of lunar surface transmitters;  EIRPnθ,φ is the equivalent isotropic radiated power of each lunar transmitter; FSPL is the free space path loss calculated using Rec. ITU-R P.525; (Grx)θ,φ is the radar victim receiver maximum antenna gain; and Nrx is the thermal noise power in the radar victim receiver bandwidth.

–	The receiver thermal noise power is calculated from the receiver noise figure (NF) using the formula provided in Recommendation ITU-R P.372-17.

–	The single and aggregate I/N results are compared to the radar protection criteria (I/N ≤ −6 dB) to verify whether the ARNS protection is met.

Figure A7.4.1.1

SRS–ARNS Scenario

Table A7.4.1.3

SRS – ARNS co-channel results

| SRS Station | SRS centre frequency | Radar | Analysis | I/N result | Margin to the ARNS protection criteria |
| --- | --- | --- | --- | --- | --- |
| Lunar Surface Transmitter | 5 600 MHz | Airborne | Single Entry | −48.0 dB | 42.0 dB |

#### A7.4.1.6	Summary and analysis of the results of Study A

The results of this study suggest that sharing between lunar SRS and ARNS in the 5 570-5 725 MHz band (co-channel) is feasible. The interference-to-noise ratio metric in the single entry static worst-case analysis shows a margin of 42 dB or more to satisfy the aeronautical radionavigation service protection criteria. This implies that the worst-case aggregate interference from 15848 lunar transmitters would still meet the ARNS protection criteria.

## A7.5	Sharing studies between maritime radionavigation service and SRS operating in the frequency band 5 570-5 725 MHz

### A7.5.1	Study A [USA]

#### A7.5.1.1	Technical and operational characteristics of SRS operating in the 5 570-5 725 MHz frequency range

The technical and operational characteristics for SRS systems are based on those available in the Report ITU-R SA.2553, including Table 3.2-4, as summarized in Table A7.4.1.1.

Table A7.5.1.1

Technical Characteristics of the SRS Systems

| Station | Lunar Surface-to-Surface Transmitter |
| --- | --- |
| Frequency range | 5 150-5 725 MHz |
| Polarization | Linear ±45º |
| Peak Gain (dBi) | 6 |
| EIRP Density (dBW/MHz) | −10 |
| Max. EIRP (dBW) | 6 |
| Channel BW (MHz) | 40 MHz |

#### A7.5.1.2	Technical and operational characteristics of maritime radionavigation service operating in the 5 570-5 725 MHz frequency range

The characteristics and protection criteria for radars operating in the maritime radionavigation service are based on the Recommendation ITU-R M.1638, Table 2, as summarized in Table A7.5.1.2.

Table A7.5.1.2

Characteristics of the maritime radionaviation radar system

| Parameter | Shipborne (Radar 20) |
| --- | --- |
| Center frequency | 5 600 MHz |
| Polarization | Horizontal |
| Antenna gain | 40 dBi |
| Receiver noise figure | 2 dB |
| Receiver IF bandwidth | 0.5 MHz |
| Protection criteria (I/N) | −6 dB |

#### A7.5.1.3	Propagation model

This sharing study includes the lunar SRS system on the surface of the Moon and a shipborne radar operating on the Earth's surface. Working Party 3J in Document 7B/58 recommends using the propagation model based on Recommendation ITU-R P.525 for this scenario.

#### A7.5.1.4	Methodology

The methodology employed in this study is based on a static analysis of both single and aggregated worst-case interference. It evaluates whether, even under conditions of perfect antenna coupling between the systems, at the minimum distance between the stations on Earth and the Moon, and in the absence of antenna pointing errors and losses such as depolarization and clutter, the interference levels comply with the established ITU-R service protection criteria.

#### A7.5.1.5	Study results

The study was conducted in accordance with the scenario outlined in Figure A7.5.1.1. For the worst-case analysis, the following assumptions were considered:

–	Main beam antenna coupling between the potentially interfering lunar SRS transmitters and the radar victim receiver.

–	Free space propagation loss assuming the minimum distance between the stations on the Earth and the Moon (355,000 km), while atmospheric, depolarization and clutter losses are not considered.

–	In the present study, it is assumed that both systems operate at the same central frequency. A bandwidth correction factor (CF) is applied when the bandwidth of the potential interferer exceeds the bandwidth of the victim receiver under analysis. If the bandwidth of the interferer is equal to or smaller than that of the victim receiver, the bandwidth correction factor is 0:

CF = 10 * log10bandwidthvictimbandwidthinterferer

–	The aggregate interference-to-noise ratio (I/N) is calculated considering Lunar SRS transmitters pointing towards the radar victim receiver using the following equation:

INTotal= 1n (EIRPnθ,φ+CF-FSPL+Grxθ,φ)-Nrx

where n is the number of lunar surface transmitters;  EIRPnθ,φ is the equivalent isotropic radiated power of each lunar transmitter; FSPL is the free space path loss calculated using Rec. ITU-R P.525; (Grx)θ,φ is the radar victim receiver maximum antenna gain; and Nrx is the thermal noise power in the radar victim receiver bandwidth.

–	The receiver thermal noise power is calculated from the receiver noise figure (NF) using the formula provided in Recommendation ITU-R P.372-17.

–	The single and aggregate I/N results are compared to the radar protection criteria (I/N ≤ −6 dB) to verify whether the maritime radionavigation service protection is met.

Figure A7.5.1.1

SRS–MRNS Scenario

Table A7.5.1.3

SRS – MRNS co-channel results

| SRS Station | SRS centre frequency | Radar | Analysis | I/N result | Margin to the MRNS protection criteria |
| --- | --- | --- | --- | --- | --- |
| Lunar Surface Transmitter | 5 600 MHz | Airborne | Single Entry | −46.5 dB | 40.5 dB |

#### A7.5.1.6	Summary and analysis of the results of Study A

The results of this study suggest that sharing between lunar SRS and maritime radionavigation service in the 5 570-5 725 MHz band (co-channel) is feasible. The interference-to-noise ratio metric in the single entry static worst-case analysis shows a margin of 40.5 dB or more to satisfy the maritime radionavigation service protection criteria. This implies that the worst-case aggregate interference from 11220 lunar transmitters would still meet the maritime radionavigation service protection criteria.

## A7.[X]	Summary and analysis of the results of studies

[Note: The summary should offer an objective overview, encompassing the results of sharing and compatibility studies, alongside any pertinent information applicable for future comparisons among studies. Additionally, it should address any mitigation techniques or proposals employed to safeguard services operating within the same band or adjacent bands, where applicable, to SRS lunar systems.]

ANNEX 8

Sharing and compatibility studies of SRS for lunar operations
 in the frequency band 5 775-5 925 MHz

## A8.1	Sharing and compatibility of radiolocation service (RLS) and SRS operating in the frequency band 5 775-5 925 MHz

[Note: Insert relevant sharing and compatibility studies for each of the identified service.]

### A8.1.1	Study A [USA]

#### A8.1.1.1	Technical and operational characteristics of SRS operating in the 5 775-5 925 MHz frequency range

The technical and operational characteristics for SRS systems are based on those available in the Report ITU-R SA.2553, including Table 3.2-4, as summarized in Table A8.1.1.1.

Table A8.1.1.1

Technical Characteristics of the SRS Systems

| Station | Lunar Surface-to-Surface Transmitter |  |
| --- | --- | --- |
| Frequency range | 5 775-5 855 MHz | 5 855-5 925 MHz |
| Polarization | Vertical | Linear ±45º |
| Peak Gain (dBi) | 6 | 3 |
| EIRP Density (dBW/MHz) | −10 | −16 |
| Max. EIRP (dBW) | 6 | 0 |
| Channel BW (MHz) | 40 MHz |  |

#### A8.1.1.2	Technical and operational characteristics of radiolocation service (RLS) operating in the 5 775-5 925 MHz frequency range

The characteristics and protection criteria for radars operating in the radiolocation service are based on the Recommendation ITU-R M.1638, Table 3, as summarized in Table A8.1.1.2.

Table A8.1.1.2

Characteristics of the radar systems

| Parameter | Ground | Ground | Shipborne |
| --- | --- | --- | --- |
| Radar | L | N | Q |
| Frequency range | 5 350-5 850 MHz | 5 400-5900 MHz | 5 450-5 825 MHz |
| Polarization | Vertical/left- hand circular | Vertical/left- hand circular | Horizontal |
| Platform height/altitude | 20 m | 20 | 40 m |
| Antenna gain | 54 dBi | 45.9 dBi | 30 dBi |
| Antenna horizontal scan type | N/A (Tracking) | N/A (Tracking) | 30-270 Sector |
| Receiver noise figure | 5 dB | 11 dB | 10 dB |
| Receiver IF bandwidth | 0.25 MHz | 2 MHz | 1 MHz |
| Protection criteria (I/N) | ‒6 dB |  |  |

#### A8.1.1.3	Propagation model

This sharing study includes the lunar SRS system on the surface of the Moon, and radiolocation service operating on Earth's surface. Working Party 3J in Document 7B/58 recommends using the propagation model based on Recommendation ITU-R P.525 for this scenario.

#### A8.1.1.4	Methodology

The methodology employed in this study is based on a static analysis of both single and aggregated worst-case interference. It evaluates whether, even under conditions of perfect antenna coupling between the systems, at the minimum distance between the stations on Earth and the Moon, and in the absence of antenna pointing errors and losses such as depolarization and clutter, the interference levels comply with the established ITU-R service protection criteria.

#### A8.1.1.5	Study results

The study was conducted in accordance with the scenario outlined in Figure A8.1.1.1. For the worst-case analysis, the following assumptions were considered:

–	Main beam antenna coupling between the potentially interfering lunar SRS transmitters and the radar victim receiver.

–	Free space propagation loss assuming the minimum distance between the stations on the Earth and the Moon (355,000 km), while atmospheric, depolarization and clutter losses are not considered.

–	In the present study, it is assumed that both systems operate at the same central frequency. A bandwidth correction factor (CF) is applied when the bandwidth of the potential interferer exceeds the bandwidth of the victim receiver under analysis. If the bandwidth of the interferer is equal to or smaller than that of the victim receiver, the bandwidth correction factor is 0:

CF = 10 * log10bandwidthvictimbandwidthinterferer

–	The aggregate interference-to-noise ratio (I/N) is calculated considering Lunar SRS transmitters pointing towards the radar victim receiver using the following equation:

INTotal= 1n (EIRPnθ,φ+CF-FSPL+Grxθ,φ)-Nrx

where n is the number of lunar surface transmitters;  EIRPnθ,φ is the equivalent isotropic radiated power of each lunar transmitter; FSPL is the free space path loss calculated using Recommendation ITU-R P.525; (Grx)θ,φ is the radar victim receiver maximum antenna gain; and Nrx is the thermal noise power in the radar victim receiver bandwidth.

–	The receiver thermal noise power is calculated from the receiver noise figure (NF) using the formula provided in Recommendation ITU-R P.372-17.

–	The single and aggregate I/N results are compared to the radar protection criteria (I/N ≤ −6 dB) to verify whether the radiolocation service protection is met.

Figure A8.1.1.1

SRS-RLS Scenario

Table A8.1.1.3

SRS – RLS co-channel results

| SRS Station | SRS centre frequency | Radar | Analysis | I/N result | Margin to the radar protection criteria |
| --- | --- | --- | --- | --- | --- |
| Lunar Surface Transmitter | 5795 MHz | Ground (L) | Single Entry | -35.7 dB | 29.7 dB |
| Lunar Surface Transmitter | 5875 MHz | Ground (N) | Single Entry | -42.9 dB | 36.9 dB |
| Lunar Surface Transmitter | 5795 MHz | Shipborne (Q) | Single Entry | -64.7 dB | 58.7 dB |

#### A8.1.1.6	Summary and analysis of the results of Study A

The results of this study suggest that sharing between SRS and radars operating in the radiolocation service in the 5 775-5 925 MHz band (co-channel) is feasible. The interference-to-noise ratio metric shows that in the single entry static worst-case analysis, there is a margin of 29.7 dB or more to satisfy the radiolocation service protection criteria. This implies that the worst-case aggregate interference from 933 lunar surface transmitters would still meet the RLS protection criteria in this band.

### A8.1.2	Study B

### A8.1.3	Study C

## A8.2	Sharing studies between fixed service and SRS for lunar operations in the frequency band 5 775 - 5 925 MHz

### A8.2.1	Study A [USA]

#### A8.2.1.1	Technical and operational characteristics of SRS operating in the 5 775-5 925 MHz frequency range

The technical and operational characteristics for SRS systems are based on those available in the Report ITU-R SA.2553, including Table 3.2-4, as summarized in Table A8.2.1.1.

Table A8.2.1.1

Technical Characteristics of the SRS Systems

| Station | Lunar Surface-to-Surface Transmitter |  |
| --- | --- | --- |
| Frequency range | 5 775-5 855 MHz | 5 855-5 925 MHz |
| Polarization | Vertical | Linear ±45º |
| Peak Gain (dBi) | 6 | 3 |
| EIRP Density (dBW/MHz) | -10 | −16 |
| Max. EIRP (dBW) | 6 | 0 |
| Channel BW (MHz) | 40 MHz |  |

#### A8.2.1.2	Technical and operational characteristics of fixed service operating in the 5 775-5 925 MHz frequency range

Document 7B/56 provides relevant technical information to support studies between SRS and Fixed Services (FS) under agenda item 1.15. The characteristics for fixed service (FS) for point-to-point (PP) systems are based on the information contained in the Report ITU-R F.2108, Table 9, as summarized in Table A8.2.1.2.

Table A8.2.1.2

Characteristics of the FS (PP) systems

| Parameter | Value |
| --- | --- |
| Frequency range | 5 850-7 025 MHz |
| Modulation | 64-QAM |
| Receiver noise bandwidth | 10 MHz |
| Antenna height | 30 m |
| Antenna pattern | Recommendation ITU-R F.699 |
| Antenna gain | 43 dBi |
| Feeder loss | 3 dB |
| Receiver thermal noise (Nrx) | −130 dBW |
| Protection criteria (I/N) | −10 dB |

#### A8.2.1.3	Propagation model

This sharing study includes the lunar SRS system on the surface of the Moon, and fixed services operating on Earth's surface. Working Party 3J in Document 7B/58 recommends using the propagation model based on Recommendation ITU-R P.525 for this scenario.

#### A8.2.1.4	Methodology

The methodology employed in this study is based on a static analysis of both single and aggregated worst-case interference. It evaluates whether, even under conditions of perfect antenna coupling between the systems, at the minimum distance between the stations on Earth and the Moon, and in the absence of antenna pointing errors and losses such as depolarization and clutter, the interference levels comply with the established ITU-R service protection criteria.

#### A8.2.1.5	Study results

The study was conducted in accordance with the scenario outlined in Figure A8.2.1.1. For the worst-case analysis, the following assumptions were considered:

–	Main beam antenna coupling between the potentially interfering lunar SRS transmitters and the FS victim receiver.

–	Free space propagation loss assuming the minimum distance between the stations on the Earth and the Moon (355,000 km), while atmospheric, depolarization and clutter losses are not considered.

–	In the present study, it is assumed that both systems operate at the same central frequency. A bandwidth correction factor (CF) is applied when the bandwidth of the potential interferer exceeds the bandwidth of the victim receiver under analysis. If the bandwidth of the interferer is equal to or smaller than that of the victim receiver, the bandwidth correction factor is 0:

CF = 10 * log10bandwidthvictimbandwidthinterferer

–	The aggregate interference-to-noise ratio (I/N) is calculated considering Lunar SRS transmitters pointing towards the FS victim receiver using the following equation:

INTotal= 1n (EIRPnθ,φ+CF-FSPL-Floss+Grxθ,φ)-Nrx

where n is the number of lunar surface transmitters;  EIRPnθ,φ is the equivalent isotropic radiated power of each lunar transmitter; FSPL is the free space path loss calculated using Rec. ITU-R P.525; Floss is the FS feeder loss; (Grx)θ,φ is the FS victim receiver maximum antenna gain; and Nrx is the thermal noise power in the FS victim receiver bandwidth.

–	The single and aggregate I/N results are compared to the FS protection criteria (I/N ≤ −10 dB) to verify whether the FS protection is met.

Figure A8.2.1.1

SRS–FS Scenario

Table A8.2.1.3

SRS - FS co-channel results

| SRS Station | SRS centre frequency | Analysis | I/N result | Margin to the FS protection criteria |
| --- | --- | --- | --- | --- |
| Lunar Surface Transmitter | 5875 MHz | Single Entry | −41.8 dB | 31.8 dB |

#### A8.2.1.6	Summary and analysis of the results of Study A

The results of this study suggest that sharing between SRS and terrestrial FS operating in the 5 775-5 925 MHz band (co-channel) is feasible. The interference-to-noise ratio metric shows that even in the static worst-case analysis, including single and aggregated potential interferers, there is a margin of 31.8 dB or more to satisfy the FS protection criteria. This implies that the worst-case aggregate interference from even 1513 lunar surface transmitters would not exceed the FS protection criterion.

## A8.3	Sharing studies between fixed satellite service (FSS) and SRS operating in the frequency band 5 775 - 5 925 MHz

### A4.6.1	Study A [USA]

In Document 7B/200, WP 4A expressed the view that sharing studies to ensure protection of FSS are not required in the 5 775 – 5 925 MHz band, provided that possible future allocations to space research service (space-to-space) in the 5 775 – 5 925 MHz band under WRC-27 agenda item 1.15 are limited to Lunar surface-to-surface communications, as specified in Report ITU-R SA.2553.

## A8.4	Sharing studies between mobile service and SRS operating in the frequency band 5 775 - 5 925 MHz

### A8.4.1	Study A [USA]

#### A8.4.1.1	Technical and operational characteristics of SRS operating in the 5 775-5 855 MHz frequency range

The technical and operational characteristics for SRS systems are based on those available in the Report ITU-R SA.2553, including Table 3.2-4, as summarized in Table A8.4.1.1.

Table A8.4.1.1

Technical Characteristics of the SRS Systems

| Station | Lunar Surface-to-Surface Transmitter |
| --- | --- |
| Frequency range | 5 775-5 855 MHz |
| Polarization | Vertical |
| Peak Gain (dBi) | 6 |
| EIRP Density (dBW/MHz) | −10 |
| Max. EIRP (dBW) | 6 |
| Channel BW (MHz) | 40 MHz |

#### A8.4.1.2	Technical and operational characteristics of mobile service operating in the 5 775-5 925 MHz frequency range

Document 7B/115 provides relevant technical information to support studies between SRS and Mobile Services (MS) under agenda item 1.15. The documents referenced by WP 5A do not contain characteristics for the mobile service in the 5 775-5 925 MHz frequency range; however, the mobile service characteristics from Table 2b of Report ITU-R M.2116-2 for the 3.4-4.2 GHz band have been applied for this study.

Table A8.4.1.2

Characteristics of the MS systems

| Parameter | Value |
| --- | --- |
| Frequency range | 5 725 – 5 825 MHz |
| Channel bandwidth | 7 MHz |
| Antenna gain | 17 dBi |
| Receiver noise figure (dBNrx) | 8 |
| Protection criteria (I/N) | -6 dB |

#### A8.4.1.3	Propagation model

This sharing study includes the lunar SRS system on the surface of the Moon, and mobile services operating on Earth's surface. Working Party 3J in Document 7B/58 recommends using the propagation model based on Recommendation ITU-R P.525 for this scenario.

#### A8.4.1.4	Methodology

The methodology employed in this study is based on a static analysis of both single and aggregated worst-case interference. It evaluates whether, even under conditions of perfect antenna coupling between the systems, at the minimum distance between the stations on Earth and the Moon, and in the absence of antenna pointing errors and losses such as depolarization and clutter, the interference levels comply with the established ITU-R service protection criteria.

#### A8.4.1.5	Study results

The study was conducted in accordance with the scenario outlined in Figure A8.4.1.1. For the worst-case analysis, the following assumptions were considered:

–	Main beam antenna coupling between the potentially interfering lunar SRS transmitters and the FS victim receiver.

–	Free space propagation loss assuming the minimum distance between the stations on the Earth and the Moon (355,000 km), while atmospheric, depolarization and clutter losses are not considered.

–	In the present study, it is assumed that both systems operate at the same central frequency. A bandwidth correction factor (CF) is applied when the bandwidth of the potential interferer exceeds the bandwidth of the victim receiver under analysis. If the bandwidth of the interferer is equal to or smaller than that of the victim receiver, the bandwidth correction factor is 0:

CF = 10 * log10bandwidthvictimbandwidthinterferer

–	The aggregate interference-to-noise ratio (I/N) is calculated considering Lunar SRS transmitters pointing towards the MS victim receiver using the following equation:

INTotal= 1n (EIRPnθ,φ+CF-FSPL-Floss+Grxθ,φ)-Nrx

where n is the number of lunar surface transmitters;  EIRPnθ,φ is the equivalent isotropic radiated power of each lunar transmitter; FSPL is the free space path loss calculated using Rec. ITU-R P.525; Floss is the FS feeder loss; (Grx)θ,φ is the MS victim receiver maximum antenna gain; and Nrx is the thermal noise power in the MS victim receiver bandwidth.

–	The single and aggregate I/N results are compared to the MS protection criteria to verify whether the MS protection is met.

Figure A8.4.1.1

SRS–MS Scenario

Table A8.4.1.3

SRS - MS co-channel results

| SRS Station | SRS centre frequency | Analysis | I/N result | Margin to the MS protection criteria |
| --- | --- | --- | --- | --- |
| Lunar Surface Transmitter | 5815 MHz | Single Entry | -75.8 dB | 69.8 dB |

#### A8.4.1.6	Summary and analysis of the results of Study A

The results of this study suggest that sharing between lunar SRS operating in the 5 775-5 925 MHz band and the mobile service (co-channel) is feasible. The interference-to-noise ratio metric shows that in the single entry static worst-case analysis, there is a margin of 69.8 dB or more to satisfy the MS protection criteria. This implies that even the worst-case aggregate interference from 9.55 million lunar surface transmitters would not exceed the MS protection criterion in this band.

## A8.[X]	Summary and analysis of the results of studies

[Note: The summary should offer an objective overview, encompassing the results of sharing and compatibility studies, alongside any pertinent information applicable for future comparisons among studies. Additionally, it should address any mitigation techniques or proposals employed to safeguard services operating within the same band or adjacent bands, where applicable, to SRS lunar systems.]

ANNEX 9

Sharing and compatibility studies of SRS for lunar operations
in the frequency band 7 190-7 235 MHz

## A9.1	Sharing and compatibility of fixed service and SRS (space-to-space) operating in the frequency band 7 190-7 235 MHz

[Note: Insert relevant sharing and compatibility studies for each of the identified service.]

### A9.1.1	Study A[CHN]

#### A9.1.1.1 	Technical and operational characteristics of SRS operating in the 7 190-7 235 MHz frequency range

The technical and operational characteristics for SRS systems are based on those available in the Report ITU-R SA.2553, including Table 10, as summarized in Table A9.1.1.1.

Table A9.1.1.1

Technical characteristics of the SRS systems

| Station | Lunar orbit space station transmitter |
| --- | --- |
| Frequency range (MHz) | 7 190-7 235 |
| Polarization | CP |
| Peak gain (dBi) | 46 |
| Transmit antenna pattern | Rec. ITU-R S.672-4 (Ls = −20 dB) |
| EIRP density (dBW/Hz) | 0.5 |
| Max. EIRP (dBW) | 57.5 |
| Channel BW (MHz) | 0.5 |

#### A9.1.1.2 	Technical and operational characteristics of fixed service operating in the 7 190-7 235 MHz frequency range

Document 7B/56 provides relevant technical information to support studies between SRS and fixed services (FS) under agenda item 1.15. The characteristics for FS for point-to-point (PP) systems are based on the information contained in Recommendation ITU-R F.758-8, Table 9, as summarized in Table A9.1.1.2.

Table A9.1.1.2

Characteristics of the FS (PP) systems

| Parameter | Value |  |
| --- | --- | --- |
| Frequency range (MHz) | 7 110-7 900 |  |
| Reference | Rec. ITU-R F.385 |  |
| Modulation format | 16-QAM | 128-QAM |
| Receiver noise bandwidth (MHz) | 3.5 | 3.5 |
| Antenna gain (dBi) | 48.6 | 48.6 |
| Feeder loss (dB) | 3 | 3 |
| Receiver noise figure (dB) | 2.5 | 2.5 |
| Protection criteria (I/N) (dB) | ‒10 | ‒10 |

#### A9.1.1.3 	Propagation model

This sharing study includes the lunar SRS system on the surface of the Moon, and fixed services operating on Earth’s surface. Working Party 3J in Document 7B/58 recommends using the propagation model based on Recommendation ITU-R P.525 for this scenario.

#### A9.1.1.4 	Methodology

The methodology employed in this study is based on a static analysis of both single and aggregated worst-case interference. It evaluates whether, even under conditions of perfect antenna coupling between the systems, at the minimum distance between the stations on Earth and the Moon, and in the absence of antenna pointing errors and losses such as depolarization and clutter, the interference levels comply with the established ITU-R service protection criteria.

#### A9.1.1.5 	Study results

The study was conducted in accordance with the scenario outlined in Figure A9.1.1.1. For the worst-case analysis, the following assumptions were considered:

–	Coupling between the sidelobe of the potentially interfering lunar SRS transmitters and the Main beam antenna of FS victim receiver.

–	Free space propagation loss assuming the minimum distance between the stations on the Earth and the Moon , while atmospheric, depolarization and clutter losses are not considered.

–	The aggregate interference-to-noise ratio (I/N) is calculated considering Lunar SRS transmitters pointing towards the FS victim receiver using the following equation:

INTotal= 1n (EIRPnθ,φ-FSPL-Floss+Grxθ,φ)-Nrx

where n is the number of lunar surface transmitters;  EIRPnθ,φ is the equivalent isotropic radiated power of each lunar transmitter; FSPL is the free space path loss calculated using Recommendation ITU-R P.525; Floss is the FS feeder loss; (Grx)θ,φ is the FS victim receiver maximum antenna gain; and Nrx is the thermal noise power in the FS victim receiver bandwidth.

–	The receiver thermal noise power is calculated from the receiver noise figure (NF) using the formula provided in Recommendation ITU-R P.372-17.

–	The single and aggregate I/N results are compared to the FS protection criteria (I/N ≤ −10 dB) to verify whether the FS protection is met.

Figure A9.1.1.1

SRS–FS Scenario

Table A9.1.1.3

SRS - FS co-channel results

| SRS Station | SRS centre frequency | Analysis | I/N result | Margin to the FS protection criteria |
| --- | --- | --- | --- | --- |
| Lunar surface transmitter | 7 212.5 MHz | Single entry | TBD | TBD |

#### A9.1.1.6 	Summary and analysis of the results of Study A

TBD

### A9.1.2	Study B

## A9.2	Sharing studies between SRS (Earth to space) and SRS (space to space) operating in the frequency band 7 190-7 235 MHz

### A9.2.1	Study A [CHN]

#### A9.2.1.1 	Technical and operational characteristics of SRS (space to space) operating in the 7 190-7 235 MHz frequency range

When lunar probes land on the moon, especially on the far side of the moon, they may not directly communicate with Earth, in that case, the probes or landers need to communicate with the satellites in lunar orbit with the same equipment in the 7 190-7 235 MHz frequency range. The technical and operational characteristics for SRS (space to space) systems are based on those available in the Report ITU-R SA.2553, including Table 10, as summarized in Table A9.2.1.1.

Table A9.2.1.1

Technical characteristics of the SRS systems

| Station | Lunar orbit space station transmitter |
| --- | --- |
| Frequency range (MHz) | 7 190-7 235 |
| Beam type | Fixed |
| Polarization | CP |
| Peak gain (dBi) | 46 |
| Transmit antenna pattern | Rec. ITU-R S.672-4 (Ls = −20 dB) |
| EIRP density (dBW/Hz) | 0.5 |
| Max. EIRP (dBW) | 57.5 |
| Orbital characteristics Apolune (km) Perilune (km) | 9 000-18 000 300-2 000 |
| Channel BW (MHz) | 0.5 |

#### A9.2.1.2 	Technical and operational characteristics of SRS (Earth to space) operating in the 7 190-7 235 MHz frequency range

Typical SRS (Earth-to- space) receiver characteristics in 7 190-7 235 MHz are summarized in Table A9.2.1.2.

Table A9.2.1.2

SRS (Earth-to- space) receiver characteristics in 7 190-7 235 MHz

|  | System 1 | System 2 |
| --- | --- | --- |
| Frequency range (MHz) | 7 190-7 235 | 7 190-7 235 |
| Beam type |  |  |
| Polarization |  |  |
| Channel BW (MHz) |  |  |
| Peak gain (dBi) |  |  |
| Receive antenna pattern |  |  |
| Receive noise temperature (K) |  |  |
| Protection criteria (I/N) (dB) | Rec. ITU-R SA.609 | Rec. ITU-R SA.609 |

#### A9.2.1.3 	Propagation model

This sharing study includes the lunar SRS system in orbit of the Moon or on Lunar surface, and SRS operating on Earth’s surface. Working Party 3J in Document 7B/58 recommends using the propagation model based on Recommendation ITU-R P.525 for this scenario.

#### A9.2.1.4 	Methodology

The methodology employed in this study is a dynamic assessment of compatibility between SRS system (space-space) and SRS system (Earth to space) in 7 190-7 235 MHz frequency band.

#### A9.2.1.5 	Study results

TBD

#### A9.2.1.6 	Summary and analysis of the results of Study A

TBD

### A9.2.2	Study B

### A9.2.3	Study C

## A9.3	Summary and analysis of the results of studies

[Note: The summary should offer an objective overview, encompassing the results of sharing and compatibility studies, alongside any pertinent information applicable for future comparisons among studies. Additionally, it should address any mitigation techniques or proposals employed to safeguard services operating within the same band or adjacent bands, where applicable, to SRS lunar systems.]

ANNEX 10

Sharing and compatibility studies of SRS for lunar operations
in the frequency band 8 450-8 500 MHz

## A10.1	Sharing and compatibility of fixed service and SRS (space-to-space) operating in the frequency band 8 450-8 500 MHz

[Note: Insert relevant sharing and compatibility studies for each of the identified service.]

### A10.1.1	Study A[CHN]

#### A10.1.1.1 	Technical and operational characteristics of SRS operating in the 8 450-8 500 MHz frequency range

The technical and operational characteristics for SRS systems are based on those available in the Report ITU-R SA.2553, including Table 8, as summarized in Table A10.1.1.1.

Table A10.1.1.1

Technical characteristics of the SRS systems

| Station | Lunar surface space station transmitter |  |
| --- | --- | --- |
|  | System A | System B |
| Frequency range (MHz) | 8 450~8 500 | 8 450~8 500 |
| Beam type | Omni | Fixed |
| Polarization | CP | CP |
| Peak gain (dBi) | −4 | 15 |
| EIRP density (dBW/Hz) | −54.5 | −45 |
| Max. EIRP (dBW) | −1.5 | 22 |
| Transmit antenna pattern | ND | Rec. ITU-R S.672-4 (Ls = −20 dB) |
| Channel BW (MHz) | 0.2 | 5 |

#### A10.1.1.2 	Technical and operational characteristics of fixed service operating in the 8 450-8 500 MHz frequency range

Document 7B/56 provides relevant technical information to support studies between SRS and fixed services (FS) under agenda item 1.15. The characteristics for FS for point-to-point (PP) systems are based on the information contained in Recommendation ITU-R F.758-8, Table 9, as summarized in Table A10.1.1.2.

Table A10.1.1.2

Characteristics of the FS (PP) systems

| Parameter | Value |  |
| --- | --- | --- |
| Frequency range | 7 725~8 500 MHz |  |
| Reference | Rec. ITU-R F.386 |  |
| Modulation format | 32-QAM | 128-QAM |
| Receiver noise bandwidth | 5 MHz | 5 MHz |
| Antenna pattern | Rec. ITU-R F.699 | Rec. ITU-R F.699 |
| Antenna gain | 12~48.6 dBi | 12~48.6 dBi |
| Feeder loss | 0~3 dB | 0~3 dB |
| Receiver noise figure | 2.5~6 dB | 2.5~6 dB |
| Protection criteria (I/N) | ‒10 dB | ‒10 dB |
| Noise long-term interference power density (dBW/MHz) | −141.5~−138.0 | −141.5~−138.0 |

#### A10.1.1.3 	Propagation model

This sharing study includes the lunar SRS system on the surface of the Moon, and fixed services operating on Earth’s surface. Working Party 3J in Document 7B/58 recommends using the propagation model based on Recommendation ITU-R P.525 for this scenario.

#### A10.1.1.4 	Methodology

The methodology employed in this study is based on a static analysis of both single and aggregated worst-case interference. It evaluates whether, even under conditions of perfect antenna coupling between the systems, at the minimum distance between the stations on Earth and the Moon, and in the absence of antenna pointing errors and losses such as depolarization and clutter, the interference levels comply with the established ITU-R service protection criteria.

#### A10.1.1.5 	Study results

The study was conducted in accordance with the scenario outlined in Figure A10.1.1.1. For the worst-case analysis, the following assumptions were considered:

–	Main beam antenna coupling between the potentially interfering lunar SRS transmitters and the FS victim receiver.

–	Free space propagation loss assuming the minimum distance between the stations on the Earth and the Moon (355 000 km), while atmospheric, depolarization and clutter losses are not considered.

–	The aggregate interference-to-noise ratio (I/N) is calculated considering Lunar SRS transmitters pointing towards the FS victim receiver using the following equation:

INTotal= 1n (EIRPnθ,φ-FSPL-Floss+Grxθ,φ)-Nrx

where n is the number of lunar surface transmitters;  EIRPnθ,φ is the equivalent isotropic radiated power of each lunar transmitter; FSPL is the free space path loss calculated using Recommendation ITU-R P.525; Floss is the FS feeder loss; (Grx)θ,φ is the FS victim receiver maximum antenna gain; and Nrx is the thermal noise power in the FS victim receiver bandwidth.

–	The receiver thermal noise power is calculated from the receiver noise figure (NF) using the formula provided in Recommendation ITU-R P.372-17.

–	The single I/N results are compared to the FS protection criteria (I/N ≤ −10 dB) to verify whether the FS protection is met.

Figure A10.1.1.1

SRS–FS scenario

Table A10.1.1.3

SRS – FS co-channel results

| SRS station | SRS centre frequency | Analysis | I/N result | Margin to the FS protection criteria |
| --- | --- | --- | --- | --- |
| Lunar surface transmitter (System A) | 8 475 MHz | Single entry | −40.4 | 30.4 |
| Lunar surface transmitter (System B) | 8 475 MHz | Single entry | −16.9 | 6.9 |

#### A10.1.1.6 	Summary and analysis of the results of Study A

The interference-to-noise ratio metric shows that in the single entry static worst-case analysis, there is a margin 6.9 dB or more to satisfy the FS protection criteria. That also implies that aggregated worst-case interfering transmitters would still meet the FS protection criteria. Thus the results of this static worst-case study suggest that sharing between SRS and terrestrial FS operating in the 8 450-8 500 MHz band (co-channel) is ensured.

### A10.1.2	Study B

### A10.1.3	Study C

## A10.2	Sharing studies between SRS (space to Earth) and SRS (space to space) operating in the frequency band 8 450-8 500 MHz

### A10.2.1	Study A [CHN]

#### A10.2.1.1 	Technical and operational characteristics of SRS (space to space) operating in the 8 450-8 500 MHz frequency range

The technical and operational characteristics for SRS systems are based on those available in the Report ITU-R SA.2553, including Table 8, as summarized in Table A10.2.1.1.

Table A10.2.1.1

Technical characteristics of the SRS systems

| Station | Lunar surface space station transmitter |  |
| --- | --- | --- |
|  | System A | System B |
| Frequency range | 8 450-8 500 MHz | 8 450-8 500 MHz |
| Beam type | Omni | Fixed |
| Polarization | CP | CP |
| Peak Gain (dBi) | −4 | 15 |
| EIRP Density (dBW/Hz) | −54.5 | −45 |
| Max. EIRP (dBW) | −1.5 | 22 |
| Transmit antenna pattern | ND | Rec. ITU-R S.672-4 (Ls = −20 dB) |
| Channel BW (MHz) | 0.2 | 5 |
| Lunar orbit space station Orbital characteristics Apolune (km) Perilune (km) | 9 000-18 000 300-2 000 |  |

#### A10.2.1.2 	Technical and operational characteristics of SRS (space to Earth) operating in the 8 450-8 500 MHz frequency range

Typical SRS (Earth-to- space) receiver characteristics in 8 450-8 500 MHz are summarized in Table A10.2.1.2.

Table A10.2.1.2

SRS (space-to-Earth) Earth station receiver characteristics in 8 450-8 500 MHz

|  | System 1 | System 2 |
| --- | --- | --- |
| Frequency range (MHz) | 8 450-8 500 MHz | 8 450-8 500 MHz |
| Beam type |  |  |
| Polarization |  |  |
| Channel BW (MHz) |  |  |
| Peak gain (dBi) |  |  |
| Receive antenna pattern |  |  |
| Receive noise temperature (K) |  |  |
| Protection criteria (I/N) | Rec. ITU-R SA.609 | Rec. ITU-R SA.609 |

#### A10.2.1.3 	Propagation model

This sharing study includes the lunar SRS system in orbit of the Moon, and MSS operating on Earth's surface. Working Party 3J in Document 7B/58 recommends using the propagation model based on Recommendation ITU-R P.525 for this scenario.

#### A10.2.1.4 	Methodology

The methodology employed in this study is a dynamic assessment of compatibility between SRS system (space-space) and SRS system (Earth to space) in 8 450-8 500 MHz frequency band.

#### A10.2.1.5 	Study results

TBD

#### A10.2.1.6 	Summary and analysis of the results of Study A

TBD

### A10.2.2	Study B

### A10.2.3	Study C

## A10.3	Summary and analysis of the results of studies

[Note: The summary should offer an objective overview, encompassing the results of sharing and compatibility studies, alongside any pertinent information applicable for future comparisons among studies. Additionally, it should address any mitigation techniques or proposals employed to safeguard services operating within the same band or adjacent bands, where applicable, to SRS lunar systems.]

ANNEX 11

Sharing and compatibility studies of SRS for lunar operations (Lunar Data Relay Satellite system) in the frequency bands
7 190-7 235 MHz and 8 450-8 500 MHz

## A11.1	Sharing and compatibility of terrestrial services and SRS LDRS systems operating in the frequency bands 7 190-7 235 MHz and 8 450-8 500 MHz

[Note: Insert relevant sharing and compatibility studies for each of the identified service.]

### A11.1.1	Study A [Russian Federation]

#### A11.1.1.1	SRS (space-space) characteristics in the frequency bands 7 190-7 235 MHz and 8 450-8 500 MHz

Information on technical and operational parameters of and concept of operation for Lunar data relay satellite (LDRS) system are provided in section 3.4 of the Report ITU-R SA.2553.

LDRS would be located in Lagrange points L1 and L2 of the Earth-Moon system, using Halo orbits with time period of 13-16 days. LDRS will provide tracking, telemetry and telecommand (TT&C) functions, as well as voice and data communication links for various Lunar space stations (LSS): Orbiters (6 000 km maximum orbit height), Landers and Rovers, as well as planned manned missions. Forward inter-orbit links (LDRS – LSS) will be operated in the frequency band 7 190-7 235 MHz. Return inter-orbit links (LSS – LDRS) will be operated in the frequency band 8 450-8 500 MHz.

LDRS will provide TT&C and low data rate/voice communications in multiple access links, involving CDMA and TDMA schemes, as well as medium data rate/voice communications in single access links.

Figure A11.1.1.1

LDRS system communication links

Technical and operational characteristics of LDRS links are provided in Table A11.1.1.1.

Table A11.1.1.1

| LDRS Links | LDRS – LSS (Multiple access) | LDRS – LSS (Single access) | LSS – LDRS (Multiple access) | LSS – LDRS (Single access) |
| --- | --- | --- | --- | --- |
| Frequency band(s) | 7 190-7 235 MHz | 7 190-7 235 MHz | 8 450-8 500 MHz | 8 450-8 500 MHz |
| Data rate per user | 8-64 kbps (TT&C, voice and low data rate) | 4-12 Mbps | 8-64 kbps (TT&C, voice and low data rate) | 4-24 Mbps |
| Number of users | Up to 8 | 1 | Up to 8 | 1 |
| Total data rate | 64 kbps | 12 Mbps | 64 kbps | 24 Mbps |
| Necessary bandwidth, MHz | 6 | 10 | 6 | 10 |
| Operating distance to LDRS, km | 51 000-80 400 | 51 000-80 400 | 51 000-80 400 | 51 000-80 400 |
| Peak Transmitting antenna gain, dBi | 22.6 | 47.7 | 29.9 | 29.9-39.5 |
| Antenna gain pattern | Rec. ITU-R S.672 (Ls –20 dB) | Rec. ITU-R S.672 (Ls –20 dB) | Rec. ITU-R S.672 (Ls –20 dB) | Rec. ITU-R S.672 (Ls –20 dB) |
| EIRP, dBW | 38.5 | 63.6 | 38.9 | 38.9-50.5 |
| Multiple access scheme | CDMA, TDMA | – | CDMA | – |
| Receiving antenna gain | 28.6 | 28.6 | 23.9 | 49 |
| Antenna gain pattern | Rec. ITU-R S.672 (Ls –20 dB) | Rec. ITU-R S.672 (Ls –20 dB) | Rec. ITU-R S.672 (Ls –20 dB) | Rec. ITU-R S.672 (Ls –20 dB) |
| System noise temperature | 350 K | 350 K | 800 K | 800 K |

#### A11.1.1.2	Assessment of power flux-density at the Earth’s surface, produced by LDRS emissions

The power flux-density at the Earth’s surface, produced by LDRS emissions is given by:

PFDSRSdBw/m2∙4 kHz= EIRPSRS(θ)-10log(BN) +36-10log104∙π∙D(δ)2-60

EIRPSRS(θ)	Off-axis EIRP of the LDRS/LSS towards the Earth surface;

D(δ)	slant distance between LDRS/LSS and the Earth surface, as a function of angle of arrival above the horizontal plane, km

BN	necessary bandwidth of emissions, MHz.

Several assumptions were made in the study:

–	Transmitting antenna of the LDRS in L2 point, operating forward inter-orbit link (LDRS – LSS) points towards the Earth surface;

–	Transmitting antenna of the LDRS in L1 point, operating forward inter-orbit link (LDRS – LSS) points towards the Earth surface with off-axis gain of 0 dBi;

–	Transmitting antenna of the LSS, communicating with LDRS in L1 point in return inter-orbit link (LSS – LDRS) points towards the Earth surface, LSS is located at the minimal distance from the Earth surface;

–	Transmitting antenna of the LSS, communicating with LDRS in L2 point in return inter-orbit link (LSS – LDRS) points towards the Earth surface with off-axis gain of 0 dBi, LSS is located at the minimal distance from the Earth surface.

Minimal distances between LDRS, LSS and the Earth surface, used in the study, are provided in Figure A11.1.1.2.

Figure A11.1.1.2

LDRS system communication links

#### A11.1.1.3	Assessment results

Tables A11.1.1.2 to A11.1.1.9 provide results of pfd assessment.

Table A11.1.1.2

Forward inter-orbit link (LDRS – LSS), L1 POINT, MA

| Angle of arrival, degrees, δ | EIRP(θ) | D(δ) | Calculated PFD | Table 21-4 PFD limit | Spreading loss | Margin |
| --- | --- | --- | --- | --- | --- | --- |
| deg | dBW | km | dBW/m2 in 4 kHz | dBW/m2 in 4 kHz | dB | dB |
| 0 | 15.9 | 326 315.7 | –197.1 | –152.0 | 181.3 | 45.1 |
| 1 | 15.9 | 326 204.4 | –197.1 | –152.0 | 181.3 | 45.1 |
| 2 | 15.9 | 326 093.2 | –197.1 | –152.0 | 181.3 | 45.1 |
| 3 | 15.9 | 325 982.0 | –197.1 | –152.0 | 181.3 | 45.1 |
| 4 | 15.9 | 325 871.1 | –197.1 | –152.0 | 181.3 | 45.1 |
| 5 | 15.9 | 325 760.3 | –197.1 | –152.0 | 181.3 | 45.1 |
| 10 | 15.9 | 325 210.0 | –197.1 | –149.5 | 181.2 | 47.6 |
| 15 | 15.9 | 324 669.1 | –197.1 | –147.0 | 181.2 | 50.1 |
| 20 | 15.9 | 324 141.6 | –197.1 | –144.5 | 181.2 | 52.6 |
| 25 | 15.9 | 323 631.3 | –197.1 | –142.0 | 181.2 | 55.1 |
| 30 | 15.9 | 323 142.2 | –197.0 | –142.0 | 181.2 | 55.0 |
| 35 | 15.9 | 322 677.9 | –197.0 | –142.0 | 181.2 | 55.0 |
| 40 | 15.9 | 322 241.7 | –197.0 | –142.0 | 181.2 | 55.0 |
| 45 | 15.9 | 321 836.9 | –197.0 | –142.0 | 181.1 | 55.0 |
| 50 | 15.9 | 321 466.4 | –197.0 | –142.0 | 181.1 | 55.0 |
| 55 | 15.9 | 321 132.9 | –197.0 | –142.0 | 181.1 | 55.0 |
| 60 | 15.9 | 320 838.9 | –197.0 | –142.0 | 181.1 | 55.0 |
| 65 | 15.9 | 320 586.4 | –197.0 | –142.0 | 181.1 | 55.0 |
| 70 | 15.9 | 320 377.3 | –197.0 | –142.0 | 181.1 | 55.0 |
| 75 | 15.9 | 320 213.1 | –197.0 | –142.0 | 181.1 | 55.0 |
| 80 | 15.9 | 320 094.9 | –197.0 | –142.0 | 181.1 | 55.0 |
| 84 | 15.9 | 320 034.1 | –197.0 | –142.0 | 181.1 | 55.0 |
| 85 | 15.9 | 320 023.6 | –197.0 | –142.0 | 181.1 | 55.0 |
| 90 | 15.9 | 320 000.00 | –197.0 | –142.0 | 181.1 | 55.0 |

Table A11.1.1.3

Forward inter-orbit link (LDRS – LSS), L1 POINT, SA

| Angle of arrival, degrees, δ | EIRP(θ) | D(δ) | Calculated PFD | Table 21-4 PFD limit | Spreading loss | Margin |
| --- | --- | --- | --- | --- | --- | --- |
| deg | dBW | km | dBW/m2 in 4 kHz | dBW/m2 in 4 kHz | dB | dB |
| 0 | 15.9 | 326 315.7 | –199.3 | –152.0 | 181.3 | 47.3 |
| 1 | 15.9 | 326 204.4 | –199.3 | –152.0 | 181.3 | 47.3 |
| 2 | 15.9 | 326 093.2 | –199.3 | –152.0 | 181.3 | 47.3 |
| 3 | 15.9 | 325 982.0 | –199.3 | –152.0 | 181.3 | 47.3 |
| 4 | 15.9 | 325 871.1 | –199.3 | –152.0 | 181.3 | 47.3 |
| 5 | 15.9 | 325 760.3 | –199.3 | –152.0 | 181.3 | 47.3 |
| 10 | 15.9 | 325 210.0 | –199.3 | –149.5 | 181.2 | 49.8 |
| 15 | 15.9 | 324 669.1 | –199.3 | –147.0 | 181.2 | 52.3 |
| 20 | 15.9 | 324 141.6 | –199.3 | –144.5 | 181.2 | 54.8 |
| 25 | 15.9 | 323 631.3 | –199.3 | –142.0 | 181.2 | 57.3 |
| 30 | 15.9 | 323 142.2 | –199.3 | –142.0 | 181.2 | 57.3 |
| 35 | 15.9 | 322 677.9 | –199.2 | –142.0 | 181.2 | 57.2 |
| 40 | 15.9 | 322 241.7 | –199.2 | –142.0 | 181.2 | 57.2 |
| 45 | 15.9 | 321 836.9 | –199.2 | –142.0 | 181.1 | 57.2 |
| 50 | 15.9 | 321 466.4 | –199.2 | –142.0 | 181.1 | 57.2 |
| 55 | 15.9 | 321 132.9 | –199.2 | –142.0 | 181.1 | 57.2 |
| 60 | 15.9 | 320 838.9 | –199.2 | –142.0 | 181.1 | 57.2 |
| 65 | 15.9 | 320 586.4 | –199.2 | –142.0 | 181.1 | 57.2 |
| 70 | 15.9 | 320 377.3 | –199.2 | –142.0 | 181.1 | 57.2 |
| 75 | 15.9 | 320 213.1 | –199.2 | –142.0 | 181.1 | 57.2 |
| 80 | 15.9 | 320 094.9 | –199.2 | –142.0 | 181.1 | 57.2 |
| 84 | 15.9 | 320 034.1 | –199.2 | –142.0 | 181.1 | 57.2 |
| 85 | 15.9 | 320 023.6 | –199.2 | –142.0 | 181.1 | 57.2 |
| 90 | 15.9 | 320 000.00 | –199.2 | –142.0 | 181.1 | 57.2 |

Table A11.1.1.4

Forward inter-orbit link (LDRS – LSS), L2 POINT, MA

| Angle of arrival, degrees, δ | EIRP(θ) | D(δ) | Calculated PFD | Table 21-4 PFD limit | Spreading loss | Margin |
| --- | --- | --- | --- | --- | --- | --- |
| deg | dBW | km | dBW/m2 in 4 kHz | dBW/m2 in 4 kHz | dB | dB |
| 0 | 38.5 | 448 332.6 | –177.3 | –152.0 | 184.0 | 25.3 |
| 1 | 38.5 | 448 221.3 | –177.3 | –152.0 | 184.0 | 25.3 |
| 2 | 38.5 | 448 110.1 | –177.3 | –152.0 | 184.0 | 25.3 |
| 3 | 38.5 | 447 999.0 | –177.3 | –152.0 | 184.0 | 25.3 |
| 4 | 38.5 | 447 887.9 | –177.3 | –152.0 | 184.0 | 25.3 |
| 5 | 38.5 | 447 777.1 | –177.3 | –152.0 | 184.0 | 25.3 |
| 10 | 38.5 | 447 226.5 | –177.3 | –149.5 | 184.0 | 27.8 |
| 15 | 38.5 | 446 684.9 | –177.3 | –147.0 | 184.0 | 30.3 |
| 20 | 38.5 | 446 156.5 | –177.2 | –144.5 | 184.0 | 32.7 |
| 25 | 38.5 | 445 645.3 | –177.2 | –142.0 | 184.0 | 35.2 |
| 30 | 38.5 | 445 155.0 | –177.2 | –142.0 | 184.0 | 35.2 |
| 35 | 38.5 | 444 689.3 | –177.2 | –142.0 | 184.0 | 35.2 |
| 40 | 38.5 | 444 251.7 | –177.2 | –142.0 | 183.9 | 35.2 |
| 45 | 38.5 | 443 845.4 | –177.2 | –142.0 | 183.9 | 35.2 |
| 50 | 38.5 | 443 473.4 | –177.2 | –142.0 | 183.9 | 35.2 |
| 55 | 38.5 | 443 138.5 | –177.2 | –142.0 | 183.9 | 35.2 |
| 60 | 38.5 | 442 843.1 | –177.2 | –142.0 | 183.9 | 35.2 |
| 65 | 38.5 | 442 589.4 | –177.2 | –142.0 | 183.9 | 35.2 |
| 70 | 38.5 | 442 379.3 | –177.2 | –142.0 | 183.9 | 35.2 |
| 75 | 38.5 | 442 214.2 | –177.2 | –142.0 | 183.9 | 35.2 |
| 80 | 38.5 | 442 095.4 | –177.2 | –142.0 | 183.9 | 35.2 |
| 84 | 38.5 | 442 034.2 | –177.2 | –142.0 | 183.9 | 35.2 |
| 85 | 38.5 | 442 023.7 | –177.2 | –142.0 | 183.9 | 35.2 |
| 90 | 38.5 | 442 000.00 | –177.2 | –142.0 | 183.9 | 35.2 |

Table A11.1.1.5

Forward inter-orbit link (LDRS – LSS), L2 POINT, SA

| Angle of arrival, degrees, δ | EIRP(θ) | D(δ) | Calculated PFD | Table 21-4 PFD limit | Spreading loss | Margin |
| --- | --- | --- | --- | --- | --- | --- |
| deg | dBW | km | dBW/m2 in 4 kHz | dBW/m2 in 4 kHz | dB | dB |
| 0 | 63.6 | 448 332.6 | –154.4 | –152.0 | 184.0 | 2.4 |
| 1 | 63.6 | 448 221.3 | –154.4 | –152.0 | 184.0 | 2.4 |
| 2 | 63.6 | 448 110.1 | –154.4 | –152.0 | 184.0 | 2.4 |
| 3 | 63.6 | 447 999.0 | –154.4 | –152.0 | 184.0 | 2.4 |
| 4 | 63.6 | 447 887.9 | –154.4 | –152.0 | 184.0 | 2.4 |
| 5 | 63.6 | 447 777.1 | –154.4 | –152.0 | 184.0 | 2.4 |
| 10 | 63.6 | 447 226.5 | –154.4 | –149.5 | 184.0 | 4.9 |
| 15 | 63.6 | 446 684.9 | –154.4 | –147.0 | 184.0 | 7.4 |
| 20 | 63.6 | 446 156.5 | –154.4 | –144.5 | 184.0 | 9.9 |
| 25 | 63.6 | 445 645.3 | –154.4 | –142.0 | 184.0 | 12.4 |
| 30 | 63.6 | 445 155.0 | –154.3 | –142.0 | 184.0 | 12.3 |
| 35 | 63.6 | 444 689.3 | –154.3 | –142.0 | 184.0 | 12.3 |
| 40 | 63.6 | 444 251.7 | –154.3 | –142.0 | 183.9 | 12.3 |
| 45 | 63.6 | 443 845.4 | –154.3 | –142.0 | 183.9 | 12.3 |
| 50 | 63.6 | 443 473.4 | –154.3 | –142.0 | 183.9 | 12.3 |
| 55 | 63.6 | 443 138.5 | –154.3 | –142.0 | 183.9 | 12.3 |
| 60 | 63.6 | 442 843.1 | –154.3 | –142.0 | 183.9 | 12.3 |
| 65 | 63.6 | 442 589.4 | –154.3 | –142.0 | 183.9 | 12.3 |
| 70 | 63.6 | 442 379.3 | –154.3 | –142.0 | 183.9 | 12.3 |
| 75 | 63.6 | 442 214.2 | –154.3 | –142.0 | 183.9 | 12.3 |
| 80 | 63.6 | 442 095.4 | –154.3 | –142.0 | 183.9 | 12.3 |
| 84 | 63.6 | 442 034.2 | –154.3 | –142.0 | 183.9 | 12.3 |
| 85 | 63.6 | 442 023.7 | –154.3 | –142.0 | 183.9 | 12.3 |
| 90 | 63.6 | 442 000.00 | –154.3 | –142.0 | 183.9 | 12.3 |

Table A11.1.1.6

Return inter-orbit link (LSS – LDRS), L1 POINT, MA

| Angle of arrival, degrees, δ | EIRP(θ) | D(δ) | Calculated PFD | Table 21-4 PFD limit | Spreading loss | Margin |
| --- | --- | --- | --- | --- | --- | --- |
| deg | dBW | km | dBW/m2 in 4 kHz | dBW/m2 in 4 kHz | dB | dB |
| 0 | 38.9 | 376 324.0 | –175.4 | –150.0 | 182.5 | 25.4 |
| 1 | 38.9 | 376 212.7 | –175.4 | –150.0 | 182.5 | 25.4 |
| 2 | 38.9 | 376 101.4 | –175.4 | –150.0 | 182.5 | 25.4 |
| 3 | 38.9 | 375 990.3 | –175.4 | –150.0 | 182.5 | 25.4 |
| 4 | 38.9 | 375 879.3 | –175.4 | –150.0 | 182.5 | 25.4 |
| 5 | 38.9 | 375 768.5 | –175.4 | –150.0 | 182.5 | 25.4 |
| 10 | 38.9 | 375 218.1 | –175.3 | –147.5 | 182.5 | 27.8 |
| 15 | 38.9 | 374 676.8 | –175.3 | –145.0 | 182.5 | 30.3 |
| 20 | 38.9 | 374 148.9 | –175.3 | –142.5 | 182.5 | 32.8 |
| 25 | 38.9 | 373 638.1 | –175.3 | –140.0 | 182.4 | 35.3 |
| 30 | 38.9 | 373 148.5 | –175.3 | –140.0 | 182.4 | 35.3 |
| 35 | 38.9 | 372 683.5 | –175.3 | –140.0 | 182.4 | 35.3 |
| 40 | 38.9 | 372 246.6 | –175.3 | –140.0 | 182.4 | 35.3 |
| 45 | 38.9 | 371 841.0 | –175.3 | –140.0 | 182.4 | 35.3 |
| 50 | 38.9 | 371 469.8 | –175.3 | –140.0 | 182.4 | 35.3 |
| 55 | 38.9 | 371 135.6 | –175.2 | –140.0 | 182.4 | 35.2 |
| 60 | 38.9 | 370 840.9 | –175.2 | –140.0 | 182.4 | 35.2 |
| 65 | 38.9 | 370 587.9 | –175.2 | –140.0 | 182.4 | 35.2 |
| 70 | 38.9 | 370 378.3 | –175.2 | –140.0 | 182.4 | 35.2 |
| 75 | 38.9 | 370 213.6 | –175.2 | –140.0 | 182.4 | 35.2 |
| 80 | 38.9 | 370 095.2 | –175.2 | –140.0 | 182.4 | 35.2 |
| 84 | 38.9 | 370 034.2 | –175.2 | –140.0 | 182.4 | 35.2 |
| 85 | 38.9 | 370 023.6 | –175.2 | –140.0 | 182.4 | 35.2 |
| 90 | 38.9 | 370 000.00 | –175.2 | –140.0 | 182.4 | 35.2 |

Table A11.1.1.7

Return inter-orbit link (LSS – LDRS), L1 POINT, SA

| Angle of arrival, degrees, δ | EIRP(θ) | D(δ) | Calculated PFD | Table 21-4 PFD limit | Spreading loss | Margin |
| --- | --- | --- | --- | --- | --- | --- |
| deg | dBW | km | dBW/m2 in 4 kHz | dBW/m2 in 4 kHz | dB | dB |
| 0 | 50.5 | 376 324.0 | –166.0 | –150.0 | 182.5 | 16.0 |
| 1 | 50.5 | 376 212.7 | –166.0 | –150.0 | 182.5 | 16.0 |
| 2 | 50.5 | 376 101.4 | –166.0 | –150.0 | 182.5 | 16.0 |
| 3 | 50.5 | 375 990.3 | –166.0 | –150.0 | 182.5 | 16.0 |
| 4 | 50.5 | 375 879.3 | –166.0 | –150.0 | 182.5 | 16.0 |
| 5 | 50.5 | 375 768.5 | –166.0 | –150.0 | 182.5 | 16.0 |
| 10 | 50.5 | 375 218.1 | –166.0 | –147.5 | 182.5 | 18.5 |
| 15 | 50.5 | 374 676.8 | –165.9 | –145.0 | 182.5 | 20.9 |
| 20 | 50.5 | 374 148.9 | –165.9 | –142.5 | 182.5 | 23.4 |
| 25 | 50.5 | 373 638.1 | –165.9 | –140.0 | 182.4 | 25.9 |
| 30 | 50.5 | 373 148.5 | –165.9 | –140.0 | 182.4 | 25.9 |
| 35 | 50.5 | 372 683.5 | –165.9 | –140.0 | 182.4 | 25.9 |
| 40 | 50.5 | 372 246.6 | –165.9 | –140.0 | 182.4 | 25.9 |
| 45 | 50.5 | 371 841.0 | –165.9 | –140.0 | 182.4 | 25.9 |
| 50 | 50.5 | 371 469.8 | –165.9 | –140.0 | 182.4 | 25.9 |
| 55 | 50.5 | 371 135.6 | –165.9 | –140.0 | 182.4 | 25.9 |
| 60 | 50.5 | 370 840.9 | –165.9 | –140.0 | 182.4 | 25.9 |
| 65 | 50.5 | 370 587.9 | –165.8 | –140.0 | 182.4 | 25.8 |
| 70 | 50.5 | 370 378.3 | –165.8 | –140.0 | 182.4 | 25.8 |
| 75 | 50.5 | 370 213.6 | –165.8 | –140.0 | 182.4 | 25.8 |
| 80 | 50.5 | 370 095.2 | –165.8 | –140.0 | 182.4 | 25.8 |
| 84 | 50.5 | 370 034.2 | –165.8 | –140.0 | 182.4 | 25.8 |
| 85 | 50.5 | 370 023.6 | –165.8 | –140.0 | 182.4 | 25.8 |
| 90 | 50.5 | 370 000.00 | –165.8 | –140.0 | 182.4 | 25.8 |

Table A11.1.1.8

Return inter-orbit link (LSS – LDRS), L2 POINT, MA

| Angle of arrival, degrees, δ | EIRP(θ) | D(δ) | Calculated PFD | Table 21-4 PFD limit | Spreading loss | Margin |
| --- | --- | --- | --- | --- | --- | --- |
| deg | dBW | km | dBW/m2 in 4 kHz | dBW/m2 in 4 kHz | dB | dB |
| 0 | 9.0 | 376 324.0 | –205.3 | –150.0 | 182.5 | 55.3 |
| 1 | 9.0 | 376 212.7 | –205.3 | –150.0 | 182.5 | 55.3 |
| 2 | 9.0 | 376 101.4 | –205.3 | –150.0 | 182.5 | 55.3 |
| 3 | 9.0 | 375 990.3 | –205.3 | –150.0 | 182.5 | 55.3 |
| 4 | 9.0 | 375 879.3 | –205.3 | –150.0 | 182.5 | 55.3 |
| 5 | 9.0 | 375 768.5 | –205.3 | –150.0 | 182.5 | 55.3 |
| 10 | 9.0 | 375 218.1 | –205.2 | –147.5 | 182.5 | 57.7 |
| 15 | 9.0 | 374 676.8 | –205.2 | –145.0 | 182.5 | 60.2 |
| 20 | 9.0 | 374 148.9 | –205.2 | –142.5 | 182.5 | 62.7 |
| 25 | 9.0 | 373 638.1 | –205.2 | –140.0 | 182.4 | 65.2 |
| 30 | 9.0 | 373 148.5 | –205.2 | –140.0 | 182.4 | 65.2 |
| 35 | 9.0 | 372 683.5 | –205.2 | –140.0 | 182.4 | 65.2 |
| 40 | 9.0 | 372 246.6 | –205.2 | –140.0 | 182.4 | 65.2 |
| 45 | 9.0 | 371 841.0 | –205.2 | –140.0 | 182.4 | 65.2 |
| 50 | 9.0 | 371 469.8 | –205.2 | –140.0 | 182.4 | 65.2 |
| 55 | 9.0 | 371 135.6 | –205.1 | –140.0 | 182.4 | 65.1 |
| 60 | 9.0 | 370 840.9 | –205.1 | –140.0 | 182.4 | 65.1 |
| 65 | 9.0 | 370 587.9 | –205.1 | –140.0 | 182.4 | 65.1 |
| 70 | 9.0 | 370 378.3 | –205.1 | –140.0 | 182.4 | 65.1 |
| 75 | 9.0 | 370 213.6 | –205.1 | –140.0 | 182.4 | 65.1 |
| 80 | 9.0 | 370 095.2 | –205.1 | –140.0 | 182.4 | 65.1 |
| 84 | 9.0 | 370 034.2 | –205.1 | –140.0 | 182.4 | 65.1 |
| 85 | 9.0 | 370 023.6 | –205.1 | –140.0 | 182.4 | 65.1 |
| 90 | 9.0 | 370 000.00 | –205.1 | –140.0 | 182.4 | 65.1 |

Table A11.1.1.9

Return inter-orbit link (LSS – LDRS), L2 POINT, SA

| Angle of arrival, degrees, δ | EIRP(θ) | D(δ) | Calculated PFD | Table 21-4 PFD limit | Spreading loss | Margin |
| --- | --- | --- | --- | --- | --- | --- |
| deg | dBW | km | dBW/m2 in 4 kHz | dBW/m2 in 4 kHz | dB | dB |
| 0 | 20.6 | 376 324.0 | –195.9 | –150.0 | 182.5 | 45.9 |
| 1 | 20.6 | 376 212.7 | –195.9 | –150.0 | 182.5 | 45.9 |
| 2 | 20.6 | 376 101.4 | –195.9 | –150.0 | 182.5 | 45.9 |
| 3 | 20.6 | 375 990.3 | –195.9 | –150.0 | 182.5 | 45.9 |
| 4 | 20.6 | 375 879.3 | –195.9 | –150.0 | 182.5 | 45.9 |
| 5 | 20.6 | 375 768.5 | –195.9 | –150.0 | 182.5 | 45.9 |
| 10 | 20.6 | 375 218.1 | –195.9 | –147.5 | 182.5 | 48.4 |
| 15 | 20.6 | 374 676.8 | –195.8 | –145.0 | 182.5 | 50.8 |
| 20 | 20.6 | 374 148.9 | –195.8 | –142.5 | 182.5 | 53.3 |
| 25 | 20.6 | 373 638.1 | –195.8 | –140.0 | 182.4 | 55.8 |
| 30 | 20.6 | 373 148.5 | –195.8 | –140.0 | 182.4 | 55.8 |
| 35 | 20.6 | 372 683.5 | –195.8 | –140.0 | 182.4 | 55.8 |
| 40 | 20.6 | 372 246.6 | –195.8 | –140.0 | 182.4 | 55.8 |
| 45 | 20.6 | 371 841.0 | –195.8 | –140.0 | 182.4 | 55.8 |
| 50 | 20.6 | 371 469.8 | –195.8 | –140.0 | 182.4 | 55.8 |
| 55 | 20.6 | 371 135.6 | –195.8 | –140.0 | 182.4 | 55.8 |
| 60 | 20.6 | 370 840.9 | –195.8 | –140.0 | 182.4 | 55.8 |
| 65 | 20.6 | 370 587.9 | –195.7 | –140.0 | 182.4 | 55.7 |
| 70 | 20.6 | 370 378.3 | –195.7 | –140.0 | 182.4 | 55.7 |
| 75 | 20.6 | 370 213.6 | –195.7 | –140.0 | 182.4 | 55.7 |
| 80 | 20.6 | 370 095.2 | –195.7 | –140.0 | 182.4 | 55.7 |
| 84 | 20.6 | 370 034.2 | –195.7 | –140.0 | 182.4 | 55.7 |
| 85 | 20.6 | 370 023.6 | –195.7 | –140.0 | 182.4 | 55.7 |

#### A11.1.1.4	Conclusions for Study A

Assessments of power flux-density at the Earth’s surface produced by emissions from a LDRS space stations, operating forward inter-orbit links in the 7 190-7 235 MHz band and return inter-orbit links in the 8 450-8 500 MHz, demonstrate that existing power-flux-density limits in Table 21-4 of the RR, applicable to the frequency band 7 250-7 900 MHz and 8 450-8 500 MHz, would be met. It is assumed that no additional conditions would be required to demonstrate compatibility of the SRS (space-to-space) with terrestrial services the frequency band 7 190-7 235 MHz and 8 450-8 500 MHz.

### A11.1.2	Study B

### A11.1.3	Study C

## A11.2	Sharing and compatibility of SRS (Earth-to-space)(space-to-Earth) and SRS LDRS systems operating in the frequency bands 7 190-7 235 MHz and 8 450-8 500 MHz

### A11.2.1	Study A

#### A11.2.1.1	SRS (space-space) characteristics in the frequency bands 7 190-7 235 MHz and 8 450-8 500 MHz

Information on technical and operational parameters of and concept of operation for Lunar data relay satellite (LDRS) system are provided in section 3.4 of the Report ITU-R SA.2553 and previous section (A11.1.1.1).

#### A11.2.1.2	SRS (space-Earth, Earth-space) characteristics in the frequency bands 7 190-7 235 MHz and 8 450-8 500 MHz

Information on technical and operational parameters of Earth-space and space-Earth links of the envisioned Lunar missions are provided in the Table A11.2.1.1

Table A11.2.1.1

| SRS link | Earth-space | Space-Earth |
| --- | --- | --- |
| Frequency band(s) | 7 190-7 235 MHz | 8 450-8 500 MHz |
| Orbit | System 1: Low polar orbit, inclination 90°, Apolune ≤ 80 km, perilune ≥50 km orbit period 114.5 min System 2: Lunar Surface (Lunar south pole) |  |
| Necessary bandwidth, MHz | 3.6 | 9.5 |
| Space station antenna gain, dBi | 29.5 | 31 |
| Antenna gain pattern | Rec. ITU-R S.672 (Ls –15 dB) | Rec. ITU-R S.672 (Ls –15 dB) |
| Max. EIRP, dBW | 97 | 40 |
| Earth station antenna gain | 56.6; 69; 71.5 | 58; 70.4; 72.9 |
| Antenna gain pattern | Rec. ITU-R SA.509 | Rec. ITU-R SA.509 |
| Location of Earth stations | Kaliningrad, Moscow Oblast, Kalyazin, Krasnodar Krai, Ussuriisk, Sahalin |  |
| System noise temperature | 500 K | 70..200 K |
| Protection criterion | Rec. ITU-R SA.609 | Rec. ITU-R SA.609 |

## A11.3	…

## A11.3[X]	Summary and analysis of the results of studies

[Note: The summary should offer an objective overview, encompassing the results of sharing and compatibility studies, alongside any pertinent information applicable for future comparisons among studies. Additionally, it should address any mitigation techniques or proposals employed to safeguard services operating within the same band or adjacent bands, where applicable, to SRS lunar systems.]

ANNEX 12

Sharing and compatibility studies of SRS for lunar operations
in the frequency band 25.25-28.35 GHz

## A12.1	Sharing and compatibility of the fixed service and SRS operating in the frequency band 25.25-28.35 GHz

[Note: Insert relevant sharing and compatibility studies for each of the identified service.]

### A12.1.1	Study A

#### A12.1.1.1 	Technical and operational characteristics of SRS operating in the 25.25 – 28.35 GHz frequency range

The technical and operational characteristics for SRS systems are based on those available in the Report ITU-R SA.2553, including table 3.2-4, as summarized in Table A12.1.1.1.

Table A12.1.1.1

Technical Characteristics of the SRS Systems

| Station | Lunar Surface-to-Surface Transmitter |
| --- | --- |
| Frequency range | 27.5 – 28.35 GHz |
| Polarization | Linear ±45º / CP |
| Peak Gain (dBi) | 29.1 |
| EIRP Density (dBW/MHz) | 16.2 |
| Max. EIRP (dBW) | 39.2 |
| Channel BW (MHz) | 200 MHz |

#### A12.1.1.2	Technical and operational characteristics of fixed service operating in the 25.25 – 28.35 GHz frequency range

Document 7B/56 provides relevant technical information to support studies between SRS and Fixed Services (FS) under Agenda Item 1.15. The characteristics for fixed service (FS) for point-to-point (PP) systems are based on the information contained in Recommendation ITU-R F.758-8, Table 10, as summarized in Table A12.1.1.2.

Table A12.1.1.2

Characteristics of the FS (PP) systems

| Parameter | Value |
| --- | --- |
| Frequency range | 24.25 – 29.50 GHz |
| Modulation | 128-QAM |
| Receiver noise bandwidth | 112 MHz |
| Antenna height | 30 m |
| Antenna pattern | Recommendation ITU-R F.699 |
| Antenna gain | 48 dBi |
| Feeder loss | 0 dB |
| Receiver noise figure | 6.5 dB |
| Protection criteria (I/N) | ‒10 dB |

#### A12.1.1.3	Propagation model

This sharing study includes the lunar SRS system on the surface of the Moon, and fixed services operating on Earth's surface. Working Party 3J in Document 7B/58 recommends using the propagation model based on Recommendation ITU-R P.525 for this scenario.

#### A12.1.1.4	Methodology

The methodology employed in this study is based on a static analysis of both single and aggregated worst-case interference. It evaluates whether, even under conditions of perfect antenna coupling between the systems, at the minimum distance between the stations on Earth and the Moon, and in the absence of antenna pointing errors and losses such as depolarization and clutter, the interference levels comply with the established ITU-R service protection criteria.

#### A12.1.1.5	Study results

The study was conducted in accordance with the scenario outlined in Figure A.12.1.1.1. For the worst-case analysis, the following assumptions were considered:

–	Main beam antenna coupling between the potentially interfering lunar SRS transmitters and the FS victim receiver.

–	Free space propagation loss assuming the minimum distance between the stations on the Earth and the Moon (355,000 km), while atmospheric, depolarization and clutter losses are not considered.

–	In the present study, it is assumed that both systems operate at the same central frequency. A bandwidth correction factor (CF) is applied when the bandwidth of the potential interferer exceeds the bandwidth of the victim receiver under analysis. If the bandwidth of the interferer is equal to or smaller than that of the victim receiver, the bandwidth correction factor is 0:

CF = 10 * log10bandwidthvictimbandwidthinterferer

–	The aggregate interference-to-noise ratio (I/N) is calculated considering three Lunar SRS transmitters pointing towards the FS victim receiver using the following equation:

INTotal= 1n (EIRPnθ,φ+CF-FSPL-Floss+Grxθ,φ)-Nrx

where n is the number of lunar surface transmitters;  EIRPnθ,φ is the equivalent isotropic radiated power of each lunar transmitter; FSPL is the free space path loss calculated using Rec. ITU-R P.525; Floss is the FS feeder loss; (Grx)θ,φ is the FS victim receiver maximum antenna gain; and Nrx is the thermal noise power in the FS victim receiver bandwidth.

–	The receiver thermal noise power is calculated from the receiver noise figure (NF) using the following equation:

Nrx = 10 log10[k  (290  10(NF/10))  B]

where k is Boltzmann’s constant; B is the receiver bandwidth, and NF is the receiver noise figure in dB.

–	The single and aggregate I/N results are compared to the FS protection criteria (I/N ≤ 10 dB) to verify whether the FS protection is met.

Figure A12.1.1.1

SRS–FS Scenario

Table A12.1.1.3

SRS - FS co-channel results

| SRS Station | SRS centre frequency | Analysis | I/N result | Margin to the FS protection criteria |
| --- | --- | --- | --- | --- |
| Lunar Surface Transmitter | 27700 MHz | Single Entry | -30.5 dB | 20.5 dB |

#### A12.1.1.6	Summary and analysis of the results of Study A

In this study the interference-to-noise ratio metric shows that even in the static worst-case analysis, for single-entry interferers, there is a margin of 20.5 dB to satisfy the FS protection criteria.

The results of this study suggest that protection of ground-based FS from SRS operating in the 27.5 – 28.35 GHz band (co-channel) is ensured.

### A12.1.2	Study B [D]

#### A12.1.2.1 	Technical and operational characteristics of SRS operating in the 27.5-28.35 GHz frequency range

The technical and operational characteristics for SRS systems are based on those available in Report ITU-R SA.2553, including Table 3.2-4, as summarized in Table A12.1.2.1.

Table A12.1.2.1

Technical Characteristics of the SRS Systems

| Station | Lunar Surface-to-Surface Transmitter (SRS 27GHz 1) | Lunar Surface-to-Surface Transmitter (SRS 27GHz 2) |
| --- | --- | --- |
| Frequency range | 27.5-28.35 GHz | 27.5-28.35 GHz |
| Polarization | Linear | Linear |
| Peak Gain (dBi) | 29.1 | 42.5 |
| EIRP Density (dBW/Hz) | 16.2 | 18.5 |
| Max. EIRP (dBW) | 39.2 | 42.5 |
| Channel BW (MHz) | 200 | 250 |

#### A12.1.2.2 	Technical and operational characteristics of fixed service operating in the 27.5 – 28.35 GHz frequency range

#### The characteristics for fixed service (FS) for point-to-point (PP) systems are based on the information contained in Recommendation ITU-R F.758-8, Tables 10 and 15, as summarized in Table A12.1.2.2 and A12.1.2.3.

Table A12.1.2.2

Characteristics of the FS (PP) systems

| Parameter | Value | Value |
| --- | --- | --- |
| Frequency range | 24.24 – 29.5 GHz | 24.24 – 29.5 GHz |
| Reference | Recommendation ITU-R F.758-8 | Recommendation ITU-R F.758-8 |
| Modulation format | 16 QAM | 128 QAM |
| Receiver noise bandwidth (MHz) | 2.5, 3.5 | 7, 14, 28, 30, 40, 56, 60, 112 |
| Antenna gain | 48 dBi | 48 dBi |
| Feeder loss | 0 dB | 0 dB |
| Receiver noise figure | 6.5 dB | 6.5 dB |
| Protection criteria (I/N) | ‒10 dB | ‒10 dB |

Table A12.1.2.3

Characteristics of the FS (PmP) systems

| Parameter | Value | Value | Value |
| --- | --- | --- | --- |
| Frequency range | 24.24 – 29.5 GHz | 24.24 – 29.5 GHz | 24.24 – 29.5 GHz |
| Reference | Recommendation ITU-R F.758-8 | Recommendation ITU-R F. 758-8 | Recommendation ITU-R F. 758-8 |
| Modulation format | 256 QAM | 16 QAM | 16 QAM |
| Receiver noise bandwidth (MHz) | 40, 50, 56, 100, 112 | 3.5, 7, 14, 28, 30, 56, 112, 40, 60 | 3.5, 7, 14, 28, 30, 56, 112, 40, 60 |
| Antenna gain | 43 dBi | 6.5 dBi | 35 dBi |
| Feeder loss | 0 dB | 0 dB | 0 dB |
| Receiver noise figure | 6 dB | 8 dB | 8 dB |
| Protection criteria (I/N) | ‒10 dB | ‒10 dB | -10 dB |

#### A12.1.2.3 	Propagation model

This study includes the lunar SRS system orbiting the Moon and fixed services operating on Earth's surface. Working Party 3J in Document 7B/58 recommends using the propagation model based on Recommendation ITU-R P.525 for this scenario.

#### A12.1.2.4 	Methodology

The methodology employed in this study is based on a static analysis of single-entry worst-case interference. It evaluates whether, even under conditions of perfect antenna coupling between the systems, at the minimum distance between the stations on Earth and the Moon, and in the absence of antenna pointing errors and losses such as depolarization and clutter, the interference levels comply with the established ITU-R service protection criteria.

#### A12.1.2.5 	Study Results

The study was conducted in accordance with the scenario outlined in Figure A12.1.2.1. For the worst-case analysis, the following assumptions were considered:

–	Main beam antenna coupling between the potentially interfering lunar SRS transmitters and the FS victim receiver.

–	Free space propagation loss assuming the minimum distance between the stations on the Earth and the Moon (355,000 km), while atmospheric, depolarization and clutter losses are not considered.

–	In the present study, it is assumed that both systems operate at the same central frequency. A bandwidth correction factor (CF) is applied when the bandwidth of the potential interferer exceeds the bandwidth of the victim receiver under analysis. If the bandwidth of the interferer is equal to or smaller than that of the victim receiver, the bandwidth correction factor is 0:

CF = 10 * log10bandwidthvictimbandwidthinterferer

–	The aggregate interference-to-noise ratio (I/N) is calculated considering Lunar SRS transmitters pointing towards the FS victim receiver using the following equation:

INTotal= 1n (EIRPnθ,φ+CF-FSPL-Floss+Grxθ,φ)-Nrx

where n is the number of lunar surface transmitters;  EIRPnθ,φ is the equivalent isotropic radiated power of each lunar transmitter; FSPL is the free space path loss calculated using Recommendation ITU-R P.525; Floss is the FS feeder loss; (Grx)θ,φ is the FS victim receiver maximum antenna gain; and Nrx is the thermal noise power in the FS victim receiver bandwidth.

–	The receiver thermal noise power is calculated from the receiver noise figure (NF) using the formula provided in Recommendation ITU-R P.372-17.

–	The single and aggregate I/N results are compared to the FS protection criteria (I/N ≤ −10 dB) to verify whether the FS protection is met.

Figure A12.1.2.1

SRS–FS Scenario

Table A12.1.2.4

SRS 27 GHz 1 – FS (PP) co-channel results

|  |  | FS 16 QAM |  |
| --- | --- | --- | --- |
| bandwidth | MHz | 2.5 | 3.5 |
| Receiver noise power | dBW | -133.5 | -132.0 |
| Single Interferer | dBW | -164.2 | -162.7 |
| I/N result | dB | -30.7 | -30.7 |
| Margin to the FS protection criterion | dB | 20.7 | 20.7 |

Table A12.1.2.5

SRS 27 GHz 1 – FS (PP) co-channel results

|  |  | FS 128 QAM |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| bandwidth | MHz | 7 | 14 | 28 | 30 | 40 | 56 | 60 | 112 |
| Receiver noise power | dBW | -129.0 | -126.0 | -123.0 | -122.7 | -121.5 | -120.0 | -119.7 | -117.0 |
| Single Interferer | dBW | -159.7 | -156.7 | -153.7 | -153.4 | -152.1 | -150.7 | -150.4 | -147.7 |
| I/N result | dB | -30.7 | -30.7 | -30.7 | -30.7 | -30.7 | -30.7 | -30.7 | -30.7 |
| Margin to the FS protection criterion | dB | 20.7 | 20.7 | 20.7 | 20.7 | 20.7 | 20.7 | 20.7 | 20.7 |

Table A12.1.2.6

SRS 27 GHz 1 – FS (PmP) co-channel results

|  |  | FS 256 QAM |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| bandwidth | MHz | 40 | 50 | 56 | 100 | 112 |
| Receiver noise power | dBW | -122.0 | -121.0 | -120.5 | -118.0 | 117.5 |
| Single Interferer | dBW | -157.1 | -156.2 | -155.7 | -153.2 | -152.7 |
| I/N result | dB | -35.2 | -35.2 | -35.2 | -35.2 | -35.2 |
| Margin to the FS protection criterion | dB | 25.2 | 25.2 | 25.2 | 25.2 | 25.2 |

Table A12.1.2.7

SRS 27 GHz 1 – FS (PmP) co-channel results

|  |  | FS 16 QAM |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| bandwidth | MHz | 3.5 | 7 | 14 | 28 | 30 | 56 | 112 | 40 | 60 |
| Receiver noise power | dBW | -130.5 | -127.5 | -124.5 | -121.5 | -121.2 | -118.5 | -115.5 | -120.0 | -118.2 |
| Single Interferer | dBW | --204.2 | -201.2 | -198.2 | -195.2 | -194.9 | -192.2 | -189.2 | -193.6 | -191.9 |
| I/N result | dB | -73.7 | -73.7 | -73.7 | -73.7 | -73.7 | -73.7 | -73.7 | -73.7 | -73.7 |
| Margin to the FS protection criterion | dB | 63.7 | 63.7 | 63.7 | 63.7 | 63.7 | 63.7 | 63.7 | 63.7 | 63.7 |

Table A12.1.2.8

SRS 27 GHz 1 – FS (PmP) co-channel results

|  |  | FS 16 QAM |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| bandwidth | MHz | 3.5 | 7 | 14 | 28 | 30 | 56 | 112 | 40 | 60 |
| Receiver noise power | dBW | -130.5 | -127.5 | -124.5 | -121.5 | -121.2 | -118.5 | -115.5 | -120.0 | -118.2 |
| Single Interferer | dBW | -175.7 | -172.7 | -169.7 | -166.8 | -166.4 | -163.7 | -160.7 | -165.1 | -163.4 |
| I/N result | dB | -45.2 | -45.2 | -45.2 | -45.2 | -45.2 | -45.2 | -45.2 | -45.2 | -45.2 |
| Margin to the FS protection criterion | dB | 35.2 | 35.2 | 35.2 | 35.2 | 35.2 | 35.2 | 35.2 | 35.2 | 35.2 |

Table A12.1.2.9

SRS 27 GHz 2 – FS (PP) co-channel results

|  |  | FS 16 QAM |  |
| --- | --- | --- | --- |
| bandwidth | MHz | 2.5 | 3.5 |
| Receiver noise power | dBW | -133.5 | -132.0 |
| Single Interferer | dBW | -161.8 | -160.4 |
| I/N result | dB | -28.4 | -28.4 |
| Margin to the FS protection criterion | dB | 18.4 | 18.4 |

Table A12.1.2.10

SRS 27 GHz 2 – FS (PP) co-channel results

|  |  | FS 128 QAM |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| bandwidth | MHz | 7 | 14 | 28 | 30 | 40 | 56 | 60 | 112 |
| Receiver noise power | dBW | -129.0 | -126.0 | -123.0 | -122.7 | -121.5 | -120.0 | -119.7 | -117.0 |
| Single Interferer | dBW | -157.4 | -154.4 | -151.4 | -151.1 | -149.8 | -148.3 | -148.0 | -145.3 |
| I/N result | dB | -28.4 | -28.4 | -28.4 | -28.4 | -28.4 | -28.4 | -28.4 | -28.4 |
| Margin to the FS protection criterion | dB | 18.4 | 18.4 | 18.4 | 18.4 | 18.4 | 18.4 | 18.4 | 18.4 |

Table A12.1.2.11

SRS 27 GHz 2 – FS (PmP) co-channel results

|  |  | FS 256 QAM |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| bandwidth | MHz | 40 | 50 | 56 | 100 | 112 |
| Receiver noise power | dBW | -122.0 | -121.0 | -120.5 | -118.0 | 117.5 |
| Single Interferer | dBW | -154.8 | -153.8 | -153.3 | -150.8 | -150.3 |
| I/N result | dB | -32.9 | -32.9 | -32.9 | -32.9 | -32.9 |
| Margin to the FS protection criterion | dB | 22.9 | 22.9 | 22.9 | 22.9 | 22.9 |

Table A12.1.2.12

SRS 27 GHz 2 – FS (PmP) co-channel results

|  |  | FS 16 QAM |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| bandwidth | MHz | 3.5 | 7 | 14 | 28 | 30 | 56 | 112 | 40 | 60 |
| Receiver noise power | dBW | -130.5 | -127.5 | -124.5 | -121.5 | -121.2 | -118.5 | -115.5 | -120.0 | -118.2 |
| Single Interferer | dBW | -201.9 | -198.9 | -195.9 | -192.9 | -192.6 | -189.8 | -186.8 | -191.3 | -189.5 |
| I/N result | dB | -71.4 | -71.4 | -71.4 | -71.4 | -71.4 | -71.4 | -71.4 | -71.4 | -71.4 |
| Margin to the FS protection criterion | dB | 61.4 | 61.4 | 61.4 | 61.4 | 61.4 | 61.4 | 61.4 | 61.4 | 61.4 |

Table A12.1.2.13

SRS 27 GHz 2 – FS (PmP) co-channel results

|  |  | FS 16 QAM |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| bandwidth | MHz | 3.5 | 7 | 14 | 28 | 30 | 56 | 112 | 40 | 60 |
| Receiver noise power | dBW | -130.5 | -127.5 | -124.5 | -121.5 | -121.2 | -118.5 | -115.5 | -120.0 | -118.2 |
| Single Interferer | dBW | -173.4 | -170.4 | -167.4 | -164.4 | -164.1 | -161.3 | -158.3 | -162.8 | -161.0 |
| I/N result | dB | -42.9 | -42.9 | -42.9 | -42.9 | -42.9 | -42.9 | -42.9 | -42.9 | -42.9 |
| Margin to the FS protection criterion | dB | 32.9 | 32.9 | 32.9 | 32.9 | 32.9 | 32.9 | 32.9 | 32.9 | 32.9 |

#### A12.1.2.6 	Summary and analysis of the results of Study B

### The interference-to-noise ratio metric shows that in the single entry static worst-case analysis, the FS protection criteria is met with a margin of 18.4 dB or more for all situations. The results of this study suggest that protection of FS from SRS in the 27.5-28.35 GHz band is ensured.

### A12.1.3	Study C

## A12.2	Sharing and compatibility of fixed services using high altitude platform stations (HAPS)and SRS operating in the frequency band 25.25-28.35 GHz

#### A12.2.1.1	Technical and operational characteristics of SRS operating in the 25.25 – 28.35 GHz frequency range

The technical and operational characteristics for SRS systems are based on those available in the Report ITU-R SA.2553, including table 3.2-4, as summarized in Table A12.2.1.1.

Table A12.2.1.1

Technical Characteristics of the SRS Systems

| Station | Lunar Surface-to-Surface Transmitter |
| --- | --- |
| Frequency range | 27.5 – 28.35 GHz |
| Polarization | Linear ±45º / CP |
| Peak Gain (dBi) | 29.1 |
| EIRP Density (dBW/MHz) | 16.2 |
| Max. EIRP (dBW) | 39.2 |
| Channel BW (MHz) | 200 MHz |

#### A12.2.1.2 	Technical and operational characteristics of fixed high altitude platform stations (HAPS) operating in the 25.25 – 28.35 GHz frequency range

Document 7B/56 provides the relevant technical information to support studies between SRS and Fixed Services (FS) under Agenda Item 1.15. The characteristics for fixed service (FS) for point-to-point (PP) systems are based on the information contained in the Report ITU-R F.2439-0, as summarized in Table A12.2.1.2.

Table A12.2.1.2

Characteristics of the FS (HAPS) systems

| Parameter | System 4a (Downlink) / Gateway (GW) |
| --- | --- |
| Frequency range | 27.9 – 28.2 GHz |
| Receiver noise bandwidth | 300 MHz |
| Antenna pattern | Rec. ITU-R S.580-6 |
| Antenna gain | 54.6 dBi |
| Receiver noise figure | 2.27 dB |
| Protection criteria (I/N) | ‒10 dB should not be exceeded for more than 20% of the time +10 dB should not be exceeded for more than 0.01% of the time |

#### A12.2.1.3	Propagation model

This sharing study includes the lunar SRS system on the surface of the Moon, and fixed services operating on Earth's surface. Working Party 3J in Document 7B/58 recommends using the propagation model based on Recommendation ITU-R P.525 for this scenario.

#### A12.2.1.4	Methodology

The methodology employed in this study is based on a static analysis of both single and aggregated worst-case interference. It evaluates whether, even under conditions of perfect antenna coupling between the systems, at the minimum distance between the stations on Earth and the Moon, and in the absence of antenna pointing errors and losses such as depolarization and clutter, the interference levels comply with the established ITU-R service protection criteria.

#### A12.2.1.5	Study results

The study was conducted in accordance with the scenario outlined in Figure A12.2.1.1. For the worst-case analysis, the following assumptions were considered:

–	Main beam antenna coupling between the potentially interfering lunar SRS transmitters and the FS (HAPS) victim receiver.

–	Free space propagation loss assuming the minimum distance between the stations on the Earth and the Moon (355,000 km), while atmospheric, depolarization and clutter losses are not considered.

–	The aggregate interference-to-noise ratio (I/N) is calculated considering three Lunar SRS transmitters pointing towards the FS (HAPS) victim receiver using the following equation:

INTotal= 1n (EIRPnθ,φ-FSPL+Grxθ,φ)-Nrx

where n is the number of lunar surface transmitters;  EIRPnθ,φ is the equivalent isotropic radiated power of each lunar transmitter; FSPL is the free space path loss calculated using Rec. ITU-R P.525; (Grx)θ,φ is the FS (HAPS) victim receiver maximum antenna gain; and Nrx is the thermal noise power in the FS (HAPS) victim receiver bandwidth.

–	The receiver thermal noise power is calculated from the receiver noise figure (NF) using the following equation:

Nrx = 10 log10 [k  (290  10(NF/10))  B]

where k is Boltzmann’s constant; B is the receiver bandwidth, and NF is the receiver noise figure in dB.

–	The single and aggregate I/N results are compared to the strictest HAPS protection criteria (I/N ≤ 10 dB) to verify whether the service protection criteria is met.

Figure A12.2.1.1

SRS–FS (HAPS) GW Scenario

Table A12.2.1.3

SRS – FS (HAPS) GW co-channel results

| SRS Station | SRS centre frequency | Analysis | I/N result | Margin to the FS protection criteria |
| --- | --- | --- | --- | --- |
| Lunar Surface Transmitter | 28050 MHz | Single Entry | -21.6 dB | 11.6 dB |

#### A12.2.1.6	Summary and analysis of the results of Study A

In this study the interference-to-noise ratio metric shows that even in the static worst-case analysis for a single lunar surface interferer, the FS (HAPS) protection criteria is met with a margin of 11.6 dB or more. The results of this study suggest that protection of FS using HAPS ground-based system operating in the 27.5 – 28.35 GHz band (co-channel) from SRS is ensured.

### A12.3	Sharing studies between mobile service and SRS operating in the frequency band 25.25-28.35 GHz

#### A12.3.1.1 	Technical and operational characteristics of SRS operating in the 25.25 – 28.35 GHz frequency range

The technical and operational characteristics for SRS systems are based on those available in the Report ITU-R SA.2553, including table 3.2-4, as summarized in Table A12.3.1.1.

Table A12.3.1.1

Technical Characteristics of the SRS Systems

| Station | Lunar Surface-to-Surface Transmitter |
| --- | --- |
| Frequency range | 27.5 – 28.35 GHz |
| Polarization | Linear ±45º / CP |
| Peak Gain (dBi) | 29.1 |
| EIRP Density (dBW/MHz) | 16.2 |
| Max. EIRP (dBW) | 39.2 |
| Channel BW (MHz) | 200 MHz |

#### A12.3.1.2	Technical and operational characteristics of the mobile service in the 27.5 – 29.5 GHz frequency range

Document 7B/105 provides relevant technical information to support studies between SRS and mobile services, under Agenda Item 1.15. The characteristics and protection criteria for the mobile service are based on the information contained in Recommendation ITU-R M.2134 (Table 2 System B), as summarized in Tables A12.3.1.2 and A12.3.1.3.

Table A12.3.1.2

MS parameters for bands between 27.5 – 29.5 GHz

|  | Base Station (System B) |
| --- | --- |
| Antenna height | 10 m |
| Antenna gain | 29 dBi |
| Antenna polarization | Linear |
| Channel bandwidth | 100 MHz |
| Noise Figure | 6 dB |

Table A12.3.1.3

MS mobile station parameters for bands between 27.5 – 29.5 GHz

|  | Mobile station |
| --- | --- |
| UE height | 1.5 m |
| Typical antenna gain | 20 dBi |
| Body loss | 4 dB |
| UE Noise Figure | 6 dB |

#### A12.3.1.3	Propagation model

This sharing study includes the lunar SRS system on the surface of the Moon, and mobile services operating on Earth's surface. Working Party 3J in Document 7B/58 recommends using the propagation model based on Recommendation ITU-R P.525 for this scenario.

#### A12.3.1.4	Methodology

The methodology employed in this study is based on a static analysis of both single and aggregated worst-case interference. It evaluates whether, even under conditions of perfect antenna coupling between the systems, at the minimum distance between the stations on Earth and the Moon, and in the absence of antenna pointing errors and losses such as depolarization, body, building and clutter, the interference levels comply with the established ITU-R service protection criteria.

#### A12.3.1.5	Study results

The study was conducted in accordance with the scenario outlined in Figure A.12.3.1.1. For the worst-case analysis, the following assumptions were considered:

–	Main beam antenna coupling between the potentially interfering lunar SRS transmitters and the MS base station victim receiver.

–	Free space propagation loss assuming the minimum distance between the stations on the Earth and the Moon (355,000 km), while atmospheric, depolarization and clutter losses are not considered.

–	The aggregate interference-to-noise ratio (I/N) is calculated considering three Lunar SRS transmitters pointing towards the MS base station victim receiver using the following equation:

INTotal= 1n (EIRPnθ,φ-FSPL+Grxθ,φ)-Nrx

where n is the number of lunar surface transmitters;  EIRPnθ,φ is the equivalent isotropic radiated power of each lunar transmitter; FSPL is the free space path loss calculated using Rec. ITU-R P.525; (Grx)θ,φ is the MS base station victim receiver maximum antenna gain; and Nrx is the thermal noise power in the MS base station victim receiver bandwidth.

–	The receiver thermal noise power is calculated from the receiver noise figure (NF) using the following equation:

Nrx = 10 log10 [k  (290  10(NF/10))  B]

where k is Boltzmann’s constant; B is the receiver bandwidth, and NF is the receiver noise figure in dB.

–	The single and aggregate I/N results are compared to the MS protection criteria (I/N ≤ -6 dB) to verify whether the MS protection is met.

Figure A12.3.1.1

SRS–MS Scenario

Table A12.3.1.4

SRS – MS co-channel results

| SRS Station | SRS centre frequency | MS Station | Analysis | I/N result | Margin to the MS protection criteria |
| --- | --- | --- | --- | --- | --- |
| Lunar Surface Transmitter | 27700 MHz | Base Station | Single Entry | -59.0 dB | 46.0 dB |
| Lunar Surface Transmitter | 27700 MHz | Mobile station | Single Entry | -65.0 dB | 52.0 dB |

#### A12.3.1.6	Summary and analysis of the results of Study

In this study the interference-to-noise ratio metric shows that even in the static worst-case analysis for a single lunar potential interferer, the MS protection criteria for the base station is met with a margin of 46 dB or more, and the MS protection criteria for the mobile station is met with a margin of 52 dB or more.

The results of this study suggest that protection of mobile services, including base stations and user equipment in the 27.5 – 28.35 GHz band (co-channel) from SRS is ensured.

## A12.4	Sharing studies between fixed satellite service (FSS) and SRS operating in the frequency band 27.5-28.35 GHz

### A12.4.1	Study A [USA]

In Document 7B/200, WP 4A expressed the view that sharing studies to ensure protection of FSS are not required in the 27.5 – 28.35 GHz band, provided that possible future allocations to space research service (space-to-space) in the 27.5-28.35 GHz band under WRC-27 agenda item 1.15 are limited to Lunar surface-to-surface communications, as specified in Report ITU-R SA.2553.

## A12.[X]	Summary and analysis of the results of studies

[Note: The summary should offer an objective overview, encompassing the results of sharing and compatibility studies, alongside any pertinent information applicable for future comparisons among studies. Additionally, it should address any mitigation techniques or proposals employed to safeguard services operating within the same band or adjacent bands, where applicable, to SRS lunar systems.]

____________________

See also the applicable Notes in 5D/734 liaison stated from WP 4A on “Technical and operational characteristics and protection criteria of FSS systems for sharing and compatibility studies on WRC-23 agenda item 1.2.”