| U.S. Radiocommunication Sector Fact Sheet |  |
| --- | --- |
| Working Party: ITU-R WP 7D | Document No: USWP7D_26Mar-doc06b |
| Ref. Document 7D/235, Annex 13 | Date: 12/19/2025 |
| Document Title: Working Document Towards a Preliminary Draft New Recommendation: Threshold levels of interference to radio astronomy in the shielded zone of the Moon [SZM_THRESHOLDS] |  |
| Author(s)/Contributors(s): Sarah Marie Bruno, Villanova University Darcy Barron, University of New Mexico | sbruno03@villanova.edu dbarron2@unm.edu |
| Purpose/Objective: To address threshold levels of permissible interference for radio astronomy facilities on the Moon in accordance with restrictions on emissions under Radio Regulations (RR) Nos. 22.22-22.25. |  |
| Abstract: This draft new Recommendation references and builds upon the scientific background and methodology for calculating threshold levels of permissible interference to radio astronomy provided in 7D/235 Annex 12. This input contribution provides updates to the document currently attached to the Chairman’s Report (7D/235 Annex 13). |  |

| Radiocommunication Study Groups |  |
| --- | --- |
| Source: Document 7D/235, Annex 13 | Document 7D/XX X March 2026 |
|  | English only |
| United States of America |  |
| working document towards a preliminary draft new RECOMMENDATION [SZM_THRESHOLDS] |  |
| Threshold levels of permissible interference to radio astronomy in the shielded zone of the Moon |  |

# Introduction

This Recommendation addresses threshold levels of permissible interference to radio astronomy in the shielded zone of the Moon in accordance with restrictions on emissions under Radio Regulations (RR) Nos. 22.22-22.25. It builds upon and references the scientific background information and methodology provided in Report 7D/235 Annex 12. Changes made relative to the previous version attached to the Chair’s Report as Annex 13 include removing the scientific background and methodology tables which are in Report 7D/235 Annex 12, cleaning up formatting, and adding two new recognizings—one which refers to Report 7D/235 Annex 12, and one which refers to Report ITU-R RA.2512. The word “harmful” is replaced by “permissible” throughout the document. Finally, some substantial revisions were made to the recommends to clarify guidance for minimizing impacts to the RAS, including the addition of an Annex 1 which will contain a table of example threshold values. The proposed changes are highlighted through track changes. This document is still under development with significant further work anticipated.

Attachment:	1

ATTACHMENT

Working Document Towards a Preliminary
Draft New Recommendation [SZM_ThresholdS]

Thresholds levels of permissible interference  to radio astronomy 
in the shielded zone of the Moon

(Question ITU-R 260/7)

(202X)

Scope

This Recommendation addresses threshold levels of permissible interference to radio astronomy in the shielded zone of the Moon given the restrictions on emissions under Radio Regulations (RR) Nos. 22.22-22.25.

The ITU Radiocommunication Assembly,

considering

a)	that radio astronomy in the shielded zone of the Moon (SZM) is a subject of RR Nos. 22.22-22.25;

b)	that the SZM is defined in RR No. 22.22.1 as comprising the area of the Moon’s surface and an adjacent volume of space which are shielded from emissions originating within a distance of 100 000 km from the centre of the Earth;

c)	that in the shielded zone of the Moon RR Nos. 22.22-22.24 prohibit emissions causing harmful interference to radio astronomy observations in the entire spectrum except in the frequency bands given there, making the SZM a uniquely advantaged location for radio astronomy observations;

d)	that in the SZM, radio astronomy may be protected in the given frequency bands by agreement between administrations concerned (RR No. 22.25);

e)	that the level of harmful interference is determined by agreement between the administrations concerned, with the guidance of the relevant ITU-R Recommendations (RR. No. 22.22.2),

recognizing

a)	that the technical and operational characteristics of radio astronomy in the SZM are similar to those subject to the relevant ITU-R Recommendations noted in RR No. 22.22.2 while benefiting from the absence of Earth’s atmosphere;

b)	that radio astronomy observations in the SZM should be able to take advantage of the long lunar night (1.2 × 106 seconds),

{Editor’s note: Report ITU-R RA.[SZM-THRESHOLDS] is under development.}

c)	that Report ITU-R RA.[SZM-THRESHOLDS] provides scientific background and a methodology for calculating nominal thresholds of permissible interference for radio astronomy in the shielded zone of the Moon.

d)	that Report ITU-R RA.2512 provides characteristics of extremely sensitive, background-limited millimeter-wave bolometric detectors and similar types of incoherent detectors with large fractional bandwidths, including reference levels for interference to telescopes employing such detectors.

recommends

that sharing and compatibility studies with radio astronomy in the SZM should use threshold levels in Annex 1 to determine the maximum level of permissible interference

ANNEX 1

Note: The methodology provided in Report ITU-R RA.[SZM-THRESHOLDS] Tables 1-4 can be used to calculate the nominal thresholds for permissible interference for any RAS facility in the SZM. Below is a table of example threshold values for three RAS facilities in the SZM.

{Editor’s note: Table 1 to be completed at a later time; this table will be further informed by Report ITU-R RA.[SZM-THRESHOLDS (under development). Numerical threshold values depend on specific instrument parameters of the RAS facilities.}

TABLE 1

Threshold values of permissible interference to radio astronomy observations in the SZM

| Facility name |  | Frequency f (Hz) |  | Bandwidth f (Hz) |  | Minimum antenna noise temperature TA. (K) |  | Receiver noise temperature TR (K) |  | System sensitivity (noise fluctuations) |  |  | Threshold interference levels |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  | Rms noise temperature T (K) | Power spectral density PS (W Hz-1) |  | Input power PH (W) | Incident pfd PH (2/4)-1 (W m-2) | Spectral pfd SH (W m-2 Hz-1) |  |

____________________