| U.S. Radiocommunication Sector Fact Sheet |  |
| --- | --- |
| Working Party: ITU-R WP 7D | Document No: USWP7D_26Mar-doc08 |
| Ref. Doc. 7D/235, Annex 5 Res. 712 (WRC-23) | Date: 01/23/2026 |
| Document Title: Draft Revision of Recommendation ITU-R RA.1631 |  |
| Author(s)/Contributors(s): Frank Schinzel (NRAO) | fschinze@nrao.edu |
| Purpose/Objective: Finalize and propose elevation of document |  |
| Abstract: This document revision was developed over the past 7D meetings and has been maturing. Minor proof-reading revisions are anticipated combined with a proposal to finalize this revision that primarily extends the typical maximum RAS antenna gain table under recommends 3 up to 275 GHz, which is of immediate relevance to agenda item 1.18, and adds a scope to the recommendation. We propose an input in support of elevation of this revision. Annex 5 is attached with list of revisions added. |  |

| Radiocommunication Study Groups |  |
| --- | --- |
| Received: | Document 7D/XXX |
|  | XX March 2026 |
|  | English only |
| United States of America |  |
| PROPOSED MINOR UPDATES AND ELEVATION OF PRELIMINARY DRAFT REVISION OF RECOMMENDATION ITU-R RA.1631 |  |
| Reference radio astronomy antenna pattern to be used for compatibility analyses between non-GSO systems and radio astronomy service stations based on the epfd concept |  |

Introduction

The revision of this recommendation was developed over the past 7D meetings and has been maturing. It is proposed to finalize this revision that primarily extends the typical maximum RAS antenna gain table under recommends 3 up to 275 GHz and adds a scope to the recommendation. This is of immediate relevance to agenda item 1.18.

A summary of revisions was added and it is proposed to elevate to draft revision status and be considered for approval at the next SG7 meeting.

Attachment

ATTACHMENT

| Radiocommunication Study Groups |  |
| --- | --- |
| Source: Annex 5 to Document 7D/235-E |  |
|  | English only |
| DRAFT REVISION OF RECOMMENDATION ITU-R RA.1631 |  |
| Reference radio astronomy antenna pattern to be used for compatibility analyses between non-GSO systems and radio astronomy service stations based on the epfd concept |  |

(Question ITU-R 146/7)

(2003-202X)

Summary of revisions:

- Added Scope to Recommendation

- Expanded frequency range of Table under recommends 3 up to 275 GHz

- Minor edits under considering

Scope

This Recommendation provides a reference antenna pattern for the radio astronomy service to be used for compatibility analyses between non-GSO systems and radio astronomy service based on the equivalent power flux-density (epfd) concept, where appropriate, for frequencies up to 275 GHz.

The ITU Radiocommunication Assembly,

considering

a)	that there is a need to determine the levels of interference which may occur at typical observatory sites, due to various sources of interference;

b)	that, to determine these levels of interference, a reference antenna pattern needs to be established;

c)	that Recommendation ITU-R SA.509 gives a reference antenna pattern which represents the side-lobe gain levels that are not expected to be exceeded at most off-axis angles in the majority of antennas used in the service;

d)	that the antenna pattern given in Recommendation ITU-R SA.509 is appropriate in some compatibility or sharing analyses;

e)	that, if the peak envelope radiation pattern such as given in Recommendation ITU-R SA.509 is used in the assessment of the aggregate interference, the predicted interference will result in values that are greater than values that are experienced in practice;

f)	that Recommendation ITU-R S.1586 and Recommendation ITU-R M.1583 provide a methodology based on the epfd concept as defined in No. 22.5C of the Radio Regulations to calculate the level of unwanted emission levels produced by a non-geostationary-satellite system at radio astronomy stations;

g)	that it is necessary to use an antenna radiation pattern representing average side-lobe levels to predict interference to a radio astronomy station from one or more fast moving stations seen under continuously variable angle such as non-GSO systems;

h)	that, a simple analytic mathematical formula is preferable to the radiation pattern representing average side-lobe levels;

i)	that, to derive the epfd resulting from unwanted emission levels produced by a non-geostationary-satellite system at radio astronomy stations, it is necessary to use the typical maximum antenna gain of radio astronomy service (RAS) stations,

recommends

1	that, in the absence of particular information concerning the radiation pattern of the radio astronomy antenna involved, the mathematical model of the average radiation pattern as stated below should be used for compatibility analyses between non-GSO systems and RAS stations for frequencies above 150 MHz:

G() Gmax – 2.5  10–3                 dBi	for	    0 				m

G() G1		for		 m 				r

G() 29 – 25 log                 dBi	for		  r 				10

G() 34 – 30 log                 dBi	for	   10				34.1

G() –12               dBi	for	34.1				80

G() –7                dBi	for	   80				120

G() –12               dBi	for	 120				180

where:

dBi

dBi

degrees

degrees

D:	diameter of the telescope (m)

:	wavelength (m);

2	that the following mathematical model of the radiation pattern may be adopted for a more accurate representation of the main beam radiation pattern for frequencies above 150 MHz:

(expressed as a ratio not in dB)

where:

J1(x):	first order Bessel function

Gmax  :	maximum gain (expressed as a ratio not in dB)

Aeff   (D/2)2:	area of the aperture of the telescope (m2)

D:	diameter of the telescope (m)

:	wavelength (m).

and where:

x  	with  off-boresight angle (degrees) (    0)

0 :	first null in this antenna pattern at 69.88/(D/) (degrees) off-boresight.

and that the following mathematical model of the radiation pattern may be adopted for a more accurate representation of the radiation pattern of near side lobes up to 1° from the boresight for frequencies above 150 MHz:

(expressed as a ratio not in dB)

where:

with  off-boresight angle (degrees) (0    1°)

D:	diameter of the telescope

:	wavelength.

and:

B = 103.2 2 ((D/2)/(180 · ))2

This main beam model corresponds to the ideal case of 100 aperture efficiency;

3	that the following typical maximum RAS antenna gain be used in compatibility analysis between non-GSO systems and RAS antenna stations.

| RAS allocated band (MHz) | Typical maximum antenna gain |
| --- | --- |
| 150.05-153 | 44 |
| 322-328.6 | 51 |
| 406.1-410 | 53 |
| 608-614 | 56 |
| 1 400-1 427 | 63 |
| 1 610.6-1 613.8 | 64 |
| 1 660-1 670 | 65 |
| 2 690-2 700 | 69 |
| 4 990-5 000 | 74 |
| 10.6-10.7 | 81 |
| 14.47-14.5 | 84 |
| 15.35-15.4 | 84 |
| 22.21-22.5 | 87 |
| 23.6-24 | 88 |
| 31.3-31.7 | 90 |
| 42.5-43.5 | 93 |
| 48.94-49.04 | 94 |
| 76-77.5 | 98 |
| 79-81 | 98 |
| 81-86 | 99 |
| 86-92 | 99 |
| 92-94 | 100 |
| 94.1-100 | 100 |
| 100-102 | 91 |
| 102-105 | 92 |
| 105-109.5 | 92 |
| 109.5-111.8 | 92 |
| 111.8-114.25 | 92 |
| 114.25-116 | 93 |
| 130-134 | 94 |
| 136-148.5 | 94 |
| 148.5-151.5 | 95 |
| 151.5-158.5 | 95 |
| 164-167 | 96 |
| 182-185 | 97 |
| 200-209 | 98 |
| 209-217 | 98 |
| 217-226 | 98 |
| 226-231.5 | 98 |
| 241-248 | 99 |
| 250-252 | 99 |
| 252-275 | 100 |

The corresponding antenna diameter may be derived using the following equations (see recommends 2):

Gmax =                 maximum gain (expressed as a ratio)

where:

Aeff = (D/2)2:	area of the aperture of the telescope (m2)

D:	diameter of the telescope (m)

:	wavelength (m).

____________________