| U.S. Radiocommunication Sector Fact Sheet |  |
| --- | --- |
| Working Party: ITU-R WP 7D | Document No: USWP7D_26Mar-doc14 |
| Ref. 7D/235, Annex 8 | Date: 01/16/2026 |
| Document Title: Updates to Considerations on draft CPM text for WRC-27 agenda item 1.18 |  |
| Author(s)/Contributors(s): Frank Schinzel Jonathan Williams Ashley VanderLey | fschinze@nrao.edu jonwilli@nsf.gov bevander@nsf.gov |
| Purpose/Objective: To provide additions and modifications to the in-process draft CPM text on agenda item 1.18. |  |
| Abstract: At the previous meeting, ITU-R WP 7D began development and consideration of draft CPM text for agenda item 1.18. This input is to provide updates and improvements to this text. This document reconciles the input received so far for CPM text on agenda item 1.18. Specifically, with revision of Report ITU-R RA.1631 and based on information provided in Report ITU-R RA.[RAS-SAT 71-235 GHZ], a method can be provided that would provide updates to Tables 1 and 2 in the Annex of Resolution 739 (WRC-19) in line with invites 2 of Resolution 712 (WRC-23). Since the draft CPM text compiled by WP 7C (resolves 1) and 7D (resolves 2), will be provided by WP 7C, once finalized, the content of this document would need to be liaised to WP 7C for combination with their draft or prepared through a joined session. |  |

| Radiocommunication Study Groups |  |
| --- | --- |
| Received: | Document 7D/XXX |
|  | XX March 2026 |
|  | English only |
| United States of America |  |
| UPDATES TO CONSIDERATIONS ON DRAFT CPM TEXT FOR WRC-27 AGENDA ITEM 1.18 |  |

Introduction

This document reconciles the input received so far for CPM text on agenda item 1.18. Specifically, with revision of Report ITU-R RA.1631 and based on information provided in Report ITU-R RA.[RAS-SAT 71-235 GHZ], a method is provided that would propose updates to Tables 1 and 2 in the Annex of Resolution 739 (WRC-19) in line with invites 2 of Resolution 712 (WRC-23). The proposed added text is highlighted through track changes.

Since the draft CPM text compiled by WP 7C (resolves 1) and 7D (resolves 2), will be provided by WP 7C, once finalized, the content of this document would need to be liaised to WP 7C for combination with their draft or prepared through a joined session.

Attachment

attachment (Version for WP 7D)

CHAPTER 4

Science issues

(Agenda items 1.15, 1.16, 1.17, 1.18, 1.19)

Agenda item 1.18

(WP 7C (resolves 1), WP 7D (resolves 2)* / WP 3J**, WP 3M**, WP 4A, WP 4C, 
WP 5A, WP 5B, WP 5C)

1.18	to consider, based on the results of ITU Radiocommunication Sector studies, possible regulatory measures regarding the protection of the Earth exploration-satellite service (passive) and the radio astronomy service in certain frequency bands above 76 GHz from unwanted emissions of active services, in accordance with Resolution 712 (WRC-23);

Resolution 712 (WRC-23) – Studies on compatibility between the Earth exploration-satellite service (passive), the radio astronomy service in certain bands above 76 GHz, and active services in adjacent and nearby frequency bands

# 4/1.18/1	Executive summary

Resolution 712 (WRC-23) is twofold and invites ITU-R:

1	to determine, based on the results of studies, any required regulatory measures regarding the protection of the EESS (passive) in the frequency bands listed in Table 1 above from unwanted emissions of active services and update Resolution 750 (Rev.WRC-19) accordingly;

Table 1

EESS (passive) frequency bands to be studied and corresponding active services to be included

| EESS (passive) frequency band | Active service frequency band | Active service |
| --- | --- | --- |
| 86-92 GHz | 81-86 GHz | Fixed-satellite service (FSS) (Earth-to-space), mobile service (MS) |
|  | 92-94 GHz | MS, radiolocation service (RLS) |
| 114.25-116 GHz | 111.8-114.25 GHz | Fixed service (FS), MS |
| 164-167 GHz | 158.5-164 GHz | FS, FSS (space-to-Earth), MS, mobile-satellite service (MSS) (space-to-Earth) |
|  | 167-174.5 GHz | FS, FSS (space-to-Earth), inter-satellite service (ISS), MS |
| 200-209 GHz | 191.8-200 GHz | FS, ISS, MS, MSS, radionavigation service (RNS), radionavigation-satellite service (RNSS) |
|  | 209-217 GHz | FS, FSS (Earth-to-space), MS |

2	to determine, based on the results of studies, any required regulatory measures regarding the protection of the RAS in the frequency bands listed in Table 2 above and update Resolution 739 (Rev.WRC-19) accordingly

Table 2

RAS frequency bands to be studied and corresponding active services to be included

| Radio astronomy frequency band | Active satellite service frequency band | Active satellite service (space-to-Earth) |
| --- | --- | --- |
| 76-81 GHz | 71-76 GHz | Fixed-satellite service (FSS), mobile-satellite service (MSS), broadcasting-satellite service (BSS) |
| 130-134 GHz | 123-130 GHz | FSS, MSS, radionavigation-satellite service (RNSS) |
| 164-167 GHz | 167-174.5 GHz | FSS |
| 226-231.5 GHz | 232-235 GHz | FSS |

# 4/1.18/2	Background

## 4/1.18/2.1	Background for resolves 1 (EESS (passive))

…/…

## 4/1.18/2.2	Background for resolves 2 (RAS)

TBD by WP 7D.

# 4/1.18/3	Summary and analysis of the results of ITU-R studies

## 4/1.18/3.1	ITU-R studies for resolves 1 (EESS (passive))

…/…

## 4/1.18/3.2	ITU-R studies for resolves 2 (RAS)

Revision of Report ITU-R RA.1631

Report ITU-R RA.[RAS-SAT 71-235 GHZ]

# 4/1.18/4	Methods to satisfy the agenda item

## 4/1.18/4.1	Methods related to resolves 1 (EESS (passive))

…/…

## 4/1.18/4.2	Methods related to resolves 2 (RAS)

### 4/1.18/4.2.1	Method A-2

### MOD	Resolution 739

Based on results presented in Report ITU-R RA.[RAS-SAT 71-235 GHZ] it is proposed to modify Tables 1 and 2 of the Annex to Resolution 739 (Rev. WRC-19) extending values to include frequencies studied under this agenda item.

TABLE 1

pfd thresholds for unwanted emissions from any GSO space station at a radio astronomy station

| Space service |  |  | Space service frequency band |  |  | Radio astronomy frequency band |  |  |  | Single dish, continuum observations |  |  |  |  |  |  |  | Single dish, spectral line observations |  |  |  |  |  |  |  |  |  | VLBI |  |  |  |  |  |  |  |  | Condition of application: API is received by BR following the entry into force of the Final Acts of: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  | pfd(1) |  |  |  | Reference bandwidth |  |  |  | pfd(1) |  |  |  |  | Reference bandwidth |  |  |  |  | pfd(1) |  |  |  |  | Reference bandwidth |  |  |  |  |  |  |  |  |  |
|  |  |  | (MHz) |  |  | (MHz) |  |  |  | (dB(W/m2)) |  |  |  | (MHz) |  |  |  | (dB(W/m2)) |  |  |  |  | (kHz) |  |  |  |  | (dB(W/m2)) |  |  |  |  | (kHz) |  |  |  |  |  |  |  |  |  |
| MSS (space-to-Earth) |  |  | 387-390 |  |  |  |  | 322-328.6 |  |  |  | -189 |  |  |  |  | 6.6 |  |  |  |  | -204 |  |  |  |  |  | 10 |  |  |  |  | -177 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 10 | WRC-07 |
| BSS MSS (space-to-Earth) |  |  | 1 452-1 492 1 525-1 559 |  |  |  |  | 1 400-1 427 |  |  |  | -180 |  |  |  |  | 27 |  |  |  |  | -196 |  |  |  |  |  | 20 |  |  |  |  | -166 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | WRC-03 |
| MSS (space-to-Earth) MSS (space-to-Earth) |  |  | 1 525-1 559 1 613.8-1 626.5 |  |  |  |  | 1 610.6-1 613.8 |  |  |  | NA |  |  |  |  | NA |  |  |  |  | -194 |  |  |  |  |  | 20 |  |  |  |  | -166 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | WRC-03 |
| RNSS (space-to-Earth) |  |  |  | 1 559-1 610 |  |  |  | 1 610.6-1 613.8 |  |  |  | NA |  |  |  |  |  | NA |  |  |  |  | -194 |  |  |  |  | 20 |  |  |  |  | -166 |  |  |  | 20 |  |  | WRC-07 |  |  |  |
| BSS FSS (space-to-Earth) |  |  |  | 2 655-2 670 |  |  |  | 2 690-2 700 |  |  |  | -177 |  |  |  |  |  | 10 |  |  |  |  | NA |  |  |  |  | NA |  |  |  |  | -161 |  |  |  | 20 |  |  | WRC-03 |  |  |  |
| FSS (space-to-Earth) |  |  |  | 2 670-2 690 |  |  |  | 2 690-2700 (in Regions 1 and 3) |  |  |  | -177 |  |  |  |  |  | 10 |  |  |  |  | NA |  |  |  |  | NA |  |  |  |  | -161 |  |  |  | 20 |  |  | WRC-03 |  |  |  |
|  |  |  |  | (GHz) |  |  |  | (GHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| BSS |  |  |  | 21.4-22.0 |  |  |  | 22.21-22.5 |  |  |  |  |  | -146 |  |  |  |  | 290 |  |  |  | -162 |  |  |  |  | 250 |  |  |  |  | -128 |  |  |  | 250 |  |  | WRC-03 for VLBI, and WRC-07 for other types of observation |  |  |  |
|  |  |  |  | (GHz) |  | (MHz) |  | (MHz) |  |
| FSS MSS BSS |  |  |  | 71 – 76 |  |  |  | 76 – 81 |  |  |  |  |  | -146 |  |  |  |  | 5 |  |  |  | -165 |  |  |  |  | 1 |  |  |  |  | -129 |  |  |  | 1 |  |  | WRC-27 |  |  |  |
| FSS MSS RNSS |  |  |  | 123 – 130 |  |  |  | 130 – 134 |  |  |  |  |  | -140 |  |  |  |  | 4 |  |  |  |  | -160 |  |  |  |  | 1 |  |  |  |  | -123 |  |  |  | 1 |  |  | WRC-27 |  |  |  |
| FSS |  |  |  | 167 – 174.5 |  |  |  | 164 – 167 |  |  |  |  |  | -138 |  |  |  |  | 3 |  |  |  |  | -158 |  |  |  |  | 1 |  |  |  |  | -121 |  |  |  | 1 |  |  | WRC-27 |  |  |  |
| FSS |  |  |  | 232 – 235 |  |  |  | 226 – 231.5 |  |  |  |  |  | -134 |  |  |  |  | 5.5 |  |  |  |  | -154 |  |  |  |  | 1 |  |  |  |  | -117 |  |  |  | 1 |  |  | WRC-27 |  |  |  |

NA: Not applicable, measurement of this type are not made in this frequency band.

(1)	Integrated over the reference bandwidth with an integration time of 2 000 s.

TABLE 2

epfd thresholds(1) for unwanted emissions from all space stations of a non-GSO satellite system at a radio astronomy station

| Space service | Space service frequency band | Radio astronomy frequency band | Single dish, continuum observations |  | Single dish, spectral line observations |  | VLBI |  | Condition of application: API is received by BR following the entry into force of the Final Acts of: |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | epfd(2) | Reference bandwidth | epfd(2) | Reference bandwidth | epfd(2) | Reference bandwidth |  |
|  | (MHz) | (MHz) | (dB(W/m2)) | (MHz) | (dB(W/m2)) | (kHz) | (dB(W/m2)) | (kHz) |  |
| MSS (space-to-Earth) | 137-138 | 150.05-153 | -238 | 2.95 | NA | NA | NA | NA | WRC-07 |
| MMSS (space-to-Earth) | 157.1875-157.3375 161.7875-161.9375 | 150.05-153 | -238 | 2.95 | NA | NA | NA | NA | WRC-19 |
| MMSS (space-to-Earth) | 157.1875-157.3375 161.7875-161.9375 | 322-328.6 | -240 | 6.6 | -255 | 10 | -228 | 10 | WRC-19 |
| MSS (space-to-Earth) | 387-390 | 322-328.6 | -240 | 6.6 | -255 | 10 | -228 | 10 | WRC-07 |
| MSS (space-to-Earth) | 400.15-401 | 406.1-410 | -242 | 3.9 | NA | NA | NA | NA | WRC-07 |
| MSS (space-to-Earth) | 1 525-1 559 | 1 400-1 427 | -243 | 27 | -259 | 20 | -229 | 20 | WRC-07 |
| RNSS (space-to-Earth)(3) | 1 559-1 610 | 1 610.6-1 613.8 | NA | NA | -258 | 20 | -230 | 20 | WRC-07 |
| MSS (space-to-Earth) | 1 525-1 559 | 1 610.6-1 613.8 | NA | NA | -258 | 20 | -230 | 20 | WRC-07 |
|  | (GHz) | (GHz) |  | (GHz) |  | (MHz) |  | (MHz) |  |
| FSS MSS BSS | 71 – 76 | 76 – 81 | -229 | 5 | -248 | 1 | -212 | 1 | WRC-27 |
| FSS MSS RNSS | 123 – 130 | 130 – 134 | -219 | 4 | -239 | 1 | -202 | 1 | WRC-27 |
| FSS | 167 – 174.5 | 164 – 167 | -219 | 3 | -239 | 1 | -202 | 1 | WRC-27 |
| FSS | 232 – 235 | 226 – 231.5 | -217 | 5.5 | -237 | 1 | -200 | 1 | WRC-27 |

NA: Not applicable, measurement of this type are not made in this frequency band.

(1)	These epfd thresholds should not be exceeded for more than 2% of time.

(2)	Integrated over the reference bandwidth with an integration time of 2 000 s.

(3)	This resolution does not apply to current and future assignments of the radionavigation-satellite system GLONASS/GLONASS-M in the frequency band 1 559-1 610 MHz, irrespective of the date of receipt of the related coordination or notification information, as appropriate. The protection of the radio astronomy service in the frequency band 1 610.6-1 613.8 MHz is ensured and will continue to be in accordance with the bilateral agreement between the Russian Federation, the notifying administration of the GLONASS/GLONASS-M system, and IUCAF, and with subsequent bilateral agreements with other administrations.

### 4/1.18/4.2.2	Method B-2

TBD by WP 7D.

…/…

4/1.18/5	Regulatory and procedural considerations

## 4/1.18/5.1	Regulatory and procedural considerations for resolves 1 (EESS (passive))

…/…

## 4/1.18/5.2	Regulatory and procedural considerations for resolves 2 (RAS)

### 4/1.18/5.2.1	For Method A-2

TBD by WP 7D.

### 4/1.18/5.2.2	For Method B-2

TBD by WP 7D.

…/…

____________________

*	Note: the draft CPM text prepared by WP 7C and WP 7D will be provided by WP 7C

**	Preparatory work in support of this agenda item will require efforts to extend the applicability of current radio-wave propagation prediction methods for sharing and compatibility studies in frequency bands up to 235 GHz. Membership is encouraged to support these critical activities for completion by 2025 in Working Parties 3J and 3M.